3GPP TSG-RAN4 Meeting #116	R4-2510126

Bengaluru, India, Aug.25th – 29th, 2025

# Source:	CMCC, ZTE Corporation, Huawei, Murata Manufacturing Co Ltd.

# Title:	TP for TR38.719-02-01_CA_n41(2A)-n79A

# Agenda Item:	6.3.3

# Document for:	Approval

- Introduction

The TP to calculate the MSD for CA_n41C-n79A can be found in [1]. In the current specification, no MSD requirements are defined for UL_n41C, as non-simultaneous Rx/Tx operation is considered. However, n41 and n79 support simultaneous Rx/Tx operation, which requires the definition of MSD for this configuration. The IMD4 and IMD6 intermodulation products, which are identified in the context of CA_n41C-n79A, are expected to affect the n79 downlink (DL) Rx.

Given that the analysis for CA_n41C-n79A using 160MHz continuous bandwidth indicates that IMD4 and IMD6 are the primary intermodulation products affecting performance, it is assumed that similar conclusions will apply for CA_n41(2A)-n79A using 140MHz non-contiguous bandwidth. However, due to the non-contiguous nature of the 140MHz configuration, further analysis and validation are required to fully assess the potential impacts of these intermodulation products in the new configuration.

This proposal outlines the introduction of CA_n41(2A)-n79A into TR38.719-02-01, based on the 2.6GHz (100MHz + 40MHz non-contiguous) and 4.9GHz (100MHz) bands for uplink and downlink, respectively. This configuration arises from operational constraints in the 2.6GHz band (D3), where part of the spectrum remains reserved for 4G use, thereby limiting the possibility of its full re-farming for 5G applications. The resulting TDD+TDD aggregation mode necessitates a comprehensive MSD analysis.

- Reference

[1] R4-2503976 TP for TR38.719-02-01_CA_n41C-n79A, ZTE Corporation, CMCC

[2] R4-2408381,TP for TR38.718-02-01_CA_n41A-n79C and CA_n41C-n79A, ZTE Corporation, Mediatek, Sanechips

[2] R4-2502955, WF on ULCA_n41C MSD test point and IMD range definition, Skyworks, Qualcomm, ZTE, Huawei

[3] TR38.718-02-01,Rel-18 NR Inter-band Carrier Aggregation/Dual Connectivity for 2 bands DL with x bands UL (x=1,2),v0.9.0

- Text Proposal

----- Start of TP -----

- 5.22	CA_n41-n79

- 5.22.1	Common for 1 band UL and 2 bands UL CA

- 5.22.1.1	Operating bands for CA

Table 5.22.1.1-1: CA band combination CA_n41-n79

| NR CA Band Combination | NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n41-n79 | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

If the band combination is TDD/TDD, is SimRx/Tx supported (YES/NO/N-A)? ==> YES.

- 5.22.1.2	Channel bandwidths per operating band for CA

Table 5.22.1.2-1: Supported bandwidths per CA band combination CA_n41-n79

| NR CA configuration | Uplink CA configuration or single uplink carrier10 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n41C-n79A | CA_n41C-n79A | n41 | CA_n41C_BCS4 and 5 | 4 and 5 |
|  |  | n79 | See n79 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41(2A)-n79A | CA_n41(2A) CA_n41A-n79A CA_n41(2A)-n79A | n41 | CA_n41(2A)_BCS4 and 5 |  |
|  |  | n79 | See n79 channel bandwidths in Table 5.3.5-1 |  |

- 5.22.1.3	UE Co-existence studies

For the UL/DL harmonic, harmonic mixing and cross band isolation caused by single band n41 or single band n79, it was already analyzed in the fallback configurations of DL CA_n41A-n79A with UL single carrier, and it can be applied for DL CA_n41C-n79A with UL single carrier.

Due to simultaneous Rx/Tx operation is supported for TDD bands n41 and n79, thus the co-existence studies caused by UL CA_n41C should be analyzed additionally for CA_n41C-n79A, respectively, which can be found in table Table 5.22.1.3.1-1:

Table 5.22.1.3.1-1: Intra-band ULCA IMD overlap with the other DL band analysis

| All in MHz | flow | fhigh | BB IMD range3 |  |  |
| --- | --- | --- | --- | --- | --- |
| n41 fUL | 2496 | 2690 | Order | flow | fhigh |
| n79 fDL | 4400 | 5000 |  |  |  |
| 2CCBW1 | Minimum | Maximum | IMD2 (1-1) | Min2CCBW | Max2CCBW |
|  | 0 | 160 |  | 0 | 160 |
| Close to UL IMD range2 |  |  | IMD4 (2-2) | 2*Min2CCBW | 2*Max2CCBW |
| Order | flow | fhigh |  | 0 | 320 |
| IMD3 (2-1) | fULlow-Max2CCBW | fULhigh+Max2CCBW | IMD6 (3-3) | 3*Min2CCBW | 3*Max2CCBW |
|  | 2336 | 2850 |  | 0 | 480 |
| IMD5 (3-2) | fULlow-2*Max2CCLBW | fULhigh+2*Max2CCBW | Close to H2 IMD range4 |  |  |
|  | 2176 | 3010 | Order | flow | fhigh |
| IMD7 (4-3) | fULlow-3*Max2CCBW | fULhigh+3*Max2CCBW | IMD4 (3-1) | 2*fULlow-Max2CCBW | 2*fULhigh+Max2CCBW |
|  | 2016 | 3170 |  | 4832 | 5540 |
| IMD9 (5-4) | fULlow-4*Max2CCBW | fULhigh+4*Max2CCBW | IMD6(4-2) | 2*fULlow-2*Max2CCBW | 2*fULhigh+2*Max2CCBW |
|  | 1856 | 3330 |  | 4672 | 5700 |
| IMD11 (6-5) | fULlow-5*Max2CCBW | fULhigh+5*Max2CCBW | Close to H3 IMD range4 |  |  |
|  | 1696 | 3490 | Order | flow | fhigh |
| IMD13 (7-6) | fULlow-6*Max2CCBW | fULhigh+6*Max2CCBW | IMD5 (4-1) | 3*fULlow-Max2CCBW | 3*fULhigh+Max2CCBW |
|  | 1536 | 3650 |  | 7328 | 8230 |
| Analysis | IMD4 and IMD6 caused by n41C would fall into n79 DL Rx. |  |  |  |  |

Due to simultaneous Rx/Tx operation is supported for TDD bands n41 and n79, thus the co-existence studies caused by UL CA_n41(2A) should be analyzed additionally for CA_n41(2A)-n79A, respectively, which can be found in table Table 5.22.1.3.1-2:

Table 5.22.1.3.1-2: Intra-band ULCA IMD overlap with the other DL band analysis

| All in MHz | flow | fhigh | BB IMD range3 |  |  |
| --- | --- | --- | --- | --- | --- |
| n41 fUL | 2496 | 2690 | Order | flow | fhigh |
| n79 fDL | 4400 | 5000 |  |  |  |
| 2CCBW1 | Minimum | Maximum | IMD2 (1-1) | Min2CCBW | Max2CCBW |
|  | 10 | 190 |  | 10 | 190 |
| Close to UL IMD range2 |  |  | IMD4 (2-2) | 2*Min2CCBW | 2*Max2CCBW |
| Order | flow | fhigh |  | 20 | 380 |
| IMD3 (2-1) | fULlow-Max2CCBW | fULhigh+Max2CCBW | IMD6 (3-3) | 3*Min2CCBW | 3*Max2CCBW |
|  | 2306 | 2880 |  | 30 | 570 |
| IMD5 (3-2) | fULlow-2*Max2CCLBW | fULhigh+2*Max2CCBW | Close to H2 IMD range4 |  |  |
|  | 2116 | 3070 | Order | flow | fhigh |
| IMD7 (4-3) | fULlow-3*Max2CCBW | fULhigh+3*Max2CCBW | IMD4 (3-1) | 2*fULlow-Max2CCBW | 2*fULhigh+Max2CCBW |
|  | 1926 | 3260 |  | 4802 | 5570 |
| IMD9 (5-4) | fULlow-4*Max2CCBW | fULhigh+4*Max2CCBW | IMD6(4-2) | 2*fULlow-2*Max2CCBW | 2*fULhigh+2*Max2CCBW |
|  | 1736 | 3450 |  | 4612 | 5760 |
| IMD11 (6-5) | fULlow-5*Max2CCBW | fULhigh+5*Max2CCBW | Close to H3 IMD range4 |  |  |
|  | 1546 | 3640 | Order | flow | fhigh |
| IMD13 (7-6) | fULlow-6*Max2CCBW | fULhigh+6*Max2CCBW | IMD5 (4-1) | 3*fULlow-Max2CCBW | 3*fULhigh+Max2CCBW |
|  | 1356 | 3830 |  | 7298 | 8260 |
| Analysis | IMD4 and IMD6 caused by n41(2A) would fall into n79 DL Rx. |  |  |  |  |

- 5.22.1.4	∆TIB and ∆RIB values

The TIB,c and RIB of CA_n41-n79 are already defined in the spec TS38.101-1.

- 5.22.1.5	REFSENs requirements

According to the co-existence study, additional IMD MSD requirements caused by UL CA_n41C should be defined for CA_n41C-n79A.

In terms of the WF R4-2502955, it was agreed that the mandatory 1RB+1RB test point and Full+Full test points are captured in the current MSD table due to IMD in section 7.3A.5 of TS 38.101-1.

The MSD value given in the WF R4-2502955 are shown in table 5.22.1.5-1.

For the IMD caused by the UL CA_n41(2A) for CA_n41(2A)-n79A, the MSD for 1RB+1RB test point can be reused from the existing MSD value. In addition to that, as similar as CA_n41C-n79A, and the MSD for full-full test points are shown in table 5.22.1.5-1:

Table 5.22.1.5-1: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC3 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n41-n79 | n41 | 2565 | 100 | 1 (RBSTART=89) | 2565 | N/A | TDD | N/A |
|  |  | 2644.8 | 60 | 1 (RBSTART=93) | 2644.8 | N/A | TDD | N/A |
|  | n79 | N/A | 10 | N/A | 4995 | 10.4 | TDD | IMD414,15 |
|  | n41 | 2565 | 100 | 270 (RBSTART=3) | 2565 | N/A | TDD | N/A |
|  |  | 2644.8 | 60 | 162 (RBSTART=0) | 2644.8 | N/A | TDD | N/A |
|  | n79 | N/A | 10 | N/A | 4995 | 6 | TDD | IMD414,15,20 |
|  | n41 | 2565 | 100 | 270 (RBSTART=3) | 2565 | N/A | TDD | N/A |
|  |  | 2655 | 40 | 100 (RBSTART=6) | 2655 | N/A | TDD | N/A |
|  | n79 | N/A | 10 | N/A | 4995 | 5.3 | TDD | IMD414,15,20 |
| NOTE 14: Applicable when n41 spectrum is restricted to 2515-2675MHz. NOTE 15: This band is subject to IMD6 also which MSD is not specified NOTE 20: This second test point is optional. |  |  |  |  |  |  |  |  |

- 5.22.1.6	OOB blocking exception requirements

No additional OOB blocking exception issue.

- 5.22.2	Specific for 2 bands UL CA

- 5.22.2.1	Maximum output power for inter-band CA

Table 5.22.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n41C-n79A | 23 | +2/-3 |
| CA_n41(2A)-n79A | 23 | +2/-3 |

- 5.22.2.2	UE co-existence studies

Due to both band n41 and n79 are TDD bands, so there are no need to check the TB IMDs caused by UL CA_n41A-n79C and CA_n41(2A)-n79A since the Tx and Rx for either n41 or n79 will not be operation at the same time.

- 5.22.2.3	REFSENS requirements

Based on the co-existence , there is no need to define the additional REFSEN exception requirements.

----- End of TP -----