3GPP TSG-RAN WG4 Meeting #116			                          	R4-2510130

Bengaluru, India, August 25th – 29th, 2025

Agenda item:	7.21.3.2

Source: 	CMCC

Title: 	Discussion on BS conformance for SBFD

Document for:	Discussion

# 1. Introduction

In TR 38.858, the impacts on SBFD requirements have been analyzed. RAN4 has identified which legacy requirements are still applicable, which legacy requirements are not applicable and which new requirements are needed. And based on the work plan, the conformance requirement is also needed to be discussed

In this contribution, we continue discuss the conformance requirement for SBFD BS.

- Discussion

2.1 Additional manufacturer declaration(s) is needed for SBFD-capable BS.

Way forward - Issue-1: Additional manufacturer declaration(s) is needed for SBFD-capable BS. - Based on the discussion so far, the following declarations shall be captured in TS 38.141-1/-2: TX power for SBFD; Bands supported for SBFD; Channel bandwidth(s) supported for SBFD, from existing channel bandwidth defined in TS 38.104; SBFD subband configuration(s) per supported channel bandwidth, including SBFD pattern (DUD or DU/UD), size of DL subband(s), size of UL subband, and size of guardband(s) between subbands. - FFS other declaration is required

Following earlier discussion on SBFD RF requirements, many SBFD BS configurations must now be supported by e manufacturer declarations. Therefore, additional declarations need to be provided during testing for SBFD BSs. Issue-1 already contains all the necessary declarations, which are also required for the performance requirements.

Proposal 1: All the declarations in the WF is needed.

2.2 RF channels to be tested for SBFD operation

Way forward - Issue-2: RF channels to be tested for SBFD operation - FFS RF channels to be tested for SBFD operation, e.g,, to guarantee SBFD carrier is configured at the left/right-most of the operating band.

Currently, TS 38.141-1/2 specifies single carrier test, occupied bandwidth test, and capable of multi-band operation, all of which use three test positions: left-most, right-most, and middle of the channel. For SBFD, new tests dedicated to SBFD BS must be introduced, but these new tests must still retain the same three test positions.

Proposal 2: New test models need to be set for SBFD BS.

Proposal 3: Guarantee SBFD carrier is configured at the left-most, right-most and middle of the operating band.

2.3 Additional test models are needed for SBFD-capable BS

Way forward - Issue-3: Additional test models are needed for SBFD-capable BS. - FFS new parameters, e.g., size of UL subband, UL subband center frequency or its relative offset from the DL, etc., are needed to describe the size and location of UL subband. - FFS new test models are required to contain SBFD configurations for UL/DL subband and guardband. - FFS the test model for SBFD DL subbband needs to be scalable - FFS signal statistics may change for SBFD test models and need to be investigated. Impact of DL subband on signal statistics needs to be analyzed. - FFS how to accommodate SBFD time domain pattern with the test model.

Since a completely new test model must be created for SBFD BS, some of SBFD specific information, such as the location of the UL/DL sub-bands, guard-band size, and sub-band size—has to be describe to the test, beyond what is already present in the existing test models. All of the above description should be clear in the spec.

Proposal 4: New parameters such as guard-band size should be described clearly in the spec.

# 3. Conclusions

In this contribution, SBFD RF requirements are discussed with following observations and proposals.

Proposal 1: All the declarations in the WF is needed.

Proposal 2: New test models need to be set for SBFD BS.

Proposal 3: Guarantee SBFD carrier is configured at the left-most, right-most and middle of the operating band.

Proposal 4: New parameters such as guard-band size should be described clearly in the spec.

# 4. Reference

[1] R4-2508707, Way Forward for [115][306] NR_duplex_evo_General