3GPP TSG-RAN WG4 Meeting #116			                          	R4-2510131

Bengaluru, India, August 25th – 29th, 2025

Agenda item:	7.21.3.3

Source: 	CMCC

Title: 	Discussion on existing Rx requirements for SBFD

Document for:	Discussion

# 1. Introduction

In TR 38.858, the impacts on SBFD requirements have been analyzed. RAN4 has identified which legacy requirements are still applicable, which legacy requirements are not applicable and which new requirements are needed.

In last meeting, WF on SBFD BS requirement is approved with several agreements. [1]

In this contribution, we continue the RF requirements discussion based on the output of study item phase.

- Discussion

2.1 ACS requirements

Following capture the agreements from TR 38.858 for ACS requirements:

- ACS requirement and the interference level shall be determined by RAN4 co-existence study, and for the definition of ACS requirement: - - Conducted ACS: Take the existing wanted signal of ACS requirement by using the existing reference sensitivity level. - - OTA ACS: The OTA sensitivity degradation shall be taken into account to determine the level of wanted signal and interference signal mean power.

Last meeting, we have the following requirements.

Issue 2-1: ACS requirement - Way forward - FFS on whether the assumptions that will be applied for in-band blocking requirements derivation can be applied for ACS requirement.

Similar to ACLR requirements, adjacent-channel co-existence simulation only focuses on not less than 10% grid shift values, and 0% grid shift is avoided. The same definition of ACS could be reused for SBFD, and the interference level of [9] dB offset should be further checked.

Proposal 1: The same definition of ACS could be reused for SBFD, and the interference level of [9] dB offset should be further checked.

2.2 In-band blocking

Last meeting we have approved the WF below:

- Way forward - For in-band blocking FR1 WA BS, FFS on the value by taking the below range as starting point: - [-28, -16.7] dBm, in which the min and max are coming from co-ex simulations based on 100% grid shift. - For requirements assumption as deployment guidance, FFS on the following options - Option 1: The in-band blocking requirements assume a [TBD] dB coupling loss between interfering NR BS transmitter operating in and NR SBFD BS receive - Option 2: Distance, need to solve the issue that different frequency bands/scenarios would have different values - Option 3: One note can be added to generally describe the deployment restriction.

For the requirements assumption, three choices are currently provided: coupling loss, distance, and a note to clarify. In our view, option 1 coupling loss conveys the intent more clearly than distance, while an appropriate note can be added to describe the deployment scenario.

Proposal 2: coupling loss conveys the intent more clearly than distance.

2.3 Receiver intermodulation

- Issue 2-5: Necessity of New RX intermodulation requirement with 1 or 2 interfering signal - Way forward - Down-select to one of the below two options in the next meeting - Option 1: New IMD requirement is not needed. Such a requirement is implicitly captured by the SBFD RX ACS/blocking requirement, i.e. the IMD between the ACS/blocking interference and own SBFD DL can be regarded as the worst case - Option 2: New IMD requirement is needed, the following scenario with 2 input signals can be regarded as the worst case - Interfering signal 1: CW - Interfering signal 2: TBD MHz NR - Power level: the final agreed in-band blocking level -9 dB

The introduction of the two new RX intermodulation requirements is primarily driven by the need to address BS2BS cross-link interference. For the general in-band blocking scenario has already accounted for the new Rx cross-link interference. So there is no need to define RX intermodulation requirement with 1 interfering signal. For the scenario for 2 input signals, it could be regarded as the worst case when setting requirement on receiver linearity. It is better to defined this requirement.

Proposal 3: It is better to define new RX intermodulation requirement with 2 interfering signals.

# 3. Conclusions

In this contribution, SBFD RF requirements are discussed with following observations and proposals.

Proposal 1: The same definition of ACS could be reused for SBFD, and the interference level of [9] dB offset should be further checked.

Proposal 2: coupling loss conveys the intent more clearly than distance.

Proposal 3: It is better to define new RX intermodulation requirement with 2 interfering signals.

# 4. Reference

[1] R4-2508713, Way Forward for [115][307] NR_duplex_evo_BSRF