3GPP TSG-RAN WG4 Meeting #116			                          	                 R4-2510134

Bengaluru, India, August 25th – 29th, 2025

Agenda item:	7.24.3.2

Source: 	CMCC

Title: 	Discussion on LP-WUS UE RF Rx requirements

Document for:	Discussion

# 1. Introduction

Revised WID of LP-WUS is approved with following RF part objectives:

- Specify the necessary RAN4 core requirement(s) to support the feature (RAN4).

  - Specify UE low-power wake-up receiver requirements, at least REFSENS, ACS and ASCS requirements with consideration of possible new methodology to assess the low-power wake-up receiver performance

    - Define guard RBs for ACS and ASCS cases

    - Study testability of above requirements

    - Consider impacts of different architecture and impairments, and set requirements that enable all types of reasonable implementation

  - Study and if necessary specify or support by declaration, the corresponding BS requirements, e.g., dynamic range for LP-WUS/LP-SS.

    - Current NR BS requirements is baseline

- Discussion

2.1 UE feature for LP-WUS

During the last meeting, RAN4 reached the following agreement below.

Issue 2-1-1: SNR for LP-WUR two types Agreements: - There is no repetition of information bits to derive OFDM-based receiver SNR. - SNR=-4.5dB for envelope detection and OFDM-based receiver. Issue 2-1-2: Whether update the equation for REFSENS Agreements: - Use legacy REFSENS approach to derive requirements in spec. For TR, some statement of this issue may be presented. Issue 2-1-3: RMC for LP-WUR requirements Agreements: - In the FR1 spec, there should be two tables for 15kHz and 30kHz; - The following parameter is agreed. Further adding other parameter (if necessary) is not precluded. - Table A.3.4.2-1 Fixed reference channel for LP-WUR requirements Parameter; Unit; Value. MR Channel bandwidth; MHz; All CBW. LP-WUS bandwidth; RB; 11. Subcarrier spacing; kHz; 15/30kHz. RM coding; bits; 16bits. CRC; No CRC. Chip rate; M=4 (4 chips in an OFDM symbol). overlaid OFDM sequence; Length 33: generated by 31-length ZC sequence with extension. Number of overlaid OFDM sequence per chip to carry information; 4. WUS duration for OOK; 8 OFDM symbols. WUS duration for OFDM; 2 OFDM symbols. Manchester coding for OOK; 1/2. Number of information bits; 5bits. Issue 2-1-6: UE feature for LP-WUS WF: - FFS if RAN4 will specify capability (or capabilities) for the two different types of LP-WUR.

In previous meeting we reached an agreement on the performance for Type1 UE and Type2 UE. However, different companies have varying interpretations of the two UE types. Our understanding is that the rationale for defining two UE types is due to the different demodulation capabilities of the receivers corresponding to the two distinct waveforms. Based on this, we think it is necessary to define two separate sets of RF requirements. Additionally, from the perspective of UE capability reporting, the UE needs to report its own UE type. In terms of UE capability, the UE type should be more explicitly defined.

Proposal 1: The UE capabilities of Type 1 and Type 2 UE should be clarified.

# 3. Conclusions

In this contribution, ASCS, ACS and REFSENSE are discussed with following observations and proposals.

Proposal 1: The UE capabilities of Type 1 and Type 2 UE should be clarified.

# 4. Reference

- R4-2507936, WF for [115][136] NR_LPWUS_UERF