# 3GPP TSG-RAN WG4 Meeting #116                              R4-2510142

# Bengaluru, India, August 25th – 29th, 2025

Agenda Item:	7.20.4.1

Source: 	CMCC

Title: 			 Discussion on RRM requirements for event-driven beam management for NR MIMO Phase 5

Document for:	Discussion

- Introduction

In RAN #102 meeting, the new WID on NR MIMO Phase 5 was approved [1], the objectives are duplicated as following.

RAN1: - Specify enhancement to facilitate UE-initiated/event-driven beam management for reducing overhead and/or latency, assuming the unified TCI while leveraging (as much as possible) legacy CSI measurement and reporting configuration frameworks, targeting FR2 and sTRP with intra- and inter-cell beam management - UL signaling content(s) (and procedure(s) as required) for UE-initiated/event-driven beam reporting facilitating fast beam switching - UL signaling medium/container considering the UE-initiated/event-driven nature of the UL transmission, designed primarily for the purpose of beam reporting - Specify CSI support for up to 128 CSI-RS ports, targeting FR1 - Type-I codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks - Type-II codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks, without modifying any codebook parameter other than introducing additional values for the number of ports codebook parameter(s) - Extension of CRI(s)-based CSI reporting (CQI/PMI/RI calculated per CRI for ≥1 CRIs) for hybrid beamforming supporting up to a total of 128 CSI-RS ports across all resources, with up to 32 CSI-RS ports per resource, without new codebook design - SRS port grouping and its association to the two codewords for the 6/8Rx low complexity receiver supporting more than 4 layers, with legacy codebook - No enhancement on codeword-to-layer mapping, DL resource allocation, CSI feedback, and DCI format - Note: Whether to support 6Rx with more than 4 layers is to be decided in RAN4 Rel-19 RF enhancements WI - Specify UE reporting enhancement for CJT deployments under non-ideal synchronization and backhaul, targeting FR1, both FDD and TDD - Inter-TRP time misalignment and frequency/phase offset measurement and reporting, assuming legacy CSI-RS design, with stand-alone aperiodic reporting on PUSCH - Specify non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions, enhancement(s) to enable 3T6R SRS antenna switching, as well as UE capability signaling for 3T3R antenna switching and 3-antenna-port non-codebook-based transmissions, without enhancement on UL full power transmission and without enhancement on SRS resource Note: UL full power transmission mode 1 and 2 are not supported. Note: Other than UE capability signaling, no other enhancement is specified for 3T3R SRS antenna switching. - Specify enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2 - Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP. - Two TAs through reusing Rel-18 specification of two TAs for multi-DCI-based multi-TRP and removing the restriction that coresetPoolIndex needs to be configured, assuming legacy PRACH resources Acronyms: - sTRP: single TRP (transmit-receive point) - mTRP: multiple TRP (transmit-receive point) - CJT: coherent joint transmission - DU: distributed unit - CRI: CSI-RS resource indicator RAN2: Specify higher layer support of the enhancements listed above. RAN4: Specify necessary core requirements for the enhancements listed above.

In last meeting, there was discussion on NR MIMO Phase 5, and a WF was agreed [2]. This contribution provides further discussion on RRM requirements for event-driven beam management.

- Discussion

According to the WID [2], one of the objectives is to specify enhancement to facilitate UE-initiated/event-driven beam management for reducing overhead and/or latency, assuming the unified TCI while leveraging (as much as possible) legacy CSI measurement and reporting configuration frameworks, targeting FR2 and sTRP with intra- and inter-cell beam management. Companies have different interpretention on this objective. Some companies think the wording “targeting FR2” means only consider FR2. However, our interpretention about this objective is that even though the target is FR2, it does not preclude FR1.

Agreements in RAN4#113 meeting (R4-2420096) Issue 1-2: How to define measurement period (evaluation time / measurement time) in Event triggered measurement reporting delay? Agreement: For specifying the RAN4 delay requirement for event 2 - Measure both the current beam and new beam after the starting point, FFS the details. - Note: this does not imply to define the delay requirement based on the sum of the current beam measurement period and new beam measurement period, which is pending on further RAN1 discussion. - Start the RAN4 RRM discussion for FR2. - Send LS to RAN1 to ask whether FR1 is covered as well. FFS the exact wording in the LS.

In RAN4 #113 meeting, it was agreed to start the RAN4 RRM discussion for FR2, and send LS to RAN1 to ask whether FR1 is covered as well. However, it was noticed that the LS was missing. And we provided the draft LS in the Annex part of our contribution submitted to RAN4 #114 meeting [3]. However, after discussion, it is noted that RAN1 discussion on feature list for Rel-19 MIMO already discuss this issue. We can further check RAN1 conclusion.

According to the latest agreed RAN1 feature list, as duplicated as following, it can be seen that there is no FR1/FR2 differentiation, which means UEIBM apply to both FR1 and FR2. It is necessary to define requirements for FR1.

Proposal 1: for UEIBM, it is proposed to define RAN4 requirements for FR1, since according to RAN1  feature list, FR1 is consiered for UE-initiated/event-driven beam management.

Issue 1-3: Whether RAN4 to specify requirements for no time window based or cover both no time window based and with time window based with eventInstanceCount-r19? <Way forward > - Option 1: (Qualcomm, Apple, LGE) - Do not define requirements for eventInstanceCount-r19 > 1 - Option 2: (China Telecom, Nokia, Xiaomi, Samsung, CMCC, Huawei, Ericsson, ZTE, MTK) - Specify requirements to cover both cases of no time window based and with time window based with configurable eventInstanceCount-r19

According to RAN1 agreements, as duplicated as below, both no time window based and with time window based are supported. Since the basic feature is without time window, it is necessary to specify requirements. For time window based case, in RAN4#113 meeting, it was agreed that it is RAN4 common understanding that no additional filtering applies for this case compared to the case without time window. However, the evalution time may be different since the report is triggerred when at least one same new beam is greater than or equal to a configurable number M. With above consideration, it is proposed to specify requirements to cover both cases of no time window based and with time window based with configurable M.

Agreements in RAN1#117 meeting Regarding the triggering event determination for Event 2: - If within a time window (which is configurable), the number of Event-2 instance(s) for at least one same new beam is greater than or equal to a configurable number M, UE initiated beam report occurs. - Note: Event-2 instance for a new beam is determined if the L1-RSRP of the new beam becomes a threshold value better than the current beam Above feature is subject to UE capability. - Basic feature: Once the L1-RSRP of the new beam becomes a threshold value better than the current beam, UE initiated beam report occurs

Proposal 2: it is proposed to specify requirements to cover the case of no time window and the case with time window with configurable eventInstanceCount-r19.

Issue 1-8: Capabilities for Support of Event Triggering and Reporting Criteria (similar Capabilities in 38.133 9.1.4)? <Way forward > - Option 1: (Samsung, Apple, Huawei, Ericsson) - For event trigger L1 reporting, do not define such capability in parallel per category like L3. - Option 2: (Nokia, MTK) - Define capabilities for support of UE-initiated/event-driven beam reporting on top of existing L3 events in section 9.1.4. For intra-frequency, numbers of event = X, where X is candidate value of [3 or 6 or 9]. Issue 1-9: Compatibility of capabilities with LTM <Way forward > - If such capabilities are defined in Issue 1-8, then discuss on: - Option 1: - The capability for supported number of events triggered L1 measurement reporting is defined separately for Rel-19 mobility and Rel-19 MIMO. - Option 2: - The capability for supported number of events triggered L1 measurement reporting is defined as common for Rel-19 mobility and Rel-19 MIMO.

For legacy event triggered L3 measurement reporting, the capabilities for support of event triggering reporting criteria are defined (section 9.1.4, TS38.133). Similarly, for Rel-19 event triggered L1 measurement reporting, it is preferred to define the capability. However, according to the discussion in last meeting, some companies mentioned that the number of events to be supported by UE shall be discussed in RAN1 first. We do not have strong view. But at least according to our knowledge, RAN1 have not discussed this. LS to RAN1 can be considered if necessary. On the other hand, some companies do not think it is needed to define such capability, since different from L3 measurement with different types of events, the measurement type is only based on RSRP for L1 measurement. And it will not be supported to measure and report the different events at the same reporting. We do not have strong view, the key consideration is that if it is agreed to define the capability in RAN4, it is not a good way to replace or update legacy capability for event triggered L3 measurement reporting. It is prefered to define a separate capability for supported number of events triggered L1 measurement reporting.

Proposal 3: if it is agreed to define capabilities for support of L1 event triggering and reporting criteria, it is proposed to define a separate capability for L1 measurement, not prefer to mix L1 measurement reporting capability with legacy L3 measurement reporting capability.

Event triggered L1 measurement reporting are considered for both Rel-19 MIMO and Rel-19 Mobility. However, considering that Rel-19 MIMO and Rel-19 Mobility are two parallel on-going work item, it is not prefered to mix the discussion. And in last meeting, this issue was discussed in WI on mobility enahncement, and it was agreed that RAN4 shall develop the two features independently at current stage. Further discussion on common capability is allowed when discussing UE feature list of R19.

Proposal 4: if it is agreed to define capabilities for support of L1 event triggering and reporting criteria, it is not prefered to mix Rel-19 mobility and Rel-19 MIMO. The capability for supported number of events triggered L1 measurement reporting is defined seperately for Rel-19 mobility and Rel-19 MIMO.

- Conclusion

This contribution provides discussion on RRM requirements for event-driven beam management for NR MIMO Phase 5. The proposals are:

Proposal 1: for UEIBM, it is proposed to define RAN4 requirements for FR1, since according to RAN1  feature list, FR1 is consiered for UE-initiated/event-driven beam management.

Proposal 2: it is proposed to specify requirements to cover the case of no time window and the case with time window with configurable eventInstanceCount-r19.

Proposal 3: if it is agreed to define capabilities for support of L1 event triggering and reporting criteria, it is proposed to define a separate capability for L1 measurement, not prefer to mix L1 measurement reporting capability with legacy L3 measurement reporting capability.

Proposal 4: if it is agreed to define capabilities for support of L1 event triggering and reporting criteria, it is not prefered to mix Rel-19 mobility and Rel-19 MIMO. The capability for supported number of events triggered L1 measurement reporting is defined seperately for Rel-19 mobility and Rel-19 MIMO.

- Reference

- RP-242394, WID revision: NR MIMO Phase 5

- R4-2508271, WF on RRM requirements for NR_MIMO_Ph5_Part1, Samsung

- R4-2501081, Discussion on RRM requirements for event-driven beam management for NR MIMO Phase 5, CMCC

- R4-2504902, WF on RRM requirements for NR_MIMO_Ph5_Part1, Samsung

- R4-2502595, WF on RRM requirements for NR_MIMO_Ph5_Part1, Samsung

- R4-2420096, WF on RRM requirements for NR_MIMO_Ph5_Part, Samsung

- R1-2410914, LS to RAN4 on event-instance evaluation periodicity for UE-initiated/event-driven beam management

- R4-2416856, WF on RRM requirements for NR_MIMO_Ph5_Part1, Samsung

- R4-2414035, WF on RRM requirements for NR_MIMO_Ph5, Samsung