# 3GPP TSG-RAN WG4 Meeting #116                              R4-2510146

# Bengaluru, India, August 25th – 29th, 2025

Agenda Item:	7.25.3.2

Source: 	CMCC

Title: 			 Discussion on CSI-RS based L1 measurement

Document for:	Discussion

- Introduction

In RAN #102 meeting, the new WID on NR mobility enhancements Phase 4 was approved [1]. In RAN #105 meeting, the revised WID was approved [2]. The detailed objectives are duplicated as following.

- Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3] - Prioritize the case when CU is acting as MN when DC is not configured - When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases: - As secondary priority, support the case where CU is acting as SN and MN is unchanged - As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released - Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM - Coordination with SA3 needed with respect to security key handling - Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support - Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management [RAN1] - Specify CSI acquisition on candidate cell(s) based on CSI-RS before or during LTM cell switch [RAN1] NOTE: RAN1 WG to decide on whether to support CSI report before or during the LTM cell switch, as part of the CSI acquisition procedure. - Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Limit specifying the conditional LTM to the scenario where the UE is in non-DC - Specify RRM requirements related to the above objectives as necessary [RAN4]

In last meetin, there was discussion on measurements related enhancements for LTM, and a WF was agreed [3]. This contribution provides discussion on CSI-RS based L1 measurement.

- Discussion

- Issue 3-2: whether step 2 is skipped for serving cell? Candidate options: - Option A: RAN4 requirements regarding ‘skipping step 2’ do not apply for the case CSI-RS based L1 measurement for LTM candidate cell is configured in legacy CSI-RS based L1 measurement in serving cell in FR2. (QC, HW) - Option A1: Requirement regarding ‘skipping step 2’ is not applicable if an FR2 candidate cell, from which CSI-RS is configured with repetition set to ‘off’, is one of the FR2 serving cells. (QC, OPPO) - Option A2: The CSI-RS based L1-RSRP measurement requirement is not applicable if an FR2 candidate cell, from which CSI-RS is configured with repetition set to 'off', is one of the FR2 serving cells (QC). - Option B: keep the current measurement procedure unchanged for serving cell when it is in the candidate cell for LTM. (MTK, Apple, E///)

In previous meeting, it was agreed that whether step 2 (SSB based L1 measurement) can be skipped for neighboring cell or not based on new UE capability reporting. For a UE supporting the capability, and if the Step 2 is skipped, UE Rx beam sweeping factor needs to be included in the CSI-RS based L1-RSRP measurement period, and the measurement period for CSI-RS based L1-RSRP will be scaled by 8. The mechanism is different from legacy serving cell L1 measurement, which is that SSB based RX beam sweeping if performed firstly. If skipping step 2 is applied to the case that SCell is configured as LTM candidate cell, it means that UE will have two different measurement approaches for the same cell, which is complex and confusing. Based on above consideration, it is proposed to keep the current measurement procedure unchanged for serving cell when it is in the candidate cell for LTM.

Proposal 1: it is proposed to keep the current measurement procedure unchanged (i.e. SSB based RX beam sweeping is not skipped for CSI-RS L1 measurement) for serving cell when it is in the candidate cell for LTM.

- Issue 3-9: early CSI acquisition Candidate options: - Option 1: Define timeline for baseline early CSI acquisition. (vivo, Nokia, E///) - Option 2: Define scheduling/measurement restriction requirements for optional early CSI acquisition. (QC) - Option 3: not define RAN4 requirements for early CSI acquisition. (MTK, HW, Apple)

RAN1 agreements about CSI acquisition are summarized as following. The baseline is that CSI-RS measurement and CSI reporting operations are performed after reception of LTM CSC MAC CE. And introduce UE capability for CSI-RS measurement can start before reception of LTM CSC MAC CE. UE determines the CSI report configuration based on the CSC. CSI report configuration for CSI acquisition is determined from the field of Target Configuration ID in CSC MAC CE. For PUSCH to convey the early CSI report, for RACH-less LTM, the first CG or DG PUSCH after CSC is used. For RACH-based LTM with CFRA, PUSCH scheduled by RAR or Msg.A is used. For RACH-based LTM with CBRA, the first CG or DG PUSCH after HARQ-ACK transmission for Msg.4 or Msg.B is used.

RAN1#119 meeting Working Assumption As baseline, CSI-RS measurement and CSI reporting operations are performed after reception of LTM CSC MAC CE. - The report is sent directly to target cell - Introduce UE capability for CSI-RS measurement can start before reception of LTM CSC MAC CE - Other than UE capability, strive for no additional spec impact compared to the baseline (only one triggering mechanism will be specified) RAN1#120 meeting Agreement For target cell CSI acquisition, - A UE is provided with RRC configurations for periodic CSI-RS resource(s) and CSI report(s) for one or more candidate cell - For a candidate cell, - down-select from the following alternatives: - Alt 1: A single CSI report configuration is configured - Alt 2: Multiple CSI report configurations can be configured - down-select from the following alternatives: - Alt X: A single CSI-RS resource for CMR is associated with a CSI report configuration - Alt Y: Multiple CSI-RS resources for CMR can be associated with a CSI report configuration - FFS: Semi-persistent CSI-RS resource - After the RRC configuration and before the reception of CSC, the UE may measure CSI based on the configured CSI-RS resource(s), which is subject to UE capability - FFS: whether or how to select a subset of CSI-RS resources to measure - FFS: when the UE may start measuring the configured CSI-RS resources - UE determines the CSI report configuration based on the CSC - After the reception of cell switch command, the UE may measure (depending on the timeline) CSI-RS resource(s) associated with determined CSI report configuration - The latest available measured CSI on target cell resource(s) is conveyed at least by a single report, and the report is sent to the target cell - Option 1: to use UCI - Option 2: to use MAC CE Note: with this agreement, the working assumption made in RAN1#119 is automatically confirmed. RAN1#120bis meeting Agreement For candidate cell CSI acquisition - In addition to periodic CSI-RS resource, semi-persistent CSI-RS resource is supported - Support of semi-persistent CSI-RS resource is subject to separate UE capability. - MAC CE is used to activate/deactivate the semi-persistent CSI-RS resource similarly to the legacy mechanism for a serving cell which will be specified in RAN2 Send an LS to RAN2 and RAN3 to inform this agreement Agreement For the container of LTM CSI report to a target cell, UCI is used with a solution to avoid blind detection at gNB if valid CSI is not available - FFS: the definition of valid CSI Agreement For the solution to avoid blind detection for LTM CSI report if valid CSI is not available, - Option 1: CSI is reported when valid CSI is not available - Option 1-1: one CQI codepoint is used to indicate that valid CSI is not available - Option 1-1-1: Codepoint with the lowest CQI index is reported Agreement Regarding CSI acquisition, for a candidate cell, - A single CSI report configuration is configured - Multiple CSI-RS resources for CMR can be associated with the CSI report configuration - The number of CSI-RS resources for CMR is subject to UE capability Agreement Following restrictions are introduced - For the codebook configurations in report configuration, typeI-SinglePanel is supported for LTM CSI acquisition - For report frequency configuration in report configuration, wideband CQI and wideband PMI are supported for LTM CSI acquisition - For the report quantity in report configuration, cri-RI-PMI-CQI is supported for LTM CSI acquisition - The supported max rank is up to separate UE capability - For the number of CSI-RS ports of CSI-RS resource(s) associated with a CSI report configuration for a candidate cell for LTM CSI acquisition - Up to 128 ports is supported - The supported max number of CSI-RS ports is up to separate UE capability RAN1#121 meeting Conclusion - From RAN1 perspective, there is no restriction with regards to the frequency location of CSI-RS used for CSI acquisition for candidate cells Send an LS to RAN4 (CC RAN2) and ask them to take the above conclusion in their further work. LS in R1-2504828. Agreement A list of interference measurement resources for candidate cells is supported for LTM CSI acquisition - If this list is not configured, CMR is used for interference measurement Agreement CSI report configuration for CSI acquisition is determined from the field of Target Configuration ID in CSC MAC CE Agreement For PUSCH to convey the early CSI report, - For RACH-less LTM, the first CG or DG PUSCH after CSC is used - For RACH-based LTM with CFRA, PUSCH scheduled by RAR or Msg.A - For RACH-based LTM with CBRA, the first CG or DG PUSCH after HARQ-ACK transmission for Msg.4 or Msg.B For the reporting LTM early CSI reporting, Table 6.3.1.1.2-7 in TS 38.212 is used as a UCI report format

In legacy, there is no RRM requirements related with CSI procedure. However, for CSI acquisition on candidate cell(s) based on CSI-RS during LTM cell switch, timeline can be specified based on the cell switch delay. LTM cell switch delay is defined as: DLTM = Tcmd + TLTM-interrupt. TLTM-interrupt = TLTM-RRC-processing + TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU ms. CSI acquisition can be performed after T/F tracking. The CSI acquisition related delay is similar as the CSI report (TCSI_reporting)for SCell activation. TCSI_reporting is the delay (in ms) including uncertainty in acquiring the first available downlink CSI reference resource, UE processing time for CSI reporting and uncertainty in acquiring the first available CSI reporting resources as specified in TS 38.331. In summary, the timeline for CSI acquisition during LTM cell switch could be Tcmd + TLTM-RRC-processing + TLTM-processing + Tfirst-RS + TRS-proc + TCSI_reporting.

Proposal 2: it is proposed to define timeline for early CSI acquisition during LTM cell switch.

Proposal 3: the timeline for early CSI acquisition during LTM cell switch is proposed as:

TCSI-acquisition = Tcmd + TLTM-RRC-processing + TLTM-processing + Tfirst-RS + TRS-proc + TCSI_reporting

- TCSI_reporting is the delay (in ms) including uncertainty in acquiring the first available downlink CSI reference resource, UE processing time for CSI reporting and uncertainty in acquiring the first available CSI reporting resources

- Conclusion

This contribution provides discussion on CSI-RS based L1 measurement. The proposals are:

Proposal 1: it is proposed to keep the current measurement procedure unchanged (i.e. SSB based RX beam sweeping is not skipped for CSI-RS L1 measurement) for serving cell when it is in the candidate cell for LTM.

Proposal 2: it is proposed to define timeline for early CSI acquisition during LTM cell switch.

Proposal 3: the timeline for early CSI acquisition during LTM cell switch is proposed as:

TCSI-acquisition = Tcmd + TLTM-RRC-processing + TLTM-processing + Tfirst-RS + TRS-proc + TCSI_reporting

- TCSI_reporting is the delay (in ms) including uncertainty in acquiring the first available downlink CSI reference resource, UE processing time for CSI reporting and uncertainty in acquiring the first available CSI reporting resources

- Reference

- RP-234036, New WID: NR mobility enhancements Phase 4

- RP-250339, Revised Work Item: NR mobility enhancements Phase 4

- R4-2508282, WF on RRM requirements for NR_Mob_Ph4_Part1, Apple

- R4-2504965, WF on RRM requirements for NR_Mob_Ph4_Part1, Apple

- R4-2502601, WF on RRM requirements for NR_Mob_Ph4_Part1, Apple

- R4-2420101, WF on RRM requirements for NR_Mob_Ph4_Part1, Apple

- R1-2409283, LS on the support of semi-persistent CSI-RS resource for LTM candidate cells

- R3-247911, Reply LS for LS on the support of semi-persistent CSI-RS resource for LTM candidate cells

- R4-2416862, WF on RRM requirements for NR_Mob_Ph4_Part1, Apple

- R4-2414060, WF on R19 Further NR mobility enhancement, Apple

- RP-233970, Revised WID on Further NR Mobility Enhancements

- R2-2404102, Report of 3GPP TSG RAN WG2 meeting #125bis

- RP-242356, Revised Work Item: NR mobility enhancements Phase 4