# 3GPP TSG-RAN WG4 Meeting #116                              R4-2510150

# Bengaluru, India, August 25th – 29th, 2025

Agenda Item:	7.26.4

Source: 	CMCC

Title: 			 Discussion on RRM performance requirements for XR (eXtended Reality)

Document for:	Discussion

- Introduction

In RAN #102 meeting, the new WID on XR (eXtended Reality) for NR Phase 3 was approved [1]. According to the WID, one of the objectives having RAN4 impact is to enable transmission/reception in gaps/restrictions that are caused by RRM measurements, the details are duplicated as following.

- Specify enhancements to enable transmission/reception in gaps/restrictions that are caused by RRM measurements (from inter-frequency RRM measurement gaps, or intra-frequency measurements, or other scheduling restrictions etc). [RAN1, RAN2, RAN4] - Specify the corresponding measurement gap and scheduling restriction to enable the identified enhancements with RRM performance impact taken into consideration, work being triggered by LS. [RAN4]

This is the first meeting to discuss XR (eXtended Reality) for NR Phase 3. This contribution provides initial discussion on this topic.

- Discussion

In RAN4#113 meeting, it was agreed to reuse existing measurement accuracy requirements. And RAN4 shall keep the number of samples for a frequency layer for L3 measurements unimpacted due to gap skipping. Measurement delay is extended due to MG skipping, but how to extend is under discussion.

Proposal 1: for measurement accuracy requirements, it is proposed to confirm previous agreements to reuse existing measurement accuracy requirements for gap cancellation.

It was agreed to define UAI, the content of UAI is a ratio (R) of gap occasions that is recommended for cancellation during a time period of 1s. However, it was agreed not to define UE behaviour and the corresponding requirements based on UAI, and RAN4 will not specify any explicit or implicit requirement or expectation on gNB behaviour in response to any XR UAI. It seems that there is no way to test UAI.

For deployment scenarios, according to the core part discussion, it was agreed to support FR1 and FR2-1, and Deprioritize FR2-2 for enabling transmission/reception in gaps/restrictions that are caused by RRM measurements. EN-DC and NE-DC for DCI-based solution is NOT supported for measurement gap skipping. And skipping requirements applies for NR-SA.

Proposal 2: it is proposed to define test cases for NR-SA.

Proposal 3: it is proposed to define test cases for FR1 and FR2-1.

For types of measurements to be considered, the agreements are intra-frequency measurement, inter-frequency measurement, and LTE inter-RAT measurement are considered for XR measurement skipping. Measurement delay is extended due to MG skipping, since measurement delay requirements are different for intra-frequency measurement, inter-frequency measurement, and LTE inter-RAT measurement, it is necessary to define test cases for all these three scenarios..

Proposal 4: it is proposed to define test cases for intra-frequency measurement, inter-frequency measurement, and LTE inter-RAT measurement.

For the type of measurement gap, as a baseline RAN4 to define requirements due to measurement skipping based on Type 1 measurement gap. In addition, MG cancelling DCI applies to concurrent MGs and cancels the MG surviving the collision handling rules. Since UE behaviour is different, it is proposed to define test cases for Type 1 measurement gap and Type 2 measurement gap, especillay for the colliding case.

Proposal 5: it is proposed to define test cases for Type 1 measurement gap and Type 2 measurement gap ( e.g. the colliding case).

For time offset requirement for measurement gap skipping, baseline is 5ms, and optional is 3ms with UE capability. It is better to define test cases for both cases. In order to reduce the number of test cases, applibility rules can be considered, e.g. the UE supporing 3ms offset only need to pass the test with 3ms timeline, the UE not supporting 3ms offset need to pass the test with 5ms timeline.

Proposal 6: it is proposed to define test cases for both 3ms offset and 5ms offset. And applibility rules can be considered, e.g. the UE supporing 3ms offset only need to pass the test with 3ms timeline, the UE not supporting 3ms offset need to pass the test with 5ms timeline.

- Conclusion

This contribution provides discussion on RRM performance requirements for XR (eXtended Reality) . The observations and proposals are:

Proposal 1: for measurement accuracy requirements, it is proposed to confirm previous agreements to reuse existing measurement accuracy requirements for gap cancellation.

Proposal 2: it is proposed to define test cases for NR-SA.

Proposal 3: it is proposed to define test cases for FR1 and FR2-1.

Proposal 4: it is proposed to define test cases for intra-frequency measurement, inter-frequency measurement, and LTE inter-RAT measurement.

Proposal 5: it is proposed to define test cases for Type 1 measurement gap and Type 2 measurement gap ( e.g. the colliding case).

Proposal 6: it is proposed to define test cases for both 3ms offset and 5ms offset. And applibility rules can be considered, e.g. the UE supporing 3ms offset only need to pass the test with 3ms timeline, the UE not supporting 3ms offset need to pass the test with 5ms timeline.

- Reference

- RP-234080, New WID: XR (eXtended Reality) for NR Phase 3

- R4-2504911, WF on RRM requirements for XR_Ph3

- R4-2502700, WF on RRM requirements for XR_Ph3

- R4-2420103, WF on RRM requirements for XR_Ph3, Nokia

- R4-2416864, WF on RRM requirements for XR_Ph3, Nokia

- R1-2209536, XR-specific capacity enhancements, Nokia, Nokia Shanghai Bell

- R4-2414043, WF on RRM requirements for XR_Ph3, Nokia

- R1-2405736, LS on UE assistance information

- R1-2407561, LS on gaps/restrictions that are caused by RRM measurements