# 3GPP TSG-RAN WG4 Meeting #116                              R4-2510159

# Bengaluru, India, August 25th – 29th, 2025

Agenda Item:	7.17.4

Source: 	CMCC

Title: 			Discussion on RRM core requirement and testing framework for beam management

Document for:	Discussion

- Introduction

In RAN #102 meeting, the WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface was approved [1]. In RAN #106 meeting, the revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface was approved [2]. The objectives for RAN4 are duplicated as following.

- Core requirements for the above two use cases for AI/ML LCM procedures and UE features [RAN4]: - Specify necessary RAN4 core requirements for the above three use cases. This includes the test setup and methodology for the beam management use case. - Specify necessary RAN4 core requirements for LCM procedures including performance monitoring.

In last meeting, there is discussion on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, and a WF was approved [3]. This contribution provides discussion on RRM requirements for beam management.

- Discussion

Issue 2-3: Rx beam knowledge and TCI handling Agreement: - if the predicted Tx beam in Set A is QCL Type-D to a known Tx beam, the corresponding Rx beam is known. - If the predicted Tx beam in Set A is not QCL Type-D to a known Tx beam, depending on UE capability, the corresponding Rx beam may be known or unknown. - Known Rx beam means no additional Rx beam sweeping is needed - Unknown Rx beam means additional Rx beam sweeping is needed. - Rx beam is known mean TCI state is known if conditions below are also met The following conditions related to whether the TCI state is known or unknown is defined as the starting point. Companies are invited to bring analysis. - The target TCI state is known when the following is met: - The TCI state switch command is received within 1280 ms upon the last transmission of the RS resource for beam reporting or measurement and the UE has sent at least one L-1 RSRP report of the target TCI state (clause 8.10.2 of 38.133) - Applies for case 1 - FFS for case 2 - Whether RS resource is only from set B should be clarified - FFS on whether prediction also impacts if a TCI state is known or unknown - Companies are invited to bring analysis on how prediction would impact whether a TCI state is known or unknown - FFS on the time delay if prediction is used as a condition for a known TCI state

According to legacy TS38.133, the known conditions for TCI state is duplicated as following:

TS38.133 clause 8.10.2 The TCI state is known if the following conditions are met: - During the period from the last transmission of the RS resource used for the L1-RSRP measurement reporting for the target TCI state to the completion of active TCI state switch, where the RS resource for L1-RSRP measurement is the RS in target TCI state or QCLed to the target TCI state - TCI state switch command is received within 1280 ms upon the last transmission of the RS resource for beam reporting or measurement - The UE has sent at least 1 L1-RSRP report for the target TCI state before the TCI state switch command - The TCI state remains detectable during the TCI state switching period - The SSB associated with the TCI state remain detectable during the TCI switching period - SNR of the TCI state ≥ -3 dB Otherwise, the TCI state is unknown.

The key point of legacy known conditions for TCI state is that UE has performed measurement on the RS resource within a period. However, these conditions are not enough to cover AI/ML based beam prediction, since set A are predicted not measured.

In last meeting, it was agreed that if the predicted Tx beam in Set A is QCL Type-D to a known Tx beam, the corresponding Rx beam is known, and if the predicted Tx beam in Set A is not QCL Type-D to a known Tx beam, depending on UE capability, the corresponding Rx beam may be known or unknown. These two agreementss can be used as conditions to identify whether TCI state is known for AI/ML based beam prediction.

There are two cases for beam prediction. BM-Case1 is spatial-domain downlink beam prediction for Set A of beams based on measurement results of Set B of beams. BM-Case2 is temporal downlink beam prediction for Set A of beams based on the historic measurement results of Set B of beams. Above two conditions for known RX beam are applied to both cases.

One more issue is the valid duration of the predicted results. For legacy definition of known TCI state, one of the conditions is that TCI state switch command is received within 1280 ms upon the last transmission of the RS resource for beam reporting or measurement. Similarly, case 1, the valid duration for predicted beams could be that TCI state switch command is received within T ms upon the report of predicted results. And the value of T can be FFS. The value of T for case 1 and case 2 can be different, considering that the report of case 2 include the predicted beams for future time instances.

Proposal 1: it is proposed that the conditions for known TCI state is updated as following to cover AI/ML based beam prediction. And the updated parts are applied to both case 1 and case 2.

The TCI state is known if the following conditions are met: - During the period from the last transmission of the RS resource used for the L1-RSRP measurement reporting for the target TCI state to the completion of active TCI state switch, where the RS resource for L1-RSRP measurement is the RS in target TCI state or QCLed to the target TCI state, or - TCI state switch command is received within 1280 ms upon the last transmission of the RS resource for beam reporting or measurement - The UE has sent at least 1 L1-RSRP report for the target TCI state before the TCI state switch command - The TCI state remains detectable during the TCI state switching period - The SSB associated with the TCI state remain detectable during the TCI switching period - SNR of the TCI state ≥ -3 dB - TCI state switch command is received within T ms upon the report of predicted results. - If the predicted Tx beam in Set A is QCL Type-D to a known Tx beam - If the predicted Tx beam in Set A is not QCL Type-D to a known Tx beam, and UE is capable of knowing RX beam after beam prediction Note: the value of T is FFS. The value of T for case 1 and case 2 can be different.

The motivation to differentiate whether TCI state is known or not is that the TCI state switch delay is different. For DCI based TCI state switch, the switch delay is only defined for konwn TCI state. For MAC-CE based TCI state switch and RRC based TCI state switch, the TCI state switch delay for known case is shorter since the L1 measurement for Rx beam refinement is not needed.

TS38.133 clause 8.10.3 MAC-CE based TCI state switch delay If the target TCI state is known, upon receiving PDSCH carrying MAC-CE for indication of UE-specific PDCCH TCI state as defined in clause 6.1.3.15 of TS 38.321 [7] in slot n, UE shall be able to receive PDCCH with target TCI state of the serving cell on which TCI state switch occurs at the first slot that is after slot n+ THARQ + 3Nslotsubframe,µ+ TOk*(Tfirst-SSB + TSSB-proc) / NR slot length. ... If the target TCI state is unknown, upon receiving PDSCH carrying MAC-CE for indication of UE-specific PDCCH TCI state as defined in clause 6.1.3.15 of TS 38.321 [7] in slot n, UE shall be able to receive PDCCH with target TCI state of the serving cell on which TCI state switch occurs at the first slot that is after slot n+ THARQ +3Nslotsubframe,µ + TL1-RSRP +TOuk*(Tfirst-SSB+ TSSB-proc) / NR slot length. ... TS38.133 clause 8.10.5 RRC based TCI state switch delay If the target TCI state is known, UE shall be able to receive PDCCH with target TCI state of the serving cell on which TCI state switch occurs at the first slot that is after slot n+ (TRRC_processing +TOk*(Tfirst-SSB + TSSB-proc)) / NR slot length, The UE is not required to receive PDCCH/PDSCH/CSI-RS or transmit PUCCH/PUSCH until the end of switching period. ... If the target TCI state is unknown, UE shall be able to receive PDCCH with target TCI state of the serving cell on which TCI state switch occurs at the first slot that is after slot n+ (TRRC_processing +TL1-RSRP +TOuk*(Tfirst-SSB + TSSB-proc)) / NR slot length, The UE is not required to receive PDCCH/PDSCH/CSI-RS or transmit PUCCH/PUSCH until the end of switching period.

Observation 1: according to current TS38.133, the TCI state switch delay for known case is shorter than unknown case, since the L1 measurement for Rx beam refinement is not needed.

For beam prediction, with the condition that if the predicted Tx beam in Set A is QCL Type-D to a known Tx beam, the corresponding Rx beam is known or if the predicted Tx beam in Set A is not QCL Type-D to a known Tx beam, and UE is capable of knowing RX beam after beam prediction, it means that no need to consider the delay for RX beam refinement in the TCI state switch delay.

Proposal 2: for TCI state switch to a predicted beam, no need to consider the delay for RX beam refinement in the TCI state switch delay with following conditions:

- if the predicted Tx beam in Set A is QCL Type-D to a known Tx beam, the corresponding Rx beam is known, or

- if the predicted Tx beam in Set A is not QCL Type-D to a known Tx beam, and UE is capable of knowing RX beam after beam prediction

Agreement in RAN4 #115meeting Issue 2-1: Metrics/KPIs for beam ID prediction - Metrics/KPIs for beam ID prediction, at least for the case if only beam ID is reported: The successful rate for the correct prediction, - The correct prediction is considered as maximum ground-truth RSRP among top-K predicted beams larger than or equal to the ground-truth RSRP of the strongest genie-aided beam(s) – x dB, - FFS on top-N (N</=K) predicted beams have to be considered in the success rate evaluation - FFS on N values - FFS on x values Note: if x=0 and N=1, it means Top-K/1 (%) : the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams"

According to RAN1 agreements on the performance moniroting metric for UE-sided AI/ML model for beam management, the Top 1 or Top K beam prediction accuracy is defined as: at least one of the Top M beam(s) of the resource set(s) for monitoring is among Top-K predicted beam(s) of Set A. M is configured by NW in CSI report configuration for monitoring. The candidate value of M is 1 or 2.

RAN1 Agreement in RAN1#120bis For UE-sided AI/ML model for beam management, for Option 2 (UE-assisted performance monitoring), the performance metric of Top 1 or Top K beam prediction accuracy is defined as: - At least one of the Top M beam(s) of the resource set(s) for monitoring is among Top-K predicted beam(s) of Set A (e.g., linked to at least one of the Top-K predicted beam(s) of Set A based on certain rule or signalling) - Where K is the number of predicted beam(s) in the corresponding inference report per time instance - Where Top M beam(s) is the best M beam(s) based on L1-RSRP measurements of the resource set(s) for monitoring - M is configured by NW in CSI report configuration for monitoring - M= 1, 2 - FFS: detailed rule or signalling

Even though RAN1 agreements are for monitoring metric, while RAN4 current discussion focus on inference metric. But both are for performance metric, the RAN1 agreements can be considered in RAN4 discussion.

The reason to introduce x in the definition of correct prediction is to allow UE to pass a test if UE’s top-1 predicted beam is adjacent to the top-1 strongest beam, since UE can not distinguish which is the best beam index due to measurement error. For example, when TE configures the setB and setA beams to be too close to each other and the actual measured RSRP of topK beams of setA lies within the range of UE’ s measurement error, UE will not be able to pass the test without x.

RAN1 agreements on Top 1 or Top K beam prediction accuracy is: at least one of the Top M beam(s) of the resource set(s) for monitoring is among Top-K predicted beam(s) of Set A. In our view, the reason to introduce M in RAN1 is also to sovle the issue that UE may not distinguish which is the best beam index due to measurement error. But RAN1 sovle this issue in another way. RAN1 definiton is similar as Top K/M (%), the percentage of "any one of the Top-M genie-aided beam is one of the Top-K predicted beams".

In RAN4 definition of successful rate for the correct prediction, one FFS is whether top-N (N</=K) predicted beams have to be considered in the success rate evaluation. For N, there are two interpretentation. One interpretention is that N is uesd to refer a subset of K, which means the maximum ground-truth RSRP among is not selected from top-K predicted beams, but selected from top-N predicted beams, N<K. The other interpretention is that N refer to the the N of Top K/N(%). According to the note that if x=0 and N=1, it means Top-K/1 (%) : the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams", it seems that the latter interpretention is correct. With the understanding that N refer to the the N of Top K/N(%), we do not think both N>1 and x of non-zero value need to be considered together, since considering both will be double relax the requirements. If x of non-zero value, N is 1. If x is zero, N is larger than 1.

Proposal 3: for Metrics/KPIs for beam ID prediction, for the FFS on top-N (N</=K) predicted beams have to be considered in the success rate evaluation, it is proposed that whether N value can be larger than 1 is related with x value.

- Either x =0 and N>1, or x of non-zero value and N=1 are OK;

- It is not necessary that N>1 and x of non-zero value are considered together

In RAN4 #114 meeting, it was agreed that if RSRP and beam ID are reported, RSRP accuracy and Beam ID prediction accuracy will be the KPIs. We think the successful rate for the correct prediction is also applied for the case that both RSRP and beam ID are reported. Some companies argue that with RSRP accuracy requirements, no need to apply beam ID prediction accuracy requirements. In our view, it depends on how to define the RSRP accuracy requirements. When the RSRP requirements are defined very tight, if UE pass the RSRP accuracy test, it is reasonable to assume that the beam prediction performance can also be guaranteed. In this case. No need to apply beam ID prediction accuracy. However, when the RSRP requirements are defined not very tight, e.g. with large margin, even if UE pass the RSRP accuracy test, the beam prediction performance cannot be guaranteed. In this case, beam ID prediction accuracy requirements need to be applied. Normally, a certain margin will be considered in the RSRP accuracy requirements. With above consdieration, it is preferred that the successful rate for the correct prediction is also applied for the case that both RSRP and beam ID are reported.

Proposal 4: for the case that both RSRP and beam ID are reported, it is proposed that both RSRP accuracy requirements and successful rate for the correct prediction are applied.

Issue 2-2: Absolute RSRP accuracy Agreement: - Absolute RSRP accuracy requirement applies to Top-1 of predicted beams at least - FFS whether to apply to other beams Issue 2-3: Relative RSRP accuracy Agreement: - RAN4 to use the follow as baseline for further discussions, if the relative RSRP accuracy requirement is defined - Relative RSRP accuracy for reported beams during inference reporting = (predicted L1-RSRP of beam index i - reported L1-RSRP of beam index n) - (ground truth of L1-RSRP of beam index i - ground truth of L1-RSRP of beam index n), [where the beam index n owns the largest reported value]. - Reported L1-RSRP can be predicted, - FFS whether reported L1-RSRP can be measured RSRP - Relative RSRP accuracy requirements apply for Case 1, and FFS on whether it can apply for Case 2

According to RAN1 agreements, it can be seen that only largest predicted RSRP is reported as absolute RSRP, other RSRP are reported as differential predicted RSRP with reference to the largest predicted RSRP. With this report mechanism, it is reasonable that absolute RSRP accuracy requirement applies to Top-1 of predicted beams. And relative accuracy requirements are applied to other beams.

RAN1 agreements in RAN1#120bis Agreement For UE-sided model inference, support the following report format (i.e., CSI field mapping order) for BM-Case1, for beam information on predicted Top K beam(s) among a set of beams and RSRP of predicted Top K beam(s) among a set of beams CRI or SSBRI #1. CRI or SSBRI #2. …. CRI or SSBRI #K. RSRP #1. differential RSRP #2. …. differential RSRP #K. - CRI or SSBRI #k is mapped to RSRP #k, where k = 1,2,…,K - RSRP #1 is absolute predicted RSRP - Differential RSRP #2~#K are differential predicted RSRP with reference to the largest predicted RSRP corresponding to CRI or SSBRI #1 Agreement For UE-sided model inference, support the following report format (i.e., CSI field mapping order) for BM-Case2, for beam information on predicted Top K beam(s) among a set of beams and RSRP of predicted Top K beam(s) among a set of beams Time instance indicator. CRI or SSBRI #1 of time instance #1. CRI or SSBRI #2 of time instance #1. …. CRI or SSBRI #K of time instance #1. CRI or SSBRI #1 of time instance #2. CRI or SSBRI #2 of time instance #2. …. CRI or SSBRI #K of time instance #2. …. CRI or SSBRI #1 of time instance #N. CRI or SSBRI #2 of time instance #N. …. CRI or SSBRI #K of time instance #N. RSRP #1 of time instance #1. Differential RSRP #2 of time instance #1. …. Differential RSRP #K of time instance #1. Differential RSRP #1 of time instance #2. Differential RSRP #2 of time instance #2. …. Differential RSRP #K of time instance #2. …. Differential RSRP #1 of time instance #N. Differential RSRP #2 of time instance #N. …. Differential RSRP #K of time instance #N. - Time instance indicator exist if N > 1 - The size of CSI field for time instance indicator is log2N - The value of time instance indicator n (n≥0) corresponds to the (n+1)-th earliest time instance in the N time instances - Time instance #1 corresponds to the time instance indicated by the time instance indicator - Time instance #2~#N are mapped to the remaining N-1 time instance(s) other than time instance #1 based on the time domain order of the time instances - where time instance #2 is mapped to the earliest time instance from the N-1 time instance(s) - CRI or SSBRI #k is mapped to RSRP #k with the same time instance, where k = 1,2,…,K - RSRP #1 of time instance #1 is absolute RSRP; and the remaining RSRP are differential RSRP with reference to the largest predicted RSRP corresponding to CRI or SSBRI #1 of time instance #1

Proposal 5: it is proposed that absolute RSRP accuracy requirement only applies to Top-1 of predicted beams. And relative accuracy requirements applies to other reported predicted beams.

One FFS for relative RSRP accuracy for case 1 is whether reported L1-RSRP can be measured RSRP. RAN1 agreed that for report content of inference results for UE-sided model for BM-Case 1, the RSRP of predicted beam(s)in the report of inference results, is the predicted RSRP, where the predicted RSRP is based on AI/ML output. It is clear that measured RSRP are not allowed to report.

RAN1 agreements in RAN1#120 Agreement For report content of inference results for UE-sided model for BM-Case 1, the RSRP of predicted beam(s)in the report of inference results, is the predicted RSRP, where the predicted RSRP is based on AI/ML output. Note: how to capture it in the spec is a separate discussion.

Proposal 6: for relative RSRP accuracy, it is proposed that the reported L1-RSRP cannot be measured RSRP, since RAN1 agreed that the RSRP of predicted beam(s)in the report of inference results is the predicted RSRP which is based on AI/ML output.

Another FFS is whether beam index n owns the largest reported value. As discussed above, RAN1 agree that only largest predicted RSRP is reported as absolute RSRP, other RSRP are reported as differential predicted RSRP with reference to the largest predicted RSRP. With the agreements, it is reasonable that beam index n owns the largest reported value.

Proposal 7: for relative RSRP accuracy, it is proposed that beam index n owns the largest reported value.

Based on above consideration, for case 1, the relative RSRP accuracy is proposed as (predicted L1-RSRP of beam index i - predicted L1-RSRP of beam index n) - (ground truth of L1-RSRP of beam index i - ground truth of L1-RSRP of beam index n), where the beam index n owns the largest reported value.

Proposal 8: for BM case 1, it is proposed that

- The relative RSRP accuracy for reported beams during inference reporting = (predicted L1-RSRP of beam index i - predicted L1-RSRP of beam index n) - (ground truth of L1-RSRP of beam index i - ground truth of L1-RSRP of beam index n), [where the beam index n owns the largest reported value]

BM case 2 is temporal downlink beam prediction for Set A of beams based on the historic measurement results of Set B of beams. The model output is the L1-RSRP prediction of furture N time instances with K beams for each time instances. According to RAN1 agreements on the inference report for BM case2, RSRP #1 of time instance #1 is absolute RSRP; and the remaining RSRP are differential RSRP with reference to the largest predicted RSRP corresponding to CRI or SSBRI #1 of time instance #1. Time instance #1 corresponds to the time instance indicated by the time instance indicator. Since multiple time instances are consdiered, the relative accuracy for BM case2 need to be updated to reflect different time instance. Based on above consideration, the relative accuracy for BM case2 is proposed as (predicted L1-RSRP of beam index i for time instance m - predicted L1-RSRP of beam index n) - (ground truth of L1-RSRP of beam index i for time instance m - ground truth of L1-RSRP of beam index n), where the beam index n owns the largest reported value, 1<=m<=M where M is the number of time instance.

Proposal 9: for BM case 2, it is proposed that

- The relative RSRP accuracy for reported beams during inference reporting = (predicted L1-RSRP of beam index i for time instance m - predicted L1-RSRP of beam index n) - (ground truth of L1-RSRP of beam index i for time instance m - ground truth of L1-RSRP of beam index n), where the beam index n owns the largest reported value among all the predicted beams. 1<=m<=M where M is the number of time instance

- Conclusion

This contribution provides discussion on RRM requirements for beam management. The proposals are:

Proposal 1: it is proposed that the conditions for known TCI state is updated as following to cover AI/ML based beam prediction. And the updated parts are applied to both case 1 and case 2.

The TCI state is known if the following conditions are met: - During the period from the last transmission of the RS resource used for the L1-RSRP measurement reporting for the target TCI state to the completion of active TCI state switch, where the RS resource for L1-RSRP measurement is the RS in target TCI state or QCLed to the target TCI state, or - TCI state switch command is received within 1280 ms upon the last transmission of the RS resource for beam reporting or measurement - The UE has sent at least 1 L1-RSRP report for the target TCI state before the TCI state switch command - The TCI state remains detectable during the TCI state switching period - The SSB associated with the TCI state remain detectable during the TCI switching period - SNR of the TCI state ≥ -3 dB - TCI state switch command is received within T ms upon the report of predicted results. - If the predicted Tx beam in Set A is QCL Type-D to a known Tx beam - If the predicted Tx beam in Set A is not QCL Type-D to a known Tx beam, and UE is capable of knowing RX beam after beam prediction Note: the value of T is FFS. The value of T for case 1 and case 2 can be different.

Proposal 2: for TCI state switch to a predicted beam, no need to consider the delay for RX beam refinement in the TCI state switch delay with following conditions:

- if the predicted Tx beam in Set A is QCL Type-D to a known Tx beam, the corresponding Rx beam is known, or

- if the predicted Tx beam in Set A is not QCL Type-D to a known Tx beam, and UE is capable of knowing RX beam after beam prediction

Proposal 3: for Metrics/KPIs for beam ID prediction, for the FFS on top-N (N</=K) predicted beams have to be considered in the success rate evaluation, it is proposed that whether N value can be larger than 1 is related with x value.

- Either x =0 and N>1, or x of non-zero value and N=1 are OK;

- It is not necessary that N>1 and x of non-zero value are considered together

Proposal 4: for the case that both RSRP and beam ID are reported, it is proposed that both RSRP accuracy requirements and successful rate for the correct prediction are applied.

Proposal 5: it is proposed that absolute RSRP accuracy requirement only applies to Top-1 of predicted beams. And relative accuracy requirements applies to other reported predicted beams.

Proposal 6: for relative RSRP accuracy, it is proposed that the reported L1-RSRP cannot be measured RSRP, since RAN1 agreed that the RSRP of predicted beam(s)in the report of inference results is the predicted RSRP which is based on AI/ML output.

Proposal 7: for relative RSRP accuracy, it is proposed that beam index n owns the largest reported value.

Proposal 8: for BM case 1, it is proposed that

- The relative RSRP accuracy for reported beams during inference reporting = (predicted L1-RSRP of beam index i - predicted L1-RSRP of beam index n) - (ground truth of L1-RSRP of beam index i - ground truth of L1-RSRP of beam index n), [where the beam index n owns the largest reported value]

Proposal 9: for BM case 2, it is proposed that

- The relative RSRP accuracy for reported beams during inference reporting = (predicted L1-RSRP of beam index i for time instance m - predicted L1-RSRP of beam index n) - (ground truth of L1-RSRP of beam index i for time instance m - ground truth of L1-RSRP of beam index n), where the beam index n owns the largest reported value among all the predicted beams. 1<=m<=M where M is the number of time instance

- Reference

- RP-234039, New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- RP-243244, Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- R4-2508080, WF on NR_AIML_air_part1, Qualcomm Incorporated

- R4-2505105, WF on requirements for AI/ML air interface normative work, Qualcomm Incorporated

- R4-2502856, WF on the requirements for AI/ML air interfance, Qualcomm Incorporated

- R4-2420513, WF on AI/ML air interface part 1, Qualcomm Incorporated

- R4-2417212, WF on requirements for AI/ML air interface, Qualcomm Incorporated

- R4-2414323, WF on AI/ML, Qualcomm Incorporated

- R4-2410570, WF on AI/ML, Qualcomm Incorporated

- R4-2406617, WF on AI/ML, Qualcomm Incorporated

[5] 3GPP TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface