# 3GPP TSG-RAN WG4 Meeting #116                              R4-2510164

# Bengaluru, India, August 25th – 29th, 2025

Agenda Item:	7.17.5

Source: 	CMCC

Title: 			 Discussion on RRM requirements for positioning accuracy enhancement

Document for:	Discussion

- Introduction

In RAN #102 meeting, the WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface was approved [1]. In RAN #106 meeting, the revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface was approved [2]. The related objectives are duplicated as following.

- Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]: - Direct AI/ML positioning: - (1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning - (2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning - (1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning - AI/ML assisted positioning - (2nd priority) Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning - (1st priority) Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning - Specify necessary measurements, signalling/mechanism(s) to facilitate LCM operations specific to the Positioning accuracy enhancements use cases, if any - Investigate and specify the necessary signalling of necessary measurement enhancements (if any) - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE for relevant positioning sub use cases - Note: RAN1 will not discuss any proposals targeting 2nd priority issues in Q4 of 2024. - Core requirements for the above two use cases for AI/ML LCM procedures and UE features [RAN4]: - Specify necessary RAN4 core requirements for the above three use cases. This includes the test setup and methodology for the beam management use case. - Specify necessary RAN4 core requirements for LCM procedures including performance monitoring.

In last meeting, there is discussion on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, and a WF was approved [3]. This contribution provides discussion on RRM core requirements for positioning accuracy enhancement.

- Discussion

Issue 3-1: Requirements for case 1 Agreement: - Discuss how to define a measurement delay requirement(reporting delay) - Do not define any positioning accuracy requirement

For beam management, it was agreed in RAN4 #113 meeting that prediction delay includes measurement for prediction, inference delay, and FFS for other components. Similarly, prediction delay need to be defined for case 1 when LMF request the results of the UE sided model inference operation. The prediction delay at least include PRS measurement for prediction, inference delay. As for the report delay, whether and how to consider it can be further discussed.

Apart from inference delay, monitoring delay can also be considered. For model performance monitoring of AI/ML positioning Case 1, RAN1 agreed that target UE side performs monitoring metric calculation is supported, and the target UE may signal the monitoring outcome to the LMF. Whether monitoring related requirements need to be defined can be further discussed.

RAN1 agreements in RAN1#119 meeting Agreement For model performance monitoring of AI/ML positioning Case 1, support at least: - Option A. The target UE side performs monitoring metric calculation. - The target UE may signal the monitoring outcome to the LMF. - FFS: content of monitoring outcome - FFS: Option B RAN1 agreements in RAN1#120bis meeting Agreement For model performance monitoring of AI/ML positioning Case 1, “FFS: content of monitoring outcome” in RAN1#119 agreement is resolved by: - the content of monitoring outcome includes at least an indication that the target UE cannot perform the Case 1 positioning method.

For LCM, RAN2 agreed that for UE-sided model, for the functionality management, the “network decision, network-initiated” AI/ML management is supported as a baseline. It means that there is interaction between network and UE, e.g. activation, deactivation, etc.

RAN2 agreements in RAN2#129bis meeting Agreements for positioning case 1 - LMF is responsible for functionality management - UE reports the applicable functionality to the LMF by the LPP provide capabilities message without any additional LMF control. - Switching/fallback to non-AI/ML positioning can be supported by including multiple positioning methods in a LPP Request Location Information message. No additional specification work is foreseen specifically for supporting "switching/fallback operation". - An AIML positioning functionality is considered “activated” once UE receives an LPP RequestLocationInformation from the LMF requesting inferred location information. - For triggered and periodical reporting, we rely on existing positioning framework mechanisms to deactivate AI/ML positioning (no spec impact is foreseen) RAN2 agreements in RAN2#127 meeting - Supported functionalities refer to functionalities that UE can indicate by using UE capability information (via RRC/LPP signalling) - Applicable functionalities refers to functionalities that the UE is ready to apply for inference - Activated functionalities refers to functionalities already enabled for performing inference

In last meeting, it was agreed to define activation delay requirements for the periodic, semi-persistent and aperiodic CSI reporting for beam prediction. Since RAN2 agreed that an AIML positioning functionality is considered “activated” once UE receives an LPP RequestLocationInformation from the LMF requesting inferred location information. It can be considered to define activation delay for case 1.

Proposal 1: for case 1, it is proposed to define delay related requirements, e.g. prediction delay, LCM related delay.

Issue 3-2: Report mapping for sample based timing measurement reporting Agreement: - Use the value of the legacy UL RTOA mapping for 1st sample - FFS for the additional sample reporting Issue 3-3: Requirements for case 3b Agreement: - Reuse legacy path based Rx-Tx accuracy requirements for case 3b - Requirement applies only for the 1st path

Case 3a is NG-RAN node assisted positioning with gNB-side model (AI/ML assisted positioning). For case 3a/3b, the agreements reached in RAN4 previous meetings are summarized in following table.

Agreements in RAN4#112bis (R4-2417212) Issue 3-3: Report applicability for existing reported metrics Agreement: Extend applicability of existing report mappings for UL-RToA and UL SRS-RSRPP to report UL measurements for AI/ML based positioning use cases 3a/3b. Agreements in RAN4#114 (R4-2502856) Issue 3-4: Report mapping for UL SRS-RSRP Agreement: - Reuse the report mapping for UL SRS-RSRP to Case 3a Agreements in RAN4#114bis (R4-2505105) Issue 3-3: Rx-Tx time difference for case 3a Agreement: - Introduce gNB Rx-Tx time difference reporting accuracy requirements - Reuse legacy accuracy requirements Issue 3-4: Report mapping for gNB Rx-Tx time difference Agreement: - Reuse the existing report mapping for gNB Rx-Tx time difference measurements Agreements in RAN4#115 (R4-2508080) Issue 3-3: Requirements for case 3b Agreement: - Reuse legacy path based Rx-Tx accuracy requirements for case 3b - Requirement applies only for the 1st path

According to current TS38.133, measurement performance requirements for NR gNB include report mapping for UL-RTOA, report mapping for gNB Rx-Tx time difference, measurement accuracy requirements for gNB Rx-Tx time difference, report mapping for UL SRS RSRP, measurement accuracy requirements for UL SRS RSRP, report mapping for UL SRS RSRPP. It can be seen that RAN4 agreed to reuse the report mapping for UL SRS-RSRP to Case 3a, but measurement accuracy requirements for UL SRS RSRP is missing for case 3a. It is proposed that to reuse legacy UL SRS RSRP measurement accuracy requirements for case 3a.

Proposal 2: It is proposed to reuse legacy UL SRS RSRP measurement accuracy requirements for case 3a.

Case 3b is NG-RAN node assisted positioning with LMF-side model (direct AI/ML positioning). The model input is the gNB channel measurements reported to LMF. And our view is that legacy positioning related measurement requirements are still applied for the model input.

In RAN4 #112bis, it was agreed that existing report mappings for UL-RToA and UL SRS-RSRPP are reused for AI/ML based positioning use cases 3b. In last meeting, it was agreed to reuse legacy path based Rx-Tx accuracy requirements for case 3b. Similarly, legacy report mapping for UL SRS-RSRP, legacy report mapping for gNB Rx-Tx time difference, and legacy UL SRS RSRP measurement accuracy requirements are reused for case 3b.

Proposal 3: same as case 3a, it is proposed that legacy report mapping for UL SRS-RSRP, legacy report mapping for gNB Rx-Tx time difference, and legacy UL SRS RSRP measurement accuracy requirements are reused for case 3b.

- Conclusion

This contribution provides discussion on RRM requirements for positioning. The proposals are:

Proposal 1: for case 1, it is proposed to define delay related requirements, e.g. prediction delay, LCM related delay.

Proposal 2: It is proposed to reuse legacy UL SRS RSRP measurement accuracy requirements for case 3a.

Proposal 3: same as case 3a, it is proposed that legacy report mapping for UL SRS-RSRP, legacy report mapping for gNB Rx-Tx time difference, and legacy UL SRS RSRP measurement accuracy requirements are reused for case 3b.

- Reference

- RP-234039, New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- RP-243244, Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- R4-2508080, WF on NR_AIML_air_part1, Qualcomm Incorporated

- R4-2505105, WF on requirements for AI/ML air interface normative work, Qualcomm Incorporated

- R4-2502856, WF on the requirements for AI/ML air interfance, Qualcomm Incorporated

- R4-2420336, WF on AI/ML air interface part 2, Ericsson

- R4-2417212, WF on requirements for AI/ML air interface, Qualcomm Incorporated

- R4-2414323, WF on AI/ML, Qualcomm Incorporated

- R4-2410570, WF on AI/ML, Qualcomm Incorporated

- R4-2406617, WF on AI/ML, Qualcomm Incorporated

- 3GPP TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface

- R1-2405389, Summary #5 of specification support for positioning accuracy enhancement

- R4-2501758, On requirements for AI/ML based positioning, Ericsson