# 3GPP TSG-RAN WG4 Meeting #116                              R4-2510166

# Bengaluru, India, August 25th – 29th, 2025

Agenda Item:	7.18.3

Source: 	CMCC

Title: 			 Discussion on testing issues for CSI compression two-sided models

Document for:	Discussion

- Introduction

In RAN #106 meeting, a new SID on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface Phase 2 was approved [1], and the objectives are same as the study aspects that were included in the REL-19 NR_AIML_air WI [2]. The objectives for RAN4 are duplicated as following.

- Testability and interoperability [RAN4]: - Finalize the testing framework and procedure for one-sided models and further analyse the various testing options for two-sided models, in collaboration with RAN1, and including at least: - Relation to legacy requirements - Performance monitoring and LCM aspects considering use-case specifics - Generalization aspects - Static/non-static scenarios/conditions and propagation conditions for testing (e.g., CDL, field data, etc.) - UE processing capability and limitations - Post-deployment validation due to model change/drift

In last meeting, there is discussion on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, and a WF was approved [3]. This contribution provides discussion on testing issues for CSI compression two-sided models.

- Discussion

According to previous discussion, for option 3 track 2, the endorsed observations are as follows:

- There is reasonable SGCS convergence when companies jointly train an encoder-decoder pair using the mixed dataset and test using the mixed testset - Around +-2% variation in SGCS; see results spreadsheet for more details - It is possible for UE vendors to train an encoder based on a frozen test decoder that is functional - When training “own” encoder with the frozen decoder, with all training (frozen decoder and encoder) and testing using the mixed dataset, there is good convergence in average SGCS results - Around - 2% to +1.4% variation in SGCS from the average - When training “own” encoder with the frozen decoder, with training of the frozen decoder using the mixed dataset, own encoder using own dataset and testing using the mixed dataset, there is convergence in average SGCS results from results presented so far - Around -+-4% variation in SGCS from the average (+-1.5% with outlier performance excluded) - When training “own” encoder with the frozen decoder, with training of the frozen decoder using the mixed dataset, own encoder using own dataset and testing using the own dataset, there is much less convergence in average SGCS results from results presented so far - Around -40% to 13% variation in SGCS from the average (+-8% to 5% with outlier performance excluded) - The results considering “own” datasets demonstrate that there is divergence in the datasets, and attention must be paid to alignment of the training datasets. - Note: Alignment of datasets does not refer to aligning UE algorithms such as channel estimation, CSI estimation etc.

According to the agreed WF, in order to further elaborate option 3 track 2 considering lower complexity encoders, interested companies may contribute simulations to RAN4#116. The following guidance, and also the guidance in R4-2508084 is provided and recommended for companies to follow in order to obtained aligned simulations. For option 3, track 2 consider the following investigations for lower complexity encoder structures.

- Performance assuming training and testing with the Samsung frozen decoder

- Performance achieved with a jointly trained low complexity encoder and decoder

- Performance achieved when a frozen decoder is trained based on all of the encoder structures (i.e. different encoder complexity)

- Consider transformer, CNN, MLP

Simulation are performed for option 3 track 2 based on the agreed simulation assumption on AI/ML CSI compression [4]. Following SCGS-0, SCGS-1, SCGS-2, SCGS-3 are simulated. Encoder 1 and encoder 3 are CNN based encoder while encoder 3 is lower-complexity. Encoder 2 is transformer based lower-complexity encoder. Encoder 4 is MLP based lower-complexity encoder.

Our simulation results are summarized as following Table 1.

Table 1: simulation results for option 3 track 2

|  |  | （1-to-1）Joint training | New encoder training with (Samsung’s) frozen decoder | （N-to-1）Joint training SGCS-4 Step-1 | New encoder training with frozen decoder from Step-1 SGCS-4 Step-2 |
| --- | --- | --- | --- | --- | --- |
| Encoder-1 | Companies' name | SGCS-0 | SGCS-1 | SGCS-4 (step-1) | SGCS-4 (step-2) |
|  | CMCC | 0.6839 | 0.6812 | 0.6736 | 0.6830 |
| Encoder-2 | Companies' name | SGCS-3 | SGCS-2 | SGCS-4 (step-1) | SGCS-4 (step-2) |
|  | CMCC | 0.7127 | 0.7005 | 0.6850 | 0.7069 |
| Encoder-3 | Companies' name | SGCS-3 | SGCS-2 | SGCS-4 (step-1) | SGCS-4 (step-2) |
|  | CMCC | 0.6739 | 0.6654 | 0.6640 | 0.6671 |
| Encoder-4 | Companies' name | SGCS-3 | SGCS-2 | SGCS-4 (step-1) | SGCS-4 (step-2) |
|  | CMCC | 0.6664 | 0.6353 | 0.6634 | 0.6649 |

- Conclusion

This contribution provides discussion on testability and interoperability issues for CSI compression. The proposals are:

Proposal 1: the simulation results for CSI compression are provided in Table 1.

|  |  | （1-to-1）Joint training | New encoder training with (Samsung’s) frozen decoder | （N-to-1）Joint training SGCS-4 Step-1 | New encoder training with frozen decoder from Step-1 SGCS-4 Step-2 |
| --- | --- | --- | --- | --- | --- |
| Encoder-1 | Companies' name | SGCS-0 | SGCS-1 | SGCS-4 (step-1) | SGCS-4 (step-2) |
|  | CMCC | 0.6839 | 0.6812 | 0.6736 | 0.6830 |
| Encoder-2 | Companies' name | SGCS-3 | SGCS-2 | SGCS-4 (step-1) | SGCS-4 (step-2) |
|  | CMCC | 0.7127 | 0.7005 | 0.6850 | 0.7069 |
| Encoder-3 | Companies' name | SGCS-3 | SGCS-2 | SGCS-4 (step-1) | SGCS-4 (step-2) |
|  | CMCC | 0.6739 | 0.6654 | 0.6640 | 0.6671 |
| Encoder-4 | Companies' name | SGCS-3 | SGCS-2 | SGCS-4 (step-1) | SGCS-4 (step-2) |
|  | CMCC | 0.6664 | 0.6353 | 0.6634 | 0.6649 |

- Reference

- RP-243245, New SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface Phase 2

- RP-242399, Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- R4-2508051, WF on AI/ML part 2, Ericsson

- R4-2508084, Simulation assumptions on AI/ML CSI compression

- R4-2505107, WF on AI/ML air interface study, Ericsson

- R4-2502953, WF on AI/ML air interface study, Ericsson

- R4-2420513, WF on AI/ML air interface part 1, Qualcomm Incorporated

- R4-2417212, WF on requirements for AI/ML air interface, Qualcomm Incorporated

- R4-2414323, WF on AI/ML, Qualcomm Incorporated

- R4-2410570, WF on AI/ML, Qualcomm Incorporated

- R4-2406617, WF on AI/ML, Qualcomm Incorporated

[5] 3GPP TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface