# 3GPP TSG-RAN WG4 Meeting #116                              R4-2510172

# Bengaluru, India, August 25th – 29th, 2025

Agenda Item:	7.22.7

Source: 	CMCC

Title: 			 Discussion on RRM performance requirements for A-IoT

Document for:	Discussion

- Introduction

In RAN #107 meeting, the revised WID on Solutions for Ambient IoT (Internet of Things) in NR was approved [1]. According to the WID, the objectives of RAN4 scope are duplicated as following.

- RAN4 scope: - Specify RF requirements for Ambient-IoT BS, device 1, and CW - RF requirements for Type 1-C Ambient-IoT BS - RF requirements for device 1 - RF requirements for CW - Specify RRM core requirements for device 1, if necessary - Study and develop OTA test methodology for A-IoT device 1 - Consider test methods specified in TR 38.870 as starting point. Take test system reuse, test system complexity and test time into account, when developing test methods suitable for Ambient IoT. - Develop the preliminary Measurement Uncertainty (MU) assessment for the test system - Use band n8 as an example band

In last meeting, there is discussion on RRM core requirements for A-IoT, and a WF was approved [2]. This meeting is the first meeting to discuss RRM performance requirements for device 1. This contribution provides initial discussion on this topic.

- Discussion

Rel-19 A-IoT focus on D1T1-B (Deployment scenario 1 with Topology 1 ), and Ambient IoT device 1 is considered. Deployment scenario 1: Device indoors, basestation indoors. In Topology 1, the Ambient IoT device directly and bidirectionally communicates with a basestation. The communication between the basestation and the ambient IoT device includes Ambient IoT data and/or signalling. For D1T1-B, CW is outside topology.

According to the scope of the WID [1], only A-IoT Paging, A-IoT Random access and A-IoT data transmission are considered. RAN4 discuss RRM impact in the core part discussion and the agreements are summarized as following.

Agreements in RAN4 #114bis meeting (R4-2504905) Issue 1-1: A-IoT paging Agreement: - RAN4 not to define RRM requirements for A-IoT paging Issue 1-2: A-IoT data transmission Agreement: - Excepting the timing issue discussed in Issue 1-4, no other potential RRM requirement impact is identified for “A-IoT data transmission” Issue 1-3: A-IoT random access procedure Agreement: - RAN4 to include RRM requirements for correct A-IoT device behaviour during CBRA and CFRA. FFS the details Agreements in RAN4 #115 meeting (R4-2508275) Issue 1-1: A-IoT random access procedure Agreement: - RAN4 to include following aspects for RRM requirements for A-IoT random access. The details of UE behaviour will be updated based on RAN2 spec. - CBRA procedure - the correct behaviour when transmitting A-IoT MSG1(Random ID transmission) - the correct behaviour when receiving A-IoT MSG2 - the correct behaviour when not receiving A-IoT MSG2 - CFRA procedure - the correct behaviour when transmitting First D2R message Issue 1-2-1: Device transmit timing Agreement: - RAN4 define D2R transmit timing requirements. - Define D2R transmit timing error requirements as follows: - For MSG1 X=1, - The reference point for the device transmission timing is Toffset1 after the end of corresponding R2D transmission as define in TS 38.219. The device transmission timing error shall be less than or equal to Te. The value of Te is equal to 10%*TSFO, where TSFO is the length in microseconds from the last edge of the corresponding R2D transmission to the starting time of MSG1. - For MSG1 X=2, follow the same principle as starting point. - Note 1: The timing error requirements for other D2R could be updated accordingly based on further RAN1 agreements. - Note 2: The end of the R2D transmission and last edge could be updated based on further RAN1 agreements if any.

It can be seen the RRM requirements for correct A-IoT device behaviour during CBRA and CFRA, and requirements for device transmit timing are defined. It is necessary to define tests for these two cases.

Proposal 1: it is proposed to define tests for following cases:

- Correct A-IoT device behaviour during CBRA and CFRA

- Device transmit timing

For legacy UE transmit timing test, the transmit timing is verified by the UE transmitting SRS. After connection set up with the cell, the test equipment will verify that the timing of the NR cell is within (NTA + NTA_offset) ×Tc ± Te of the first detected path of DL SSB. Similarly, for A-IoT, the transmit timing is verified by device D2R transmission. Taking RACH as an example, the test equipment will verify that the timing of the device is within  a period, e.g. (Toffset1 ± Te_SFO),  of the end the corresponding R2D transmission triggering random access.

Proposal 2: For RACH, the transmit timing is verified by the device D2R transmission, and the test equipment will verify that the timing of the device is within a period of the end the corresponding R2D transmission triggering random access, where the period is related with Toffset1 /Toffset2/Toffset3/Toffset4 and timing error due to SFO.

- Conclusion

This contribution provides discussion on RRM requirements for A-IoT. The proposals are:

Proposal 1: it is proposed to define tests for following cases:

- Correct A-IoT device behaviour during CBRA and CFRA

- Device transmit timing

Proposal 2: For RACH, the transmit timing is verified by the device D2R transmission, and the test equipment will verify that the timing of the device is within a period of the end the corresponding R2D transmission triggering random access, where the period is related with Toffset1 /Toffset2/Toffset3/Toffset4 and timing error due to SFO.

- Reference

- RP-250796, Revised WID: New Work Item: Solutions for Ambient IoT (Internet of Things) in NR, CMCC, Huawei, T-Mobile USA Inc

- R4-2508275, WF on RRM requirements for Ambient_IoT_Solutions, CMCC

- R4-2504905, WF on RRM requirements for Ambient_IoT_Solutions, CMCC