3GPP TSG-RAN WG4 Meeting #116	R4-2510187

Bengaluru, India, 25 August – 29 August, 2025

Source:	vivo

Agenda item:			7.14.3.1

Title:	Discussion on remaining issues on FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor

Document for:	Discussion

- Introduction

In the RAN#105 meeting, the revised WID on enhancements for NR and MR-DC RRM requirements was approved [1]. The objectives for RAN4 in the WID are as following

- FR2-1 SSB based L3 measurement delay reduction for connected mode - For UE supporting multiple-Rx simultaneous reception on single carrier: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - Rx beam sweeping factor - The CSSF requirement optimization is defined assuming a UE not operating with multiple-Rx based simultaneous L3 measurement: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - CSSF outside gap in CA/DC scenarios - Baseline assumption on number of searchers is 2 - Note: Rel-19 CSSF optimization applies for CA/DC scenario independently of the UE support of multi-Rx capabilities. - Fast SCell activation for UE supporting Rel-18 EMR - Study and, if feasible, to reduce the SCell activation delay with valid EMR reporting - Apply fast scell activation in FR1 and FR2-1 - Note: RAN4 to start this work from Q3’2024 and aim for completion in Dec’2024. Workplan for this bullet can be discussed in May’2024

In the last RAN4 meeting, the topic of FR2-1 SSB based L3 measurement delay reduction for connected model is discussed and the way forwards are captured in [2]. In this contribution, we provided our views on FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor.

- Discussion

At RAN4 114bis meeting, the following agreements are available:

Issue 1-1-1: Conditions to trigger UE to quit “L3 measurement delay reduction by optimizing Rx BSF”

Agreement:

For the quitting condition of FBS:

- Introduce a new UAI indicating “overheating” issue:

  - RAN4 will not discuss the UE behaviour after reporting UAI

- Introduce a timer-based solution:

  - UE is allowed to fall back to normal measurement when it has performed fast measurement for at least T1 from the time point when UE enters the FBS mode after meeting the FBS entry condition

    - Further discuss the candidate numbers for T1.

Way forward:

For re-evaluation mechanism for scenario 1/2/3/4:

- Check whether the following mechanism is agreeable:

  - UE re-evaluates the activation condition after the timer expires, where the timer starts after UE quits from Multi-Rx mode.

Issue 1-1-2: Status report of fast beam sweeping

Agreement:

  - No need to define UE status report of fast beam sweeping.

Issue 1-1-3: UE capability for Rx BSF optimization

Agreement:

For FBS capability design:

  - UE capability is per-band and optional

  - TDD only

  - FR2-1 only

  - FFS: in the same FG, whether to define separate UE capability for (intra-frequency) and (intra-frequency + inter-frequency)

In this contribution we discuss the following open issues:

- Sub-topic 1-1 Conditions to apply L3 measurement delay reduction by optimizing Rx BSF

In the last meeting, following issues are captured in the WF

Issue 1-1-1: Conditions to trigger UE to quit “L3 measurement delay reduction by optimizing Rx BSF”

Way forward:

For re-evaluation mechanism for scenario 1/2/3/4:

- Check whether the following mechanism is agreeable:

  - UE re-evaluates the activation condition after the timer expires, where the timer starts after UE quits from Multi-Rx mode.

For the re-evaluation mechanism, if the UE leave FBS mode because the measurement quality is less than the threshold, it is expected that the measurement quality will not exceed the threshold in a short period hence even without any timer, the UE will not re-enter the FBS mode immediately. If the UE leave FBS mode due to expiry of a timer or overheating, then it is hard to define a timer since when that UE has the will to re-enter the FBS mode is difficult to be known. Hence it is suggested that for re-evaluation mechanism for scenario 1/2/3/4, it is up to UE implementation and do not need introduce any new timer.

Proposal 1: For re-evaluation mechanism for scenario 1/2/3/4 after a UE leaves FBS mode, it is up to UE implementation and do not need introduce any new timer.

- Summary

In this contribution, we provided our views on FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor. Based on above analysis, following proposals are present.

Proposal 1: For re-evaluation mechanism for scenario 1/2/3/4 after a UE leaves FBS mode, it is up to UE implementation and do not need introduce any new timer.

- References

- RP-242380, Revised WID: NR Radio Resource Management (RRM) Phase 5, RANP 105

- R4-2508266, WF on RRM requirements for NR_RRM_Ph5_Part2, RAN4 115, CATT