3GPP TSG-RAN WG4 Meeting #116	R4-2510188

Bengaluru, India, 25 August – 29 August, 2025

Source: 	vivo

Title: 	Discussion on remaining RRM requirements for UE-to-UE CLI handling

Agenda Item:			7.21.4.1

Document for:	   Discussion

- Introduction

In RAN#102, a new R19 WI on Evolution of NR duplex operation: Sub-band full duplex (SBFD) was approved, and the revised WID at RAN#104 can be found in [1].

The updated objective in [1] which are related to RRM are copied below for reference:

- Specify enhancements for CLI handling [RAN1, RAN2, RAN3, RAN4]

  - Information exchange among gNBs, including [RAN3]

    - Semi-static cell-specific SBFD time and frequency location configuration

    - Measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS

    - Strongest DL beam information

    - CLI-mitigation request

  - UL resource muting for PUSCH, including [RAN1, RAN2, RAN4]

    - Indication/determination of UL resource muting for PUSCH based on semi-static configuration, assuming comb-2 for both DFT-S-OFDM and CP-OFDM in each allocated PRB and up to 2 symbols in time domain

    - PUSCH resource mapping, i.e., rate-matching around the muted REs

    - UCI resource determination in symbols with muted REs

  - L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, including [RAN1, RAN2, RAN4] (Reuse major principles)

    - Periodic, semi-persistent, or aperiodic measurement resource (set), i.e., SRS-RSRP resource or CLI-RSSI resource, and configuration/determination of ‘typeD’ QCL assumptions for the CLI measurement resource

    - At least aperiodic reporting

    - New report quantities, e.g., L1-SRS-RSRP, L1-CLI-RSSI and/or measurement resource indices

    - UCI bits generation

    - Priority rules for multiple CSI reporting

    - CLI measurement accuracy requirement

- Specify applicable RRM core requirements for CLI handling mechanisms [RAN4]

- Specify other RRM core requirements for SBFD operation, if identified [RAN4]

In this contribution, we provide our considerations on RRM requirements for SBFD based on [2] and [3].

- Discussion

- Sub-topic 1-1: General issues

At previous meeting, the following agreements are achieved based on [2].

Issue 1-1-1: Requirement applicability

- Agreement (Monday online session)

  - For method #2, 3 and non-basic UE operation of method #1

    - The L1 CLI measurement requirements are applicable for RRC_CONNECTED intra-frequency:

      - when SRS-RSRP measurement resource is configured within active DL BWP

      - when CLI-RSSI measurement resource is configured within active DL BWP

  - FFS for basic UE operation for method #1

  - Note: The assumption for basic UE operation is that UE cannot receive simultaneous PDSCH reception on two DL subbands, and whether the basic UE operation exists or not is pending on RAN1 discussion.

Issue 1-1-2: SCS for performing measurement

- Agreement

  - UE shall perform L1-CLI-RSSI and L1-SRS-RSRP measurements with same SCS as the active DL BWP.

  - Note: this is same as existing CSI-RS based L1-RSRP measurement.

Issue 1-1-4: Collision between L1 CLI and L3 CLI measurements

- Agreement

  - Close the issue based on following RAN1 agreement in RAN1#121

Agreement A UE does not expect to be simultaneously configured with L3 and L1 UE-to-UE CLI measurement and reporting.

Based on the WF [2], the following agreements are available for discussion:

Issue 1-1-3: L1 CLI measurement in non-SBFD symbols

- Proposals

  - Proposal 1 (Nokia):

    - RAN4 to wait, and potentially take into consideration, RAN1 decision on whether a UE can be configured to perform L1 UE-to-UE CLI measurements in non-SBFD symbols in SBFD operation.

To our understanding RAN1 has already determined to support this functionality. When RAN4 defines requirements for L1 UE-to-UE CLI measurements in non-SBFD symbols, due to the existence of SBFD symbols, the requirements should be extended and similar to requirements for SBFD symbols, NR-U methods should be used.

Proposal 1: For requirements for L1 UE-to-UE CLI measurements in non-SBFD symbols, the measurement period should be extended due to the existence of SBFD symbols. NR-U methods should be used for the measurement period on L1 UE-to-UE CLI measurements in non-SBFD symbols.

- Sub-topic 1-3: L1-CLI-RSSI

At previous meeting, the following agreements are achieved based on [2].

Issue 1-3-2: Scheduling restriction

- Agreement (Tuesday AH session)

  - For Method #3

    - Do not define DL scheduling restriction on symbol after RSSI resource for the switching between UL subband and DL subband.

Issue 1-3-4: Nominal BW and SCS for reporting

- Agreement

  - UE shall scale the measured CLI-RSSI to report a nominal RSSI equivalent to 6RB measurement with 15 kHz SCS.

  - Note: this is same as existing report mapping for L3 CLI-RSSI as defined in clause 10.1.22.2.2.

Based on the WF [2], the following agreements are available for discussion:

Issue 1-3-1: Measurement period

- Proposals

  - Proposal 1 (HW):

    - RAN4 to confirm that the number of samples for DUD case with method #1 and resource type#2 is 1, at least for the case when UE supports DL reception in two DL subbands.

    - RAN4 to wait for RAN1 conclusion on whether the case when UE does not support DL reception in two DL subbands exists or not, and if so, discuss whether and how to define requirements.

  - Proposal 2 (ZTE):

    - For L1-CLI-RSSI measurement based on P/SPS resource, 1 sample or 3 samples is performed and up to the configuration of timeRestrictionForChannelMeasurement.

- Recommended WF

  - Discuss P2, i.e. whether to use 3 samples for measurement period for L1-CLI-RSSI when timeRestrictionForChannelMeasurement is not configured.

  - For P1, suggest to wait for RAN1 conclusion on UE capability for simultaneous DL reception in two DL subbands.

Regarding issue 1-3-1, for P2, even with the configuration timeRestrictionForChannelMeasurement it is still possible to define requirement based on 1 sample although we do not have strong view.

Proposal 2: For L1-CLI-RSSI measurement based on P/SPS resource, 1 sample or 3 sample is performed and up to the configuration of timeRestrictionForChannelMeasurement.

.

- Conclusions

In this contribution we provided our initial considerations on RRM requirements for SBFD and have the following proposals:

Proposal 1: For requirements for L1 UE-to-UE CLI measurements in non-SBFD symbols, the measurement period should be extended due to the existence of SBFD symbols. NR-U methods should be used for the measurement period on L1 UE-to-UE CLI measurements in non-SBFD symbols.

Proposal 2: For L1-CLI-RSSI measurement based on P/SPS resource, 1 sample is performed and up to the configuration of timeRestrictionForChannelMeasurement.

- Reference

[1] RP-241614, Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD), Huawei

[2] R4-2508274, WF on RRM requirements for NR_duplex_evo, Huawei, RAN4 115

[3] R4-2505568,	Topic summary for [115][219] NR_duplex_evo, Huawei, RAN4 115