3GPP TSG-RAN WG4 Meeting #116	R4-2510191

Bengaluru, India, 25 August – 29 August, 2025

Source: 	vivo

Title: 	Discussion on remaining issues for adaptation on common signal in R19 NES

Agenda Item:	7.23.3.2

Document for:	Discussion

Introduction

At RAN4 114bis, the WFs and topic summary [1], [2] are approved.

Based on all above information, we provide our views on RRM requirements for common signal or channel adaptation in R19 NES.

At RAN1 120bis meeting the following agreements are available for SSB adaptation.

Agreement

For adaptation of SSB in time-domain, for DCI 2_9-based SSB burst periodicity adaptation for an SCell,

- The DCI is scrambled a new RNTI,

  - Same search space and DCI size as that of cell DTX/DRX DCI if gNB configures both

Agreement

For DCI 2_9-based SSB burst periodicity adaptation for an SCell,

- the starting location of the information block for SSB burst periodicity indication for a SCell within the DCI format 2_9 is configured using a new RRC parameter,

- the length of the information block is given by ceil(log2(1+X)), where UE is configured with X additional SSB burst periodicities for the SCell.

Agreement

For adaptation of SSB in time-domain, UE can be configured with X (<=Xmax) additional SSB burst periodicities for an SCell.

- Xmax=2

Agreement

Separate configuration of the following parameters for the additional PRACH resources at least for 4-step RACH is supported

- CB-PreamblesPerSSB

At RAN1 121 meeting the following related agreements are available:

Agreement

For DCI 2_9-based SSB burst periodicity adaptation for an SCell for the case when cell DTX/DRX is not configured, reuse existing search space configuration parameter for DCI 2_9-based monitoring and existing DCI 2_9 size configuration parameter and update in specification that these are also applicable to SSB burst periodicity adaptation (when configured).

Agreement

Value d (in the WA from RAN1#120bis) is the number of slots for the SCS of the active DL BWP of the first serving cell in Table 11.5-1 of TS 38.213.

At RAN2 129bis meeting the following agreements related to SSB adaptation are available:

For SSB adaptation, multiple additional SMTC configurations are configured to UE via RRC, together with the mapping between SSB burst periodicity and SMTC configuration.

Upon reception of DCI format 2_9 for SSB burst periodicity switching, UE selects one SMTC configuration accordingly.

At RAN2 130 meeting the following related agreements are available:

For SSB adaptation, R2 concludes the support of L3 measurement configuration based on a single MO. No RAN2 spec impact is foreseen.

The following agreements are achieved at RAN4 115 meeting:

Issue 1-1-1: Processing time for SSB adaption

Agreement

RAN4 to define the additional delay requirement in TfirstSSB for L1 transition period measurement for SSB adaptation.

- FR1: 2ms FR2: 3ms

Issue 1-2-2: L1 requirements after transition  - DRX

Agreement:

- For L1 requirements impacts due to SSB adaptation, when DRX is configured, follow legacy principle to define requirements, which means the SSB adaption only impacts TSSB in the measurement period

Issue 1-2-3: L1 requirements - whether to release the measurement results from resources before transition

Agreement:

- No spec. impact is identified for releasing the measurement results.

- It’s up to UE implementation.

Issue 1-3-1: L3 requirements for SSB adaptation – transition for serving cell

Agreement

RAN4 to define L3 requirements at transition for SSB adaptation same as legacy L3 transition period requirement.

- The cell identification and measurement period requirements apply based on the longer period before or after the transition.

Issue 1-3-2: L3 requirements for SSB adaptation – after transition for serving cell

Agreement

- Subsequent to transition, serving cell identification and measurement period requirements are based on the additional SMTC configuration according to SSB adaptation mapped with SSB periodicity.

Issue 1-4-1: SCell activation requirement for SSB adaptation – SSB adaptation command related

Agreement:

- For the conditions for SCell activation with SSB adaptation, SSB adaptation command comes no later than SCell activation command.

Discussion

In this contribution we discuss the following open topics:

- Sub-topic 1-1 SSB adaptation – General

- Sub-topic 1-2 SSB adaptation – L1 Requirements impacts

Issue 1-2-4: L1 requirements – Measurement restriction impact

- Proposals

  - Option 1: RAN4 to discuss whether existing L1 measurement restriction can apply to the case when adapted SSB colliding with CSI-RS. i.e. Whether UE can choose to measure one of them or only to measure SSB. (Huawei)

  - Option 2: When adapted SSB colliding with CSI-RS, existing L1 measurement restriction can apply. (CTC, Vivo)

- Recommended WF:

  - Discuss above option.

9.5.5.1	Measurement restriction for SSB based L1-RSRP

For FR1, if UE support rxTimingDiff-r18, when [the SSB for L1-RSRP measurement and CSI-RS for RLM, BFD, CBD or L1-RSRP measurement are overlapping or partially overlapping in OFDM symbols],

-	If SSB and CSI-RS have same SCS, UE shall be able to measure the SSB for L1-RSRP measurement without any restriction;

-	If SSB and CSI-RS have different SCS,

-	If UE supports simultaneousRxDataSSB-DiffNumerology, UE shall be able to measure the SSB for L1-RSRP measurement without any restriction;

-	If UE does not support simultaneousRxDataSSB-DiffNumerology, UE is required to measure one of but not both SSB for L1-RSRP measurement and CSI-RS. Longer measurement period for SSB based L1-RSRP measurement is expected, and no requirements are defined.

9.5.5.2	Measurement restriction for CSI-RS based L1-RSRP

For FR1, when the SSB for RLM, BFD, CBD or L1-RSRP measurement is within the active BWP and has different SCS than CSI-RS for L1-RSRP measurement, the UE shall be able to perform CSI-RS measurement with restrictions according to its capabilities:

-	If the UE supports simultaneousRxDataSSB-DiffNumerology the UE shall be able to perform CSI-RS measurement without restrictions.

-	If the UE does not support simultaneousRxDataSSB-DiffNumerology, UE is required to measure one of but not both CSI-RS for L1-RSRP measurement and SSB. Longer measurement period for CSI-RS based L1-RSRP measurement is expected, and no requirements are defined.

Based on the current spec a UE is required to measure one of but not both SSB for L1-RSRP measurement when SSB and CSI-RS for L1 measurement collide. For the SSB adaptation, the periodicity of SSB is changed after adaptation whereas other properties of SSB is the same as before. From requirements point of view, the overlapping issue between CSI-RS and SSB, whether the SSB adaptation is supported or not, is same and the same principle should be used. Hence the legacy measurement restriction can apply.

Proposal 1: For SSB adaptation, measurement restriction for SSB based L1-RSRP can be the same as that of legacy when SSB collides with CSI-RS.

- Sub-topic 1-3 SSB adaptation – L3 Requirements impacts

Issue 1-3-4: Colliding with MG

- Proposals

  - Option 1: If the SMTC occasion of the selected additional SMTC configuration collides with MG occasion, the UE is allowed to skip measurements in the overlapping SMTC occasion of the serving SCell.

For the SMTC configuration colliding with MG after adaptation, the same principle as legacy when SMTC collides MG can be reused. Option 1 is ok.

Proposal 2: For the SMTC configuration colliding with MG after adaptation, option 1 is ok.

Conclusions

Based on above analysis, we have following observations and proposals.

Proposal 1: For SSB adaptation, measurement restriction for SSB based L1-RSRP can be the same as that of legacy when SSB collides with CSI-RS.

Proposal 2: For the SMTC configuration colliding with MG after adaptation, option 1 is ok.

- References

[1] R4-2508441, WF on RRM requirements for Netw_Energy_NR_enh_Part2, Huawei, RAN4 115

[2] R4-2505571, Topic summary for [115][222] Netw_Energy_NR_enh_Part2, Huawei, RAN4 115