3GPP TSG-RAN WG4 Meeting #116	R4-2510195

Bengaluru, India, 25 August – 29 August, 2025

Source: 	vivo

Title: 	Consideration on performance part for LP-WUR

Agenda Item:	7.24.6

Document for:	Discussion

Introduction

In [1], the time budget for LP-WUR performance part is included and the TU plan is copied below for reference.

LP-WUR performance part work has already started at RAN4 115 meeting. In this contribution, we provide our considerations on the LP-WUR performance part.

Discussion

Discussion

Issue 3-2-1: Test case categories/scenarios

- Proposals

  - P1: Consider test case for the following categories: Category 1: FR1 LP-WUR monitoring related test case; Category 2: FR1 RRM offloading related test case; Category 3: FR1 RRM relaxation related test case (vivo)

  - P2: Introduce a test case for to test LR Evaluation (delay) requirements. (Nokia)

  - P3: Design test cases for LP-WUR measurement and evaluation at RRC_IDLE/INACTIVE state; design test case for relaxed MR measurement for serving cell/neighbour cell measurements (CT)

  - P4: Define test cases for evaluation of exit conditions for the following cases. (Huawei)

    - Case 1: UE exist Case 1 to legacy

    - Case 2: UE exist Case 3 to legacy

    - Case 3: UE exist Case 1 to Case 3

  - P5: RAN4 to define the test cases to verify both MR RRM relaxation and MR RRM offloading scenarios. RAN4 to introduce the RRM relaxation test cases similar as Rel-16 cell reselection (Ericsson)

  - P6: RAN4 to discuss whether to introduce an applicability to the LP-WUR UE which supports both LP-SS and SSB. (Ericsson)

Recommendations:

Regarding test case categories and scenarios, firstly we think test case for offloading scenarios and RRM relaxation scenario should be considered. The detail of offloading related test and RRM relaxation related test will be discussed later. For the LP-WUR operation, when the entry conditions are satisfied, there is no mandatory on UE behavior, for example as the following:

“The gNB configures in system information entry and exit conditions to monitor LP-WUS. The UE may start monitoring LP-WUS when measurements using the MR are above the configured entry threshold(s), and the measurements using the LR are above the entry threshold(s) if configured. Entry conditions for LP-WUS monitoring are based on MR and optionally LR measurements as specified in TS 38.304.”

Hence for test cases for offloading and RRM relaxation, only the exit from one state to another state needs be verified. This means at the beginning of a test case, the UE will already in the offloading or RRM relaxation or LP-WUR monitoring mode.

For the LP-WUR monitoring related test, except for the evaluation requirement for entry/exit, there is no other LP-WUR monitor requirements in RRM part. The verification on miss detection/false alarm etc. should be considered in demodulation session. The LP-WUR related paging reception duration, i.e., the time when the LP-WUR detect the paging indicator till the time when MR feedback to network after decoding paging information, could be verified and the related part is the wake up delay of the MR, i.e., the time when the LP-WUR detect the paging indicator till the time when MR is ready for paging reception. However the issue is there in no directly RAN4 requirement on wake up delay hence whether and how to verify the wake up delay needs further study.

In summary, we have the following proposals:

Proposal 1: Define test cases for offloading and RRM relaxation, for offloading and RRM relaxation, only the exit from one state to another state needs be verified. And at the beginning of a test case, the UE will already in the offloading or RRM relaxation or LP-WUR monitoring mode

Proposal 2: Whether and how to verify MR wake up need further study, except for it, no need for test cases for LP-WUR monitoring like miss detection/false alarm.

Issue 3-2-2: Test case for OOK based LR and OFDM based LR

- Proposals

  - P1: Introduce separate sets of test cases for UE supporting PSS/SSS based LP-WUR and UE supporting LP-SS based LP-WUR. (Nokia Huawei)

  - P2: RAN4 to introduce a single test for both LP-SS and SSB based LP-WUR UE (Ericsson)

Regarding OOK based LR or OFDB based LR, since the reference signal is different, separate sets of test cases are needed. Of course the number of test case for each set could be further discussed. For the OOK based LP-WUR, in the test case, only the LP-SS will be sent and SSB will not be sent.

Proposal 3: Define separate test cases for LP-SS based LR and SSB based LR. In the test case for LP-SS based LR, only LP-SS will be sent and the SSB will not be sent.

For the RRM relaxation test case, the Rel-16 procedure could be followed and the following test case could be introduced:

  - intra-frequency cell re-selection (LP-SS based LR and/or SSB based LR)

  - inter-frequency cell re-selection for lower and equal priority re-selection (LP-SS based LR and/or SSB based LR)

  - inter-RAT cell re-selection (LP-SS based LR and/or SSB based LR)

In order to limit the test case number, whether to introduce all the former 3 test cases for each LR type, i.e., the LP-SS based LR or the SSB based LR, could be further discussed.

Proposal 4: Suggest to introduce the following test cases for RRM relaxation:

  - intra-frequency cell re-selection (for LP-SS based LR and/or SSB based LR)

  - inter-frequency cell re-selection for lower and equal priority re-selection (for LP-SS based LR and/or SSB based LR)

  - inter-RAT cell re-selection (for LP-SS based LR and/or SSB based LR)

Proposal 5: whether to introduce all the former 3 test cases for each LR type, i.e., the LP-SS based LR or the SSB based LR, could be further discussed.

Issue 3-2-5: Test case design for offloading case

- Proposals

  - P1: For RRM offloading case, a joint procedures can be considered to verify both the LR measurement requirement and whether a MR is triggered in time. (vivo)

  - P2: RAN4 to discuss the test methodology about how to verify MR RRM offloading scenario, such as whether to introduce a new test mode. (Ericsson)

  - P3: Introduce the test cases where the exit conditions can be tested: UE meets the cell reselection requirements with initial conditions being WUR operating in the full offloading mode. (Qualcomm)

Recommendations:

Regarding the RRM offloading case, when the UE exit the offloading case and triggers the MR, actually no feedback signal was sent from a UE to network hence the total duration is untestable. A joint procedures can be considered, for example, at the beginning time point A, a UE is in the RRM offloading case, a particular condition can be designed to trigger the MR at time point B to move the UE to the legacy state, then a procedure (for example a cell reselection procedure)which could provide feedback signal to the TE is followed immediately at the time point C. In the end at time point D the UE feedback to TE. The total delay can be derived based on the delay of each procedure the exit from offloading to legacy state can be verified.

Proposal 6: A joint procedures can be considered to verify the exit from offloading to legacy state. For example, a cell reselection procedure can be triggered immediately or after some time after the UE exit from the offloading state.

Issue 3-2-4: Test case design for RRM relaxation

- Proposals

  - P1: Introduce a test case for MR reselection with case 3 and with 16x relaxation. (Nokia)

  - P2: Define Cell reselection to FR1 intra-frequency NR case for UE fulfilling relaxed measurement criterion; Cell reselection to FR1 inter-frequency NR case for UE fulfilling relaxed measurement criterion. (vivo)

  - P3: For Case#3, at least to introduce following tests to guarantee the test coverage (CMCC)

    - intra-frequency cell re-selection

    - inter-frequency cell re-selection including lower and equal priority re-selection and higher priority re-selection

    - inter-RAT cell re-selection

  - P4: In case 3, the test cases for cell reselection shall be considered and the specific number of test cases could be discussed in this meeting. (ZTE)

  - P5: Introduce the test cases where the exit conditions can be tested: UE meets the cell reselection requirements with initial conditions being WUR operating in the relaxed measurement mode. (Qualcomm)

Regarding higher priority frequency layer cell reselection, suggest no test case since the test duration is quite long.

Proposal 7: Suggest do not define test case for higher priority frequency layer cell reselection.

Conclusions

Based on above analysis, we have following observations and proposals.

Proposal 1: Define test cases for offloading and RRM relaxation, for offloading and RRM relaxation, only the exit from one state to another state needs be verified. And at the beginning of a test case, the UE will already in the offloading or RRM relaxation or LP-WUR monitoring mode

Proposal 2: Whether and how to verify MR wake up need further study, except for it, no need for test cases for LP-WUR monitoring like miss detection/false alarm.

Proposal 3: Define separate test cases for LP-SS based LR and SSB based LR. In the test case for LP-SS based LR, only LP-SS will be sent and the SSB will not be sent.

Proposal 4: Suggest to introduce the following test cases for RRM relaxation:

  - intra-frequency cell re-selection (for LP-SS based LR and/or SSB based LR)

  - inter-frequency cell re-selection for lower and equal priority re-selection (for LP-SS based LR and/or SSB based LR)

  - inter-RAT cell re-selection (for LP-SS based LR and/or SSB based LR)

Proposal 5: whether to introduce all the former 3 test cases for each LR type, i.e., the LP-SS based LR or the SSB based LR, could be further discussed.

Proposal 6: A joint procedures can be considered to verify the exit from offloading to legacy state. For example, a cell reselection procedure can be triggered immediately or after some time after the UE exit from the offloading state.

Proposal 7: Suggest do not define test case for higher priority frequency layer cell reselection.

- References

[1] RP-250821, TU budget after RP107, RAN#107