3GPP TSG-RAN WG4 Meeting #116				                                　 R4-2510200

Bengaluru, India, August 25th – 29th, 2025

Source: 	Huawei, HiSilicon

Title: 	MPR reduction for upper 6GHz by ACLR relaxation

Agenda Item:	4.2.2

Document for:	Discussion/Decision

# Introduction

This contribution is a companion CR of [1] to elaborate the motivation and technical details for the CR.

# Discussion

## Review of a R17 SI

A release 17 SI of “Study on International Mobile Telecommunications (IMT) parameters for 6.425 - 7.025 GHz, 7.025 - 7.125 GHz and 10.0 - 10.5 GHz” performed co-existence study. Note that the SI is called FS_NR_IMT_param hereafter in this contribution.

According to the corresponding TR 38.921 for FS_NR_IMT_param, it was agreed to specify 26 dB ACLR for the 6.425~7.125 GHz frequency range assuming UE PC3. This value, however, has not yet been incorporated into TS38.101-1, even though the study results for BS side was updated accordingly.

Observation 1: TR 38.921 for FS_NR_IMT_param agreed to specify not 30 dB, but rather 26 dB ACLR for the 6.425~7.125 GHz frequency range assuming UE PC3, while it has not been incorporated into TS38.101-1.

## Relation between ACLR and MPR

MPR may be needed depending on relation between various configured/scheduled parameters such as the number of RBs, its position, MCS, waveform and gating factors such as ACLR, SEM, EVM, etc.

For lower order modulation(s) like QPSK, ACLR can be a gating factor for outer RBs allocation especially for wider RBs allocations. Thus, relaxing ACLR can reduce MPR for those cases. According to our initial analysis, it would be possible to reduce MPR by up to 0.5~1.0 dB for QPSK and 16QAM for DFT-s-OFDM and those for CP-OFDM, respectively. See the detailed simulation results in Annex. It is noted that 64QAM for both DFT-s-OFDM and CP-OFDM as well as 256QAM for DFT-s-OFDM may have possibility to reduce MPR by 0.5 dB. We are open to further discuss those aspects.

Observation 2: 4 dB ACLR relaxation would enable MPR to be reduced by 0.5~1.0 dB for QPSK and 16QAM for DFT-s-OFDM and those for CP-OFDM, respectively.

## Specification impact

### Release aspect

In case the relaxed ACLR of 26 dB is introduced into n104, n104C and band combinations (BCs) including n104, some important aspects are from which release it is introduced and if there is a non-backward compatibility (NBC) issue or not. As far as we know, even single carrier of operation for n104 has not been deployed yet so that there are no UEs implementing n104 as well as n104C and BCs including n104. Therefore, at least we haven’t seen NBC issues. Hence, we propose to introduce ACLR of 26 dB from Release 17.

Observation 3: There would not be any deployments for n104. Hence, no NBC issue is found to introduce ACLR of 26 dB from release 17.

### Specification impact

For single carrier operation and intra band contiguous UL CA of n104C, thus far there is no additional spectrum emission requirements for n104. Therefore, the impacted requirements are ACLR of 30 dB, which must be replaced with 26 dB for n104 and n104C PC3 and corresponding MPRs.

For inter-band CA including n104, only a requirement impacted by ACLR would be MSD for cross-band isolation. There is a band combination of CA_n78-n104, which as MSD of n78 from interreference from n104. Therefore, a new MSD value must be evaluated.

Observation 4: For single carrier operation of n104 and intra band contiguous UL CA of n104C, the impacted requirements are ACLR of 30 dB, which must be replaced with 26 dB and corresponding MPR. For inter-band UL CA including n104, i.e., CA_n78-n104, MSD value for cross band isolation due to interference from n104 must be revisited.

## Conclusion

From Observation 1-4, there is no NBC issue to correct the current ACLR value of 30 dB to 26 dB from release 17. In addition, the specification impact on the introduction of 26 dB into single carrier, intra band contiguous UL CA and inter band UL CA including n104 are quite limited.

Proposal : Introduce a relaxed ACLR of 26 dB for PC3 n104, n104C and BCs including n104 together with reduced MPRs for n104 and n104C, respectively and a new MSD value for CA_n78-n104 from R17 and beyond.

# Summary

We obtained following observations and proposal. A companion CR [1] is also provided in this meeting.

Observation 1: TR 38.921 for FS_NR_IMT_param agreed to specify not 30 dB, but rather 26 dB ACLR for the 6.425~7.125 GHz frequency range assuming UE PC3, while it has not been incorporated into TS38.101-1.

Observation 2: 4 dB ACLR relaxation would enable MPR to be reduced by 0.5~1.0 dB for QPSK DFT-s-OFDM and 1.0~1.5 dB for QPSK CP-OFDM, respectively.

Observation 3: There would not be any deployments for n104. Hence, no NBC issue is found to introduce ACLR of 26 dB from release 17.

Observation 4: For single carrier operation of n104 and intra band contiguous UL CA of n104C, the impacted requirements are ACLR of 30 dB, which must be replaced with 26 dB and corresponding MPR. For inter-band UL CA including n104, i.e., CA_n78-n104, MSD value for cross band isolation due to interference from n104 must be revisited.

Proposal: Introduce a relaxed ACLR of 26 dB for PC3 n104, n104C and BCs including n104 together with reduced MPRs for n104 and n104C, respectively and a new MSD value for CA_n78-n104 from R17 and beyond.

Correction CR for single carrier is prioritized firstly for the above proposal and a corresponding CR is provided in [1].

# References

[1] R4-2510201, (NR_6GHz-Core) CR to TS 38.101-1: Correct the Tx requirements for n104, Huawei, HiSilicon, RAN4#116

# Annex 1 (QPSK)

| ACLR (dB) | CP-OFDM | DFT-s-OFDM |
| --- | --- | --- |
| 30 |  |  |
| 26 |  |  |

# Annex 2 (16QAM)

| ACLR (dB) | CP-OFDM | DFT-s-OFDM |
| --- | --- | --- |
| 30 |  |  |
| 26 |  |  |

# Annex 3 (64QAM)

| ACLR (dB) | CP-OFDM | DFT-s-OFDM |
| --- | --- | --- |
| 30 |  |  |
| 26 |  |  |

# Annex 4 (256QAM)

| ACLR (dB) | CP-OFDM | DFT-s-OFDM |
| --- | --- | --- |
| 30 |  |  |
| 26 |  |  |

For reference: Table 6.2.2-1 Maximum power reduction (MPR) for power class 3

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK w/ Rel-15 DMRS | ≤ 3.51 | ≤ 1.21 | ≤ 0.21 |
|  |  | ≤ 0.52,3 | ≤ 0.52 | 02,4 |
|  | Pi/2 BPSK w Pi/2 BPSK DMRS | ≤ 0.52,3 | 02 | 02,4 |
|  | QPSK | ≤ 1 |  | 05 |
|  | 16 QAM | ≤ 2 |  | ≤ 1 |
|  | 64 QAM | ≤ 2.5 |  |  |
|  | 256 QAM | ≤ 4.5 |  |  |
| CP-OFDM | QPSK | ≤ 3 |  | ≤ 1.5 |
|  | 16 QAM | ≤ 3 |  | ≤ 2 |
|  | 64 QAM | ≤ 3.5 |  |  |
|  | 256 QAM | ≤ 6.5 |  |  |
| NOTE 1: Applicable for UE operating in TDD mode with Pi/2 BPSK modulation and UE indicates support for UE capability powerBoosting-pi2BPSK and if the IE powerBoostPi2BPSK is set to 1 and 40 % or less slots in radio frame are used for UL transmission for bands n40, n41, n77, n78 and n79. The reference power of 0 dB MPR is 26 dBm. NOTE 2: Applicable for conditions where note 1 does not apply. NOTE 3: For 3 MHz channel bandwidth the Pi/2 BPSK edge allocation MPR is 1 dB NOTE 4: For a UE indicating support for UE capability powerBoosting-pi2BPSK-QPSK-r18 or powerBoosting-pi2BPSK-QPSK-Modified-r18 and if the IE powerBoostPi2BPSK-r18 is set to 1, the reference power is increased by [ΔPPowerBoost - ΔPPowerClass] NOTE 5: For a UE indicating support for UE capability powerBoosting-pi2BPSK-QPSK-r18 or powerBoosting-pi2BPSK-QPSK-Modified-r18 and if the IE powerBoostQPSK-r18 is set to 1, the reference power is increased by [ΔPPowerBoost - ΔPPowerClass] |  |  |  |  |