3GPP TSG-RAN WG4 Meeting #116			R4-2510205

Bengaluru, India, 25-29 Aug 2025

Agenda item:	5.8.1

Title:	Discussion on UE RF A-MPR for B254

Source:	Nordic Semiconductor ASA

Document for:	Discussion

# Introduction

TS 36.102 is uncomplete for b254. A-MPRs are set to TBD.

6.2B.3 UE additional maximum output power reduction for category NB1 and NB2 UE Additional ACLR and spectrum emission requirements can be signalled by the network to indicate that the UE shall also meet additional requirements in a specific deployment scenario. To meet these additional requirements, Additional Maximum Power Reduction (A-MPR) is allowed for the output power are specified. For the agreed E-UTRA bands for category NB1 and NB2 UE an A-MPR of 0 dB shall be allowed unless specified otherwise. For UE Power Class 3 and 5 the specific requirements and identified subclauses are specified in Table 6.2B.3-1 along with the allowed A-MPR values that may be used to meet these requirements. The allowed A-MPR values specified below in Table 6.2B.3-1 are in addition to the allowed MPR requirements specified in subclause 6.2B.2-1. Table 6.2B.3-1: Additional Maximum Power Reduction (A-MPR) for category NB1 and NB2 UE Network Signalling value; Requirements (subclause); E-UTRA Band; A-MPR (dB). NS_01; 6.5B.4.2; Table 5.2-1; N/A. NS_02N; 6.5B.4.4.2; 255; N/A. NS_03N; 6.5B.4.4.4; 254; N/A. NS_04N; 6.5B.4.4.5; 254; TBD. NS_05N; 6.5B.4.4.6; 254; TBD. NS_24; 6.5B.4.4.3; 256; PC3; PC5. ≤ 3.5; ≤ 0.5.

In RAN4#115 partial progress has been achieved, the following agreements have been reached.

# FFS on 3kHz measurement BW

Using commercial module, we measured SEM with 3kHz instead of 30kHz BW @1526.5kHz. It has been observed that average measured power is 10dB lower (as expected, since measured bandwidth is 10x smaller), and measurements themselves become challenging as even small CFO impacts measurement significantly.  To summarize, we have not seen any benefit from changing measurements to 3kHz measurement BW.

Proposal-1: Do not adopt 3kHz BW for measurements.

# A-MPR

Using randomly picked single commercial NTN IoT module, we measured SEM with standard test equipment using standard  30kHz measurement BW @1526.5 MHz, and measured the values in the following table. In practice some margin is needed for the UEs to ensure that every device can pass the requirement in practice with 100% probability.

| NS_05N | 12-tone | 6-tone edge | 3-tone edge | 1-tone edge 15kHz |
| --- | --- | --- | --- | --- |
| Legacy A-MPR | 2dB | 1dB | 0dB | 0dB |
| Guard band 0kHz + 10kHz (legacy) | 4.3dB | 9.88 | 12.14 | 16.59dB |
| Guard band 50kHz + 10kHz (legacy) | 0dB | 0dB | 0dB | [0]dB |

If the more detailed A-MPR would be standardized, our proposed values would be the following for the stricter NS_05N, and could be 3dB lower for the NS_04N.

Table 1: Potential A-MPR for NS_05N, 15 kHz SCS for 0 kHz additional guard-band

| Modulation | QPSK |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Tone positions for single tone allocation | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |
| A-MPR (dB) | ≤ 18 |  | ≤18 | ≤18 | ≤2 | ≤2 | ≤2 | ≤2 | ≤ 18 | ≤18 | ≤18 |  |
| Tone positions for 3 Tones allocation | 0-2 |  |  | 3-5 and 6-8 |  |  |  |  |  | 9-11 |  |  |
| A-MPR (dB) | ≤ 13 |  |  | ≤ 2 |  |  |  |  |  | ≤ 13 |  |  |
| Tone positions for 6 Tones allocation | 0-5 and 6-11 |  |  |  |  |  |  |  |  |  |  |  |
| A-MPR (dB) | ≤ 11 |  |  |  |  |  | ≤ 11 |  |  |  |  |  |
| Tone positions for 12 Tones allocation | 0-11 |  |  |  |  |  |  |  |  |  |  |  |
| A-MPR (dB) | ≤ 5.5 |  |  |  |  |  |  |  |  |  |  |  |

For the stricter NS_05N requirement, to accommodate the NB-IoT UE implementations, based on our measurements, and for the edge 15kHz tone of b254, the A-MPR required is 18dB and for NS_04N 3dB less, i.e. 17dB

Observation-1: With legacy 10kHz NB-IoT guard-band for the stricter NS_05N:

- A-MPR for NS_05N for edge 1-tone 15kHz would need to be >18dB, to provide enough margin

- A-MPR for NS_05N for edge 3-tone 15kHz would need to be >13dB, to provide enough margin

- A-MPR for NS_05N for edge 6-tone 15kHz would need to be >11dB, to provide enough margin

- A-MPR for NS_05N for edge 12-tone 15kHz would need to be 5.5dB, to provide enough margin

It does not make any sense to operate with AMPRs >10dB. Therefore, we suggest the following with respect to Option1 vs Option 2 which is FFS:

Proposal-1: If Option 2 is to be pursued, only the following transmission modes are supported/defined:

- 12-tone 15kHz with 5.5dB A-MPR for NS_05N

- 3-tone 15kHz tone position 3-5 and 6-8 with 2dB A-MPR for NS_05N

- 1-tone 15kHz tone position 4-8 with 2dB A-MPR for NS_05N

One can argue that UE could do more filtering and pulse-shaping to improve performance. However, it should be noted that when filtering starts to be longer than CP and too much pulse-shaping is used. EVM starts to raise, such device may need less A-MPR, but does not anymore pass performance requirements for BLER.

If there is a will to reduce A-MPRs for NB-IoT, additional guard-band needs to be offered to the NB-IoT devices. Offering >50kHz of additional guard-band may reduce the A-MPRs to 0dB for PC3 devices. However, considering higher power classes, 100kHz additional guard-band would be a more forward compatible option for b254

Proposal-2: If there is desire to operate NB-IoT in b254 (0dB A-MPRs are desired), NB-IoT UE must be offered additional guard band in b254. At least 50kHz (optimally 100kHz) additional guard band must be defined for NB-IoT SEM.  And NS_04N  and NS_05N  A-MPR is set to N/A.

Our text proposal for 36.102.:

##### 6.5B.3.3.3	Requirements for network signalling value "NS_04N"

As specified in 6.5A.3.3.2, where (ΔfOOB) starting from the  edge  100kHz of the assigned category NB1 or NB2 channel bandwidth.

##### 6.5B.3.3.4	Requirements for network signalling value “NS_05N"

As specified in 6.5A.3.3.3, where (ΔfOOB) starting from the  edge  100kHz of the assigned category NB1 or NB2 channel bandwidth.

…..

Table 6.2B.3-1: Additional Maximum Power Reduction (A-MPR) for category NB1 and NB2 UE

| Network Signalling value | Requirements (subclause) | E-UTRA Band | A-MPR (dB) |  |
| --- | --- | --- | --- | --- |
| NS_01 | 6.5B.4.2 | Table 5.2-1 | N/A |  |
| NS_24 | 6.5B.4.4.3 | 256 | PC3 | PC5 |
|  |  |  | ≤ 3.5 | ≤ 0.5 |
| NS_02N | 6.5B.4.4.2 | 255 | N/A |  |
| NS_03N | 6.5B.4.4.4 | 254 | N/A |  |
| NS_04N | 6.5B.4.4.5 | 254 | N/A |  |
| NS_05N | 6.5B.4.4.6 | 254 | N/A |  |
| NS_06N | 6.5B.4.4.7 | 252 | N/A |  |
| NS_07N | 6.5B.4.4.7 | 252 | N/A |  |
| NS_08N | 6.5B.4.4.7 | 252 | N/A |  |
| NS_11N | 6.5B.4.4.5 | 254 |  |  |
| NS_12N | 6.5B.4.4.6 | 254 |  |  |

# Conclusion

Observation-1: With legacy 10kHz NB-IoT guard-band for the stricter NS_05N:

- A-MPR for NS_05N for edge 1-tone 15kHz would need to be >18dB, to provide enough margin

- A-MPR for NS_05N for edge 3-tone 15kHz would need to be >13dB, to provide enough margin

- A-MPR for NS_05N for edge 6-tone 15kHz would need to be >11dB, to provide enough margin

- A-MPR for NS_05N for edge 12-tone 15kHz would need to be 5.5dB, to provide enough margin

Proposal-1: If Option 2 is to be pursued, only the following transmission modes are supported/defined:

- 12-tone 15kHz with 5.5dB A-MPR for NS_05N

- 3-tone 15kHz tone position 3-5 and 6-8 with 2dB A-MPR for NS_05N

- 1-tone 15kHz tone position 4-8 with 2dB A-MPR for NS_05N

Proposal-2: If there is real desire to operate NB-IoT in b254 (0dB A-MPRs are desired) in the future, NB-IoT UE must be offered additional guard band in b254. At least 50kHz (optimally 100kHz) additional guard band must be defined for NB-IoT SEM.  And NS_04N and NS_05N A-MPR is set to N/A.