3GPP TSG-RAN WG4 Meeting #116           	R4-2510216

Bengaluru, IN, 25th-29th, Aug, 2025

Agenda item:	7.13.4

Source:	Samsung

Title:	View on the remaining issue for BS demodulation requirement with MMSE-IRC receiver

Document for:	Discussion

- Introduction

In the last RAN4 meeting, the remaining issue of Rel-19 WI on demodulation performance requirements enhancement with MMSE-IRC receiver was further discussed.

In this contribution, the view on the remaining issue of test setup for BS demodulation with MMSE-IRC were provided.

# 2	Discussion

Additional applicability rule

For test scenario, both HomNet and HetNet scenarios were agreed to cover the performance requirement of MMSE-IRC receiver. Regarding the applicability for requirement in different scenarios, it will be further discussed with the following

Whether to consider additional applicability rule on how the test is performed - Option 1 - If a BS declared to support Wide Area BS, Medium Range BS and pass the tests for HomNet, the test for HetNet can be skipped - Option 2 - Not to consider additional applicability rule on how the test is performed for different test scenarios or BS classes

In the previous meeting, RAN4 agreed that only HomNet requirements are not appliable for local Area BS. That means both HetNet and HomNet requirements can be applied for wide area and medium area BS. From the baseband processing perspective of MMSE-IRC, there is no difference foreseen. Since the test applicability for different HomNet and HetNet requirements will be considered, which means if BS is declared to support wide area BS, Medium Area BS, pass the tests of HomNet (DIP Set 1), the test for HetNet (DIP Set 2) can be skipped. Therefore, based on the declaration of BS categories, to reduce the test effort, we are fine to consider additional applicability rule for perform the testing on HetNet and HomNet requirement based on BS declaration on BS classes.

Proposal 1: If a BS declared to support Wide Area BS, Medium Range BS and pass the tests for HomNet, the test for HetNet can be skipped.

# 3	Conclusion

In this contribution, the view on the remaining issues of BS demodulation requirement with MMSE-IRC receiver was provided.

Proposal 1: If a BS declared to support Wide Area BS, Medium Range BS and pass the tests for HomNet, the test for HetNet can be skipped.

# References

R4-2508618, Way Forward for [115][322] NR_demod_Ph5_Part1_General_BS