3GPP TSG-RAN WG4 Meeting #116	R4-2510219

# Bengaluru, India, August 25th – 29th, 2025

Agenda item:			7.24.3.2

Source:	Samsung

Title:	TP to TR 38.774 on FR2 UE RF of LP-WUR

Document for:	Approval

# 1  Introduction

In this paper, we provide text proposal to TR 38.774 on FR2 UE RF of LP-WUR based on agreements of previous meetings, it can be revised during this meeting with updated meeting outcome.

# 2  Text propsoal

## <<Start of Change>>

# 6	Simulation assumptions and evaluation

## 6.1	Performance metric

<Editor’s note: performance metric to specify RF requirements. >

For RF requirements and conformance testing, the performance metric is 1% MDR. Meanwhile, the FAR assumption for MDR evaluation is </=1%.

## 6.2	Simulation assumption

### 6.2.1	General

To evaluate the RF performance of LP-WUS/WUR, the link level simulation is performed in this WI. This clause captures the simulation assumption for LP-WUS UE RF and BS RF analysis.

### 6.2.2	Simulation parameters

For RF performance analysis, the following simulation assumptions are considered.

Table 6.2.2-1 General Link level simulation assumption for FR1 LP-WUR

| Attributes | Assumptions |
| --- | --- |
| Waveform | OOK-4 waveform |
| Center frequency | 900MHz, 2.6GHz and 3.5GHz |
| Channel structure | 5 bits |
| RM coding | 8/16/32bit for OOK |
| CRC | NO |
| Chip rate | M=4 |
| overlaid OFDM sequence | 4 candidates overlaid sequences for M=4 |
| WUS duration | OOK：RM coding length*2/M OFDM：ceil(5/log2(4)/(M/2)) |
| Coding | 1/2 rate Manchester coding |
| Time error | 0/0.9 us |
| Residual Frequency error | 0/10/20 ppm |
| SCS | 30kHz |
| UE Channel BW | 20MHz (51 RB)-case 1 10MHz (24 RB)-case 2 5MHz (11 RB)-case 3 |
| WUS RB | - Fixed 11RB ~ 3.96MHz for 10MHz and 20MHz cases |
| Position within channel | - For 10/20MHz CBW, Center for ASCS, edge for ACS [assume no ASCS impact] - For 5MHz CBW, fixed center of channel |
| Guardband of NR channel, both wanted cell and interfer cell (ACS) | - For wanted signal: 505kHz for 5MHz, 665kHz for 10MH, 805kHz for 20Mhz - For interference cell2 5MHz: fixed 505kHz |
| Guard RB | For ASCS: 0 or 1RB on each side of LP-WUS bandwidth For ACS: 1/2/3/4 RB |
| Filter | 3th/5th Order lowpass Butterworth matching fixed 3.96MHz RF bandwidth for 10MHz/20MHz case - Other order lowpass filter is not precluded The filter bandwidth is adapted with actual WUS RBs, for 5MHz case |
| ASCS | PDSCH mapped on RBs not used for LP-WUS and Guard RB; EPRE of PDSCH /EPRE of LP-WUS = 0 dB Same PSD with WUS signal |
| ACS | PDSCH mapped on interference RBs (11RB for 5MHz CBW), one side; EPRE of PDSCH /EPRE of in-band LP-WUS = [20~33] dB NOTE: decide the interference level depending on SNR |
| Wanted signal level | For ACS, REFSENS + 14 dB for LP-WUS as baseline, different wanted power level can be also considered |
| Sampling rate | 7.68MHz |
| ADC bit width | 4/8 bits ADC for ASCS/ACS Encourage companies to provide simulation results with both options for comparison |
| Phase noise | Reciprocal mixing can be optionally considered for ASCS/ACS. Note that actual implementation choice of phase noise is driven by jammer requirement and power consumption target. |
| Non-linearities | Not modelled |
| Power boosting | EPRE ratio: 0dB/3dB for OOK-1/OOK-4 NOTE: 3dB is optional for simulation |
| Channel Model | AWGN |
| Performance metric | - 1% MDR - false alarm rate 1% as side condition |
| Note: The detailed Receiver algorithm is UE implementation . |  |

Table 6.2.2-2 General Link level simulation assumption for FR2 LP-WUR

| Attributes | Assumptions |  |  | Notes |
| --- | --- | --- | --- | --- |
| Case name (waveform) | OOK-1 waveform |  | OOK-4 waveform |  |
| Center frequency | 24300MHz |  |  | n258 example band |
| WUS structure | Total 8/16/32 bits |  |  | (placeholder for RAN1 decision) |
| Chip rate | M=1 | M=1/2/4 |  |  |
| Coding | 1/2 rate Manchester coding |  |  |  |
| Time error | up to UE implementation |  |  | Difficult to separate time and frequency errors in realistic receivers |
| Residual Frequency error | 0/10/20 ppm |  |  |  |
| SCS | 120kHz |  |  |  |
| UE Channel BW | 200MHz (132 RB)-case 1 100MHz (66 RB)-case 2 |  |  |  |
| WUS RB | - Fixed 11RB |  |  | Based on agreement in RAN1#118-Bis |
| Position within channel | - Center for ASCS, edge for ACS [assume no ASCS impact] |  |  |  |
| Guardband of NR channel, both wanted cell and interfer cell (ACS) | - Refer to 5.3.3 in TS38.101-2 |  |  |  |
| Guard RB | - For ASCS: 0 or 1RB on each side of LP-WUS bandwidth - For ACS: 1/2/3/4 RB |  |  |  |
| Filter | - 3th/5th Order lowpass Butterworth - Other order lowpass filter is not precluded |  |  |  |
| ASCS | PDSCH mapped on RBs not used for LP-WUS and Guard RB; EPRE of PDSCH /EPRE of LP-WUS = 0 dB Same PSD with WUS signal |  |  |  |
| ACS | PDSCH mapped on interference RBs (11RB for 5MHz CBW), one side; NOTE: decide the interference level depending on SNR |  |  |  |
| Wanted signal level | For ACS, REFSENS + 14 dB for LP-WUS |  |  |  |
| Sampling rate | TBD |  |  |  |
| ADC bit width | 4/8 bits ADC for ASCS/ACS Encourage companies to provide simulation results with both options for comparison |  |  |  |
| Phase noise | FFS |  |  |  |
| Non-linearities | Not modelled |  |  |  |
| Power boosting | 0dB/3dB for OOK-1/OOK-4 NOTE: 3dB is optional for simulation |  |  |  |
| Channel Model | Option 1: TDL-C 100 Option 2: AWGN Note: encourage companies to provide simulation results with both options |  |  | The procedure for using CDL channel models in demod analysis is not fully established. |

## 6.3	Simulation evaluation results

<Editor’s note: detailed simulations from each company. FFS detailed structure for each RF requirement>

### 6.3.1	General

To evaluate some RF performance, link level simulations are performed to collect analysis input.

### 6.3.2	SNR simulations

In this sub-clause, SNR simulation results from different companies are collected for analysis of target SNR for LP-WUR.

Table 6.3.2-1 SNR simulation summary for FR1 Envelop-detection LP-WUR

| Companies Input | SNR | RM coding | ADC | Filter order | Timing error | Note |
| --- | --- | --- | --- | --- | --- | --- |
| A |  |  |  |  |  |  |
| B |  |  |  |  |  |  |
| C |  |  |  |  |  |  |

Observations and Summary of SNR simulation outcome for envelop-detection receiver: TBA

Table 6.3.2-2 SNR simulation summary for FR1 OFDM-based LP-WUR

| Companies Input | SNR | RM coding | ADC | Filter order | Timing error | Note |
| --- | --- | --- | --- | --- | --- | --- |
| A |  |  |  |  |  |  |
| B |  |  |  |  |  |  |
| C |  |  |  |  |  |  |

Observations and Summary of SNR simulation outcome for OFDM-based receiver: TBA

### 6.3.3	ASCS simulations

In this sub-clause, ASCS simulation results from different companies are collected for analysis of in-band interference performance of LP-WUS. This is for information only, because actual implementation choice of filtering is driven by jammer requirements.

Table 6.3.3-1 ASCS simulation summary for FR1 Envelop-detection LP-WUR

| Companies Input | SNR degradation | Guard RB | RM coding | ADC | Filter order | Timing error | Phase noise | Note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A |  |  |  |  |  |  |  |  |
| B |  |  |  |  |  |  |  |  |
| C |  |  |  |  |  |  |  |  |

Summary of ASCS simulation outcome: TBA

Table 6.3.3-2 ASCS simulation summary for FR1 OFDM-based LP-WUR

| Companies Input | SNR degradation | Guard RB | RM coding | ADC | Filter order | Timing error | Phase noise | Note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A |  |  |  |  |  |  |  |  |
| B |  |  |  |  |  |  |  |  |
| C |  |  |  |  |  |  |  |  |

### 6.3.4	ACS simulations

In this sub-clause, ACS simulation results from different companies are collected for analysis of ACS performance of LP-WUR. It was agreed that the interference level of LP-WUR ACS is the same as MR, for both type1 and type 2 LP-WUR. This is for information purpose only, because ACS requirements are driven by agreed coexistence requirements applied to the LR.

Table 6.3.4-1 ACS simulation summary for FR1 Type-1 LP-WUR

| Companies Input | Detection | ACS | Guard RB | Interference level | Wanted signal level | Filter order | Timing error | Phase noise | Note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A | Envelop detection |  |  |  |  |  |  |  |  |
|  | OFDM |  |  |  |  |  |  |  |  |
| B | Envelop detection |  |  |  |  |  |  |  |  |
|  | OFDM |  |  |  |  |  |  |  |  |
| C | Envelop detection |  |  |  |  |  |  |  |  |
|  | OFDM |  |  |  |  |  |  |  |  |

Summary of ACS simulation outcome: TBA

### 6.3.5	FR2 simulations

### 6.3.6	Other aspects

# 7	RF requirements

## 7.1	UE RF

<Editor’s note: discussions and analysis for UE RF requirements related. Mainly focus on key aspects and conclusions, rather than detailed simulations from each company>

### 7.1.1	General

The minimum requirements will be determined assuming there is only one receiver for FR1. For FR2, 1RX is assumed at baseband but two selection-based Rx chains are assumed at RF domain. A 1% MDR criterion will be used for verification of all core RF requirements. Following LP-WUS parameters will be used

Table 7.1.1-1. Common reference channel parameters for FR1

| Parameter | Unit | Value |
| --- | --- | --- |
| MR Channel bandwidth | MHz | All CBW |
| LP-WUS bandwidth | RB | 11 |
| Subcarrier spacing | kHz | 15/30kHz |
| RM coding | Bits | 16 |
| CRC |  | No CRC |
| Chip rate |  | M=4 (4 chips in an OFDM symbol) |
| Overlaid OFDM sequence |  | Length 33: generated by 31-length ZC sequence with extension |
| Number of overlaid OFDM sequence per chip to carry information |  | 4 |
| WUS duration for OOK |  | 8 OFDM symbols |
| WUS duration for OFDM |  | 2 OFDM symbols |
| Manchester coding for OOK |  | 1/2 |
| Number of information bits | Bits | 5 |

Table 7.1.1-2. Common reference channel parameters for FR2

TBD

In the specification, tables will be defined corresponding to 15 KHz and 30 KHz SCS for FR1, and 120kHz SCS for FR2.

### 7.1.2	Rx SNR evaluations

To derive SNR performance of LP-WUS, it was agreed to select OOK-4 M=4 under AWGN channel model with 1% MDR without repetition as worst case for FR1. M=1 is adopted for FR2.

### 7.1.3	Architecture and NF considerations

The basic architecture considered for the LP_WUR is a zero-IF architecture for both envelope and sequence based detectors. This assumption is common for both FR1 and FR2. Multiple frequency conversion in RF domain is not precluded for FR2. This is just an assumption made to derive parameters which dictate the requirements. This assumption does not preclude any other RF implementations.

In case of FR1, only single RX is assumed. For FR2 requirement derivation, the baseline assumption is an OFDM based receiver with two receiver chains with mutually orthogonally polarized antennas. The combining strategy of FR2 LP-WUR is selection-based diversity for the two Rx chains. Antenna gain is that of a single element.

In RAN4, there had been no consensus on what IM includes, and no agreement was achieved on individual values of NF and IM, so a joint number was agreed. There are two sets of NF + IM values agreed, and these will be used for REFSENS calculations. The values are,

-	Set 1: 18 dB,

-	Set 2: 13.5 dB.

The above mentioned numbers are for FR1.

For FR2, the NF + IM value is agreed as [12]dB which is 1dB degradation compared with MR assumption at 28GHz.

### 7.1.4	REFSENS requirements

It was agreed to derive the REFSENS for LP-WUR in the typical RF manner with AWGN being the channel for which SNR is derived. Thus, the REFSENS for LR is defined as

REFSENSLR (dBm) = -174 + 10log10(BWLR)+ (NFLR + IMLR) + SNRLR – Diversity Gain

Here, BWLR corresponds to the bandwidth of the LP-WUS signal in hertz. RAN1 has decided to have 11 RBs for LP-WUS irrespective of SCS, thus the LP-WUS bandwidth will be 1.98 MHz and 3.96 MHz, for 15 and 30 KHz SCS, respectively for FR1, and 15.84MHz for 120kHz SCS for FR2. Further, there are two set of values for NFLR + IMLR, i.e. 18.0dB and 13.5dB for Type 1 and Type 2 UEs, respectively for FR1, and NFLR + IMLR,.is agreed as [12]dB for FR2. There is only single receiver for FR1 so Diversity Gain is 0dB for FR1; For FR2, the Diversity Gain is agreed as -3dB since two receiver chains with mutually orthogonally polarized antennas are selection-based diversity (Note: real 2RX diversity in FR2 leads to 0dB diversity gain).

In the REFSENS test, only the RBs allocated for LP-WUS are populated with the LP-WUS and the rest of the RBs for a given channel bandwidth are left empty. This configuration, however, does not correspond to the real-life case where NR signals in other RBs are generally present and thus can create a coverage shortfall. Separately, the ASCS requirement allows a maximum degradation of 0.5 dB and the side conditions incorporate populated adjacent RBs.

Given the small value of the ASCS degradation factor (0.5 dB), it was agreed not to include this factor in the REFSENS.

For FR1, a SNR value of -4.5 dB for both Type 1 and Type 2 UEs was selected by RAN4 to be used for the REFSENS equation.

### 7.1.5	ASCS requirements

Adjacent subcarrier selectivity (ASCS) is a measure of a receiver's ability to receive an LP-WUS signal at its configured channel frequency in the presence of adjacent in-band NR signal(s). The in-band LP-WUS and NR signal should be same PSD and be separated by a given frequency offset (guard RB) between LP-WUS and NR, and the NR signal occupies the remaining RB resources within the maximum transmission bandwidth configuration, excluding the LP-WUS RBs.

### 7.1.6	ACS requirements

### 7.1.7	IBB requirements

### 7.1.8	OBB requirements

### 7.1.9	other Rx requirements

The spurious emission requirements for LP-WUR are considered same as MR for both FR1 and FR2.

## <<End of Change>>