3GPP TSG-RAN WG4 Meeting #116	R4-2510226

# Bengaluru, India, August 25th – 29th, 2025

Title:	Discussion on test tolerance for multi-panel transmission

Source:	Samsung

Agenda item:	7.11.3

Document for:	Discussion

1.	Introduction

Test tolerance due to finite measurement resolution is the only remaining issue for this study item. In last meeting the splitting principle and detailed assumption was agreed for further simulation.

In this contribution, we share our simulation results and proposals based on simulation results.

2. 	Discussion

The agreements of last meeting are as following [1, R4-2508663]:

How to define additional Test Tolerance, TTFiniteRes for min Peak EIRP - Agreements: - AoA1 and AoA2 will have the same TTFiniteRes, i.e., we are not assessing TTFiniteRes separately for the two different scenarios (AoA1 declared vs Beam peak direction re-used (default)) - The TTFiniteRes will be assessed similar to [R4-2506847] [R4-2506974] but with [6x6, 4x2] antenna configs and beam steering assumptions for the PC1/PC5 from spherical coverage analyses in RAN5, i.e., 4o beam steering granularity [R5-227982]; the resulting relative antenna gain difference will be split in half and applied equally as TTFiniteRes for AoA1 and AoA2; consideration of the 0.7dB systematic MU for beam peak search in this TTFiniteRes is TBD.

How to handle the 0.7dB systematic MU due to finite measurement grid is still TBD. In our understanding, TTFiniteRes is the test tolerance due to finite 2AoA separations, which is an additional item on top of the legacy 0.7dB due to finite measurement grid. So the 0.7dB systematic MU should be kept as is.

Proposal 1:	The legacy 0.7dB systematic MU due to finite measurement grid should be kept as is.

Regarding the simulation of TTFiniteRes, in our previous contribution [2, R4-2506974], the case when two panels located in adjacent faces have been simulated. In this paper, we mainly focus on the case when two panels are located in same face or in opposite faces.

According to the WF, 6x6 and 4x2 antenna arrays with 4o beam steering granularity are simulated and the maximum gain drop occurs with 30deg AoA separation for the case when two panels are located in same face, and with 150deg AoA separation for the case when two panels are located in opposite faces, and the simulation results would be very simliar. For simplicity, the simulation results with 30deg AoA separation is provided.

Table 1. simulation results with configurations in previous WF

|  | 6x6, 4deg beam steering granularity | 4x2, 4deg beam steering granularity |
| --- | --- | --- |
| Gain drop from peak | 1.45dB | 1.38dB |

However, it should be noted that 4x2 array with 4deg beam steering granularity has never been used as assumption for FWA UE, but has been used for handheld UE. In this study item, only non-handheld UE is within scope and PC1 and PC5 FWA UE are prioritized.

Proposal 2:	Do not consider 4x2 array with 4deg beam steering assumption when deriving test tolerance for PC1 and PC5 FWA UE.

According to the RAN5 assumption for PC1 and PC5 FWA UE, both 12x12 and 6x6 have been considered [3][4]. Accordingly, the simulation results are shown as Table 2.

Table 2. simulation results with RAN5 assumption for PC1 and PC5 FWA

|  | 12x12, 4deg beam steering granularity | 6x6, 4deg beam steering granularity |
| --- | --- | --- |
| Gain drop from peak | 1.80dB | 1.45dB |

As indicated in the WF, the resulting relative antenna gain difference will be split in half and applied equally as TTFiniteRes for AoA1 and AoA2, so the TTFiniteRes can be 1.8/2 = 0.9dB.

Proposal 3:	RAN4 to take both 12x12 and 6x6 array as assumption for PC1 and PC5 FWA UE in the test tolerance simulation, and accordingly the TTFiniteRes = 0.9dB.

It is also noticed that the 4deg beam steering granularity is rather small for PC5 UE. Usually the smaller antenna array, the larger beam steering granularity. 22.5deg beam steering granularity which is the assumption for PC3 handheld UE is also simulated for reference, as shown in Table 3.

Table 3. simulation results with larger beam steering granularity

|  | 6x6, 22.5deg beam steering granularity | 4x2, 22.5deg beam steering granularity |
| --- | --- | --- |
| Gain drop from peak | 3.13dB | 2.09dB |

By comparing the simulation results from Table 1 to 3, it can be observed that the test tolerance is not only sensitive to array size, but more sensitive to beam steering granularity.

Observation 1:	TTFiniteRes is not only sensitive to array size, but more sensitive to beam steering granularity

Given this study item is to study non-handheld UE only and just prioritize PC1 and PC5 FWA UE, it is okay to set TTFiniteRe as 0.9dB, but in TR the applicable device types should be explicitly stated. For other device types rather than PC1 and PC5 FWA, further analysis is needed based on suitable antenna configuration and beam steering granularity.

Proposal 4:	RAN4 to explicitly state in TR that the TTFiniteRes value is only applicable to PC1 [and PC5] FWA UE. The same value can not be extended to other device types without further analysis with suitable antenna configuration and beam steering granularity

3. 	Conclusion

Proposal 1:	The legacy 0.7dB systematic MU due to finite measurement grid should be kept as is.

Proposal 2:	Do not consider 4x2 array with 4deg beam steering assumption when deriving test tolerance for PC1 and PC5 FWA UE.

Proposal 3:	RAN4 to take both 12x12 and 6x6 array as assumption for PC1 and PC5 FWA UE in the test tolerance simulation, and accordingly the TTFiniteRes = 0.9dB.

Observation 1:	TTFiniteRes is not only sensitive to array size, but more sensitive to beam steering granularity

Proposal 4:	RAN4 to explicitly state in TR that the TTFiniteRes value is only applicable to PC1 [and PC5] FWA UE. The same value can not be extended to other device types without further analysis with suitable antenna configuration and beam steering granularity

4. 	References

- R4-2508663    “WF for [115][338] NR_FR2_OTA”, Qualcomm

- R4-2506974    “On additional Test Tolerance for STxMP testing”, Samsung

- R5-198203    “MU Assumptions for PC1 Fixed Wireless Access (FWA) UE”, Qualcomm

- R5-227982    “PC5 device assumptions”, Qualcomm