3GPP TSG-RAN WG4 Meeting #116	R4-2510241

# Bengaluru, India, August 25th – 29th, 2025

Agenda Item:	7.18.2

Source:	CAICT, Ericsson, Qualcomm, APPLE, Huawei, Hisilicon, OPPO, CATT, CMCC, NTT DOCOMO, INC., Vivo, NTU, Nokia, Xiaomi, Mediatek, Rohde & Schwarz, Samsung, Intel, ZTE Corporation, Sanechips, Korea Testing Laboratory

Title:	Proposed update for TR 38.843 with RAN4 part

Document for:	Discussion

1	Introduction

In this contribution, we provide update for TR 38.843 with RAN4 part with the agreements till RAN4#115.

2	Text Proposal

------------------------------------------- Change --------------------------------------------------------

3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

…….

AI/ML model delivery: A generic term referring to delivery of an AI/ML model from one entity to another entity in any manner. Note: An entity could mean a network node/function (e.g., gNB, LMF, etc.), UE, proprietary server, etc.

AI/ML model Inference:  A process of using a trained AI/ML model to produce a set of outputs based on a set of inputs.

AI/ML model testing: A subprocess of training, to evaluate the performance of a final AI/ML model using a dataset different from one used for model training and validation. Differently from AI/ML model validation, testing does not assume subsequent tuning of the model.
NOTE: the term is not applicable for performance/conformance testing.

……….

AI/ML model transfer: Delivery of an AI/ML model over the air interface in a manner that is not transparent to 3GPP signalling, either parameters of a model structure known at the receiving end or a new model with parameters. Delivery may contain a full model or a partial model.

AI/ML model validation: A subprocess of training, to evaluate the quality of an AI/ML model using a dataset different from one used for model training, that helps selecting model parameters that generalize beyond the dataset used for model training.
NOTE: the term is not applicable for performance/conformance testing.

Data collection: A process of collecting data by the network nodes, management entity, or UE for the purpose of AI/ML model training, data analytics and inference.

……..

Functionality activation: A process of enabling an applicable functionality to perform inference.

Functionality deactivation: A process of network deactivating an active functionality.

……

7.4	Interoperability and testability aspects

7.4.1	Introduction

In this section, the study of requirements and testing frameworks to validate AI/ML based performance enhancements and ensuring that UE and gNB with AI/ML meet or exceed the existing minimum requirements, if applicable, are documented.

The need and implications for AI/ML processing capabilities definition is considered.

7.4.2	Common framework

7.4.2.1 General

The general requirements and testing frameworks for AI/ML based performance enhancements mainly focus on

- how to define requirements and tests for inference

- evaluate feasibility and necessity of requirements/tests for LCM

- requirements for data collection (in particular for training) could/need be defined

Requirements/tests for training will not be studied unless training procedures are defined. The design of test should ensure performance is guaranteed and avoid that a UE can pass the test but perform poorly in the field.

The testing goal is to verify whether the minimum performance of AI/ML functionality/feature can be achieved

- FFS on whether and how many different test configurations/parameters are used for tests

- LCM would be tested

7.4.2.2 Principles on the definition of requirements

For the definition of AI/ML requirements, the following cases related to legacy performance should be considered

- For the cases with the existing legacy performance

  - Take the legacy performance as baseline for existing use cases/procedures/functionalities /measurements that are to be enhanced by AI/ML based methods

    - Further study may be needed on what is baseline performance in conditions different to the requirement condition but within the expected range of operation.

  - New or enhanced performance requirements/tests could be considered for existing use cases/procedures/functionalities/measurements that are to be enhanced by AI/ML based methods

- For the cases without the existing legacy performance

  - New performance requirements/tests could be considered for the use cases/procedures/functionalities/measurements that are carried out or are to be enhanced by AI/ML based methods

The following procedure can be considered for defining core requirements

- Performance monitoring procedure, including performance evaluation and decision-making procedure for AI/ML functionalities/models

- Functionality/Model management procedure, including functionality/model selection/activation/deactivation, and functionality/model switching/fallback/transfer/delivery/update

- Latency/interruption requirement for above procedures

Inference latency can be implicitly captured in the delay of corresponding procedures (e.g. reporting delay) and/or performance requirements. Inference latency is part of delay of the corresponding procedure and no separate requirement will be specified for inference latency. Delay requirements for UE sided reporting is defined in case-by-case manner.

The following LCM related requirements can be considered:

- Model/Functionality select/switch/activate/deactivate/fallback

- Model/Functionality monitoring

- On whether requirements for data collection (in particular for training) could/need be defined:

  - Data collection requirements would only be defined if data collection procedure is defined in 3GPP specifications.

- On requirements for model transfer/update:

  - Requirements would only be defined if model transfer/update would be defined in 3GPP specifications.

Legacy RRM requirements (non-AI/ML) are applicable to the corresponding legacy RRM procedures even during the AI/ML operation mode and RAN4 to assess what RRM requirements are needed for each case. The transition from AI to non-AI would be UE initiated or network initiated. The legacy framework for RRC/MAC-CE/DCI based core requirements (e.g., define delay requirements based on multiple delay components) can be used as the baseline for LCM procedures if the LCM related requirements are agreed to be introduced. If new procedures which legacy framework is not applicable to are introduced, additional core requirement framework can be discussed.

LCM related tests should consider how the framework can address the possibility of updates/activation/deactivation /switching to the functionalities/models after the deployment of the devices in the field.

Both static and non-static scenarios/configurations could be needed for AI testing.

- Static: channel model and SNR settings are fixed and do not change over the test, specific channel realizations may be dynamic

- Non-static: Non-static scenarios/configuration can be further considered in application to use cases. The details of models are FFS and may include non-stationary SNR and other conditions.

How to use them, including whether to use static scenarios/configurations as baseline need to be discussed and decided case by case. Static scenarios are assumed by default. Non-static scenarios are introduced on a use-case specific basis if needed for testing.

7.4.2.3 Reference block diagrams for testing

Reference block diagrams provide test modules/functionalities of TE/DUT and testing framework for different use cases. Both reference block diagrams for 1-sided model and 2-sided model are studied.

7.4.2.3.1 Reference block diagram for 1-sided model

Figure 7.4.2.3.1-1 provides the reference block diagram for 1-sided model. LCM in the figure includes functionality and/or model ID based LCM. The link between TE and DUT are physical and not logical. The logical link will depend on the functionality being tested. The scope of the figure includes both performance and potentially LCM tests. Offline training is assumed and some blocks may not be used in some of the tests. LCM may not be tested depending on the purpose of the test.

LCM

Verification

AI/ML functions

Test configuration/controller

Signal generator

inference

LCM

DUT

TE

Figure 7.2.3.1-1 Reference block diagram for 1-sided model

7.4.2.3.2 Reference block diagram for 2-sided model

Figure 7.4.2.3.1-2 provides the reference block diagram for 2-sided model. LCM in the figure includes functionality and/or model ID based LCM. The link between TE and DUT are physical and not logical. The logical link will depend on the functionality being tested. The scope of the figure includes both performance and potentially LCM tests. Offline training is assumed and some blocks may not be used in some of the tests. LCM may not be tested depending on the purpose of the test.

LCM

Verification

AI/ML functions

Test configuration/controller

Signal generator

inference

LCM

DUT

TE

inference

Figure 7.2.3.1-2 Reference block diagram for 2-sided model

7.4.2.4 Test encoder/decoder for 2-sided model

(NOTE: At the current stage the framework in this session applies to CSI compression case.)

In order to determine the test encoder/decoder, the following issues are considered:

- Common assumptions for proposals of the test decoder / encoder (and the paired encoder/ decoder) for tester

- The need for and potential definition and derivation procedure of intermediate KPI for decoder evaluation and selection

- Data collection/generation for decoder evaluation, and the common assumptions/environment needed for data collection/generation

- How to minimize the impact of possible variations/differences in the test decoder/ test encoder design/implementation on UE/ gNB performance verification

- The impact of test decoder/ encoder for testing complexity to UE/gNB performance verification, and the advantage/disadvantage analysis of high/low complexity decoders.

The test decoder/encoder design should take into account complexity limitations based on e.g., feasibility of TE implementation and complexity levels considered feasible by network vendors/UE vendors for decoder/encoder deployment.

The choice of test decoder/encoder should aim as much as possible to avoid limiting the implementation choices, including e.g. complexity, back-bone model etc, of UE/gNB encoders/decoders operating in the field (this principle may not be fully achievable in practice).

Specification on the test may include some high-level parameters for the test decoder/encoder (e.g. parameters related to processing complexity, model structure, etc).

Following the above principles, the considered options of test decoder are listed below

- Option 1: DUT provides the decoder

- Option 2: Infra vendor provides the decoder

- Option 3: Full decoder specification in standard

- Option 4: TE vendor provides the decoder

Option 3 target is that a single decoder defined in the specifications for at least a single test for any DUTs.

For option 4, the following aspects should be considered

- TE vendor should be able to develop the decoder based on the specifications

- Test repeatability should be ensured (variation among TE vendor implementations should be bound)

- Other vendors should also be able to develop such a decoder and which can deliver similar performance

- Interoperability should be ensured based on the parameters that need to be specified

  - Parameters that need to be specified are FFS

- Candidate parameters/conditions that may be considered for defining test decoder include

  - Training data set for TE decoder training

  - Model structure (Activation function is included in the model structure)

  - Performance parameters for the TE decoder (e.g. cosine similarity, loss function, etc)

  - Maximum FLOPs allowed for the test decoder

  - Maximum number/size of model parameters

  - Compression ratio of decoder (output size/input size)

  - Quantization level

  - Other parameters are not precluded and to be further discussed.

  - Note: Feasibility of definition of parameters needs further investigated.

Option 4 target is that a single decoder implemented by each TE vendor will be enough for at least a single test for any DUTs. TE vendor should be able to implement the test decoder for Option 4 without any involvement from another party. If this is found infeasible, another option in which TE vendors need to collaborate with DUT/infra vendors to implement the decoder could be considered.

Further clarifications and analysis of the four options of test decoder are included in table 7.4.2.3-1. It is assumed that for Option 4 the TE vendors can implement the decoder just based on the specifications (no other party involved). The table would need to be revised if collaboration between TE vendor and DUT/infra vendor is needed.

Table 7.4.2.4-1 Comparison of the four options of test decoder

|  | Option 1 | Option 2 | Option 3 | Option 4 |
| --- | --- | --- | --- | --- |
| Clarification of options |  |  |  |  |
| Source of the test decoder | DUT vendor | Decoder vendor (infra vendor in case of testing UEs) | RAN4 specifications | TE vendor, decoder developed based on RAN4 specifications |
| Source of decoder training data | Up to DUT vendor (no need to be specified) | Up to decoder implementer (infra vendor) FFS whether coordination with encoder vendor is required | Not needed, decoder fully specified (used as part of the RAN4 procedure to specify the decoder) | FFS Could be specified depending on how Option 4 will be defined |
| DUT vendor knowledge of the test decoder | Full knowledge | No or partial or enough or full knowledge based on alignment with infra vendors or specifications | Full knowledge based on the specifications | Partial knowledge – based on the RAN4 specification |
| Supported training collaboration type between DUT and decoder provider (source of training data should be consistent with the collaboration type) |  |  |  |  |
| Test decoder performance verification procedure at TE | Need to ensure that decoder performance is not degraded (as intended by the decoder provider) on the TE | - Need to ensure that decoder performance is not degraded (as intended by the decoder provider) on the TE - Need to ensure that decoder performance is good enough to enable a DUT that meets the minimum requirements to pass the test | Not needed as long as the standardized model implementation can be similar enough between TE vendors | Not needed as long as the model implementation can be similar enough between TE vendors |
| Feasibility of test decoder verification procedure | FFS | FFS | FFS | FFS |
| Pros/Cons analysis |  |  |  |  |
| Reflection on the real deployment (likelihood that test decoder would be used in actual field deployments ) |  |  |  |  |
| TE requirements to deploy the decoder (e.g. training, complexity, interoperability) | Higher than Option 3/4 in terms of that maybe more than one decoder are implemented by TE Lower than Option 3/4 in terms of that no training at TE is required | Higher than Option 3/4 in terms of that maybe more than one decoder are implemented by TE Lower than Option 3/4 in terms of that no training at TE is required | Lower complexity than Option 1/2 in terms of that only one decoder is implemented by TE Lower than Option 4 in terms of that no training at TE is required | Lower complexity than Option 1/2 in terms of that only one decoder is implemented by TE Higher than Option 3 in terms of that training at TE is required Note: How to ensure compatibility/interoperability between TE and DUT needs further study. |
| Specification Effort (defining test decoder and requirements) | Low | Low | Highest RAN4 needs to standardize the entire decoder | High RAN4 needs study and decide on what to standardize |
| Confidentiality/ IP issues in the testing procedure(after specs are published) |  |  | No | No |
| Applicability to different scenarios/conditions/ configurations |  |  |  |  |
| Complexity of testing for the ecosystem | Testing the encoder at DUT Higher than Option 3/4 Need for interaction between TE vendor | Testing the encoder at DUT Higher than Option 3/4 Testing complexity higher also than option 1. | Testing the encoder at DUT Low – no need for interaction between TE vendors and other parties | Testing the encoder at DUT Low – no need for interaction between TE vendors and other parties |
| Complexity of verifying/testing the test decoder | Higher than option 3/4 FFS compared to option 2 | Higher than Option 3/4 FFS compared to Option 1 | Low | Low |
| Complexity of deploying for the ecosystem |  |  |  |  |
| Friendly to STOA(state of the art) model test / Forward compatibility when new AI models are invented |  |  |  |  |
| Relationship with reference decoder/encoder(used by RAN4 to define the performance requirements) for defining requirement |  |  |  |  |
| Whether model transfer/delivery is needed during the test procedure |  |  |  |  |

7.4.2.4.1 Terms for feasibility study

The following terms are defined for the feasibility study of test encoder/decoder for 2-sided model

Reference decoder: A decoder used in RAN4 discussions at least for simulation alignment/requirement derivation and/or verification of the decoder implemented by the TE.

Reference encoder: An encoder used in RAN4 discussions at least for simulation alignment/requirement derivation, test decoder derivation and/or test decoder verification.

7.4.2.4.2 Evaluation assumptions for option 3 and 4 feasibility study

Table 7.4.2.4-2 provides the agreed parameters for verifying the feasibility of aligning model among companies and additional parameters of the encoder/decoder and feedback are given in Table 7.4.2.4-3.

Table 7.4.2.4-2 System-level simulation parameters for verifying the feasibility of aligning model

| Parameter |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD, OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Dense Urban (Macro only) |
| Frequency Range |  | FR1 only, [2GHz, 4GHz] |
| Inter-BS distance |  | 200m |
| Channel model |  | According to TR 38.901 |
| Antenna setup and port layouts at gNB |  | Companies need to report which option(s) are used between - 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ |
| Antenna setup and port layouts at UE |  | 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for (rank 1-4) |
| BS Tx power |  | 44dBm for 20MHz |
| BS antenna height |  | 25m |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | Baseline: 15kHz for 2GHz; Optional: 30kHz for 4GHz |
| Simulation bandwidth |  | Baseline: 10 MHz for 15kHz Optional: 20 MHz for 30kHz |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| MIMO scheme |  | SU-MIMO |
| MIMO layers |  | Baseline: 1 Optional: 2 |
| CSI feedback |  | Feedback assumption at least for baseline scheme - CSI feedback periodicity (full CSI feedback): 5 ms (baseline) - Scheduling delay (from CSI feedback to time to apply in scheduling): 4 ms |
| Traffic load (Resource utilization) |  | Baseline: 50% Optional: 20/70% |
| UE distribution |  | CSI compression: 80% indoor (3 km/h), 20% outdoor (30 km/h) |
| UE receiver |  | MMSE-IRC as the baseline receiver |
| Feedback assumption |  | Realistic |
| Channel estimation |  | Realistic or ideal channel estimation |

Table 7.4.2.4-3 additional parameters of the encoder/decoder and feedback

| Assumptions |  | Value |
| --- | --- | --- |
| CSI generation part | AI/ML model backbone | MLP, CNN, Transformer |
|  | Pre-processing | SVD to get channel eigenvectors |
| CSI reconstruction part | AI/ML model backbone | [MLP, CNN, Transformer] |
| Common description | Input type | Eigenvectors of channel matrix |
|  | Output type | Eigenvectors of channel matrix |
|  | Quantization /dequantization method | Scalar quantization |
| Dataset description |
|  | Train/Test | 10% of the training dataset as baseline |
|  | Ground-truth CSI quantization method (including scalar/codebook based quantization, and the parameters) | Floating point (float 32) |
| Other parameters | Laten/reporting size | [X: below 80bits, Y: 100 – 140 bits] |

Logistical issues:

For AI/ML model formats:

- Use at least ONNX format for sharing.

- Companies can also share PyTorch model

- The models in different formats from one company shall be the same

- Use version v1.16 or later

- ONNX version and opset version number to be included in the file

For dataset format

- Use NumPy for dataset sharing

- Do not use pickled data(compression mechanism)

- Use npy – single array in each file

Dataset/model input file format:

- N (samples) X 2 (IQ) X nSB (number of subbands) X nPorts (number of CSI-RS ports) X nLayers (number of layers)

        - In other words, dataset should content following info:

        - 1st dimension: Number of samples

        - 2nd dimension: Real and imaginary

        - 3rd dimension: Number of sub-bands

        - 4th dimension: Number of antenna ports

        - 5th dimension: Number of layers – present only if >1 layer

        - Note: Each element of the dataset will be a float32 real number

The dataset includes eigenvectors calculated based on average covariance channel matrix for the given MIMO layer for each sub-band based on pre-processing assumptions in WF R4-2414447

Single file for fixed scalar quantization(only dataset), TBD whether additional file is needed for variable/trainable quantization

Dataset files can be split into multiple files to enable easier upload.

- Use the 2 digits for split files starting from 00, increment for each additional file

Split files and then archive each file.

For file naming scheme

- Folders for AI/ML data sharing and current WI/use case to be created under “RAN4 folder”

- Subfolder created for each meeting

File naming scheme (ML model file and dataset file)

- a unique identifier for the company (4 characters – list to be maintained by RAN4 secretary)

- meeting number

- differentiate model and dataset by identifier (2 characters ml and ds)

- additional identifier with 2 characters (TBD)

        - ec for encoder

        - dc for decoder

        - ei for encoder input

        - do for decoder output

        - lt for latent (decoder input or encoder output)

        - others to be added as needed

- dataset can be split in multiple files – 2 digits

- files to be compressed to zips and uploaded

Files to be shared, to be decided based on option under study

- 	Dataset containing encoder input per subband

- 	Encoder and/or decoder model

7.4.2.4.3 Feasibility study of Option 3

The basic step for the feasibility study of Option 3 is as follows:

- Step 1: Based on the test decoder and reference encoder AI/ML model structure, companies individually jointly train the reference encoder and test decoder based on a dataset. Check on performance alignment -> see simulation results from contributing companies

  - The dataset used for jointly training can be company specific training dataset or mixed training dataset

  - repeat simulations until good alignment is achieved

  - move to next step after alignment

- Step 2: Share models(encoder or decoder or both)/datasets (training/testing/inference)

  - framework to share data among companies(dataset/model) to be discussed

- Step 3: Select one or more decoder for further analysis

  - selection criteria to be discussed

- Step 4: Each company brings results for training of “own encoder” with selected decoder(s)

  - performance alignment to be checked/discussed

  - using “own” data or data shared by other companies or mixed test dataset

- Step 5: Conclude on overall feasibility of Option 3

  - consider the conditions under which Option 3 is feasible if found feasible

  - FFS on what the feasibility criteria

It should be noted that parameters agreed are just for the feasibility study of testing options and if/when RAN4 discusses requirement definition, RAN4 will define a new test decoder which may or may not reuse any of the parameters agreed in the feasibility study.

Two options are listed for test decoder derivation procedure:

- Option 1: companies bring encoder + decoder set based on agreed parameters. RAN4 chooses one of the decoders and interested companies further check if an encoder can be trained with this decoder to obtain similar performance/complexity (or other evaluation criteria)

- Option 2: companies bring training data for encoder + decoder pair, interested companies train an encoder + decoder pair based on the aggregated dataset from all companies to check decoder derivation feasibility and performance/complexity (or other evaluation criteria)

Option 1 is agreed to work on. MLP and CNN are chosen for further evaluation.

For decoder(s) selection in step 3, Choose the decoders with Low, medium, high SGCS among the decoders submitted by companies. Continue the feasibility study on the two tracks below:

Track 1(Decoder trained over company own dataset): Select 3 decoders, which are trained in step-1 with companies’ own training dataset. Companies train own encoders (with own Eigenvector dataset) and check performance against decoders (with at least own test dataset)

- The 3 decoders are selected based on low, medium, high mean SGCS out of available decoders

- At least an encoder with the agreed model structure should be considered when making own encoder. Optionally companies can try own encoder with their preferred structure. Companies should report the structure if different.

Track 2(Decoder trained over mixed dataset): Create a mixed dataset, companies train decoders based on the mixed dataset in Step-1. One or more decoder selected and companies develop encoders and check encoder performance with own or the mixed dataset checked against decoder(s).

- At least an encoder with the agreed model structure should be considered when making own encoder. Optionally companies can try own encoder with their preferred structure. Companies should report the structure if different.

For track 1, decoders with 2 bit quantization, and with no quantization are provided

- No quantization aware training when model is unquantized

- Quantization aware training when model is quantized

- For the model with quantization:

  - Agree on a scaling and quantization codebook

  - Include Sigmoid in model file in the encoder, and inverse Sigmoid in the decoder

  - Codebook (1/8, 3/8, 5/8, 7/8) in model file

  - Do not including mapping to 2 bits. Quantizer function of converting to 2 bits should not be in the model file

- For the model without quantization

  - For no quantization models, do not include sigmoid at the model output and no inverse Sigmoid in the decoder

For the input / output dimensionality of track 1,

- define Encoder Input and Decoder Output as data dimension (n, 2, 13, 32) with data type float32 where n is the dynamic batch size

- define Encoder Output and Decoder Input as data dimension (n, 32) with data type float

- No additional inputs

Take mixed dataset comprising Mediatek, Ericsson, Vivo, Oppo, Nokia.

CSI_Feasibility Table1 in attached Spreadsheets provide simulation results for the feasibility study of option 3.

Preliminary observations for Dataset

- Datasets consist of separate training data and separate test data.

- All companies have produced datasets based on the same simulation assumptions, however differences can be observed between performance with different companies datasets.

- The “mixed dataset” provides much more consistent results than training or testing using different individual company datasets

- The exercise demonstrates that in order to agree on test decoder(s) and or reference encoder(s), some effort is needed to align dataset(s).

The focus in this study has been on convergence between SGCS reported for different company UE encoders when operating with test decoders, in order to understand whether it is feasible to specify a test decoder (or, for option 4, information used to create a test decoder) that can be used for consistent performance testing across multiple UE vendor implementations. Absolute SGCS has not been considered, since the assumed model structure is somewhat arbitrary and the purpose of the study has been to consider testing.

Observations for option 3 track 1

- It is possible for UE vendors to train an encoder based on another company’s frozen test decoder that is functional

- The performance of the encoder is very dependent on the dataset used for training the encoder and the dataset used for training the frozen decoder, and the testset

  - If the encoder is trained using a dataset of the UE vendor, the UE vendor dataset should contain the [features] captured in the dataset used to develop the test decoder.

- The variation in the performance depending on the training datasets and test datasets can be as much as 30-50% in SGCS. Further details are available in the results spreadsheet.

- For option 3, track 1 to be useful, more careful attention would be needed to the alignment of datasets

- Only one encoder structure was considered for option 3 track 1. Other lower complexity encoder structures were not examined.

Observations for option 3 track 2

- There is reasonable SGCS convergence when companies jointly train an encoder-decoder pair using the mixed dataset and test using the mixed testset

  - Around +-2% variation in SGCS; see results spreadsheet for more details

- It is possible for UE vendors to train an encoder based on a frozen test decoder that is functional

- When training “own” encoder with the frozen decoder, with all training (frozen decoder and encoder) and testing using the mixed dataset, there is good convergence in average SGCS results

  - Around - 2% to +1.4% variation in SGCS from the average

- When training “own” encoder with the frozen decoder, with training of the frozen decoder using the mixed dataset, own encoder using own dataset and testing using the mixed dataset, there is convergence in average SGCS results from results presented so far

  - Around -+-4% variation in SGCS from the average (+-1.5% with outlier performance excluded)

- When training “own” encoder with the frozen decoder, with training of the frozen decoder using the mixed dataset, own encoder using own dataset and testing using the own dataset, there is much less convergence in average SGCS results from results presented so far

  - Around -40% to 13% variation in SGCS from the average (+-8% to 5% with outlier performance excluded)

- The results considering “own” datasets demonstrate that there is divergence in the datasets, and attention must be paid to alignment of the training datasets.

  - Note: Alignment of datasets does not refer to aligning UE algorithms such as channel estimation, CSI estimation etc.

Potential specification impact

For option 3, for each test, a test decoder needs to be captured in the specs, a reference encoder might be needed to derive the test decoder and/or requirements and/or to validate the test decoder implementation in the TE. the same decoder might be used in multiple tests or each test could have a difference decoder. In option 3 the reference decoder is the test decoder. The training dataset used to train the test decoder in 3GPP should be captured and FFS whether channel, Eigenvectors or something else.

The need to define a reference encoder was discussed for the test decoder for option 3 and it is agreed that

- To verify the performance of CSI compression test decoder for calibration of test equipment, agree to define the reference encoder including encoder structure and parameters in RAN4

  - FFS on whether to specify it in TS or TR, and capture the conclusion in the TR conclusion part

  - FFS on criterion for selection of reference encoder

  - FFS on decision whether RAN4 or RAN5 will specify it

- The encoder that corresponds to the standardized test decoder is stored within 3GPP

  - TBC whether in a TR, or in any TS, whether in RAN4 or RAN5, or whether simply kept in a 3GPP database (with no reference from TR or TS)

  - FFS whether several encoders would be captured (e.g. due to different complexity)

7.4.2.4.4 Feasibility study of Option 4

The feasibility of two option 4 sub-options is studied:

- Option 4a (Dataset based): specify at least dataset based on which the test decoder can be implemented by TE vendors

  - FFS whether anything else needs to be specified (e.g. model structure for decoder or a reference encoder)

- Option 4b (Encoder based): encoder is documented in the specifications, used to derive a decoder to be implemented by TE vendors

  - FFS whether training dataset (channel information) needs to be specified

The basic step for the feasibility study of Option 4a is as follows

- Step 1-3: Reuse results of Option 3

- Step 4: Select one or more Eigenvector dataset(s) for further analysis based option 3

  - Selection criteria: select the dataset(s) generated from the selected encoder and decoder pair(s) from Option 3 track 1

  - Or mixed dataset from track 2

  - If multiple datasets are selected (as was the case of Option 3 track 1), the subsequent steps of the feasibility procedure will be applied on each one of the selected datasets separately

- Step 5: Label selected dataset with encoder input/output using encoder corresponding to the selected test decoder from option 3.

- Step 6: Companies bring results for training of “own encoder(s) and decoder(s)” with selected dataset(s)

  - Performance alignment to be checked/discussed

- Step 7: Conclude on overall feasibility of Option 4a

  - Feasibility criteria to be discussed (e.g. Perform testing of all the UE encoders using all test decoders (replicating the process of testing UE’s against RAN4 requirements with different TE vendor decoders, others given in R4-2415376, etc ).

  - Dataset for feasibility evaluation to be discussed; may be common test dataset or own test dataset

The basic step for the feasibility study of Option 4b is as follows:

- Step 1-3: Reuse results of Option 3

- Step 4: Select one or more encoder(s) for further analysis based on option 3

  - Selection criteria: select the encoder(s) from the selected encoder and decoder pair(s) of Option 3 track 1

  - Or encoder(s) from the selected encoder and decoder pair(s) of Option 3 track 2

- Step 5: Company brings results for training of “own decoder(s)” with selected encoder(s)

  - Performance alignment to be checked/discussed

  - FFS what is the dataset assumed for the training

- Step 6: Company trains “own encoder(s)” with “own decoder(s)” from step 5

  - FFS what is the dataset assumed for the training

- Step 7: Conclude on overall feasibility of Option 4b

  - Feasibility criteria to be discussed (e.g. Perform testing of all the UE encoders using all test decoders (replicating the process of testing UE’s against RAN4 requirements with different TE vendor decoders, others given in R4-2415376, etc ).

  - Dataset for feasibility evaluation to be discussed; may be common test dataset or own test dataset

Encoder in step 5 should at least have the agreed structure. Companies can also bring analysis/results with other encoders (using different structures)

CSI_Feasibility Table2 in attached Spreadsheets provide simulation results for the feasibility study of option 4.

Observations for option 4

- For option 4b: It is feasible to train a functional “own” decoder based on the frozen encoder, and then a functional “own” encoder based on the “own” decoder when the mixed dataset is used

- For option 4a: For the mixed dataset, a similar observation to option 4b can be expected (in this case, the labelled dataset would be the same latent space as the selected encoder (created using joint training) acting on the mixed dataset, and so option 4a converges to option 4b)

- If the frozen encoder is created using the mixed dataset, TE/UE vendors create decoders using the mixed dataset, UE vendors create an encoder using the mixed dataset and testing uses the mixed testset then good convergence is seen in SGCS

  - Around +- 1.5% from average SGCS

  - UE encoders were shown to be interoperable with all companies own decoders

  - This represents the situation where there is a well defined dataset, and UE vendors use the same dataset to train their encoder models

- Some companies observed that

  - if the frozen encoder is created using the mixed dataset,

  - TE/UE vendors create decoders using the mixed dataset,

  - UE vendors create an encoder using their own dataset and

  - testing uses the mixed testset

then [good] convergence is seen in SGCS, but there is more variation than when mixed dataset is used for training the encoder

  - Around [+- 3%] from average SGCS

    - [Further SGCS results may be provided in the next meeting]

  - [UE encoders were shown to be interoperable with all companies own decoders]

  - This represents the situation where there is a well defined dataset that is used for all 3GPP responsibilities (Creating the frozen decoder, providing information for the TE vendor to train the decoder, providing test data), but the UE vendor uses their own dataset to create the encoder

- Some companies observed that if the frozen encoder is created using the mixed dataset,

  - TE/UE vendors create decoders using the mixed dataset,

  - UE vendors create an encoder using their own dataset and

  - testing uses the own testset

then [convergence is seen in SGCS,] but there is more variation than when mixed dataset is used for training the encoder. The variation is similar between using the mixed dataset and own dataset for testing.

  - Around [+- 1.6 to +4.7%] from average SGCS

  - [Further SGCS results may be provided in the next meeting]

  - [UE encoders were shown to be interoperable with all companies own decoders]

  - This represents the situation where there is a well defined dataset that is used for all 3GPP responsibilities (Creating the frozen decoder, providing information for the TE vendor to train the decoder, providing test data), but the UE vendor uses their own simulation to create the encoder

- Two lower complexity encoders were considered and tested based on the frozen decoder.

  - A transformer with 3.63 MFlops

  - A CNN encoder with 4.14 MFlops

- When the lower complexity encoders were examined using the mixed dataset for all training and the mixed test set for testing, SGCS convergence (- 1 to 3.5%) and interoperability with all companies “own” decoders was observed for the considered structures.

Potential specification impact

For option 4a, the following aspects should be considered to be specified:

- Dataset is specified for option 4a.

- Assume reference decoder is captured.

- Reference encoder should be captured

- FFS whether the labelled dataset is based on Eigenvector, channel estimation or something else

- The study has only considered the case of a common assumption on model structure for the “own” test decoder. This corresponds to standardized model structure.

  - Structure refers to backbone, numbers of layers, type of layers and all description of the model, but not the parameters.

  - Interoperability in the case that the structure of the TE decoder is not specified has not been investigated

For option 4b, the following aspects should be considered to be specified:

- Assume reference decoder is captured.

- Specify dataset to train frozen encoder and decoder

  - TE vendor shall use the dataset to train the test decoder

  - This is the same dataset as used to train the frozen encoder

  - FFS whether the datasets consist of Eigenvector, channel or something else

- The study has only considered the case of a common assumption on model structure for the “own” test decoder. This corresponds to standardized model structure.

  - Structure refers to backbone, numbers of layers, type of layers and all description of the model, but not the parameters.

  - Interoperability in the case that the structure of the TE decoder is not specified has not been investigated

7.4.2.5 Data collection/generation for testing

Different generating methods of test dataset can be used for different tests. The following candidate methods are to be considered:

- Dataset based on TR 38.901, e.g. UMa channel, UMi channel, CDL channel, “legacy approach”, etc.

  - “Legacy approach” refers legacy test in which a channel model is used

- Field dataset (data collected directly from field measurements)

- TE generates dataset for test based on assumptions/parameters defined by RAN4 (e.g. by defining some rules/function to generate data)

- Other methods are not precluded

Synthetic channels are used as baseline, and check whether it can be used for the individual use case. An analysis for each use case to determine the reliability of using synthetic channels for test data in evaluating models trained on real data may be conducted and the field data can be considered for the analysis.

7.4.2.6 Data collection/generation for training

Some conditions and/or accuracy requirements for the training dataset or training data generation could only be introduced if the training procedure is defined in 3GPP specifications.

7.4.2.7 Generalization/scalability aspects

The necessity and feasibility of defining requirements or test to verify the generalization of AI/ML is studied.

The goals of generalization test are to verify whether the minimum level of performance of AI/ML functionality/model can be achieved/maintain under the identified scenarios and/or configurations, while the performance won’t be significantly degraded in other scenarios and/or configurations. The following aspects should be considered for generalization/scalability related testing:

- details about the scenarios and/or configurations for test and the corresponding AI/ML models/functionality

- what the minimum level performance for each identified scenario and/or configuration is

- what the significant degradation for other scenarios and/or configurations is

It should also be considered that generalization and/or scalability related requirements for different scenarios/ configurations can be implicitly handled in the test case definition.

The high level and general guidelines for the generalization test are provided as follow. The exact decisions are to be made case by case.

- define requirements for each AI/ML functionality

- Define one test per UE capability as a minimum

  - FFS >1 test per UE capability

- Define a minimum set of test configurations (including NW sided conditions if any) as mandatory for testing of AI/ML-enabled Feature

As for the handling of generalization tests, the following option is considered as baseline:

Signaling based LCM procedures and performance monitoring are considered in dedicated test cases and are excluded in tests verifying generalization. RAN4 may define multiple tests with different conditions. In each of the test, TE configures the same specified UE configuration, and therefore the same specified UE configuration is tested under different conditions to verify its generalizability. (environment differs in each test but not changing dynamically during the test)

- Specified UE configuration includes functionality and/or model ID if defined

There shall be consistency between applicable conditions signalling and testing. No test will be defined that implies a change of network condition or associated ID during the test. Whether a mixed dataset can be created for testing generalization, and whether such a mixed dataset would be a static or non-static scenario should be considered.

Potential areas to consider for generalization testing includes:

- gNB array parameters

  - port layouts, array size, antenna virtualization, codebook

- propagation conditions

- Deployment scenarios

  - Speeds

  - Indoor/outdoor

- SNR

7.4.2.8 AI/ML processing capability

The practical processing capability and implementation complexity for device under test should be assumed when specifying RAN4 requirements.

- The UE capability may be needed to handle different complexity for one side and two-side models.

- The complexity of UE should also be studied when making assumption on gNB side model, and vice versa.

7.4.2.9 Post deployment handling

When operating in the field, two aspects of UE operation may impact performance:

- Update or fine tuning or addition/removal of models, if applicable, resulting in a change of functionality which may impact functionality performance

- Data drift / mismatch between the conditions encountered by the UE in the field and the training data, which may impact functionality performance.

For dealing with drift / mismatch between the conditions encountered by the UE in the field and the training data for the model, which may impact functionality performance, monitoring is needed.

  - Performance monitoring will be designed in other groups

  - RAN4 may consider the need and feasibility of requirements and tests to ensure consistency and accuracy of monitoring metrics or other monitoring related data sent from the UE, and set requirements as feasible/needed.

  - Monitoring can be used for managing fallback, change in functionality, model update/model switching/model transfer, if applicable

For dealing with potential changes in the performance of functionalities, two options may be available. The options are not mutually exclusive:

- Option 1 (Post-deployment pre-activation functionality/configuration update testing): Conduct the validation of a change in AI model/functionality before its deployment/activation in already deployed UEs

  - Validation takes into account the UE hardware in which the model is to be deployed/activated.

  - FFS whether the validation takes place in the field (i.e., on each individual device) or in the lab conditions (i.e., per device type/model)

  - FFS whether validation can be done at the device along with inference for another active functionality(ies)

  - FFS on the feasibility

  - FFS on one possibility for consideration for option 1 is to capture model input (and if needed other test data such as ground truth) during conformance testing. This stored data can later be used to validate new or updated models.

  - Other aspects not precluded.

- Option 2 (Post-deployment post-activation functionality testing based on performance monitoring): Using performance monitoring and LCM procedures

  - Performance monitoring will be designed in other groups

  - RAN4 may consider the need and feasibility of requirements and tests to ensure consistency and accuracy of monitoring metrics or other monitoring related data sent from the UE, and set requirements as feasible/needed.

  - FFS on monitoring can be used for managing fallback, changes in functionality, model update/model switching/model transfer, if applicable

FFS: RAN4 needs to clarify whether changed functionalities that did not pass conformance testing are expected to be activated at the device.

Option2 is prioritized. Reliable and testable monitoring procedures can be established for all cases

7.4.3	CSI feedback enhancement

Both time domain CSI prediction and spatial-frequency domain CSI compression are considered.

PMI reporting framework (follow PMI vs. random PMI test, use of γ as criteria, etc.) is taken as starting point for CSI related tests. Other metrics/framework is not precluded.

For metrics for CSI requirements/tests, the following test metrics are identified:

- Option 1: Throughput/relative throughput

- Option 2: SGCS, NMSE

- Option 3: CSI prediction accuracy

Option 1 should be used as baseline. For option 3, further discuss is needed on the feasibility to define the CSI prediction accuracy in WI. For metrics for CSI monitoring, further discussion is needed in WI.

For CSI prediction, relative throughput is used as requirement metric. AI/ML based performance requirement should not be worse than the legacy (non-AI/ML based) if a relevant equivalent legacy requirement/feature exists.

7.4.4	Beam management

Both spatial-domain DL beam prediction and temporal DL beam prediction are considered.

For metrics for beam management requirements/tests, the following test metrics are identified and could be considered

- Option 1: RSRP accuracy

- Option 2: Beam prediction accuracy

  - Top-1 (%) : the percentage of “the Top-1 strongest beam is Top-1 predicted beam”

  - Top-K/1 (%) : the percentage of “the Top-1 strongest beam is one of the Top-K predicted beams”

  - Top-1/K (%) : the percentage of “the Top-1 predicted beam is one of the Top-K strongest beams”

- Option 3: The successful rate for the correct prediction which is considered as maximum RSRP among top-K predicted beams is larger than the RSRP of the strongest beam – x dB,

  - Related measurement accuracy can be considered to determine x

- Option 4: combinations of above options

The overhead/latency reduction should be considered for the requirements as the side condition. In addition, measurements period, TCI states and other requirements if needed should be defined

- Measurement period for prediction (how long will set B be measured before making a prediction)

  - Use “legacy” L1-RSRP measurement as the baseline for Case 1

    - FFS whether same values for M,N,P should be reused

  - FFS for case 2

- TCI state known/unknown

- Prediction delay includes measurement for prediction, inference delay and Other components

The Rx beam assumptions is listed:

- if the predicted Tx beam in Set A is QCL Type-D to a known Tx beam, the corresponding Rx beam is known.

- If the predicted Tx beam in Set A is not QCL Type-D to a known Tx beam, depending on UE capability, the corresponding Rx beam may be known or unknown.

- Known Rx beam means no additional Rx beam sweeping is needed

- Unknown Rx beam means additional Rx beam sweeping is needed.

- Rx beam is known mean TCI state is known if conditions below are also met

The following conditions related to whether the TCI state is known or unknown is defined as the starting point:

- The target TCI state is known when the following is met:

  - The TCI state switch command is received within 1280 ms upon the last transmission of the RS resource for beam reporting or measurement and the UE has sent at least one L-1  RSRP report of the target TCI state (clause 8.10.2 of 38.133)

  - Applies for case 1

  - FFS for case 2

  - Whether RS resource is only from set B should be clarified

    - FFS on whether prediction also impacts if a TCI state is known or unknown

      - Companies are invited to bring analysis on how prediction would impact whether a TCI state is known or unknown

      - FFS on the time delay if prediction is used as a condition for a known TCI state

The ground truth for the predicted RSRP is the ideal measurement of RSRP on the predicted Tx beam. The ground truth in RAN4 is the approximate as the reported RSRP measurement under the certain SNR on the predicted Tx beam and the following aspects should be considered

- SNR level to ensure that SNR is high enough for sufficient accuracy of reported RSRP

- impact of multiple-AoA test setup

- the channel condition

- whether the ground truth will be changing or not

If the absolute RSRP is agreed to be metric for the beam management prediction, the absolute RSRP accuracy for AI/ML based beam prediction = predicted L1-RSRP of beam index i – groundtruth of L1-RSRP of beam index i. The index i may be any beam index [in top-K beams based on groundtruth L1-RSRP]. Absolute RSRP accuracy requirement applies to Top-1 of predicted beams at least and FFS whether to apply to other beams.

If the relative RSRP accuracy requirement is defined, relative RSRP accuracy for reported beams during inference reporting = (predicted L1-RSRP of beam index i - reported L1-RSRP of beam index n) -  (ground truth of L1-RSRP of beam index i - ground truth of L1-RSRP of beam index n), [where the beam index n owns the largest reported value].

- Reported L1-RSRP can be predicted,

- FFS whether reported L1-RSRP can be measured RSRP

Relative RSRP accuracy requirements apply for Case 1, and FFS on whether it can apply for Case 2

Vendors may take into account the test environment/conditions defined by RAN4 when training the UE. Sufficient test environment/conditions should be defined to enable vendors to create the data needed for training. It is strived to make the test conditions similar to field deployment conditions. Augment training data to avoid overfitting of UE models to the test environment could be considered.

If RSRP and beam ID are reported:

- RSRP accuracy will be one of the KPIs.

  - How option 1 is applied is FFS

- Beam ID prediction accuracy will be one of the KPIs

  - FFS whether beam ID prediction accuracy refers to a combination of Option 2 and/or Option 3 or modified Option 2/3 or other options

If only beam ID is reported:

- Beam ID prediction accuracy will be used as the KPI

Beam ID prediction accuracy is the successful rate for the correct prediction, at least for the case if only beam ID is reported

- The correct prediction is considered as maximum ground-truth RSRP among top-K predicted beams larger than or equal to the ground-truth RSRP of the strongest genie-aided beam(s) – x dB,

- FFS on top-N (N</=K) predicted beams have to be considered in the success rate evaluation

  - FFS on N values

- FFS on x values

Test Setup

For the test setup, issues listed below are investigated in order to determine the capabilities of the test equipment:

- set B Tx beams; set A Tx beams

  - number of beams transmitted simultaneously

  - overall number of beams transmitted during the test

- AoA

- AoD

- UE rotation during the test

The maximum number of set B Tx beams that test system should be able to emulate: [8-16]. The maximum number of set A Tx beams that test system should be able to emulate: [64-128]. UE rotation/repositioning between different tests should be supported.

For Channel model, the following options considering TE feasibility is selected for down-select

- Option1: TDL

- Option 2: simplified CDL

  - CDL channel model with a small number of clusters. CDL simplification needs to be further discussed

For CDL or simplified CDL channel model, the relationship between AoD range [and granularity], number of beams and number of probes is studied

- Set A and B should be generated in the chamber for ground-truth extraction.

For TDL channel model, the relationship between [dynamic power range], number of beams and number of probes is studied.

- Set A and B should be generated in the chamber for ground-truth extraction.

  - For single AoA, FSS how to derive Set A

CDL-based channel model is used as starting point.

Measurement error study

RAN4 study the impact of measurement error for the input to inference and dataset for training (set B measurement error) on prediction accuracy. LLS is performed to evaluate the RSRP prediction accuracy and other KPIs with UE Model trained using ideal measurements.

[TBD]

7.4.5	Positioning accuracy enhancements

Both direct AI/ML positioning and AI/ML assisted positioning are considered.

For metrics for positioning requirements/tests, the candidate options include

- Option 1: positioning accuracy: Ground truth vs. reported

  - only option available for direct positioning

- Option 2: CIR/PDP, channel estimation accuracy

- Option 3: ToA, RSTD and RSRP, and RSRPP

- Option 4: others (e.g., intermediate KPIs, LoS/NLoS)/combinations of the above

The feasibility and testability of different options should be further justified in WI.

For case 1, do not define any positioning accuracy requirement,

For case 2a, no positioning accuracy requirements will be defined because positioning is LMF based. Existing requirements for existing metrics will be taken as baseline. How to define requirement for any new reported metrics needs further study.

For case 3a/3b, no positioning accuracy requirements will be defined. Extend applicability of existing report mappings for UL-RToA and UL SRS-RSRPP to report UL measurements for AI/ML based positioning use cases 3a/3b.