3GPP TSG-RAN4 Meeting #116	R4-2510242

Bangaluru, India, 25th – 29th August, 2025

Agenda item:			7.1.2.2.1

Source:	vivo

Title:	Discussion on UE RF power enhancement for NR single carrier

Document for:	Discussion

- Introduction

In RAN#115 meeting, the WF[1] has been agreed, and the remaining issues are as follows:

### Sub-topic 1-1: Applicable requirements #### Issue 1-1-1: Extension values/ratios - Agreement in main session: - At least ½ and ¼ are introduced as the extension ratio to apply to both single-sided and dual-sided cases - It is FFS on the details of symmetric and asymmetric cases for dual-sided case where non-zero extension is applied for both sides. - It is FFS on details of the single side case. #### Issue 1-1-2: Guard band for extended CBW - WF - GB for extended CBW is a virtual concept to consider the boundary applying the shifted OOBE, which is not reflected in the specification and it can be calculated by extended CBW, UE CBW, NRB and extended NRB - Extended CBW is not limited to the existing UE CBW - RAN4 studies whether the extended BW emissions can be guaranteed by only measuring the existing CBW supported in the band (close to the extended BW) given the relaxed extended GB and preliminary measurements performed suggests #### Issue 1-1-3: Derivation of new inner RB region in extended CBW - WF - The following approach is considered as starting point to be further discussed in next meeting, notation should be further considered as well - For NRB based RB_start coordinate system origin, the formula of new inner region can be changed as below: - RBStart,Low_new = max(0, RBStart,Low – RBshift_low); - RBStart,High_new = min(NRB – LCRB, NRB + RBshift_high – RBStart,Low_new – LCRB); - RBStart,Low_new ≤ RBStart ≤ RBStart,High_new, and LCRB ≤ ceil((NRB+ RBshift_low + RBshift_high)/2); - RBshift_low, RBshift_high ≤ NRB/2 - Other approach is not precluded - The approach shall be generic to cover all possible extension cases, including symmetric, asymmetric and different extension ratios ### Sub-topic 1-2: Applicable requirements #### Issue 1-2-1: Derivation of the extended IBE - Agreement in main session: - Fixed IBE used for extended CBW at each side of UE CBW as the baseline, the value is derived from the outmost allocated RB of UE CBW (illustrated in a figure to be developed) - The details and the related test cases can be discussed and decided later. - If significant issue is identified, other options can be further considered. - WF Further discuss whether to apply the extended IBE in the GB of extended CBW. - Option1: Yes - Option2: No #### Issue 1-2-2: Relaxation of the extended IBE - Options - Option 1: using BPSK EVM in the IBE formula for all extended RBs - Option 2: no relaxation is needed. - WF - If there is no obvious system performance impact observed, option 1 is adopted ### Sub-topic 1-3: Signaling #### Issue 1-3-1: NW signaling - Agreement in main session: - The power enhancement feature is enabled by the gNB RRC signalling according to UE capabilities, - The details of the signaling including the contents are FFS - WF - It is up to the NW to judge the gap width on both sides of the UE - NW sends info of extension ratio (at least 1/2) per side #### Issue 1-3-2: UE capability - Agreement in main session: - No separate UE capabilities for scenario 1 and scenario 2 - Capability 1: Indicates whether UE supports MPR reduction for single UL carrier with 1/2*UE CBW extension only - Both dual-sided and single sided cases are supported. - Capability 2: Indicates whether UE supports MPR reduction for single UL carrier with both ½ and 1/4*UE CBW extension - Both dual-sided symmetric and single sided cases are supported. - For dual-sided case, it is FFS if asymmetric cases and a separated capability is introduced. - Note: the other ratio number depends on discussion of issue 1-1-1

- Discussion

## 2.1 Parameters influenced by the extension ratio, OOBE & IBE applying range

Through the indication information it should be clear to the UE that the extended RB per side is ceil(x*NRB), e.g., x=1/4, 1/2. However, whether the UE can synchronously obtain the information of the extended CBW is still FFS. And the evaluation range of OOBE and IBE is associated with the parameters influenced by the extension ratio.

Option1：If the extension ratio only multiples NRB, i.e., extended RB number = ceil (extension ratio * NRB).

Fig1. OOBE applying range, when the extension ratio only multiples NRB

- For OOBE,

The applying range would start from the edge of the last extended RB, e.g.,

Regarding two-side extension cases, the spectrum emission mask of the UE applies to frequencies (ΔfOOB) starting from the  edge of the [12*SCS*(RB_Extended_Low + NRB + RB_Extended_High)]. The NR ACLR measurement starts from  edge of the [12*SCS*(RB_Extended_Low + NRB + RB_Extended_High)]. The spurious emission limits apply for the frequency ranges that are more than FOOB (MHz) in Table 6.5.3.1-1 from the  edge of [12*SCS*(RB_Extended_Low + NRB + RB_Extended_High)].

- For IBE of the extension part,

There is no ambiguity that outside NRB it would only apply to the extended RBs. And IBE will connect with OOBE beyond the last extended RB.

Option 2：If the extension ratio multiples not only NRB but also CBW, i.e., extended RB number = ceil (extension ratio * NRB), & extended CBW = extension ratio * assigned CBW.

Fig 2. OOBE applying range, when the extension ratio multiples NRB & CBW

- For OOBE,

The applying range would start from the edge of the extended CBW, e.g.,

Regarding two-side cases, the SEM applies to frequencies (ΔfOOB) starting from the  edge of (CBW_Extended_Low + the assigned NR channel bandwidth + CBW_Extended_High). The NR ACLR measurement starts from  edge of (CBW_Extended_Low + the assigned NR channel bandwidth + CBW_Extended_High). The spurious emission limits apply for the frequency ranges that are more than FOOB (MHz) in Table 6.5.3.1-1 from the  edge of (CBW_Extended_Low + the assigned NR channel bandwidth + CBW_Extended_High).

- For IBE of the extension part,

Extending both CBW and RB would unavoidably introduce the GB as (extension ratio * CBW) would always be wider than (extension ratio * NRB). Therefore, regarding whether to apply the extended IBE in the GB of extended CBW there are two different schemes remaining from the last WF.

From our perspective, we prefer not to apply any requirement in the GB of the extended CBW.

From one side, (extended CBW + assigned NR channel bandwidth) would not always be integer of RB. So once the concept of extended CBW is introduced and OOBE begins to take effect beyond the extended CBW, there will always be a small portion of the bandwidth that remains unevaluated by any requirement.

Observation 1: Once the concept of extended CBW is introduced and OOBE begins to take effect beyond the extended CBW, there will always be a small portion of the extension bandwidth that remains unevaluated by any requirement.

From the other side, in the legacy transmission process without extension indication, there are also GBs on both sides of  assigned CBW where no requirements work inside. And the bandwidth after extension is kind of an analogy of a wider  assigned CBW and could also follow the similar evaluation scheme.

Furthermore, comparing one extended channel and one assigned channel with the same bandwidth and same allocated RB number, the IBE requirement of the unallocated RBs in the assigned channel is more stringent than that of the extended RBs in the extended channel. Therefore, with respect to the extension range, the emission would be more readily achievable in accordance with the IBE. And observation of the simulation results also indicates that the emission level would continue to decrease as the extended RB index moves farther from the allocated RBs. Therefore, the RBs in the GB of the extension channel would predictably easy to satisfy the IBE requirement which maintains more stable and lenient than that of the same RB index in the assigned channel. Consequently, the establishment of an evaluation metric for the extended virtual GBs is of little significance.

Based on the analysis above, it seems that there is no need to evaluate the emission of the GB.

Observation 2: Comparing one extended channel and one assigned channel with the same bandwidth and same allocated RB number, either the extended RBs or the RBs in the GB of the extended channel would predictably easier to satisfy the IBE requirement which maintains more stable and lenient than that of the same RB index in the assigned channel.

Proposal 1: Regarding the parameters impacted by the extension ratio, there are two options and option 1 is recommended:

- Option 1: The extension ratio only multiples NRB, i.e., extended RB number = ceil (extension ratio * NRB):

  - The applying range of OOBE would start from the edge of the last extended RB.

  - There would be no GB in the extension part.

- Option 2: The extension ratio multiples not only NRB but also CBW, i.e., extended RB number = ceil (extension ratio * NRB), & extended CBW = extension ratio * assigned CBW.

  - The applying range of OOBE would start from the edge of the extended CBW.

  - There would be GB in the extended CBW.

Proposal 2: If the extension ratio multiples not only NRB but also CBW, the IBE should only apply to the extended RBs except for the GB of extended CBW.

## 2.2 Derivation of the extended IBE

According to the WF in #115 meeting, the IBE in the extended RBs could be derived from the following formula:

Proposal 3: For the extended RBs the IBE could take the following form:

IBE_extension = max(PRB - 30 dB, )

- NRB is the Transmission Bandwidth Configuration (see clause 5.3).

- LCRB is the Transmission Bandwidth (see clause 5.3).

- ∆RB is the frequency offset between the allocated RB and the measured outmost non-allocated RB within the assigned NR channel bandwidth, ∆RB = 0 for full RB allocation.

- EVM is fixed to 30%, which is the EVM requirement defined for Pi/2-BPSK.

## 2.3. New inner region

One essential issue is that, the extended RBs are used to help re-define a new inner region based on the legacy allocated RB recourses. Therefore, the derivation of the new inner RB region within the extended CBW should not alter the reference system for RB allocation, but only modify the boundary of the inner region.

Observation 3: The derivation of the new inner RB region within the extended CBW should not alter the reference system for RB allocation, but only modify the boundary of the inner region.

In the current spec, the definition of inner region is captured as follows:

Based on the corresponding concepts and definition format, considering the diverse extension ratios and also the asymmetric extension scenarios, the definition of the new inner region is categorized as the following four cases:

Case 1. Only the higher side of original UE is extended by a*NRB.

Case 2. Only the lower side of original UE is extended by a*NRB.

Case 3. Symmetrical extension on both sides of original UE by a*NRB.

Case 4. Asymmetric extension on both sides of original UE, a*NRB on the lower side and b*NRB on the higher side.

The detailed analysis process could be found in [2] and are omitted here. In total, the previous four cases can actually be consolidated into Case 4. Specifically:

- For higher-side-only extension, set a = 0.

- For lower-side-only extension, set b = 0.

- For symmetric extension on both sides, set a = b.

Therefore, the unified formula for the inner region after bandwidth extension is as follows, which is generic to cover all possible extension cases, including symmetric, asymmetric and different extension ratios:

Proposal 4: If the UE supports [MPR reduction for single carrier with single value] or [MPR reduction for single carrier with multiple values], and if at least one of IE [ExtendLowerSideRel19] and [ExtendHigherSideRel19] is set to non-zero value, the RB allocation is an Inner RB allocation if the following conditions are met:

max(0, RBStart,Low – RB_Extended_Low) ≤  RBStart  ≤  min(NRB – LCRB, NRB + RB_Extended_High – RBStart,Low – LCRB)

and LCRB  ≤  ceil((NRB+ RB_Extended_Low + RB_Extended_High )/2)

  - RB_Extended_Low = ceil (a*NRB), and RB_Extended_High = ceil (b*NRB) are the number of extended RBs from the lower edge of NRB and upper edge of NRB, respectively.

## 2.4 UE capability & signaling

According to the WF[1] and feature list[3], two UE capabilities have been specified as follows:

- Capability 1 (MPR reduction for single carrier with single value): Indicates whether UE supports MPR reduction for single UL carrier with 1/2*[UE CBW] extension for dual-sided symmetric and single-sided case. - Capability 2 (MPR reduction for single carrier with multiple values): [Indicates whether UE supports MPR reduction for single UL carrier with 1/X*UE CBW extension for dual-sided symmetric and single-sided case, where 1/X is a set of ratios equal and smaller than 1/2 [{1/4, …}].]

There are two different possible options regarding the IE encoding issue:

Option 1. If the encoding only considers the extension ratio information：

When the gap on one side of the UE CBW is wide enough, the NW should send the signaling [ExtendLowerSideRel19], or [ExtendHigherSideRel19], or [ExtendLowerSideRel19] + [ExtendHigherSideRel19] to the UE to indicate the extension ratio on the left or right or both sides. [ExtendLowerSideRel19] and [ExtendHigherSideRel19] both contain 2bit binary code, for example: 00 represents non-extension, 01 represents the extended NRB [& extended CBW] = 1/4 * NRB [& UE CBW], 10 represents the extended NRB [& extended CBW] = 1/2 *NRB [& UE CBW]. It is a more compatible solution that works well no matter whether asymmetric extension cases are supported or not.

Table 1. Possible IE encoding when only considering the extension ratio

| Possible IE encoding | Extension ratio |
| --- | --- |
| 00 | 0 |
| 01 | 1/4 |
| 10 | 1/2 |

Option 2. If the encoding considers both the extension side and the extension ratio information：

Then the required number of bits will increase, but the IE number that the NW needs to indicate will be reduced. When the gap on one side of the UE CBW is wide enough, the NW should send one signaling [ExtendUERel19] to the UE to indicate the extension on the left or right or both sides.

- When only considering dual-sided symmetric and single-sided cases:

Table 2. Possible IE encoding when only introducing dual-sided symmetric and single-sided extension

| Possible IE encoding | Extension side | Extension ratio |
| --- | --- | --- |
| 000 | / | 0 |
| 001 | Only lower side | 1/4 |
| 010 | Only lower side | 1/2 |
| 011 | Only higher side | 1/4 |
| 100 | Only higher side | 1/2 |
| 101 | Dual-sided symmetric extension | 1/4 |
| 110 | Dual-sided symmetric extension | 1/2 |

- When dual-sided asymmetric cases are also considered:

Table 3. Possible IE encoding when introducing dual-sided asymmetric extension

| Possible IE encoding | Extension side | Extension ratio |
| --- | --- | --- |
| 0000 | / | 0 |
| 0001 | Only lower side | 1/4 |
| 0010 | Only lower side | 1/2 |
| 0011 | Only higher side | 1/4 |
| 0100 | Only higher side | 1/2 |
| 0101 | Dual-sided symmetric extension | 1/4 |
| 0110 | Dual-sided symmetric extension | 1/2 |
| 0111 | Dual-sided asymmetric extension | 1/4 for lower, 1/2 for higher |
| 1000 | Dual-sided asymmetric extension | 1/2 for lower, 1/4 for higher |

It could be found in the above two tables that, both schemes incur coding redundancy, but table 3’s is higher. Therefore, take a comprehensive view we recommend to take option 1 as the encoding form.

Proposal 5: When the gap on one side of the UE CBW is wide enough, the NW should send the signaling [ExtendLowerSideRel19], or [ExtendHigherSideRel19], or [ExtendLowerSideRel19] + [ExtendHigherSideRel19] to the UE to indicate the extension on the left or right or both sides.

- Use the 2bit binary code to indicate the extension on per side, e.g., : 00 represents non-extension, 01 represents the extended NRB [& extended CBW] = 1/4 * NRB [& UE CBW], 10 represents the extended NRB [& extended CBW] = 1/2 *NRB [& UE CBW].

- Conclusion

Observation 1: Once the concept of extended CBW is introduced and OOBE begins to take effect beyond the extended CBW, there will always be a small portion of the extension bandwidth that remains unevaluated by any requirement.

Observation 2: Comparing one extended channel and one assigned channel with the same bandwidth and same allocated RB number, either the extended RBs or the RBs in the GB of the extended channel would predictably easier to satisfy the IBE requirement which maintains more stable and lenient than that of the same RB index in the assigned channel.

Proposal 1: Regarding the parameters impacted by the extension ratio, there are two options and option 1 is recommended:

- Option 1: The extension ratio only multiples NRB, i.e., extended RB number = ceil (extension ratio * NRB):

  - The applying range of OOBE would start from the edge of the last extended RB.

  - There would be no GB in the extension part.

- Option 2: The extension ratio multiples not only NRB but also CBW, i.e., extended RB number = ceil (extension ratio * NRB), & extended CBW = extension ratio * assigned CBW.

  - The applying range of OOBE would start from the edge of the extended CBW.

  - There would be GB in the extended CBW.

Proposal 2: If the extension ratio multiples not only NRB but also CBW, the IBE should only apply to the extended RBs except for the GB of extended CBW.

Proposal 3: For the extended RBs the IBE could take the following form:

IBE_extension = max(PRB - 30 dB, )

- NRB is the Transmission Bandwidth Configuration (see clause 5.3).

- LCRB is the Transmission Bandwidth (see clause 5.3).

- ∆RB is the frequency offset between the allocated RB and the measured outmost non-allocated RB within the assigned NR channel bandwidth, ∆RB = 0 for full RB allocation.

- EVM is fixed to 30%, which is the EVM requirement defined for Pi/2-BPSK.

Observation 3: The derivation of the new inner RB region within the extended CBW should not alter the reference system for RB allocation, but only modify the boundary of the inner region.

Proposal 4: If the UE supports [MPR reduction for single carrier with single value] or [MPR reduction for single carrier with multiple values], and if at least one of IE [ExtendLowerSideRel19] and [ExtendHigherSideRel19] is set to non-zero value, the RB allocation is an Inner RB allocation if the following conditions are met:

max(0, RBStart,Low – RB_Extended_Low) ≤  RBStart  ≤  min(NRB – LCRB, NRB + RB_Extended_High – RBStart,Low – LCRB)

and LCRB  ≤  ceil((NRB+ RB_Extended_Low + RB_Extended_High )/2)

  - RB_Extended_Low = ceil (a*NRB), and RB_Extended_High = ceil (b*NRB) are the number of extended RBs from the lower edge of NRB and upper edge of NRB, respectively.

Proposal 5: When the gap on one side of the UE CBW is wide enough, the NW should send the signaling [ExtendLowerSideRel19], or [ExtendHigherSideRel19], or [ExtendLowerSideRel19] + [ExtendHigherSideRel19] to the UE to indicate the extension on the left or right or both sides.

- Use the 2bit binary code to indicate the extension on per side, e.g., : 00 represents non-extension, 01 represents the extended NRB [& extended CBW] = 1/4 * NRB [& UE CBW], 10 represents the extended NRB [& extended CBW] = 1/2 *NRB [& UE CBW].

- References

- R4-2507930 WF on NR_ENDC_RF_Ph4_part1, Huawei, HiSilicon, RAN4 #115.

- R4-2506837 Discussion on UE RF power enhancement for NR single carrier, vivo, RAN4#115.

- R4-2508077 Rel-19 RAN4 UE feature list for NR, CMCC, RAN4#115.