3GPP TSG-RAN WG4 Meeting # 116 	R4-2510244

Bengaluru, India, August 25th – 29th, 2025

Source: 	vivo

Title: 	Discussion on the remaining issue of FR2 OTA

Agenda Item:	7.11.3

Document for:	Approval

- Introduction

- AoA1 and AoA2 will have the same TTFiniteRes, i.e., we are not assessing TTFiniteRes separately for the two different scenarios (AoA1 declared vs Beam peak direction re-used (default))

- The TTFiniteRes will be assessed similar to [R4-2506847] [R4-2506974] but with [6x6, 4x2] antenna configs and beam steering assumptions for the PC1/PC5 from spherical coverage analyses in RAN5, i.e., 4o beam steering granularity [R5-227982]; the resulting relative antenna gain difference will be split in half and applied equally as TTFiniteRes for AoA1 and AoA2; consideration of the 0.7dB systematic MU for beam peak search in this TTFiniteRes is TBD.

In [1], the applicability rules and evaluation assumption of test tolerance was agreed for sTxMP:

This contribution provides the evaluation and analysis of the TTFiniteRes

- Discussion

In [2], we provide the initial analysis of the additional tolerance for the AoA2 (i.e., the AoA not declared by UE or align with the beam beak of single carrier) but it was pointed out that the assumption is not align with the previous PC5 evaluation. We update the simulation assumption and the results are shown in Figure 1

Figure 1 updated simulation results based on [1]

Since usually the FWA/CPE assume 6x6 or 12x12, and 4x2 is more likely used in smartphone, to avoid unnecessary misleading, we suggest to only keep the assumption of 6x6.

Proposal 1: Only keep the results of 6x6 antenna assumption.

Due to finer beam granularity, the degradation is reduced obviously, for 6x6 antenna assumption, the maximum antenna gain degradation is around 1.5 dB.

Observation 1: The maximum antenna gain degradation for AoA2 is around 1.5 dB.

As we agreed in the last meeting, we can equally split this degradation and apply it to AoA1 and AoA2 separately.

Proposal 2: The 0.8 dB TTFiniteRes is applied to both AoA1 and AoA2.

Another remaining issue is the MU of beam peak search. In our understanding, the MU of beam peak search comes from finite test grid, and no matter UE chose to declaration or reuse the beam peak of single carrier, such uncertainty still exist

Proposal 3: The 0.7 dB systematic MU for beam peak search is still applied to sTxMP.

- Conclusion

In this contribution, we provide our views on remaining issue of OTA test method of sTxMP

Proposal 1: Only keep the results of 6x6 antenna assumption.

Observation 1: The maximum antenna gain degradation for AoA2 is around 1.5 dB.

Proposal 2: The 0.8 dB TTFiniteRes is applied to both AoA1 and AoA2.

Proposal 3: The 0.7 dB systematic MU for beam peak search is still applied to sTxMP.

- References

- R4-2508663	Way Forward for [115][338] NR_FR2_OTA, Qualcomm, RAN4#115

- R4-2506847	Discussion on the FR2 OTA test method, vivo, RAN4#115