3GPP TSG-RAN WG4 Meeting # 116											   R4-2510252

Bengaluru, India, August 25th – 29th, 2025

Source: 	vivo

Title: 	draft TP to TS 38.194 on CW frequency error and unwanted emission

Agenda Item:	7.22.3.3

Document for:	Approval

- Introduction

In this contribution, we update the TR for CW unwanted emission and frequency error

- Text proposal

## <<Start of Change>>

## 8.3	Frequency error

#### 8.3.1	General

The requirements in clause 7.3 apply to the transmitter ON period.

Frequency error is the measure of the difference between the actual carrier wave transmit frequency and the assigned frequency. This requirement shall be applied at the antenna connector supporting transmission in the operating band.

#### 8.3.2	Minimum requirement

The frequency of carrier wave measurements shall be accurate to within ±0.1 ppm observed over a period of 1 ms compared to the carrier frequency declared by the manufacturer.

## <<Unchanged parts are omitted>>

## 8.5	Unwanted emission

#### 8.5.1	General

Unwanted emissions consist of phase noise, operating band unwanted emissions and spurious emissions.

The phase noise applies to a certain frequency offset from centre frequency of carrier wave declared by the manufacturer. The maximum offset of the operating band unwanted emissions mask from the operating band edge is 10 MHz. The Operating band unwanted emissions define all unwanted emissions in each supported uplink operating band plus the frequency ranges 10 MHz above and 10 MHz below each band. Unwanted emissions outside of phase noise and operating band unwanted emissions are limited by a spurious emissions requirement.

Unless otherwise stated, all requirements are measured as mean power (RMS).

#### 8.5.2	Phase noise

#### 8.5.2.1	General

The phase noise is the unwanted emissions outside the centre frequency of carrier wave resulting from random fluctuations in the phase of signal in the transmitter but excluding spurious emissions. The phase noise of the CW applies to ±[120] kHz frequency offset (ΔfOOB) from centre frequency of carrier wave.

For the CW equipment declared capable of performing phase noise cancellation, the requirement in 8.5.2.2 is not applied.

#### 8.5.2.2	Minimum requirement

The phase noise power of CW transmitter shall not exceed the levels specified in Table 8.5.1-1

Table 8.5.1-1: CW phase noise emission limit

| ΔfOOB (kHz) | Phase noise emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| ± 120 | [-30] | [7.5 kHz] |

#### 8.5.4	Spurious emission

#### 8.5.4.1	General

The transmitter spurious emission limits shall apply from 9 kHz to 12.75 GHz, excluding the frequency range from ΔfOOB below the centre frequency of carrier wave declared by the manufacturer, up to ΔfOOB above centre frequency of carrier wave declared by the manufacturer, where the ΔfOOB is defined in table 7.5.1-1. For some operating bands, the upper limit is higher than 12.75 GHz in order to comply with the 5th harmonic limit of the downlink operating band, as specified in ITU-R recommendation SM.329 [x].

#### 8.5.3.2	Minimum requirement

The basic limits of either table 8.5.3.2-1 (Category A limits) or table 8.5.3.2-2 (Category B limits) shall apply. Unless otherwise stated, all requirements are measured as mean power (RMS).

Table 8.5.3.2-1: General CW transmitter spurious emission limits in FR1, Category A

| Spurious frequency range | Basic limit | Measurement bandwidth | Notes |
| --- | --- | --- | --- |
| 9 kHz – 150 kHz |  | 1 kHz | Note 1 |
| 150 kHz – 30 MHz |  | 10 kHz | Note 1 |
| 30 MHz – 1 GHz |  | 100 kHz | Note 1 |
| 1 GHz 12.75 GHz | -13 dBm | 1 MHz | Note 1, Note 2 |
| 12.75 GHz – 5th harmonic of the upper frequency edge of the DL operating band in GHz |  | 1 MHz | Note 1, Note 2, Note 3 |
| 12.75 GHz - 26 GHz | -13 dBm | 1 MHz | Note 1, Note 2, Note 4 |
| NOTE 1: Measurement bandwidths as in ITU-R SM.329 [x], s4.1. NOTE 2: Upper frequency as in ITU-R SM.329 [2], s2.5 table 1. NOTE 3: Applies for Band for which the upper frequency edge of the DL operating band is greater than 2.55 GHz and less than or equal to 5.2 GHz.. NOTE 4: Applies for Band for which the upper frequency edge of the DL operating band is greater than 5.2 GHz. |  |  |  |

Table 8.5.3.2-2: General CW transmitter spurious emission limits in FR1, Category B

| Spurious frequency range | Basic limit | Measurement bandwidth | Notes |
| --- | --- | --- | --- |
| 9 kHz – 150 kHz |  | 1 kHz | Note 1 |
| 150 kHz – 30 MHz | -36 dBm | 10 kHz | Note 1 |
| 30 MHz – 1 GHz |  | 100 kHz | Note 1 |
| 1 GHz – 12.75 GHz |  | 1 MHz | Note 1, Note 2 |
| 12.75 GHz – 5th harmonic of the upper frequency edge of the DL operating band in GHz | -30 dBm | 1 MHz | Note 1, Note 2, Note 3 |
| 12.75 GHz - 26 GHz | - 30 dBm | 1 MHz | Note 1, Note 2, Note 4 |
| NOTE 1: Measurement bandwidths as in ITU-R SM.329 [x], s4.1. NOTE 2: Upper frequency as in ITU-R SM.329 [x], s2.5 table 1. NOTE 3: Applies for Band for which the upper frequency edge of the DL operating band is greater than 2.55 GHz and less than or equal to 5.2 GHz. NOTE 4: Applies for Band for which the upper frequency edge of the DL operating band is greater than 5.2 GHz. |  |  |  |

## <<End of Change>>

- Reference