3GPP TSG-RAN WG4 Meeting # 116	R4-2510264

Bengaluru, India, August 25th – 29th, 2025

Source: 	vivo

Title: 	NR NTN HPUE Rx Requirements

Agenda Item:	7.6.3.2.2

Document for:	Approval

Introduction

In the previous meeting, PC2 RSD for n255 was concluded, and the main remaining issues are the RSD for PC1.5 and PC1 non-handheld devices, as in the WF [2]:

Issue 2-2: RSD for PC1.5/PC1 non-handheld devices

Agreement:

- Consider [0] dB RSD for PC1.5/PC1 non-handheld devices for the current NTN bands with the following.

  - To document 66~70 dB filter rejection would be needed for the non-handheld 0dB RSD in the TR.

  - Revisit this value with more inputs in next meeting.

Apart from the [0]dB MSD, the main proposal for non-zero values are as following which is from earlier summary.

  - Option 1: For PC1.5 2Tx (MTK)

| Operating Band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) |
| --- | --- | --- | --- | --- |
| n256 | 2.2 | 2.2 | 2.2 | 2.2 |
| n255 | 3.2 | 3.2 | 3.3 | 3.3 |
| n254 | 2.2 | 2.2 | 2.2 |  |

  - Option 2: For PC1 1Tx (MTK)

| Operating Band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) |
| --- | --- | --- | --- | --- |
| n256 | [3] | [3] | [3] | [3] |
| n254 | [3] | [3] | [3] |  |

In this paper, some of the proposals and analysis are provided.

HPUE Impact for NR-NTN RSD

Similar to [3] which submitted in last meeting, it should be noted that the scenarios needed to be defined for RSD is related to what Tx requirements would be defined, the following table was copied below from [3]:

Table 1: Current requirements applicability in R19 (Uncertain part in [])

|  | Antenna Numbers | Handheld and/or non-handheld |
| --- | --- | --- |
| PC1 | 1Tx | Non-handheld |
|  | [4Tx] | Non-handheld |
| PC1.5 | FFS if time allows |  |
|  | 2Tx | Non-handheld |
| PC2 | 1Tx | Both |
|  | 2Tx | Both |

Currently, there are mainly two options for PC1.5 and PC1 non-handheld device, zero-value and non zero value.

As discussed before, the current non-handheld requirements defined in 36.101 and TRs such as 37.880, are based on some duplexer which is comparable to what base station implementation. However, actually those type of devices are barely commercialized so far.

Observation 1: The FDD PC1 device were barely commercialized, and the components studied in the TR may not be typical and optimized.

It should be noted that even non-handheld device also may have expectation on size, cost etc. If we completely rely on those component assumptions, there would be a danger that the bar for the non-handheld device would be high, and there might be fewer choices on the components in the future. This is also a risk, apart from the lower performance.

Observation 2: High performance components, such as filter, may also have higher risk of size, cost, thermal etc.

In fact, those risks may be even more considerable during the implementation process. Based on these considerations, introduce some non-zero RSD values which corresponds to slightly lower performance might also be reasonable. The original MTK proposals might be considered as starting point.

Proposal 1: Considering to introduce non-zero RSD value for non-handheld type, particularly for PC1.

However, if no more and agreements on this can be reached, 0dB can still be used since this is in line with legacy cases. In this case, some assumptions can be written into TR.

Conclusion

In this paper, the RSD requirements have been discussed for NR NTN HPUE.

Observation 1: The FDD PC1 device were barely commercialized, and the components studied in the TR may not be typical and optimized.

Observation 2: High performance components, such as filter, may also have higher risk of size, cost, thermal etc.

Proposal 1: Considering to introduce non-zero RSD value for non-handheld type, particularly for PC1.

- References

[1] R4-2507567, Topic summary for [115][128] NR_IoT_NTN_HPUE_part1, Moderator (Samsung), RAN4#115

[2] R4-2508008, WF on NR_IoT_NTN_HPUE_part1, Samsung, RAN4#115

[3] R4-2503841, NR NTN HPUE Rx Requirements, vivo, RAN4#115