3GPP TSG-RAN WG4 Meeting # 116	R4-2510266

Bengaluru, India, August 25th – 29th, 2025

Source: 	vivo

Title: 	Iot NTN HPUE Rx Requirements

Agenda Item:	7.6.3.3.2

Document for:	Approval

Introduction

In the previous meeting, PC2 1Tx RSD for 256 (full duplex eMTC based) was concluded, and the only remaining issue is PC1 RSD for non-handheld.

Issue 3-1-2: PC1 RSD (Non-handheld only, full duplex eMTC based)

- Ad-hoc agreement:

  - Consider [0] dB RSD for PC1 non-handheld devices for the current NTN bands with the following.

    - To document 66~70 dB filter rejection would be needed for the non-handheld 0dB RSD in the TR.

    - Revisit this value with more inputs in next meeting.

In this paper, some of the proposals and analysis are provided.

HPUE Impact for Iot-NTN RSD

It should be noted that the only Similar to [3] which submitted in last meeting, it should be noted that the scenarios needed to be defined for RSD is related to what Tx requirements would be defined, the following table was copied below from [3]:

For eMTC based Iot-NTN, actually the PC1 shares pretty much the same condition with NR NTN. The analysis and proposal in [4] also holds for Iot-NTN, which is also copied here:

Observation 1 (Reference): The FDD PC1 device were barely commercialized, and the components studied in the TR may not be typical and optimized.

Observation 2 (Reference): High performance components, such as filter, may also have higher risk of size, cost, thermal etc.

Proposal 1 (Reference): Considering to introduce non-zero RSD value for non-handheld type, particularly PC1.

However, if no more and agreements on this can be reached, 0dB can still be used since this is in line with legacy cases. In this case, some assumptions can be written into TR. Generally, eMTC based Iot-NTN can be aligned with NR-NTN in the definition of PC1 RSD.

Proposal 2: Align eMTC based Iot-NTN and NR-NTN in PC1 RSD requirements.

In addition, there is one more issue specific for eMTC based Iot-NTN, that is the MRP and A-MPR for certain bands are not thoroughly studied, thus may face some difficulty to be agreed. There is a possibility that PC1 for eMTC based Iot-NTN would not be defined. If the requirements go this way, then the RSD for PC1 would also be not needed.

Observation 3: If eMTC based Iot-NTN PC1 was not pursued, the corresponding RSD would also not be needed.

Conclusion

In this paper, the RSD requirements have been discussed for NR NTN HPUE.

Observation 1 (Reference): The FDD PC1 device were barely commercialized, and the components studied in the TR may not be typical and optimized.

Observation 2 (Reference): High performance components, such as filter, may also have higher risk of size, cost, thermal etc.

Proposal 1 (Reference): Considering to introduce non-zero RSD value for non-handheld type, particularly PC1.

Proposal 2: Align eMTC based Iot-NTN and NR-NTN in PC1 RSD requirements.

Observation 3: If eMTC based Iot-NTN PC1 was not pursued, the corresponding RSD would also not be needed.

- References

[1] R4-2507568, Topic summary for [115][129] NR_IoT_NTN_HPUE_part2, Moderator (vivo), RAN4#115

[2] R4-2508122, WF on NR_IoT_NTN_HPUE_part2, vivo, RAN4#115

[3] R4-2506859, Iot NTN HPUE Rx Requirements, vivo, RAN4#115

[4] R4-2510264, NR NTN HPUE Rx Requirements, vivo, RAN4#116