3GPP TSG RAN WG4 #116	R4-2510271

Bangalore, India Aug 25th – 29th, 2025

Title:	Reply LS on the RAN simulation assumptions for ULBC

Response to:	R4-2509024

Release:	Release 20

Work Item:	FS_ULBC

Source:	RAN WG4

To:	SA WG4

Cc:	RAN WG2, RAN WG1, SA WG2

Contact Person:

Name:	Sanjun Feng

E-mail Address:	fengsanjun@vivo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	none

RAN4 would like to thank SA4 for the LS.

Here is a detailed status about supported power classes for NB-Iot based Iot-NTN in current and future releases:

In Rel-18, the default power class (PC3, 23dBm) has been introduced with Iot-NTN UE RF requirements specification 36.102, and this default power class is applied to all Iot-NTN bands.

In Rel-19, power class 2 (26dBm) and power class 1 (31dBm) were agreed to be introduced. The targeted bands are band 254/255/256. The feature CR to introduce into specification is planned to be agreed in this RAN4 meeting, and the following table which referenced from draft CR can be used as a reference.

Table 6.2B.1-1: UE Power Class

| EUTRA band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 256 | 31 | +2/-3 | 26 | +/-2 | 23 | +/-2 | 20 | +/-2 |
| 255 | 31 | +2/-3 | 26 | +/-2 | 23 | +/-2 | 20 | +/-2 |
| 254 | 31 | +2/-3 | 26 | +/-2 | 23 | +/-2 | 20 | +/-2 |
| 253 |  |  |  |  | 23 | +/-2 | 20 | +/-2 |

For Rel-20, a new UE power class which can transmit power higher than PC1 (e.g., up to 37dBm) will be studied and specified if deemed feasible. The feasibility and specific power would be under further study and this work has not been started yet. There are also no designated bands yet for this higher than PC1 study.

In addition, power class 1.5 (29dBm) is also included in the Rel-20 RAN4 led-package WF and highly likely to be officially agreed in next RAN plenary meeting. There is no discussion on applicable bands yet.

# 2. Actions:

# To SA4:

RAN4 kindly inform SA4 to take the above response into account.

# 3. Dates of Next RAN4 Meetings:

RAN4#116bis				13th to 17th October 2025			Prague, CZ

RAN4#117				17th to 21st November 2025			Dallas, US