3GPP TSG-RAN4 Meeting #116	R4-2510279

# Bengaluru, India, 25th – 29th August, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 2955 | rev | - | Current version: | 17.18.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_newRAT-Core) Addition of higher harmonic exceptions for UE co-existence requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2025-08-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | An accomanying discussion paper is in R4-2510276 and the following are the key contents: There is a note for harmonic exception for UE co-existence from LTE. However, it has not been updated with introduction even higher frequency band for a long time. Traditionally, all the possible harmonic cases were treated as exception. In Rel-8/9, only 2nd and 3rd harmonics were considered, which is as following: When band 22(3.5GHz) were introduced in R4-114762, there begins to have overlapping of 4th harmoincs for band 8, and the note2 was updated as following and captured into 36.101 V10.4.0: After that, the first 1 MHz frequency range immediately outside the harmonic emission was also studied and incorporated into the note2 as exception, in the mean time, the harmoincs were extended to [5]th harmonics. The discussion paper is in R4-147329, and CR R4-148034 was agreed and introduced into 36.101 V10.17.0.The note has not been updated ever since. In later stage, more bands in higher range were introduced, such as band n79. The possible 6th harmonics for some lower bands (e.g. band 8/28 etc) could be overlapping with n79 and may also apply the exception. However, this note was never updated for a long time, and was directly reused from LTE for NR without even remove the square brackets of [5th]. For certain band e.g. band 28/n28, the 6th harmonics might fall into n79, and Note 2 is also needed for the UE protection. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding 6th harmonics in the Note 2 of UE co-existence requirements. Change the applicability of band n79 in the band n28 UE-coexistence. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The latest need of higher harmoincs for higher frequency was not considered in UE co-existence. This would bring more difficulty and unnecessary complexity for UE implementation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 38.521-1CR … |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

6.5.3.2	Spurious emissions for UE co-existence

This clause specifies the requirements for NR bands for coexistence with protected bands.

Table 6.5.3.2-1: Requirements for spurious emissions for UE co-existence

| NR Band | Spurious emission for UE co-existence |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| <unchanged part omitted> |  |  |  |  |  |  |  |
| n28, n83 | E-UTRA Band 1, 4, 22, 32, 42, 43, 50, 51, 65, 66, 74, 75, 76 NR Band n77, n78, n79, n100, n101 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 19, 25 |
|  | E-UTRA Band 2, 3, 5, 7, 8, 18, 19, 20, 25, 26, 27, 31, 34, 38, 39, 40, 41, 52, 72, 73 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 19, 24 |
|  | Frequency range | 470 | - | 694 | -42 | 8 | 15, 35 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 34 |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 15 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 15 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8, 19 |
| <unchanged part omitted> |  |  |  |  |  |  |  |
| NOTE 1: FDL_low and FDL_high refer to each E-UTRA frequency band specified in Table 5.5-1 NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th, 5th or 6th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5, 6 for the 2nd, 3rd, 4th, 5th or 6th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: N/A NOTE 4: N/A <unchanged part omitted> |  |  |  |  |  |  |  |  |

< End of changes >