3GPP TSG-RAN4 Meeting #116	R4-2510283

# Bengaluru, India, August 25th – 29th, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | draft | rev | - | Current version: | 18.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR 38.101-1 R18 cleanup for ModifiedMPR table |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2025-08-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The leagcy bits of ModifiedMPR is to provide transition time for UE to comply with, it’s no longer needed in new release. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Unecessary bit setting of Modified MPR is reserved. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Table is very hard to understand and incorrect. Description column content may cause confusion. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex L |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*********************************** Start of Changes **************************************

# L.1	Indication of modified MPR behavior

This annex contains the definitions of the bits in the field modifiedMPR-Behaviour indicated per supported NR band in the IE RF-Parameters [7] by a UE supporting an MPR or A-MPR modified in a given version of this specification. A modified MPR or A-MPR behaviour can apply to a supported NR band in stand-alone operation (including CA and NN-DC operation) or in non-standalone operation with the said NR band as part of an EN-DC or NE-DC band combination.

NOTE 1:	In the present release, the modifiedMPR-Behaviour is indicated [7] by an 8-bit bitmap per supported NR band.

Table L.1-1: Definitions of the bits in the field modifiedMPR-Behaviour

| NR Band | Index of field (bit number) | Definition (description of the supported functionality if indicator set to one) | Notes |
| --- | --- | --- | --- |
| n30 | 0 (leftmost bit) | Requirements for network signalling value NS_21 as defined in Clause 6.5.2.3.y of 38.101-1 v17.6.0 and A-MPR as defined in Clause 6.2.3.14 of 38.101-1 v17.6.0. | This bit is reserved. |
| n34 | 0 (leftmost bit) | PC 1.5 MPR as defined in Table 6.2D.2-3 | This bit may be set to 1 by a UE of any release supporting power class 1.5. This bit is intended to be set by larger form factor FWA devices. If the bit is not set for a Rel-17 and later UE, PC 1.5 MPR as defined in Table 6.2D.2-2 applies. If the bit is not set for a Rel-16 and earlier UE, MPR in Table 6.2.2-4 of 38.101-1 v16.5.0 applies. |
| n39 | 0 (leftmost bit) | PC 1.5 MPR as defined in Table 6.2D.2-3 | This bit may be set to 1 by a UE of any release supporting power class 1.5. This bit is intended to be set by larger form factor FWA devices. If the bit is not set for a Rel-17 and later UE, PC 1.5 MPR as defined in Table 6.2D.2-2 applies. If the bit is not set for a Rel-16 and earlier UE, MPR in Table 6.2.2-4 of 38.101-1 v16.5.0 applies. |
| n40 | 0 (leftmost bit) | PC 1.5 MPR as defined in Table 6.2D.2-3 | This bit may be set to 1 by a UE of any release supporting power class 1.5. This bit is intended to be set by larger form factor FWA devices. If the bit is not set for a Rel-17 and later UE, PC 1.5 MPR as defined in Table 6.2D.2-2 applies. If the bit is not set for a Rel-16 and earlier UE, MPR in Table 6.2.2-4 of 38.101-1 v16.5.0 applies. |
| n41 | 0 (leftmost bit) | EN-DC contiguous intraband MPR as defined in clause 6.2B.2.1 of 38.101-3 v15.5.0 | This bit is reserved. |
|  | 1 | EN-DC non-contiguous intraband MPR as defined in clause 6.2B.2.2 of 38.101-3 v15.5.0 | This bit is reserved. |
|  | 2 | EN-DC contiguous and non-contiguous intraband MPR and A-MPR as defined in 38.101-3 v16.4.0. If this bit is not set the UE uses Rel-15 MPR or A-MPR for EN-DC contiguous and non-contiguous intraband MPR and A-MPR | This bit is reserved. |
|  | 3 | PC 1.5 MPR as defined in Table 6.2D.2-3 | This bit is reserved. |
| n71 | 0 (leftmost bit) | EN-DC contiguous intraband MPR as defined in clause 6.2B.2.1 of 38.101-3 v15.5.0 | - This bit is reserved. |
| n77 | 0 (leftmost bit) | PC 1.5 MPR as defined in Table 6.2D.2-3 | This bit is reserved. |
| n78 | 0 (leftmost bit) | PC 1.5 MPR as defined in Table 6.2D.2-3 | This bit is reserved. |
| n79 | 0 (leftmost bit) | PC 1.5 MPR as defined in Table 6.2D.2-3 | This bit is reserved. |
| n96 | 0 (leftmost bit) | Support of all band n96 network signalling labels as defined in Table 6.2F.3.1-1 of 38.101-1 Rel-17 | This bit may be set to 1 by a Rel-16 UE to indicate support of all network signalling labels for n96 as defined in Table 6.2F.3.1-1 of 38.101-1 Rel-17 |
|  | 1 | Support of all band n96 network signalling labels as defined in Table 6.2F.3.1-1 of 38.101-1 Rel-18 | This bit may be set to 1 by a Rel-16 and Rel-17 UE to indicate support of all network signalling labels for n96 as defined in Table 6.2F.3.1-1 of 38.101-1 Rel-18 |
| n102 | 0 (leftmost bit) | Support of all band n102 network signalling labels as defined in Table 6.2F.3.1-1 of 38.101-1 Rel-17 | This bit may be set to 1 by a Rel-16 UE to indicate support of all network signalling labels for n102 as defined in Table 6.2F.3.1-1 of 38.101-1 Rel-17 |
|  | 1 | Support of all band n102 network signalling labels as defined in Table 6.2F.3.1-1 of 38.101-1 Rel-18 | This bit may be set to 1 by a Rel-16 and Rel-17 UE to indicate support of all network signalling labels for n102 as defined in Table 6.2F.3.1-1 of 38.101-1 Rel-18 |

*********************************** End of Changes **************************************