3GPP TSG-RAN4 Meeting #116 R4-2510293 Bangalore, India, 25 Aug - 29 Aug, 2025 CR-Form-v12.3. CHANGE REQUEST. 38.101-1; CR; 2958; rev; -; Current version:; 19.2.0. For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests.. Proposed change affects:; UICC apps; ME; X; Radio Access Network; Core Network. Title:; (NR_LPWUS-Core) Big CR for LP-WUS UE RF requirements in Rel-19. Source to WG:; vivo. Source to TSG:; R4. Work item code:; NR_LPWUS-Core; Date:; 2025-08-05. Category:; B; Release:; Rel-19. Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900.; Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20). Reason for change:; This is big CR to introduce RF requirements for LP-WUS.. Summary of change:; The following changes: - Definitions - Symbols - Abbreviations - Suffix - new sub-clause 5.2M operating bands - new sub-clause 5.3M UE channel - new sub-clause 7.1M Reference sensitivity - new sub-clause 7.4M Max input level - new sub-clause 7.5M Adjacent channel and subcarrier selectivity - new sub-clause 7.6M Blocking - new sub-clause 7.7M Spurious response - new sub-clause 7.8M intermodulation - new sub-clause 7.9M Spurious emissions - new sub-clause A.3M RMC. Consequences if not approved:; The spec is not appicable for LP-WUS/WUR feature.. Clauses affected:; 3.1, 3.2, 3.3, 4.3, 5.2M, 5.3M, 7.1M, 7.2M, 7.3M, 7.4M, 7.5M, 7.6M, 7.7M, 7.8M,7.9M, A.3M. Y; N. Other specs; X; Other core specifications; TS/TR ... CR .... affected:; X; Test specifications; TS/TR ... CR .... (show related CRs); X; O&M Specifications; TS/TR ... CR .... Other comments:. This CR's revision history:.

<<< Skip Unchanged Sections >>>

## <<< START OF CHANGES >>>

3	Definitions, symbols and abbreviations

3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Aerial UE: A UE supporting UAS (Uncrewed Aircraft Systems) as indicated by the capability aerialUE-Capability-r18 [15] and that has an aerial subscription as described in TS 23.256 [17]. The UE is considered to have access to UAS services after the UE ha performed a successful authentication and authorization with the USS as described in TS 23.256 [17].

Aggregated Allocation Bandwidth: Total bandwidth of all allocated RBs in a transmission occasion. Can be calculated for two aggregated CCs as LCRB, 1* 12* SCS1 + LCRB,2 * 12 * SCS2 .

Aggregated Channel Bandwidth: The RF bandwidth in which a UE transmits and receives multiple contiguously aggregated carriers.

ATG UE: The terminals or user equipments which are mounted in aircraft and support ATG feature (i.e. UE capability airToGroundNetwork-r18) as defined in clause 4.2.2 from TS38.306[15].

Carrier aggregation: Aggregation of two or more component carriers in order to support wider transmission bandwidths.

Carrier aggregation band: A set of one or more operating bands across which multiple carriers are aggregated with a specific set of technical requirements.

Carrier aggregation bandwidth class: A class defined by the aggregated transmission bandwidth configuration and maximum number of component carriers supported by a UE.

Carrier aggregation configuration: A combination of CA operating band(s) and CA bandwidth class(es) supported by a UE.

Concurrent operation: The simultaneous transmission and reception of sidelink and Uu interfaces while operation is agnostic of the service used on each interface.

Contiguous carriers: A set of two or more carriers configured in a spectrum block where there are no RF requirements based on co-existence for un-coordinated operation within the spectrum block.

Contiguous resource allocation: A resource allocation of consecutive resource blocks within one carrier or across contiguously aggregated carriers. The gap between contiguously aggregated carriers due to the nominal channel spacing is allowed.

Contiguous spectrum: Spectrum consisting of a contiguous block of spectrum with no sub-block gap(s).

Enhanced channel raster: channel raster with a 10 kHz granularity in bands with a 100 kHz channel raster.

eRedCap UE: The UE with enhanced reduced capabilities as defined in clause 4.2.22.1 from TS38.306 [15].

Inter-band carrier aggregation: Carrier aggregation of component carriers in different operating bands.

NOTE:	Carriers aggregated in each band can be contiguous or non-contiguous.

Inter-band concurrent operation: Operation of NR Uu carrier and NR Sidelink carrier in different operating bands.

Intra-band contiguous carrier aggregation: Contiguous carriers aggregated in the same operating band.

Intra-band non-contiguous carrier aggregation: Non-contiguous carriers aggregated in the same operating band.

Intra-band SL CA UE: UE that supports NR SL CA operation in a single band

LP-WUS power boost: difference between the average power of LP-WUS REs (which occupy certain REs within a NR transmission bandwidth configuration) and the average power over all REs (from both LP-WUS and the NR carrier containing the LP-WUS REs)

NR SL CA: Aggregation of two or more NR Sidelink component carriers in order to support wider transmission bandwidths

NR SL inter-band concurrent operating Band：Band combinations of NR Uu carrier and NR Sidelink carrier in different operating bands.

NR SL-U UE: UE that supports NR Sidelink operation in unlicensed bands (e.g. n46, n96, n102).

Railway Mobile Radio: railway operations encompassing GSM-R and its successor(s), including the Future Railway Mobile Communication System (FRMCS); in the context of this specification the Railway Mobile Radio is limited to NR operation in band n100, or n101.

RedCap UE: The UE with reduced capabilities as defined in clause 4.2.21.1 from TS38.306 [15].

Sub-band:  For a UE that supports shared spectrum channel access in wideband operation, a sub-band is the set of RBs within an approximately 20 MHz segment of the channel where the wideband channel is uniformly divided into an integer number of 20 MHz sub-bands.  Sub-bands may be separately allocated in uplink and downlink.

Sub-block: This is one contiguous allocated block of spectrum for transmission and reception by the same UE. There may be multiple instances of sub-blocks within an RF bandwidth.

Sub-block bandwidth: The bandwidth of one sub-block.

Sub-block gap: A frequency gap between two consecutive sub-blocks within an RF bandwidth, where the RF requirements in the gap are based on co-existence for un-coordinated operation.

Two Rx antenna port XR UE: A non-(e)RedCap XR UE that is equipped with only two Rx antenna ports in frequency band(s) where 4 Rx antenna ports are required. The UE is intended to be worn on human head. When in use, is intended to be supported only by/behind the ears and by a nose-bridge resulting in a constrained form factor with limited volume available for Rx chains.

UE transmission bandwidth configuration: Set of resource blocks located within the UE channel bandwidth which may be used for transmitting or receiving by the UE.

Vehicular UE: A UE embedded in a vehicle, permanently connected to an embedded antenna system that radiates externally for NR operating bands.

NOTE:	Vehicular UE does not refer to other UE form factors placed inside the vehicle.

Wideband operation: For a UE that supports shared spectrum channel access, wideband operation refers to operation within a channel larger than 20 MHz in which intra-cell guard bands may be configured to distinguish individual RB-sets

3.2	Symbols

For the purposes of the present document, the following symbols apply:

ΔFGlobal	Granularity of the global frequency raster

ΔFRaster	Band dependent channel raster granularity

ΔfOOB	Δ Frequency of Out Of Band emission

ΔFTX-RX	Maximum deviation to the Tx-Rx carrier center frequency separation for asymmetric uplink/downlink channel bandwidth operation

∆MPRc	Allowed Maximum Power Reduction relaxation for serving cell c

ΔPPowerClass	Adjustment to maximum output power for a given power class

RB	The starting frequency offset between the allocated RB and the measured non-allocated RB

ΔRIB,c	Allowed reference sensitivity relaxation due to support for inter-band CA operation, for serving cell c

ΔRIBC	Allowed relaxation to the power class 3 reference sensitivity level due to support for intra-band contiguous CA operation

ΔRIBNC	Allowed relaxation to the power class 3 reference sensitivity level due to support for intra-band non-contiguous CA operation

ΔRIB,4R	Reference sensitivity adjustment due to support for 4 antenna ports

ΔRIB,8R	Reference sensitivity adjustment due to support for 8 antenna ports

ΔR1R	Reference sensitivity adjustment due to support for 1 antenna ports

ΔRXR,2R	Reference sensitivity adjustment for two antenna ports XR UEs on bands defined in Table 7.3.2-2b

ΔShift	Channel raster offset

TC	Allowed operating band edge transmission power relaxation

TC,c	Allowed operating band edge transmission power relaxation for serving cell c

ΔTIB,c	Allowed maximum configured output power relaxation due to support for inter-band CA operation, inter-band NR-DC operation and due to support for SUL operations, for serving cell c

BWChannel	Channel bandwidth

BWChannel,block	Sub-block bandwidth, expressed in MHz. BWChannel,block= Fedge,block,high- Fedge,block,low

BWChannel_CA	Aggregated channel bandwidth, expressed in MHz

BWChannel,max	Maximum channel bandwidth supported among all bands in a release

BWGB	max(GBChannel,low, GBChannel,high)

BWDL	Channel bandwidth for DL

BWUL	Channel bandwidth for UL

BWinterferer	Bandwidth of the interferer

Ceil(x)	Rounding upwards; ceil(x) is the smallest integer such that ceil(x) ≥ x

Floor(x)	Rounding downwards; floor(x) is the greatest integer such that floor(x) ≤ x

FC	Center frequency of a carrier for a numerology defined by the RF reference frequency on the channel raster mapped to the carrier according to sub-clause 5.4.2.2FC,block, high	Fc of the highest transmitted/received carrier in a sub-block

FC,block, low	Fc of the lowest transmitted/received carrier in a sub-block

FC,low	The Fc of the lowest carrier, expressed in MHz

FC,high	The Fc of the highest carrier, expressed in MHz

FDL_low	The lowest frequency of the downlink operating band

FDL_high	The highest frequency of the downlink operating band

FUL_low	The lowest frequency of the uplink operating band

FUL_high	The highest frequency of the uplink operating band

Fedge,block,low	The lower sub-block edge, where Fedge,block,low = FC,block,low - Foffset, low.

Fedge,block,high	The upper sub-block edge, where Fedge,block,high = FC,block,high + Foffset, high.

Fedge , low	The lower edge of aggregated channel bandwidth, expressed in MHz. Fedge,low = FC,low - Foffset,low.

Fedge, high	The higher edge of aggregated channel bandwidth, expressed in MHz. Fedge,high = FC,high + Foffset,high.

FInterferer (offset)	Frequency offset of the interferer (between the center frequency of the interferer and the carrier frequency of the carrier measured). For intra-band contiguous CA, the FInterferer (offset) is the frequency separation of the center frequency of the carrier closest to the interferer and the center frequency of the interferer

FInterferer	Frequency of the interferer

FIoffset	Frequency offset of the interferer (between the center frequency of the interferer and the closest edge of the carrier measured)

Foffset	Frequency offset from FC_high to the higher edge or FC_low to the lower edge.

Foffset,high	Frequency offset from FC,high to the upper UE RF Bandwidth edge, or from FC,block, high to the upper sub-block edge

Foffset,low	Frequency offset from FC,low to the lower UE RF Bandwidth edge, or from FC,block, low to the lower sub-block edge

FOOB	The boundary between the NR out of band emission and spurious emission domains

FREF	RF reference frequency

FREF-Offs	Offset used for calculating FREF

FREF, shift	RF reference frequency for Supplementary Uplink (SUL) bands, the uplink of all FDD bands, and TDD bands

Fuw (offset)	The frequency separation of the center frequency of the carrier closest to the interferer and the center frequency of the interferer

Gn100post connector	Declared value of the post chipset unit antenna connector gain for band n100, used for conversion of the radiated requirement into a conducted requirement (see principles described in annex M)

Gn101post connector 	Declared value of the post chipset unit antenna connector gain for band n101, used for conversion of the radiated requirement into a conducted requirement (see principles described in annex M)

GBChannel	Minimum guard band defined in clause 5.3.3, expressed in kHz

GBChannel(i)	Minimum guard band defined in clause 5.3.3 of carrier i

GBChannel,low	Minimum guard band defined in clause 5.3.3 for the lowest assigned component carrier in clause 5.3A.3

GBChannel,high	Minimum guard band defined in clause 5.3.3 for the highest assigned component carrier in clause 5.3A.3

LCRB	Transmission bandwidth which represents the length of a contiguous resource block allocation expressed in units of resources blocks

Max()	The largest of given numbers

Min()	The smallest of given numbers

Physical resource block number

NRACLR	NR ACLR

NRB	Transmission bandwidth configuration, expressed in units of resource blocks

NRB_agg	The number of the aggregated RBs within the fully allocated aggregated channel bandwidth

NRB_agg=i=1jNRBi*2μi for carrier 1 to j, where μ is defined in TS 38.211 [6]

NRB,c	The transmission bandwidth configuration of component carrier c, expressed in units of resource blocks

NRB,cj=NRBj*2μj for carrier j, where μ is defined in TS 38.211 [6]

NRB,LP-WUS	Number of RBs for LP-WUS

NRB,largest BW	The largest transmission bandwidth configuration of the component carriers in the bandwidth combination, expressed in units of resource blocks

NRB,low	The transmission bandwidth configurations according to Table 5.3.2-1 for the lowest assigned component carrier in clause 5.3A.1

NRB,high	The transmission bandwidth configurations according to Table 5.3.2-1 for the highest assigned component carrier in clause 5.3A.1

NREF	NR Absolute Radio Frequency Channel Number (NR-ARFCN)

NREF-Offs	Offset used for calculating NREF

PCMAX	The configured maximum UE output power

PCMAX, c	The configured maximum UE output power for serving cell c

PCMAX, f, c	The configured maximum UE output power for carrier f of serving cell c in each slot

PEMAX	Maximum allowed UE output power signalled by higher layers

PEMAX, c	Maximum allowed UE output power signalled by higher layers for serving cell c

PInterferer	Modulated mean power of the interferer

Plargest BW	Power of the largest transmission bandwidth configuration of the component carriers in the bandwidth combination

PPowerClass	The nominal UE power (i.e., no tolerance)

Pmax,c,AC	Maximum output power defined as the sum of measurement of all antenna connectors

Pmax,c,TABC	Maximum carrier output power defined as the sum of measurement of all TAB connectors

Prated,c,AC	Rated maximum output power defined as the sum of power over all antenna connectors

Prated,c,TABC	Rated maximum output power defined as the sum of power over all TAB connectors

P-MPRc	Power Management Maximum Power Reduction for serving cell c

PRB	The transmitted power per allocated RB, measured in dBm

PREFSENS_SL 	The REFSENS power for Sidelink

PUMAX	The measured configured maximum UE output power

Puw	Power of an unwanted DL signal

Pw	Power of a wanted DL signal

RBstart	The lowest RB index of transmitted resource blocks

RBstart_CA	The lowest RB index of transmitted resource blocks for intra-band contiguous CA

SCSc	SCS for the component carrier c, expressed in kHz

SCSlargest BW	SCS for the largest transmission bandwidth configuration of the component carriers in the bandwidth combination, expressed in kHz

SCSlow	SCS for the lowest assigned component carrier in clause 5.3A.1, expressed in kHz

SCShigh	SCS for the highest assigned component carrier in clause 5.3A.1, expressed in kHz

tp	Transient Period value signalled by the UE

tpstart	Start position of transient period relative to the symbol boundary

T(PCMAX, f, c)	Tolerance for applicable values of PCMAX, f, c for configured maximum UE output power for carrier f of serving cell c

TL,c	Absolute value of the lower tolerance for the applicable operating band as specified in clause 6.2.1

SSREF	SS block reference frequency position

UTRAACLR	UTRA ACLR

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

ACLR	Adjacent Channel Leakage Ratio

ACS	Adjacent Channel Selectivity

A-MPR	Additional Maximum Power Reduction

ASCS	Adjacent Subcarrier selectivity

ATG	Air-To-Ground

BS	Base Station

BW	Bandwidth

BWP	Bandwidth Part

CA	Carrier Aggregation

CA_nX-nY	Inter-band CA of component carrier(s) in one sub-block within Band nX and component carrier(s) in one sub-block within Band nY where nX and nY are the applicable NR operating bands

CC	Component Carriers

CG	Carrier Group

CP-OFDM	Cyclic Prefix-OFDM

CW	Continuous Wave

DC	Dual Connectivity

DFT-s-OFDM	Discrete Fourier Transform-spread-OFDM

DM-RS	Demodulation Reference Signal

DTX	Discontinuous Transmission

E-UTRA	Evolved UTRA

EIRP	Equivalent Isotropically Radiated Power

(e)RedCap	Redcap or eRedCap

eRedCap	enhanced Reduced Capability

EVM	Error Vector Magnitude

FAR	False Alarm Rate

FR	Frequency Range

FRC	Fixed Reference Channel

FRMCS	Future Railway Mobile Communication System

FWA	Fixed Wireless Access

GSCN	Global Synchronization Channel Number

HD	Half Duplex

IBB	In-band Blocking

IDFT	Inverse Discrete Fourier Transformation

ITS	Intelligent Transportation System

ITU-R	Radiocommunication Sector of the International Telecommunication Union

LP-WUR	Low Power-Wake Up Receiver

LP-WUS	Low Power-Wake Up Signal

LP-SS	Low Power-Synchronization Signal

LR	LP-WUR

MBW	Measurement bandwidth

MCG	Master Cell Group

MDR	Miss-Detection Rate

MOP	Maximum Output Power

MPR	Allowed maximum power reduction

MR	Main Radio

MSD	Maximum Sensitivity Degradation

NR	New Radio

NR-ARFCN	NR Absolute Radio Frequency Channel Number

NS	Network Signalling

OCNG	OFDMA Channel Noise Generator

OOB	Out-of-band

OOK	On-Off keying

P-MPR	Power Management Maximum Power Reduction

PRB	Physical Resource Block

PS	Public Safety

PSBCH	Physical Sidelink Broadcast CHannel

PSCCH	Physical Sidelink Control CHannel

PSFCH	Physical Sidelink Feedback CHannel

PSSCH	Physical Sidelink Shared CHannel

QAM	Quadrature Amplitude Modulation

RE	Resource Element

REFSENS	Reference Sensitivity

RedCap	Reduced Capability

RF	Radio Frequency

RMR	Railway Mobile Radio

RMS	Root Mean Square (value)

RSRP	Reference Signal Receiving PowerRx	Receiver

Rx	Receiver

SC	Single Carrier

SCG	Secondary Cell Group

SCS	Subcarrier spacing

SDL	Supplementary Downlink

SEM	Spectrum Emission Mask

SL	Sidelink

SL-MIMO	Sidelink-Multiple Antenna transmission

SL-U	Sidelink at unlicensed band

SNR	Signal-to-Noise Ratio

SRS	Sounding Reference Symbol

SS	Synchronization Symbol

S-SSB	Sidelink Synchronization Signal Block

SUL	Supplementary uplink

TAB	Transceiver Array Boundary

TAE	Time Alignment Error

TAG	Timing Advance Group

Tx	Transmitter

TxD	Tx Diversity

UAS	Uncrewed Aircraft Systems

UAV	Uncrewed Aerial Vehicle

UL MIMO	Uplink Multiple Antenna transmission

ULFPTx	Uplink Full Power Transmission

USS	UAS Service Supplier

V2X	Vehicle to Everything

XR	eXtended Reality

<<< Skip Unchanged Sections >>>

4.3	Specification suffix information

Unless stated otherwise, the suffixes shown in Table 4.3-1 are used for indicating at 2nd level clause. For shared spectrum channel access, suffixes A, B, and D are used for indicating at 3rd level clause. For V2X, suffixes A and F are used for indicating at 3rd level clause.

Table 4.3-1: Definition of suffixes

| Clause suffix | Variant |
| --- | --- |
| None | Single Carrier |
| A | Carrier Aggregation (CA) |
| B | Dual-Connectivity (DC) |
| C | Supplementary Uplink (SUL) |
| D | UL MIMO |
| E | V2X |
| F | Shared spectrum channel access |
| G | Tx Diversity (TxD) |
| H | Carrier Aggregation (CA) with UL MIMO |
| I | (e)RedCap |
| J | ATG |
| K | Aerial UE (UAV) |
| L | Carrier Aggregation (CA) with Tx Diversity |
| M | LP-WUR |

<<< Skip Unchanged Sections >>>

5.2M	Operating bands for LP-WUR

LP-WUS/WUR is designed to operate in the operating bands defined in Table 5.2-1, excluding bands n46, n47, n96, n102 and SDL bands.

<<< Skip Unchanged Sections >>>

5.3M		UE channel bandwidth for LP-WUR

5.3M.1	General

The LP-WUS carrier bandwidth corresponding to the UE channel bandwidth for LP-WUS/WUR is defined as the sum of resource blocks (RBs) occupied by the LP-WUS signal and the guard RBs separating it from the NR signal. The LP-WUS carrier is embedded within the NR channel and is flexibly positionable, provided alignment with the NR PRB grid is maintained.

A guard RB is referred to as an ASCS guard RB when located between an NR RB and an LP-WUS RB, and as an ACS guard RB when positioned between the NR guardband and an LP-WUS RB.

5.3M.2 	Maximum transmission bandwidth

The maximum transmission bandwidth NRB_WUR for LP-WUS within each NR UE channel bandwidth and subcarrier spacing is specified in Table 5.3M.2-1.

Table 5.3M.2-1: Maximum transmission bandwidth configuration NRB_LP-WUS for WUR

| SCS (kHz) | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NRB_LP-WUS | NRB_ LP-WUS | NRB_ LP-WUS | NRB_ LP-WUS | NRB_ LP-WUS | NRB_ LP-WUS | NRB_ LP-WUS | NRB_ LP-WUS | NRB_ LP-WUS | NRB_ LP-WUS | NRB_ LP-WUS | NRB_ LP-WUS | NRB_ LP-WUS | NRB_ LP-WUS | NRB_ LP-WUS | NRB_ LP-WUS |
| 15 | N/A | 11 | 11 | 11 | 11 | 11 | 11 | 11 | 11 | 11 | 11 | 11 | 11 | 11 | 11 | 11 |
| 30 | N/A | 11 | 11 | 11 | 11 | 11 | 11 | 11 | 11 | 11 | 11 | 11 | 11 | 11 | 11 | 11 |
| 60 | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |

<<< Skip Unchanged Sections >>>

## 7.1M	General for LP-WUR

The minimum requirements are specified assuming only one receiver for the wake-up signal. The criterion for verifying all receiver core RF requirements shall be MDR of the LP-WUS, which shall not exceed 1% with the LP-WUS parameters given in Annex A.3M.

## 7.2M	Diversity characteristics for LP-WUR

There is no diversity gain for the LP-WUR.

<<< Skip Unchanged Sections >>>

## 7.3M	Reference sensitivity for LP-WUR

7.3M.1	General

The reference sensitivity power level REFSENS is the minimum mean power applied to the UE antenna port, at which the MDR criterion with the side conditions specified in 7.1M shall be met.

The requirements are specified for different LP-WUR types. The LP-WUR types are specified in Table 7.3M.1-1 below.

Table 7.3M.1-1: The definitions of LP-WUR types

| LP-WUR type | Type description |
| --- | --- |
| 1 | The LP-WUR is designed for [ultra-low power consumption], assuming combined noise figure (NF) and implementation margin (IM) of 18dB. |
| 2 | The LP-WUR is designed for [moderate power consumption], assuming combined noise figure (NF) and implementation margin (IM) of 13.5dB. |
| NOTE 1: The LP-WUR types are expected to support OOK and/or OFDM-based LP-WUS signals, depending on the indicated UE capability. |  |

7.3M.2	Reference sensitivity power level for LP-WUR

The REFSENS for the two types of LR shall be specified as in Table 7.3M.2-1a. The requirement shall apply to all bands for which LP-WUS is specified in clause 5.2M and supported by the UE. The requirement applies for all channel BW included in sub-clause 5.3M.2.

Table 7.3M.2-1: One antenna port reference sensitivity for FDD and TDD bands for LP-WUS

| RX parameter | SCS kHz | Value (dBm) |
| --- | --- | --- |
| REFSENSLP-WUS for UEs supporting Type1 LR | 15 | -97.5 |
|  | 30 | -94.5 |
| REFSENSLP-WUS for UEs supporting Type2 LR | 15 | -102.0 |
|  | 30 | -99 |

Due to the different NF assumptions, reference sensitivity for LP-WUR in Table 7.3M.2-1 shall be modified by the amount given in ΔRNF in Table 7.3M.2-2 for the applicable operating bands.

Table 7.3M.2-2: Reference sensitivity allowance ΔRNF

| Operating band | ΔRNF (dB) |
| --- | --- |
| n41 | 2 |
| n77 | 1 |
| n78, n79 | 1.5 |
| n104 | 3 |

<<< Skip Unchanged Sections >>>

## 7.4M	Maximum input level for WUR

Maximum input level is defined as the maximum mean power received at the UE antenna port, at which the MDR criterion shall be met with the side conditions in clause 7.1M with parameters specified in Table 7.4M-1 and with parameters specified in Table 7.4M-2a and Table 7.4M-2b. The requirement applies for all channel BW included in sub-clause 5.3M.2.

Table 7.4M-1: Maximum input level for Type1 LR and Type2 LR

| Rx Parameter | Units |  | Maximum input level |  |
| --- | --- | --- | --- | --- |
| Power in LP-WUS | dBm |  | -25 |  |

Table 7.4M-2a: Maximum input level for Type1 LR

| RX parameter |  | Units |  | Value |
| --- | --- | --- | --- | --- |
| Power in LP-WUS |  | dBm |  | -25 |
| ASCS Guard RB1 |  | PRB |  | 1 |
| LP-WUS power boost |  | dB |  | 0 |
| NOTE 1: ASCS Guard RBs are zero-power and exist when NRB,LP-WUS < NRB. 1 Guard RB is configured for type-1 LR at the both sides of LP-WUS RB as specified in section 7.5M.3. NOTE 2: The interfering NR RBs consists of CP-OFDM QPSK with the same SCS as LP-WUS, in all PRB in the DL transmission BW configuration not occupied by either LP-WUS or Guard RB. |  |  |

Table 7.4M-2b: Maximum input level for Type2 LR

| RX parameter |  | Units | Value |  |  |
| --- | --- | --- | --- | --- | --- |
| Power in LP-WUS |  | dBm | -25 |  |  |
| ASCS Guard RB1 |  | PRB | 0 |  |  |
| LP-WUS power boost |  | dB | 0 |  |  |
| NOTE 1: ASCS Guard RBs are zero-power and exist when NRB,LP-WUS < NRB. 0 Guard RB is configured for type-2 LR as specified in section 7.5M.3. NOTE 2: The interfering NR RBs consists of CP-OFDM QPSK with the same SCS as LP-WUS, in all PRB in the DL transmission BW configuration not occupied by either LP-WUS or Guard RB. |  |  |  |  |  |

<<< Skip Unchanged Sections >>>

7.5M	Adjacent channel and subcarrier selectivity for LP-WUR

7.5M.1	General

Adjacent channel selectivity (ACS) is a measure of a receiver's ability to receive an LP-WUS signal at its configured channel frequency in the presence of an adjacent channel signal at a given frequency offset from the centre frequency of the assigned channel. ACS is the ratio of the receive filter attenuation on the configured channel frequency to the receive filter attenuation on the adjacent channel(s).

Adjacent subcarrier selectivity (ASCS) is a measure of a receiver's ability to receive an LP-WUS signal at its configured channel frequency in the presence of adjacent NR signal(s) at a given frequency offset (guard RB) between LP-WUS and NR.

7.5M.2	Adjacent channel selectivity for LP-WUR

The UE shall fulfil the minimum requirements specified in Table 7.5M.2-1 and Table 7.5M.2-2 for LP-WUR type 1 UE and the minimum requirements specified in Table 7.5M.2-3 and Table 7.5M.2-4 for LP-WUR type 2 UE. These requirements apply for all values of an adjacent channel interferer up to -25 dBm and for any SCS specified for the channel bandwidth of the wanted signal. For these test parameters, the Miss Detection Rate (MDR) for LP-WUS shall be ≤ 1 % with the reference measurement channels as specified in Annexes A.3M.1. The requirement applies for all channel BW included in sub-clause 5.3M.2.

Table 7.5M.2-1: Test parameters for LP-WUR type 1 UE, case 1

| RX parameter | Units | Value |
| --- | --- | --- |
| Power in LP-WUS | dBm | REFSENSLP-WUS + Y dB |
| Pinterferer | dBm | REFSENSLP-WUS + X dB |
| BWinterferer | MHz | 5 |
| Finterferer (offset) | MHz | BWChannel /2 + 2.5 / -(BWChannel /2 + 2.5) |
| NOTE 1: LP-WUS is configured at the edge of the NR carrier with 0 guard RBs, along with the minimum guardband specified in Table 5.3.3-1 for NR channel bandwidth. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with 15 kHz SCS. NOTE 3: The interferer consists of the NR interferer RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1. |  |  |

Table 7.5M.2-2: Test parameters for LP-WUR type 1 UE, case 2

| RX parameter | Units | Value |
| --- | --- | --- |
| Power in LP-WUS | dBm | TBD |
| Pinterferer | dBm | -25 |
| BWinterferer | MHz | 5 |
| Finterferer (offset) | MHz | BWChannel /2 + 2.5 / -(BWChannel /2 + 2.5) |
| NOTE 1: LP-WUS is configured at the edge of the NR carrier with 0 guard RBs, along with the minimum guardband specified in Table 5.3.3-1 for NR channel bandwidth. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with 15 kHz SCS. NOTE 3: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 |  |  |

Table 7.5M.2-3: Test parameters for LP-WUR type 2 UE, case 1

| RX parameter | Units |  | Value |  |  |
| --- | --- | --- | --- | --- | --- |
| Power in LP-WUS | dBm |  | REFSENSLP-WUS + M dB |  |  |
| Pinterferer | dBm |  | REFSENSLP-WUS + N dB |  |  |
| BWinterferer | MHz |  | 5 |  |  |
| Finterferer (offset) | MHz |  | BWChannel /2 + 2.5 / -(BWChannel /2 + 2.5) |  |  |
| NOTE 1: LP-WUS is configured at the edge of the NR carrier with 0 guard RBs, along with the minimum guardband specified in Table 5.3.3-1 for NR channel bandwidth. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with 15 kHz SCS. NOTE 3: The interferer consists of the NR interferer RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1. |  |  |

Table 7.5M.2-4: Test parameters for LP-WUR type 2 UE, case 2

| RX parameter | Units | Value |
| --- | --- | --- |
| Power in LP-WUS bandwidth | dBm | -56.5 |
| Pinterferer | dBm | -25 |
| BWinterferer | MHz | 5 |
| Finterferer (offset) | MHz | BWChannel /2 + 2.5 / -(BWChannel /2 + 2.5) |
| NOTE 1: LP-WUS is configured at the edge of the NR carrier with 0 guard RBs, along with the minimum guardband specified in Table 5.3.3-1 for NR channel bandwidth. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with 15 kHz SCS. NOTE 3: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 |  |  |

7.5M.3	Adjacent subcarrier selectivity for LP-WUR

The UE shall fulfil the minimum requirements specified in Table 7.5M.3-1 and 7.5M.3-2 for LP-WUR. The Miss Detection Rate (MDR) for LP-WUS shall be ≤ 1 % with the reference measurement channels as specified in Annexes A.3M.1. The requirement applies for all channel BW included in sub-clause 5.3M.2.

Table 7.5M.3-1: ASCS parameters for LP-WUR type 1 UE

| RX parameter | Units | Value |
| --- | --- | --- |
| Power in LP-WUS | dBm | REFSENSLP-WUS+0.5 |
| LP-WUS power boost2 | dB | 0 |
| NOTE 1: LP-WUS is configured at the center of NR carrier with 1 guard RB on each side of LP-WUS signal for NR channel bandwidth ≥ 5 MHz. NOTE 2: The NR signal occupies the remaining RB resources within the maximum transmission bandwidth configuration as interferer, excluding the LP-WUS RBs and the guard RB between the NR and LP-WUS signals. 0 power boost means equal PSD between NR RBs and LP-WUS RBs. |  |  |

Table 7.5M.3-1: ASCS parameters for LP-WUR type 2 UE

| RX parameter | Units | Value |
| --- | --- | --- |
| Power in LP-WUS bandwidth | dBm | REFSENSLP-WUS |
| LP-WUS power boost2 | dB | 0 |
| NOTE 1: LP-WUS is configured at the center of NR carrier with 0 guard RB on each side of LP-WUS signal for NR channel bandwidth ≥ 5 MHz. NOTE 2: The NR signal occupies the remaining RB resources within the maximum transmission bandwidth configuration as interferer, excluding the LP-WUS RBs and the guard RB between the NR and LP-WUS signals. 0 power boost means equal PSD between NR RBs and LP-WUS RBs. |  |  |

<<< Skip Unchanged Sections >>>

7.6M	Blocking characteristics for LP-WUR

7.6M.1	General

The blocking characteristic is a measure of the receiver's ability to receive a wanted signal at its assigned channel frequency in the presence of an unwanted interferer on frequencies other than those of the spurious response or the adjacent channels, without this unwanted input signal causing a degradation of the performance of the receiver beyond a specified limit. The blocking performance shall apply at all frequencies except those at which a spurious response occurs.

7.6M.2	In-band blocking for LP-WUR

The UE shall fulfil the minimum requirements specified in Table7.6M.2-1 and Table 7.6M.2-2 for LP-WUR type 1 UE and LP-WUR type 2 UE respectively. The test parameters for the interferer are chosen as in Table 7.6M.2-3 for verification of the requirements specified in Table 7.6M.2-1 and Table 7.6M.2-2. For these test parameters, the Miss Detection Rate (MDR) for LP-WUS signal shall be ≤ 1 % with the reference measurement channels as specified in Annexes A.3M.1. The requirement applies for all channel BW included in sub-clause 5.3M.2.

Table 7.6M.2-1: In-band blocking parameters for LP-WUR type 1 UE

| RX parameter | Units | Value |
| --- | --- | --- |
| Power in LP-WUS | dBm | TBD |
| BWinterferer | MHz | 5 |
| FIoffset, case 1 | MHz | 7.5 |
| FIoffset, case 2 | MHz | 12.5 |
| NOTE 1: LP-WUS is configured at the edge of the NR carrier, with the minimum guardband specified in Table 5.3.3-1 for NR channel bandwidth. NOTE 2: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 |  |  |

Table 7.6M.2-2: In-band blocking parameters for LP-WUR type 2 UE

| RX parameter | Units | Value |
| --- | --- | --- |
| Power in LP-WUS | dBm | TBD |
| BWinterferer | MHz | 5 |
| FIoffset, case 1 | MHz | 7.5 |
| FIoffset, case 2 | MHz | 12.5 |
| NOTE 1: LP-WUS is configured at the edge of the NR carrier, with the minimum guardband specified in Table 5.3.3-1 for NR channel bandwidth. NOTE 2: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 |  |  |

Table 7.6M.2-3: In-band blocking for LP-WUR

| Parameter | Unit |  | Case 1 |  | Case 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Pinterferer | dBm |  | -56 |  | -44 |  |
| Finterferer (offset) | MHz |  | -BW/2 – FIoffset, case 1 and BW/2 + FIoffset, case 1 |  | ≤ -BW/2 – FIoffset, case 2 and ≥ BW/2 + FIoffset, case 2 |  |

7.6M.3	Out-of-band blocking for LP-WUR

The out-of-band band blocking is defined for an unwanted CW interfering signal falling outside a frequency range 15 MHz below or above the UE receive band. The Miss Detection Rate (MDR) for LP-WUS signal shall be ≤ 1 % with the reference measurement channels as specified in Annexes A.3M.1 with parameters for the wanted signal as specified in Table 7.6M.3-1 and Table 7.6M.3-2 for LP-WUR type 1 and type 2 UE respectively, and for the interferer as specified in Table 7.6M.3-3. The requirement applies for all channel BW included in sub-clause 5.3M.2.

Table 7.6M.3-1: Out-of-band blocking parameters LP-WUR type 1 UE

| LP-WUS bandwidth (RBs) | Power in LP-WUS (dBm) |  |
| --- | --- | --- |
| 11 | REFSENSLP-WUS + TBD dB |  |

Table 7.6M.3-2: Out-of-band blocking parameters LP-WUR type 2 UE

| LP-WUS bandwidth (RBs) | Power in LP-WUS (dBm) |  |
| --- | --- | --- |
| 11 | REFSENSLP-WUS + TBD dB |  |

Table 7.6M.3-3: Out of-band blocking for LP-WUR

| Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- |
| Pinterferer | dBm | -44 | -30 | -15 |
| Finterferer (CW) | MHz | -60 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -85 < f – FDL_low ≤ -60 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 85 or FDL_high + 85 ≤ f ≤ 12750 |

For interferer frequencies across ranges 1, 2 and 3 in Table 7.6M.3-3, a maximum of

max24,6⋅n⋅NRB/6minn⋅NRB/10,5

exceptions are allowed for spurious response frequencies in each assigned frequency channel when measured using a step size of 5 MHz with NRB the number of resource blocks in the downlink transmission bandwidth configuration of NR signal, BWChannel the bandwidth of the frequency channel in MHz and n = 1, 2 for SCS = 15, 30 kHz, respectively. For these exceptions, the requirements in clause 7.7M apply.

7.6M.4	Narrow band blocking for LP-WUR

This requirement is measure of a receiver's ability to receive a LP-WUS signal at its assigned channel frequency in the presence of an unwanted narrow band CW interferer at a frequency, which is less than the nominal channel spacing.

The Miss Detection Rate (MDR) for LP-WUS signal shall be ≤ 1 % with the reference measurement channels as specified in Annexes A.3M.1 with parameters specified in Table 7.6M.4-1 and Table 7.6M.4-2 for LP-WUR type 1 and type 2 UE respectively.

Table 7.6M.4-1: Narrow Band Blocking for LP-WUR type 1 UE

| Parameter | Unit | Value |  |  |
| --- | --- | --- | --- | --- |
| Pw | dBm | PREFSENS_LP-WUS + TBD |  |  |
| Puw (CW) | dBm | -55 |  |  |
| Fuw (offset SCS= 15 kHz) | MHz | BWChannel2+0.2SCS+0.5+0.5SCS |  |  |
| NOTE 1: Fuw shall be rounded to half of SCS NOTE 2: LP-WUS is configured at the edge of the NR carrier, with the minimum guardband specified in Table 5.3.3-1 for NR channel bandwidth ≥ 5 MHz. |  |  |  |  |

Table 7.6M.4-2: Narrow Band Blocking for LP-WUR type 2 UE

| Parameter | Unit | Value |  |  |
| --- | --- | --- | --- | --- |
| Pw | dBm | PREFSENS_LP-WUS _+ TBD |  |  |
| Puw (CW) | dBm | -55 |  |  |
| Fuw (offset SCS= 15 kHz) | MHz | BWChannel2+0.2SCS+0.5+0.5SCS |  |  |
| NOTE 1: Fuw shall be rounded to half of SCS NOTE 2: LP-WUS is configured at the edge of the NR carrier, with the minimum guardband specified in Table 5.3.3-1 for NR channel bandwidth ≥ 5 MHz. |  |  |  |  |

<<< Skip Unchanged Sections >>>

7.7M	Spurious response for LP-WUR

Spurious response is a measure of the ability of the receiver to receive a wanted signal on its assigned channel frequency without exceeding a given degradation due to the presence of an unwanted CW interfering signal at any other frequency for which a response is obtained, i.e. for which the out-of-band blocking limit as specified in clause 7.6M.3 is not met.

The MDR criterion with the side conditions specified in 7.1M shall be met with parameters for LP-WUS as specified in Table 7.7M-1 and Table 7.7M-2 for Type 1 LP-WUR and Type 2 LP-WUR respectively, and for the interference as specified in Table 7.7M-3. The requirement applies for all channel BW included in sub-clause 5.3M.2.

Table 7.7M-1  Spurious response parameters for Type 1 LP-WUR

| RX parameter | Units | Value |
| --- | --- | --- |
| Power in LP-WUS | dBm | REFSENSLP-WUS + [TBD]dB |
| NOTE 1: LP-WUS is configured at the center of the NR carrier without adjacent sub-carrier interference. |  |  |

Table 7.7M-2  Spurious response parameters for Type 2 LP-WUR

| RX parameter | Units | Value |
| --- | --- | --- |
| Power in LP-WUS | dBm | REFSENSLP-WUS + [TBD]dB |
| NOTE 1: LP-WUS is configured at the center of the NR carrier without adjacent sub-carrier interference. |  |  |

Table 7.7M-3  Spurious response for LP-WUR

| Parameter | Unit | Level |
| --- | --- | --- |
| PInterferer (CW) | dBm | -44 |
| FInterferer | MHz | Spurious response frequencies |

<<< Skip Unchanged Sections >>>

7.8M	Intermodulation characteristics for LP-WUR

7.8M.1	General

Intermodulation response rejection is a measure of the capability of the receiver to receive a wanted signal on its assigned channel frequency in the presence of two or more interfering signals which have a specific frequency relationship to the wanted signal.

7.8M.2	Wide band Intermodulation for LP-WUR

The wide band intermodulation requirement is defined using a CW carrier and modulated NR signal as interferer 1 and interferer 2 respectively.

The MDR criterion with the side conditions specified in 7.1M shall be met with parameters specified in Table 7.8M.2-1 and Table 7.8M.2-2 for Type 1 LP-WUR and Type 2 LP-WUR respectively. The requirement applies for all channel BW included in sub-clause 5.3M.2.

Table 7.8M.2-1: Wide band intermodulation parameters for Type 1 LP-WUR

| Rx parameter | Units | Value |
| --- | --- | --- |
| Pw in LP-WUS | dBm | REFSENSLP-WUS + [TBD]dB |
| PInterferer 1 (CW) | dBm | -46 |
| PInterferer 2 (Modulated) | dBm | -46 |
| BWInterferer 2 | MHz | 5 |
| FInterferer 1 (Offset) | MHz | -BWChannel/2 – 7.5 / +BWChannel/2 + 7.5 |
| FInterferer 2 (Offset) | MHz | 2*FInterferer 1 |
| NOTE 1: LP-WUS is configured at the center of the NR carrier without adjacent sub-carrier interference. NOTE 2: The modulated interferer consists of the Reference measurement channel specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 and the same SCS as the wanted signal. NOTE 3: The Finterferer 1 (offset) is the frequency separation of the center frequency of the NR carrier closest to the interferer and the center frequency of the CW interferer and Finterferer 2 (offset) is the frequency separation of the center frequency of the NR carrier closest to the interferer and the center frequency of the modulated interferer. |  |  |

Table 7.8M.2-2: Wide band intermodulation parameters for Type 2 LP-WUR

| Rx parameter | Units | Value |
| --- | --- | --- |
| Pw in LP-WUS | dBm | REFSENSLP-WUS + [TBD]dB |
| PInterferer 1 (CW) | dBm | -46 |
| PInterferer 2 (Modulated) | dBm | -46 |
| BWInterferer 2 | MHz | 5 |
| FInterferer 1 (Offset) | MHz | -BWChannel/2 – 7.5 / +BWChannel/2 + 7.5 |
| FInterferer 2 (Offset) | MHz | 2*FInterferer 1 |
| NOTE 1: LP-WUS is configured at the center of the NR carrier without adjacent sub-carrier interference. NOTE 2: The modulated interferer consists of the Reference measurement channel specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 and the same SCS as the wanted signal. NOTE 3: The Finterferer 1 (offset) is the frequency separation of the center frequency of the NR carrier closest to the interferer and the center frequency of the CW interferer and Finterferer 2 (offset) is the frequency separation of the center frequency of the NR carrier closest to the interferer and the center frequency of the modulated interferer. |  |  |

<<< Skip Unchanged Sections >>>

7.9M	Spurious emissions for LP-WUR

For LP-WUS UE, the spurious emissions as specified in clause 7.9 applies.

<<< Skip Unchanged Sections >>>

# A.3M	LP-WUR reference channel

## A.3M.1	General

Unless otherwise stated, Table A.3M.1-1 is applicable for all receiver requirements of LP-WUR when LP-WUS is generated with 15 KHz SCS. Table A.3M.1-2 is applicable for all receiver requirements of LP-WUR when LP-WUS is generated with 30 KHz SCS.

Table A.3M.1-1. Common reference channel parameters for 15 KHz SCS.

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| MR Channel bandwidth |  | MHz | All CBW |
| LP-WUS bandwidth |  | RB | 11 |
| Subcarrier spacing |  | KHz | 15 |
| RM coding |  | Bits | 16 |
| CRC |  |  | No CRC |
| Chip rate |  |  | M=4 (4 chips in an OFDM symbol) |
| Overlaid OFDM sequence |  |  | Length 33: generated by 31-length ZC sequence with extension |
| Number of overlaid OFDM sequence per chip to carry information |  |  | 4 |
| LP-WUS duration for OOK |  |  | 8 OFDM symbols |
| LP-WUS duration for OFDM |  |  | 2 OFDM symbols |
| Manchester coding for OOK |  |  | 1/2 |
| Number of information bits |  | Bits | 5 |

Table A.3M.1-2. Common reference channel parameters for 30 KHz SCS.

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| MR Channel bandwidth |  | MHz | All CBW |
| LP-WUS bandwidth |  | RB | 11 |
| Subcarrier spacing |  | KHz | 30 |
| RM coding |  | Bits | 16 |
| CRC |  |  | No CRC |
| Chip rate |  |  | M=4 (4 chips in an OFDM symbol) |
| Overlaid OFDM sequence |  |  | Length 33: generated by 31-length ZC sequence with extension |
| Number of overlaid OFDM sequence per chip to carry information |  |  | 4 |
| LP-WUS duration for OOK |  |  | 8 OFDM symbols |
| LP-WUS duration for OFDM |  |  | 2 OFDM symbols |
| Manchester coding for OOK |  |  | 1/2 |
| Number of information bits |  | Bits | 5 |

## <<< END OF CHANGES >>>