3GPP TSG-RAN4 Meeting #116	R4-2510294

# Bangalore, India, 25 Aug - 29 Aug, 2025

Source:	vivo

Title:	TP for TR 38.774 on LP-WUS UE RF General

Agenda item:			7.24.3.1

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 38.774 to add general part, simulation summary and other RF related content for LP-WUS.

This is the last meeting of LP-WUS WI core part, the TR 38.774 should be finalized, and it will be formally released in next RAN plenary.

This contribution captures some simulations from companies in RAN4#115, might be revised during meeting and further updated based on companies’ input.

2. Reference

[1] TR 38.774 v0.4.0

# 3. Text Proposal

<Start of Text Proposal>

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Main radio (MR): the Tx/Rx module operating for NR signals/channels apart from signals/channel related to low-power wake-up.

LP-WUR (LR): The Rx module operating for receiving/processing signals/channel related to low-power wake-up.

LP-WUS power boost: difference between the average power of LP-WUS REs (which occupy certain REs within a NR transmission bandwidth configuration) and the average power over all REs (from both LP-WUS and the NR carrier containing the LP-WUS REs)

3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

NRB	Transmission bandwidth configuration, expressed in units of resource blocks

NRB,LP-WUS	Number of RBs for LP-WUS

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

<ABBREVIATION>	<Expansion>

ACS	Adjacent Channel Selectivity

ADC	Analog to Digital Converter

ASCS	Adjacent Subcarrier selectivity

BB	Base Band

BLER	Block Error Rate

BPF	Band Pass Filter

BS	Base Station

CFO	Center frequency offset

FAR	False Alarm Rate

FR1	Frequency range 1

FR2	Frequency range 2

ICS	In-channel Selectivity

IF	Intermediate Frequency

LP-WUS	Low Power-Wake Up Signal

LP-WUR	Low Power-Wake Up Receiver

LP-SS	Low Power- Synchronization Signal

LO	Local Oscillator

LNA 	Low Noise Amplifier

LPF 	Low Pass Filter

LR	LP-WUR

MDR	Miss Detection Rate

MR	Main Radio

NF	Noise Figure

OOB	Out-of-band

OOK	On-Off keying

OFDM	Orthogonal Frequency Division Multiplexing

RE	Resource Element

REFSENS	Reference Sensitivity

RF	Radio Frequency

SCS	Subcarrier spacing

SINR	Signal to Interference plus Noise Ratio

SNR	Signal to Noise Ratio

UE	User Equipment

Tx	Transmitter

<Skip>

6.3	Simulation evaluation results

### 6.3.1	General

To evaluate some RF performance, link level simulations are performed to collect analysis input.

### 6.3.2	SNR simulations

In this sub-clause, SNR simulation results from different companies are collected for analysis of target SNR for LP-WUR.

Table 6.3.2-1 SNR simulation summary for FR1 Envelop-detection LP-WUR (OOK-4 M=4)

| Companies Input | SNR |  | RM coding | Timing error |  | Note |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| A | -6 to -2dB |  | 32/16/8 | 4us |  | -2.15 dB (8 bit), -4.63 dB (16 bit), -5.94 dB (32 bit) |  |
| B | -7 to -3dB |  |  | 0us |  | 5th order of lowpass Butterworth |  |
| C | -7.5 dB to -2.5dB |  | 32/16/8 | 0us |  | simulated SNR with 4bit ADC or 8bit ADC is nearly the same |  |
| D | -7.1 to -2.5dB |  | 32/16/8 |  |  | OOK SNR: RM coding, 8bit with -2.5dB, 16bit with -5.3dB, 32bit with -7.1dB. |  |
| F | -7.7dB to -3.2dB |  | 32/16/8 | 0/0.9us |  | 0.9us TE will introduce less than 1dB degradation |  |
| G | -9.2 to -5.9dB |  |  |  |  | 8bit with -5.9dB, 16bit with -7.5dB, 32bit with -9.2dB. |  |
| H | -8.0 dB |  | 32 bits |  |  | under 4us TE has no impact |  |
| I | -7.8 to -5.9 dB |  | 32/16/8 |  |  | Payload: 5 bits with RM, w/o MC |  |

Observations and Summary of SNR simulation outcome for envelop-detection receiver: most companies perform analysis based on RM coding of 8/16/32 bits, and majority views prefer to use 16bit RM coding as RMC and derive target SNR. The final target SNR is agreed as -4.5dB for LP-WUS envelop-detection.

Table 6.3.2-2 SNR simulation summary for FR1 OFDM-based LP-WUR

| Companies Input | SNR | Timing error |  | Note |  |
| --- | --- | --- | --- | --- | --- |
| A | -4dB |  |  | no big difference for the applicable SNR for both OOK based and OFDM based signals |  |
| B | -4dB |  |  |  |  |
| C | -5.5dB |  |  |  |  |
| D | -7.4dB |  |  |  |  |
| F | -8.0dB |  |  | OFDM-based receiver outperforms OOK-based receiver by 2.5~3dB SNR |  |
| G | -13.8 to -9.9dB |  |  |  |  |
| H | -3.5dB |  |  |  |  |

Observations and Summary of SNR simulation outcome for OFDM-based receiver: There is no repetition for OFDM-based receiver, the simulated SNR from companies is not much far from the simulated performance of LP-WUS envelop-detection. With consideration of other impacted aspects, the final target SNR of OFDM-based receiver is also specified as -4.5dB.

### 6.3.3	ASCS simulations

In this sub-clause, some ASCS simulation results from different companies are collected for analysis of in-band interference performance of LP-WUS. This is for information only, because actual implementation choice of filtering is driven by jammer requirements.

In SI phase, RAN4 reach the following conclusion for ASCS: Based on the following analysis, RAN4 observed that for 5th order filter, the guard RB number for LP-WUS ASCS is in the range of 0RB ~ 1RBs for 30KHz SCS, or 0RBs ~2RBs for 15KHz SCS. Similar number of guard RBs could be applied also with lower filter orders as performance was observed to remain similar with 3rd order filter.

In the WI phase, further detailed analysis is performed by companies, these analyses show a similar trend that the ASCS degradation is not that significant (0~1dB), even no guard RB is reserved.

Table 6.3.3-1 ASCS simulation summary for FR1 Envelop-detection and OFDM-based LP-WUR

| Companies Input | SNR degradation | Guard RB | Phase noise | Note |  |
| --- | --- | --- | --- | --- | --- |
| A | unnegligible | 1 | Considered |  |  |
| B | negligible | 0 | N/A | the number of guard RB does not affect the MDR performance significantly |  |
| C | Less than 0.2dB | 0 | Considered | benefit of guard RB in ASCS case is minor |  |
| F | negligible | 0 | Considered | With 1 guard RB, there is minor improvement on the required SNR to achieve 1% BLER compared to 0 guard RB |  |
| H | 1dB | 0 | WiFi phase noise | an approximately 1 dB of degradation in sensitivity level at MDR=1% for both AWGN channel and for the TDL-C300 channel |  |
| J | ~0.5dB | 0 | N/A | Maximum of 1 guard RB at each side |  |

### 6.3.4	ACS simulations

In this sub-clause, ACS simulation results from different companies are collected for analysis of ACS performance of LP-WUR. It was agreed that the interference level of LP-WUR ACS is the same as MR, for both type1 and type 2 LP-WUR. This is for information purpose only, because ACS requirements are driven by agreed coexistence requirements applied to the LR.

Table 6.3.4-1 ACS simulation summary for FR1 Type-1 LP-WUR [This table will be updated based on latest input]

| Companies Input | Detection | ACS | Guard RB | Interference level | Wanted signal level | Filter order | Timing error | Phase noise | Note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A | Envelop detection |  |  |  |  |  |  |  |  |
|  | OFDM |  |  |  |  |  |  |  |  |
| B | Envelop detection |  |  |  |  |  |  |  |  |
|  | OFDM |  |  |  |  |  |  |  |  |
| C | Envelop detection |  |  |  |  |  |  |  |  |
|  | OFDM |  |  |  |  |  |  |  |  |

Summary of ACS simulation outcome:

### 6.3.5	FR2 simulations

[To be added based on latest input]

### 6.3.6	Other aspects

TBA

7.1.3	Architecture and NF considerations

The basic architecture considered for the LP_WUR is a zero-IF architecture for both envelope and sequence based detectors. This assumption is common for both FR1 and FR2. This is just an assumption made to derive parameters which dictate the requirements. This assumption does not preclude any other RF implementations.

In case of FR1, only single RX is assumed. For FR2 requirement derivation, the baseline assumption is an OFDM based receiver with two receiver chains with mutually orthogonally polarized antennas.

In RAN4, there had been no consensus on what IM includes, and no agreement was achieved on individual values of NF and IM, so a joint number was agreed. There are two sets of NF + IM values agreed, and these will be used for REFSENS calculations. The values are,

-	Set 1: 18 dB,

-	Set 2: 13.5 dB.

The above mentioned numbers are for FR1.

Based on above differentiation of LP-WUR, RAN4 will specify two sets of RF requirement, i.e., type-1 LR requirements and type-2 LR requirement. The type-1 and type-2 requirement is not directly mapped to OOK-based and OFDM-based receiver. A UE can only support either Type-1 or Type-2 but not both.

<Skip>

8.1	Testability for UE Performance verification

8.1.1	General Framework

It has been agreed that same LP-WUS RF requirements will apply to all RRC states, but only one RRC state needs to be tested. It is agreed to verify LP-WUR based on 1% MDR of LP-WUS which can be tested based on UE’s response to the NW/TE upon successfully detecting the LP-WUS (e.g. ACK/NACK in CONNECTED state or MSG1/3 in IDLE state or other methods). The ultimate test method including the potential test mode will be decided by RAN5 based on RAN4 input.

For demodulation requirements FAR will also be used.

8.1.2	other

FR2 LP-WUR spherical coverage can be skipped if the reference sensitivity requirement is met.

For idle-mode only or connected-mode only LP-WUR, the test should be done accordingly. For LP-WUR support both idle-mode and connected-mode, RF requirements testing under connected mode is sufficient

Given LP-WUS RF requirement is different from traditional Rx testing of MR based on connected close-loop condition, there for the testing time reduction for performance verification is important when specify LP-WUS conformance test case in RAN5.

<End of Text Proposal>