3GPP TSG-RAN WG4 Meeting #116	R4-2510299

Bengaluru, India, August 25th – August 29th, 2025

Source:	Nokia

Title:	System parameters for LP-WUR

Agenda item:	7.24.3.1

Document for:	Discussion

Introduction

In this document we discuss the feature list for LP-WUS.

Discussion

Feature list for LP-WUS

In the Wuhan meeting [1], the following agreement was made regarding UE capabilities with LP-WUR types.

Issue 2-1-5: How to map two LP-WUR types with UE capability Agreement: - RAN4 define two LP-WUR types (Type 1 and Type 2), no need 1-to-1 mapping between waveform and LP-WUR type. - Requirement applicability should be clearly defined. - RAN4 need to further discuss how to design the RMC for each type, how to ensure the performance of each type properly.

Further, it is RAN4 understanding that type1 UE requirements are derived based on demodulation of OOK signal and Type2 UE is based on demodulation of OFDM signal.

Issue 2-3-1: ASCS requirements for OOK-based receiver Agreement: - For Type1 UE requirements, which are derived based on the assumption that demodulation of OOK content of LP-WUS is targeted - 1 RB guard for ASCS and 0.5dB de-sens - For Type2 UE requirements, which are derived based on the assumption that demodulation of OFDM content of LP-WUS is targeted - 0 RB guard for ASCS and 0 dB de-sens - For Type 1 and Type2 UE are not limited to specific LP-WUS waveform. - FFS how to define test case and requirement applicability for Type 1 and Type 2 - FFS how type 1 and type 2 capability related to RAN1 UE feature - A UE can only support either Type 1 or Type 2 but not both for LP-WUR

This understanding matches with what RAN1 has agreed regarding the feature list regarding LP-WUR types with UE [2].

- 62-1: LP-WUS operation in IDLE/INACTIVE mode based on OOK signal - 62-1a: LP-WUS operation in IDLE/INACTIVE mode based on OFDM overlaid sequence - 62-2: LP-WUS operation in CONNECTED mode based on OOK signal - 62-2a: LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence

Thus, our understanding is that there is no need for any additional feature list from RAN4 regarding distinguishing Type1 and Type2 LR.

Agree to have no addition to the feature list for distinguishing Type1 and Type2 LR from RAN4 UE RF side regarding LP-WUS.

Feature group 62-1 and 62-2 shall apply for Type1 LR requirements and feature group 62-1a and 62-2a shall apply for RAN4 Type 2 LR requirements.

Conclusion

The following Observations and Proposals were made:

Proposal 1: Agree to have no addition to the feature list for distinguishing Type1 and Type2 LR from RAN4 UE RF side regarding LP-WUS.

Proposal 2: Feature group 62-1 and 62-2 shall apply for Type1 LR requirements and feature group 62-1a and 62-2a shall apply for RAN4 Type 2 LR requirements.

- References

- R4-2505234, WF for [114bis][131] NR_LPWUS_UERF, Vivo, RAN4#114bis, Wuhan, Hubei, China, 07th – 11th April, 2025.

- R1-2504673, Updated RAN1 UE features list for Rel-19 NR after RAN1 #121, Moderators (AT&T, NTT DOCOMO, INC.), RAN1#121, St Julian’s, Malta, 19th – 23rd May, 2025.