3GPP TSG-RAN WG4 Meeting #116	R4-2510301

Bengaluru, India, August 25th – August 29th, 2025

Source:	Nokia

Title:	Other RX requirements for LP-WUR

Agenda item:	7.24.3.3

Document for:	Discussion

Introduction

In this document we present our views on issues regarding other LP-WUR specific requirements. Following agreements were made in the last RAN4 meeting [1].

### Sub-topic 3-1 IBB, OBB and Intermodulation requirements for UE RF Issue 3-1-4: detailed requirements and table for test parameters of IBB, OBB, NBB, Intermodulation test case WF: - Finalize detailed test parameters next meeting. endorse the definition and clause structure, requirement table (with requirement TBD) this meeting. ### Sub-topic 3-2 Other Rx requirements Issue 3-2-1: spurious response requirements for FR1 LP-WUR WF: - Finalize detailed test parameters next meeting. endorse the definition and clause structure, requirement table (with requirement TBD) this meeting. Issue 3-2-2: spurious emission requirements for FR1 LP-WUR Agreements: - For FR1, the spurious emission requirements for LP-WUR will be considered same as MR. - For FR2, the spurious emission requirements for LP-WUR will be considered same as MR. Issue 3-2-3: Maximum input level requirements for FR1 LP-WUR Agreements: - In spec, for requirements using -25dBm as single value. - The max. input power level is specified for all channel bandwidths. FFS the side condition - in the presence of legacy RBs with the same PSD as the LPWUS RBs. - And no NR RBs in the CBW Issue 3-2-5: detailed table for test parameters of spurious response, spurious emission, Maximum input level, and other test cases WF: - Finalize detailed test parameters next meeting. endorse the definition and clause structure, requirement table (with requirement TBD) this meeting.

Discussion

Blocking requirements

Blocking characteristics are measure of the receiver ability to receive the wanted signal in presence of interference on frequencies other than its designated frequencies. Given LP-WUR will observe the same radio environment as MR, it should be able to withstand interferer with similar level as of MR. Thus, the interferer power levels, and configuration should be the same as of MR.

Reuse Table 7.6.2-2, 7.6.2-4, 7.6.3-2, 7.6.3-4 for in-band and out-of-band blocking levels for LP-WUR.

Endorse the draft CR presented in R4-2510305.

Spurious emission

Spurious emissions are the narrow band CW emissions appearing at the UE antenna connector. Receiver spurious emissions mainly come from the LO in the receiver. As illustrated in Figure 1 marked with the green line can the LO leakage end at the antenna or antenna connector. The main LO frequency will dominate in-band spurious emissions, and the harmonic will dominate out-band spurious emissions. In a design with potential low isolation from LO to RF input an LNA will be required to attenuate the in-band spurious and a band pass filter to ensure out-band spurious is suppressed. Given that the current requirements are regulatory in nature, they should apply to the LP-WUR.

Reuse the spurious emission requirements from TS 38.101-1 for LP-WUR.

Figure 1Illustration of a DC receiver.

Conclusion

The following Observations and Proposals were made:

Proposal 1: Reuse Table 7.6.2-2, 7.6.2-4, 7.6.3-2, 7.6.3-4 for in-band and out-of-band blocking levels for LP-WUR.

Proposal 2: Endorse the draft CR presented in R4-2510305.

Proposal 3: Reuse the spurious emission requirements from TS 38.101-1 for LP-WUR.

- References

- R4-2507936, WF for [115][136] NR_LPWUS_UERF, Vivo, RAN4#115, Malta, MT, 19th – 23rd May, 2025.

- R4-2505234, WF for [114bis][131] NR_LPWUS_UERF, Vivo, RAN4#114bis, Wuhan, China, 7th – 11th April, 2025

- R4-2503028, WF on UE RF requirements for LP-WUS, Vivo, RAN4#114, Athens, Greece, 17th – 21st Feb, 2025.