3GPP TSG-RAN4 Meeting #116	R4-2510304

# Bengaluru, India, 25th August 2025 – 29th August 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | draftCR | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS 38.101-1 on REFSENS requirements for LP-WUR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_LPWUS-Core |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Adding the REFSENS requirements for LP-WUS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Updating the previously agreed table with numbers. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | LP-WUS will not be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3M |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | This draft CR is on top of the previously endorsed big draft CR R4-2506882 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

************************************ START OF CHANGES ************************************

## 7.3M	Reference sensitivity for LP-WUR

### 7.3M.1	General

The reference sensitivity power level REFSENS is the minimum mean power applied to the UE antenna port, at which the MDR criterion with the side conditions specified in 7.1M shall be met.

### 7.3M.2	Reference sensitivity power level for LP-WUR

The REFSENS for the two types of LR shall be as specified in Table 7.3M.2-1a and 7.3M.2-1b.

Table 7.3M.2-1a: One antenna port reference sensitivity for FDD and TDD bands with FDL_high < 2700 MHz

| RX parameter | SCS kHz | Channel bandwidth (MHz) |
| --- | --- | --- |
|  |  | 3, 5, 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70,80, 90, 100 MHz (dBm) |
| REFSENS for UEs supporting [Type1 LR] | 15 | -97.5 |
|  | 30 | -94.5 |
| REFSENS for UEs supporting [Type2 LR] | 15 | -102.0 |
|  | 30 | -99.0 |

Table 7.3M.2-1b: One antenna port reference sensitivity for FDD and TDD bands with FDL_low ≥ 3300 MHz

| RX parameter | SCS kHz | Channel bandwidth (MHz) |
| --- | --- | --- |
|  |  | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70,80, 90, 100 MHz (dBm) |
| REFSENS for UEs supporting [Type1 LR] | 15 | -97.0 |
|  | 30 | -94.0 |
| REFSENS for UEs supporting [Type2 LR] | 15 | -101.6 |
|  | 30 | -98.5 |

************************* UNCHANGED CLAUSES HAVE BEEN OMITTED *************************

************************************ END OF CHANGES ************************************