3GPP RAN4 Meeting #116		R4-2510320

Bengaluru, India, 25th – 29th August, 2025

Agenda item:	7.22.5

Source:	Ericsson

Title:	Discussion on OTA testing for A-IoT device 1

WI/SI:	Ambient_IoT_Solutions-Core

Release:	Rel-19

Document for:	Approval

# Introduction

During RAN4 #115 meeting, there were a lot of discussions concerning performance metric for A-IoT device 1. This contribution provides our views related to this issue.

# Discussion

## Performance metric for A-IoT device 1

The use of 3D spherical scan-based performance metrics offers a comprehensive and unbiased method of evaluating radiated power and sensitivity. By measuring the average radiated power or sensitivity across all directions over a 3D sphere, this approach ensures that each direction is treated equally, without any preference. This direction-agnostic methodology provides a holistic assessment of a device's radiated power and sensitivity, making it particularly suitable for A-IoT devices.

However, A-IoT device 1 presents a unique challenge in that it lacks an internal RF transmitter and relies solely on backscattering an external Continuous Wave (CW) signal generated by a CW Transmitter (CWT) node. To accurately measure 3D spherical scan-based performance metrics for A-IoT device 1, the relative angle between the CWT node and the Device Under Test (DUT) must remain fixed during the DUT's rotation. This can be achieved if the CWT is positioned either below or above the DUT, allowing both to rotate only in the azimuth plane while the testing antenna moves along a boom or uses multiple probes mounted on a boom, as shown in Figure 1. This setup can yield precise and reproducible measurement results, but it excludes dual-axis rotation anechoic chamber measurement systems. Additionally, it is limited to testing only the CW normal incidence case.

Figure 1. Performing 3D scanning for A-IoT device 1 testing

As an alternative, performance metrics for A-IoT devices can be derived from a 2D scan by fixing both the DUT and CWT node and moving the testing antenna along a boom or using multiple probes on a boom, as shown in Figure 2. This method produces accurate and reproducible results, although it differs from the traditional 3D Total Radiated Power (TRP) or Total Radiated Sensitivity (TRS) approaches. Importantly, it can be implemented in all types of anechoic chamber measurement systems. It can also be measured across different CW incident angles.

Figure 2. Performing 2D scanning for A-IoT device 1 testing

The most straightforward alternative is measuring a single-point performance metric, which is feasible in all types of anechoic chamber measurement systems. However, this approach provides only a partial view of the DUT's performance, lacking the comprehensive insight offered by 3D or 2D scanning methods.

To study the effect of the scanning method on the estimated backscattering loss, let us take the example of a meander dipole designed for n8 (@900MHz). The antenna geometry is shown in Figure 3 and its simulated input reflection coefficient magnitude in dB is given in Figure 4. This design was chosen because it is widely used for RFID tags, and it will probably also be used widely for A-IoT tags. The tag was simulated in ANSYS HFSS and optimized for the backscattering efficiency, showing that the required loads for the two switching states are 0Ω and -250jΩ. The tag was, then, simulated using a normal incidence illumination using a plane wave. Since the backscattering loss is proportional to Radar Cross Section (RCS), we will focus our analysis on the RCS. Figure 5 shows the simulated 3D bistatic RCS in the two switching states and Figure 6 shows the simulated 2D bistatic RCS for different azimuth angles. The obtained results are summarized in Table 1. These results show that even though the tag presents a quasi-omnidirectional RCS pattern, there is still a null in the excitation access (OY direction). Hence, only considering a peak value will over-estimate the tag performances. Indeed, the peak values are around 1.5dB higher than the average values (calculated from 3D scanning method). Performing a 2D cut in the H-plane (ϕ=0°) will also lead to around 1.4dB overestimation in the RCS. Meanwhile, performing a cut in the E-plane (ϕ=90°) will lead to an underestimation of around 1.8dB in the RCS. Combining the results from the two cuts (E-plane and H-plane) gives an accurate estimation of the average RCS. However, since this is not possible while maintaining the same CW incident angle, performing a cut 45° rotated from E-plane will also give an accurate estimation of the average RCS as can be seen from Figure 7 showing the effect of the scanning angle on average RCS estimation error.

Figure 3. An example meander dipole tag for n8 band

Figure 4. Antenna simulated input reflection coefficient magnitude in dB

(a)                                       (b)

Figure 5. Simulated 3D bistatic RCS in dBsm for the two switching states. (a) State 1 and (b) state 2

(a)                                       (b)                                         (c)

Figure 6. Simulated 2D bistatic RCS in dBsm for the two switching states. (a) ϕ=0°, (b) ϕ=45° and (c) ϕ=90°

Table 1. Average simulated RCS in dB vs the scanning method

|  | Switch state 1 | Switch state 2 |
| --- | --- | --- |
| Single peak value | -10.5 (+1.5) | -26 (+1.5) |
| 3D scan | -12 (reference) | -27.5 (reference) |
| 2D scan, H-plane (ϕ=0°) | -10.6 (+1.4) | -26.1 (+1.4) |
| 2D scan, ϕ=45° | -12 (0) | -27.6 (-0.1) |
| 2D scan, E-plane (ϕ=90°) | -13.8 (-1.8) | -29.3 (-1.8) |

Figure 7. Effect of scanning angle on the estimation error

Observation 1: there are three alternatives for specifying performance metrics for A-IoT Device 1:

Option 1: 3D scan-based metric

This approach involves positioning the CWT either below or above the DUT, which allows both components to rotate solely in the azimuth plane while the testing antenna moves along a boom. This setup can produce precise and reproducible measurement results. However, it excludes the use of dual-axis rotation anechoic chamber measurement systems and is limited to testing only under CW normal incidence conditions.

Option 2: 2D scan-based metric

In this method, both the DUT and the CWT node are fixed, and the testing antenna is moved along a boom. This approach yields accurate and reproducible results and can be implemented in all types of anechoic chamber measurement systems. It also offers the flexibility to measure performance across different CW incident angles, although it deviates from traditional 3D TRP or TRS methodologies.

Option 3: single-point metric

This metric is feasible across all types of anechoic chamber measurement systems, making it widely applicable. However, it provides only a partial view of the DUT's performance, lacking the comprehensive insights afforded by 3D or 2D scanning methods.

Considering the advantages and disadvantages of these three options, our preference is for Option 2. This choice balances accuracy with ease of measurement and allows for evaluation across different CW incident angles, making it a versatile and effective method for assessing the performance of A-IoT device 1.

Observation 2: legacy UE TRP / TRS performance metrics are typically conducted on a grid of 15° increment. This means that the boom antennas are placed at angles such as 0, 15°, 30°, and so forth around DUT. One of these antennas, such as the one positioned at 30°, can be repurposed as CWT node.

Observation 3: performing 2D scanning in a plane 45° rotated from E-plane yields the most precise estimation of the average radiated performances.

Proposal 1: implement 2D scan-based performance metrics for A-IoT device 1 with a CW incident angle of 30°.

Proposal 2: 2D scanning should be performed in a plane 45° rotated from E-plane.

Proposal 3: these 2D scan-based performance metrics are only for A-IoT device 1, which lack internal RF transmitters, and do not set a precedent for future “active” A-IoT devices.

# Conclusions

This contribution provides our views on the performance metric for A-IoT device 1. The contribution makes the following observations and proposals.

Observation 1: there are three alternatives for specifying performance metrics for A-IoT Device 1:

Option 1: 3D scan-based metric

This approach involves positioning the CWT either below or above the DUT, which allows both components to rotate solely in the azimuth plane while the testing antenna moves along a boom. This setup can produce precise and reproducible measurement results. However, it excludes the use of dual-axis rotation anechoic chamber measurement systems and is limited to testing only under CW normal incidence conditions.

Option 2: 2D scan-based metric

In this method, both the DUT and the CWT node are fixed, and the testing antenna is moved along a boom. This approach yields accurate and reproducible results and can be implemented in all types of anechoic chamber measurement systems. It also offers the flexibility to measure performance across different CW incident angles, although it deviates from traditional 3D TRP or TRS methodologies.

Option 3: single-point metric

This metric is feasible across all types of anechoic chamber measurement systems, making it widely applicable. However, it provides only a partial view of the DUT's performance, lacking the comprehensive insights afforded by 3D or 2D scanning methods.

Considering the advantages and disadvantages of these three options, our preference is for Option 2. This choice balances accuracy with ease of measurement and allows for evaluation across different CW incident angles, making it a versatile and effective method for assessing the performance of A-IoT device 1.

Observation 2: legacy UE TRP / TRS performance metrics are typically conducted on a grid of 15° increment. This means that the boom antennas are placed at angles such as 0, 15°, 30°, and so forth around DUT. One of these antennas, such as the one positioned at 30°, can be repurposed as CWT node.

Observation 3: performing 2D scanning in a plane 45° rotated from E-plane, yields the most precise estimation of the average radiated performances.

Proposal 1: implement 2D scan-based performance metrics for A-IoT device 1 with a CW incident angle of 30°.

Proposal 2: 2D scanning should be performed in a plane 45° rotated from E-plane.

Proposal 3: these 2D scan-based performance metrics are only for A-IoT device 1, which lack internal RF transmitters, and do not set a precedent for future “active” A-IoT devices.

# References