3GPP RAN4 Meeting #116		R4-2510321

Bengaluru, India, 25th – 29th August, 2025

Agenda item:	7.10.3.1

Source:	Ericsson

Title:	Discussion on XR performance metric

WI/SI:	TRP_TRS_MIMO_OTA_Ph3-Core

Release:	Rel-19

Document for:	Approval

# Introduction

During RAN4 #115 meeting, the following agreement was made in WF [1].

This contribution provides our views related to this issue.

# Discussion

## Performance metric for XR devices

When assessing the performance of XR devices, Total Radiated Power (TRP) emerges as the most suitable metric due to its comprehensive and unbiased nature. TRP measures the average radiated power across all directions over a 3D sphere, treating each direction equally without preference. This direction-agnostic approach ensures a holistic evaluation of the device's radiated power, making it an ideal choice for XR devices, which benefits from a balanced performance across varied usage scenarios.

In contrast, metrics like Weighted Total Radiated Power (Weighted TRP) and Near Horizontal Part Radiated Power (NHPRP) introduce biases by emphasizing specific directions or conditions. Weighted TRP assigns varying importance to different directions based on factors such as usage scenarios, body blocking, or network directionality. While this may suit certain specialized applications, it complicates the evaluation process and may not accurately reflect the versatile capabilities of XR devices, which is designed for diverse environments where uniform performance is critical.

Similarly, NHPRP focuses on radiated power in near-horizontal directions, which might align with particular use cases but fails to capture the full spectrum of XR devices capabilities. XR devices are intended to function effectively across a wide range of orientations and scenarios, making a direction-neutral measure like TRP more appropriate.

Moreover, TRP offers practical advantages in terms of measurement. It can be readily assessed in both Anechoic Chambers (AC) and Reverberation Chambers (RC), which are standard testing environments. This ease of measurement ensures consistent and reliable data, facilitating straightforward comparisons and analysis. Conversely, the bias inherent in Weighted TRP and NHPRP complicates their measurement in RCs, potentially leading to inconsistencies and challenges in obtaining accurate assessments.

In conclusion, TRP stands out as the optimal metric for evaluating XR devices, providing a comprehensive, unbiased, and easily measurable representation of the device's radiated power. By focusing on TRP, one can ensure that XR devices meet the demands of diverse usage scenarios without being constrained by directional biases inherent in other metrics.

Observation 1: TRP provides a comprehensive, direction-neutral evaluation of radiated power, which is essential for XR devices that requires balanced performance across diverse environments and usage scenarios.

Observation 2: TRP can be easily measured in standard testing environments such as Anechoic Chambers (AC) and Reverberation Chambers (RC), while both weighted TRP and NHPRP cannot be measured in RC.

Observation 3: Total Received Sensitivity (TRS) serves as the equivalent metric of TRP on the receive side. TRS quantifies the average sensitivity of a device to incoming signals from all directions over a 3D sphere.

Proposal 1: Use TRP and TRS as performance metric for XR devices.

# Conclusions

This contribution provides our views on the performance metric regarding XR devices. The contribution makes the following observations and proposals.

Observation 1: TRP provides a comprehensive, direction-neutral evaluation of radiated power, which is essential for XR devices that requires balanced performance across diverse environments and usage scenarios.

Observation 2: TRP can be easily measured in standard testing environments such as Anechoic Chambers (AC) and Reverberation Chambers (RC), while both weighted TRP and NHPRP cannot be measured in RC.

Observation 3: Total Received Sensitivity (TRS) serves as the equivalent metric of TRP on the receive side. TRS quantifies the average sensitivity of a device to incoming signals from all directions over a 3D sphere.

Proposal 1: Use TRP and TRS as performance metric for XR devices.

# References

R4-2508780, “Way Forward for [115][337] TRP_TRS_MIMO_OTA_Ph3” vivo, 3GPP RAN4 #115.