3GPP TSG-RAN WG4 Meeting#116		R4-2510322

Bengaluru, India, Aug 25th -29th, 2025

Agenda item:	7.3.3

Source: 	Spreadtrum, UNISOC

Title: 	Remaining issues on UE RF requirements for LB-LB CA switching

Document for:	Discussion

Introduction

In RAN4#115 meeting, a WF [1] was approved. In this contribution, we want to provide some views on UE RF requirements for LB-LB CA switching.

- Way Forward Issue 1: Agreements related to the time mask requirement - A text proposal to TR38.768 in R4-2507932 captures the relevant agreements - How to specify the DL time mask: - The UE DL behaviour following the switch operation can be discussed in the RRM session - Considering that only one RAN4 meeting is left before the conclusion of the work item, companies are encouraged to exchange views via the RAN4 email reflector in advance of RAN4 #116 to converge on the technical content of the CR to 38.101-1 - RAN4 to further discuss whether the time mask should show the switching period/gap location and whether the diagram similar to Figure 5.2.2-1 and 5.2.3-1 in TR38.768 should be considered. Issue 2: Agreements related to UE capabilities - A new feature group is included in the Rel-19 feature list for NR_LBCA_Sw - Related to concurrent Rx operation, the following aspect is included in the feature group: - [The con-current reception on FDD band and SDL band is not supported. The con-current operation between FDD UL and SDL is not supported.] - Only switchedDL option can be considered for low-low CA downlink carrier switching in this WID. - Support 70us as a candidate value of switching gap for 15kHz SCS in addition to 35us and 140us

Discussion

- UL Time mask

According to the last meeting WF, RAN4 needs to discuss whether the time mask should show the switching period /gap location and whether the diagram similar to Figure 5.2.1-1 and 5.2.3-1 in TR 38.768 should be considered. In our understanding, the UL time mask needs to show the switching period location which was decided by RAN4 and show the switching from SDL to FDD and from FDD to SDL. The off to on state of UL represents the switching from SDL to FDD band, and the on to off state of UL represents the switching from FDD to SDL band. The on to off state will be interrupted at the end of slot due to the switching period (switching gap defined in RAN1). Two states show that the low-low band CA switching completely. The main difference between UL time mask for LB-LB CA switching and general ON/OFF time mask is the demonstration of switching period location. To sum up, we have proposed to describe the UL time mask in Figure 1.

Proposal 1: Take the Figure 1 as the UL time mask for LB-LB CA switching with non-CA in the UL.

Conclusions

In this paper, we provide our views on UE RF requirements for LB-LB CA, we have made the following proposals.

Proposal 1: Take the Figure 1 as the UL time mask for LB-LB CA switching with non-CA in the UL.

- Reference

- R4-2508118, WF on UE RF requirements for LB CA via switching, Apple. TSG-RAN4#115 meeting