3GPP TSG RAN WG4 Meeting #116	R4-2510326

Bengaluru, India, 25th – 29th August, 2025

Agenda item:			7.26.3

Source:	vivo

Title:	Further discussion on RRM requirements for XR

Document for:	Discussion

# Introduction

In the RAN4#115 meeting, there were further discussions on RRM requirements impact for XR. Progress was made during the meeting. Following agreements were captured in the WF [1].

<Agreement> - MG cancelling DCI applies to concurrent MGs and cancels the MG surviving the collision handling rules. - The timeline X ms for the MG cancelling the colliding MGs is based on the one that survives collision handling rules. - DCI skipping cannot be applied if any gap is configured with gapAssociationPRS - FFS the applicability of the gap cancellation if two non colliding concurrent gaps are separated by less than X ms <Agreement> - MG cancelling DCI doesn’t apply to MGs configured via GapConfig-r17 with ncsgInd-r17. <Agreement> - MG cancelling DCI doesn’t apply to MGs configured via GapConfig-r17 with preConfigInd-r17. <Agreement> - MG cancelling DCI doesn’t apply to Rel-18 concurrent gaps. <Agreement>: <Agreement>: Capture the following agreement in the LS to RAN1/2. Meanwhile, it is not precluded to update the number of 1 s if any issue will be identified in the future meeting. - The information in UAI: a ratio (R) of gap occasions that is recommended for cancellation during a time period of 1 s. - R ∈ {0, 20, 40, 60}%. - The UE may provide the UAI related information, for the UAI in the agreement captured above, per MG configuration. - per-FR gap if gap is configured with gapConfig with per-FR gaps - per gap configuration if gap is configured with gapConfig-r17 <Agreement>: - From RAN4 perspective, no consensus to support semi-static solutions in Rel-19. <Agreement>: - It is RAN4 common understanding that: If UE is configured with per-FR gap, the MG skipping only applies to the same FR and has no impacts on UE behavior on the other FR. If UE is configured with per-UE gap, the MG skipping applies to both FRs. - Further discuss whether it has already been specified in RAN1 CR, and whether/how to capture in RAN4 spec. <Agreement>: - Use “Measurement gap occasion cancelation” to refer to measurement gaps that were skipped or cancelled by DCI. - Use Lcancel for number of skipping occasions - Indication of which formula/table to use - For the UE supporting [FFS feature name] provided that the configuration and conditions in clause 9.1.y.4 are met (new) - Use RAN2 pseudo code approach for new clause 9.1.y <Agreement>: - Define optional UE feature for support of UAI with per UE capability, [and no FR1/FR2 differentiation].

There are also open issues in the WF [1] needed to be solved.

In [2, 3, 4, 5, 6], we provided our views on the RRM requirements impact and issues raised in the RAN1 LSs. In this contribution, we further provide our views on RRM requirements impact for XR in Rel-19.

# RRM requirements impact

## 2.1 Concurrent gap

In the last RAN4 meeting, following agreement was made.

<Agreement> - MG cancelling DCI applies to concurrent MGs and cancels the MG surviving the collision handling rules. - The timeline X ms for the MG cancelling the colliding MGs is based on the one that survives collision handling rules. - DCI skipping cannot be applied if any gap is configured with gapAssociationPRS - FFS the applicability of the gap cancellation if two non colliding concurrent gaps are separated by less than X ms

In the last meeting, it was agreed that MG cancelling DCI applies to concurrent measurement gaps and cancels the measurement gap surviving the collision handling rules. When UE is configured with concurrent measurement gaps, two measurement gap occasions are considered colliding if at least one of the following conditions is met:

- the two occasions are fully or partially overlapping in time domain, or

- the distance between the two occasions is equal to or smaller than 4 ms.

However, due to time offset X for DCI indication, the DCI should be sent 5ms or optional 3ms before the gap so the gap can be skipped. If the distance between the gaps occasion is just larger than 4ms, it would be complicated for NW to indicate skipping of the 2nd gap. Thus, there is not much benefit to consider concurrent gaps for gap skipping.

Fig 1. Indication of non-colliding gaps separated less that Xms

As depicted in Fig 1, cancellation of gap2 can only be indicated within the Xms before gap1, where X is 5ms and optionally 3ms. This may complicate NW implementation and may lead to unnecessary cancellation of gaps. On the other hand, this seems to be corner case and configuration of concurrent gap is under control of NW, there is no need to further specify requirements for such case. The requirements should be applicable based on DCI indication rule and gap collision handling rule.

In the last RAN1 meeting, below conclusion on cancellation for concurrent measurement gaps was made.

Conclusion RAN1 does not discuss the measurement gap occasion cancellation for concurrent measurement gaps further unless triggered by RAN4.

TP for indication for cancellation of measurement gaps was agreed in RAN1 also.

Indication for cancelation of RRM measurement gaps A measurement gap cancellation field in a DCI format [5, TS 38.212] provided by a PDCCH reception is associated to an earliest RRM measurement gap occasion that starts at least 3 msec or 5 msec, as determined by a reported UE capability, after the end of the PDCCH reception. The indication by the measurement gap cancellation field is applied to the associated RRM measurement gap occasion on all cells in the applicable range of the measurement gap occasion that include the one or more scheduled cells associated with the DCI format [10, TS 38.133] as described below. A UE can be indicated by a value ‘1’ of a measurement gap cancellation field in a DCI format [5, TS 38.212] provided by a PDCCH reception to cancel an associated RRM measurement gap occasion. If a UE indicates “Option 1” for FG 64-1, the UE ignores a value ‘0’ of the measurement gap cancellation field in a DCI format. If a UE indicates “Option 2” for FG 64-1, a value ‘0’ of the measurement gap cancellation field in a DCI format provided by a PDCCH reception indicates to the UE that the UE behavior for the associated RRM measurement gap occasion is as described in [10, TS 38.133]. When the UE detects more than one DCI formats that include the measurement gap cancellation field and are associated with a same associated RRM measurement gap occasion, the UE applies the indication provided by the measurement gap cancellation field of a last DCI format, from the more than one DCI formats.

Based on agreed indication, there would be no need for RAN1 to further discuss concurrent gaps if RAN4 specifies that cancellation of concurrent measurement gaps is after handling of gap collision. The indication can be applied for concurrent gaps.

Proposal 1: Cancellation of concurrent gaps applies after gap collision handling and regardless of gap distance.

## 2.2 RRM requirements

In the previous meeting, it was agreed to use NR-U-like approach to define requirements for gap skipping when extended measurement delay is needed.

- Follow NR-U-like approach for designing the formulas for measurement delay extension.

However, how the delay is extended has not been decided yet.

Issue 3-1: Formulas for measurement delay extension without DRX <Way forward> - RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as - Option 1: Max(200 ms × CSSFinter, 8 × Max(MGRP , SMTC period) × CSSFinter + Nskip × Max(MGRP , SMTC period)), - Option 2: Max( 200 ms, ( 8 + Nskip ) ´ Max(MGRP , SMTC period) ) ´ CSSFinter - Option 3: - Tidentify_inter_without_index = (TPSS/SSS_sync_inter + TSSB_measurement_period_inter+ Tskip) ms - Tidentify_inter_with_index = (TPSS/SSS_sync_inter + TSSB_measurement_period_inter + TSSB_time_index_inter+ Tskip) ms - where Tskip = Nskip ´ Max(MGRP , SMTC period) - Other options are not precluded - where Nskip is the number of skipped MG occasions in the measurement period. - FFS: CSSFinter Issue 3-2: Formulas for measurement delay extension with DRX <Way forward>: - Option 1: RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as - DRX cycle ≤ 320 ms - Max(200 ms CSSFinter, Ceil(8 1.5) Max(MGRP, SMTC period, DRX cycle) CSSFinter + Nskip × Max(MGRP, SMTC period, DRX cycle) ) - DRX cycle > 320 ms - 8 DRX cycle CSSFinter + Nskip DRX cycle - Option 2: Inter-frequency cell detection period requirements with MG cancellation due to XR can be as in the table below for FR1 and FR2-1, respectively: - DRX cycle ≤ 320 ms - Max(200 ms, Ceil((8+XPSS/SSS,gaps) 1.5) Max(MGRP, SMTC period, DRX cycle)) CSSFinter - DRX cycle > 320 ms - Ceil(8+XPSS/SSS,gaps) DRX cycle CSSFinter - Option 3: RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as - Tidentify_inter_without_index = (TPSS/SSS_sync_inter + TSSB_measurement_period_inter+ Tskip) ms - Tidentify_inter_with_index = (TPSS/SSS_sync_inter + TSSB_measurement_period_inter + TSSB_time_index_inter+ Tskip) ms - Where - DRX cycle ≤ 320 ms, Tskip = Nskip ´ Max(MGRP , SMTC period, DRX cycle) - DRX cycle > 320 ms, Tskip = Nskip ´ DRX cycle - Other options are not precluded Issue 3-3: Formulas for measurement delay extension inter-RAT <Way forward>: - Discuss how to extend the formulas in 3-1 and 3-2 for inter-RAT

The difference between option 1 and option 2 is whether the number of skipped occasions needs to be multiplied by CSSF. In principle, the measurement delay is specified for each inter-frequency layer measured with gaps. With option 1, the same number of gap occasions in which the frequency layer can be measured are given. Thus, regardless of which frequency layer is supposed to be measured in the skipped occasions, same number of additional gap occasions can be used for the measurements so that UE can complete the measurements without compromising accuracy. However, this approach could have significant impact on UE implementation, e.g., UE needs to re-schedule the measurements after gap skipping which may take longer time for processing and increase UE complexity. It is better to reuse NR-U like requirements, i.e., option 2.

For the DRX case, in general same principle can be reused that delay extension should be scaled by CSSF. On the other hand, optimization can be considered. Since UE typically measures once in one DRX cycle, it would be reasonable to consider that the number of skipped gaps is one per DRX cycle in the delay requirements even if a greater number of gap occasions are indicated for skipping.

Proposal 2: Option 2 is used to define requirements for gap skipping for both DRX and non-DRX cases.

Proposal 3: The number of skipped gap occasions is one per each DRX cycle in the delay requirements.

For the inter-RAT measurement, requirements were defined for E-UTRA FDD and E-UTRA-TDD. The measurements within gap are reflected by Tinter1 in the requirements, which is the minimum available time for inter-RAT measurements. When gap skipping is enabled, some of the gap occasions would not be possible for inter-RAT measurements. Then, the available time for inter-RAT measurements could be reduced. The delay should be extended accordingly same as for intra-frequency and inter-frequency NR measurements.

From requirements structure perspective, the straightforward way would be to define a new Tinter1 to account for gap skipping during 480ms. Another approach would be to define a scaling factor for gap skipping considering MGRP and MGL additionally. It seems not very suitable for existing requirements structure for inter-RAT measurement.

Proposal 4: Define new Tinter1 for gap skipping in inter-RAT measurement requirements.

Regrading conditions for measurement skipping, following was discussed.

<Way forward>: - Option 1: No conditions are defined - Option 2: Define the maximum number of gaps skipped during the measurement period. - Option 3: Define a maximum separation between two non-skipped gaps for measurement

Since NR-U like requirements is defined for measurement gap skipping, it is necessary to limit the max number of gaps skipped during the measurement period. Otherwise, the mobility performance cannot be ensured. The max number of gaps to be cancelled should not be larger than UE capability reported via UAI.

Proposal 5: For measurement gap skipping, the max number of gaps skipped during the measurement period should be limited/specified which should not be larger than UE capability reported via UAI.

Regarding skipping indication on frequency domain, following agreement was made in the last meeting.

## Skipping indication on frequency domain <Agreement>: - It is RAN4 common understanding that: If UE is configured with per-FR gap, the MG skipping only applies to the same FR and has no impacts on UE behavior on the other FR. If UE is configured with per-UE gap, the MG skipping applies to both FRs. - Further discuss whether it has already been specified in RAN1 CR, and whether/how to capture in RAN4 spec.

TP for indication for cancellation of measurement gaps was agreed in RAN1 as copied in section 2.1.

The indication by the measurement gap cancellation field is applied to the associated RRM measurement gap occasion on all cells in the applicable range of the measurement gap occasion that include the one or more scheduled cells associated with the DCI format [10, TS 38.133]

With highlighted part, it would be that RAN4 common understanding is captured in RAN1 spec from DCI indication point of view. On the other hand, it may be clearer if it is also captured in the RAN4 specification from RRM requirements perspective.

Proposal 6: Common understanding on per-FR/per-UE gap application of gap cancellation is also captured in RAN4 specification from RRM requirements perspective.

Applicability of requirements for gap cancellation in terms of UE power class was discussed.

<Way forward> - Option 1: The requirements for inter-frequency measurement gap cancellation apply for UE power class 3.

There is no need to restrict the UE power class for applying gap cancellation requirements. The feature can be supported by UE with other power class, e.g., PC2, PC1.5 etc. It is anyway up to both UE and NW whether the feature can be used or not.

Proposal 7: No restriction on UE power classes whether the requirements for gap cancellation are applicable or not.

The RAN4 UE feature list was agreed in [7]. The UE feature for XR is copied as below.

| 57.NR_XR_Ph3 | [53-1] | [Recommended ratio of gap occasions for cancelation report. ] | [Indicates the UE is capable of reporting recommended ratio of measurement gap occasions for cancelation. ] | [64-1] | [Yes] | [N/A] | [The UE does not support reporting recommended ratio for gap cancelation] | [Per UE] | [no] | [no] |  |  | [Optional with capability signalling] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

The UE feature is generally fine, except that the name of feature group needs to be revised a bit. The bracket can be removed then.

Proposal 8: UE feature for XR is revised in the Table.

| 57.NR_XR_Ph3 | 53-1 | Recommended ratio of gap occasions for cancelation | Indicates the UE is capable of reporting recommended ratio of measurement gap occasions for cancelation | 64-1 | Yes | N/A | The UE does not support reporting recommended ratio for gap cancelation | Per UE | no | no |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

# Summary

In this contribution, we further provided views on RRM requirements impact for XR in Rel-19. Following proposals and observations are present.

Proposal 1: Cancellation of concurrent gaps applies after gap collision handling and regardless of gap distance.

Proposal 2: Option 2 is used to define requirements for gap skipping for both DRX and non-DRX cases.

Proposal 3: The number of skipped gap occasions is one per each DRX cycle in the delay requirements.

Proposal 4: Define new Tinter1 for gap skipping in inter-RAT measurement requirements.

Proposal 5: For measurement gap skipping, the max number of gaps skipped during the measurement period should be limited/specified which should not be larger than UE capability reported via UAI.

Proposal 6: Common understanding on per-FR/per-UE gap application of gap cancellation is also captured in RAN4 specification from RRM requirements perspective.

Proposal 7: No restriction on UE power classes whether the requirements for gap cancellation are applicable or not.

Proposal 8: UE feature for XR is revised in the Table.

| 57.NR_XR_Ph3 | 53-1 | Recommended ratio of gap occasions for cancelation | Indicates the UE is capable of reporting recommended ratio of measurement gap occasions for cancelation | 64-1 | Yes | N/A | The UE does not support reporting recommended ratio for gap cancelation | Per UE | no | no |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- References

- R4-2508285	WF on RRM requirements for NR_XR_Ph3, Nokia

- R4-2415198	Further discussion on RRM impacts for XR and reply LS, vivo

- R4-2418759	Further discussion on RRM impacts for XR and reply LS, vivo

- R4-2500827	Further discussion on RRM requirements for XR, vivo

- R4-2504027	Further discussion on RRM requirements for XR, vivo

- R4-2506456	Further discussion on RRM requirements for XR, vivo

- R4-2508077	R19 RAN4 UE feature list for NR, CMCC