3GPP TSG RAN WG4 Meeting #116	R4-2510329

Bengaluru, India, 25th – 29th August, 2025

Agenda item:			7.25.3.1

Source:	vivo

Title:	On RRM requirements for LTM event-triggered L1-RSRP reporting

Document for:	Discussion

# Introduction

In the RAN4#115 meeting, RAN4 discussed RRM requirements for LTM event-triggered L1-RSRP reporting. Following agreements were made and captured in the WF [1].

#### Issue 2-2: Event triggered reporting delay for L1 reporting Agreement: - The event triggered measurement reporting delay shall be no larger than the maximum of L1-RSRP measurement periods on the cells corresponding to the event. - If serving cell is involved in event evaluation, L1-RSRP measurement period of serving cell refers to TL1-RSRP_Measurement_Period_SSB and TL1-RSRP_Measurement_Period_CSI-RS respectively for SSB based and CSI-RS based L1-RSRP measurement, as defined in 9.5, assuming TReport = 0 - If neighbor cell is involved in event evaluation: - L1-RSRP measurement period of neighbor cell refers to TL1-RSRP_Measurement_Period_SSB_intra for SSB based intra-frequency measurement, as defined in 9.14, assuming TReport = 0. - L1-RSRP measurement period of neighbor cell refers to TL1-RSRP_Measurement_Period_SSB_inter for SSB based inter-frequency measurement, as defined in 9.15, assuming TReport = 0. - L1-RSRP measurement period of neighbor cell refers to [TL1-RSRP_Measurement_Period_CSI_intra] for CSI-RS based intra-frequency measurement, as defined in [9.x], assuming TReport = 0.

In this contribution, we further provide our views on RRM requirements for event-triggered L1-RSRP reporting in R19 LTM.

# Discussion

Event triggered reporting delay for L1 reporting

#### Issue 2-1: applicability of L1-RSRP reporting delay Candidate options: - Option 1: RAN4 considers following conditions in the test for L1-RSRP reporting delay requirements: (vivo, Xiaomi) - No indicate TCI change during the evaluation period. - No configuration change w.r.t measurement RS during the evaluation period - After the condition that would trigger the event takes effect, the condition is assumed to be fixed during the whole L1 measurement period - Option 2: Seems no need to capture assumptions listed under option-1 into core requirements. (Apple) - Option 2a: The proposed L1-RSRP reporting assumptions are not captured in core requirements. During performance phase, RAN4 can consider to ensure serving beam kept unchanged (e.g., no indicate TCI change, and no configuration change) for L1 event trigger reporting test cases. (HW)

Per proposals in the last meeting, companies all think there is no need to capture these conditions in the core requirements. It is better to consider these conditions in the test. In general, at least there should be no change on TCI states during L1-RSRP measurement and reporting delay in the test. This can be discussed in the performance part then.

Proposal 1: RAN4 discuss at least following condition for the test in the performance part:

- No indication of TCI change during the evaluation period.

Reporting criteria

#### Issue 2-3: whether to introduce/update requirements for reporting criteria per measurement category Candidate options: - Option 1: Define a separate capability for supported number of events triggered L1 measurement reporting. (CTC, Nokia, Xiaomi, CATT, vivo, MTK, ZTE, CMCC, E///) - Details: - Option 1a:(vivo) - RAN4 to specify the reporting capabilities for SSB-based L1 measurements for LTM by considering intra-frequency measurement and inter-frequency measurement. For CSI-RS based L1 measurements for LTM, consider 1 layer of measurement for the reporting capability. - Reporting criteria for LTM event triggered reporting is 4 per frequency layer. - The total number of reporting criteria for LTM event triggered reporting is 4*n, where n is the total number frequency layers for L1-RSRP measurement, including intra-frequency layer(s), inter-frequency layer(s) for SSB-based L1-RSRP measurement and 1 frequency layer for CSI-RS based L1-RSRP measurement if configured. - Option 1b: E/// - RAN4 to support a common LTM Ecat capability to be shared between intra- and inter-frequency candidate cells. - RAN4 to agree that Ecat capability for LTM L1 measurements includes both PCell event, and candidate cells’ events. The detail number of Ecat is FFS - Option 2: update the existing requirements to cover L1 event triggered reporting criteria per measurement category. (OPPO) - Option 3: assuming new UE capabilities regarding support of event triggering needs to be introduced, RAN4 shall discuss whether it is still necessary to update/introduce new Event Triggering and Reporting Criteria for LTM. (Apple)

In R18, LTM-CSI-ReportConfig is configured under csi-MeasConfig of ServingCellConfig. The max number of report configs is limited by UE capability supportedMaxAperiodic-LTM-CSI-ReportConfig-r18, supportedMaxPeriodic-LTM-CSI-ReportConfig-r18, and supportedMaxSemiPersistent-LTM-CSI-ReportConfig-r18, which are reported per-BC. In each LTM-CSI-ReportConfig, only one report type, either periodic, semi-persistent or aperiodic, is allowed. For event-triggered L1 reporting, how to configure this new kind of reporting is subject to RAN1/2 discussion. But in our view, it is highly possible that this new kind of reporting is treated as a new report type, while the max number of report config supported by UE shall be discussed and determined by RAN1/2.

So far, RAN1/2 has made the following agreements about the resource type that can be configured in an LTM event.

RAN2 #126 Agreements - Support the following LTM events based on beam specific quality of serving cell and candidate cells as the L1 LTM measurement events. - Event LTM2: Beam of serving cell becomes worse than absolute threshold; - Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell; - Event LTM4: Beam of candidate cell becomes better than absolute threshold; - Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2. - Support the beam config of both SSB and CSI-RS in L1 measurement resource configuration in LTM config. Working assumption: Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5. RAN2 #127 Agreements - Event LTM1 is not defined. - Current beam (i.e. a beam corresponding to the indicated TCI state) is used for event evaluation in L1 measurement reporting for serving cell. - Any beam in candidate RS configuration can be used for LTM event evaluation. RAN2 #127bis Agreements - Confirms WA (Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5). RAN2 #128 Agreements - Either CSI-RS or SSB could be configured as candidate beam and the measurement RS of the serving cell beam is determined based on the candidate beam to ensure same RS type, i.e. the RS for current beam of serving cell is same as or QCLed with the QCL RS of the indicated TCI state, according to what is agreed in RAN1. - Network can configure which RS type (SSB or CSI-RS) is used for LTM event evaluation.

RAN2 #129 Agreements - Confirm on the previous agreement that "For measurement resource configuration, R18 LTM CSI resource configuration is reused". - Continue to carry the bits for SSB resource set when CSI-RS resource set is configured. FFS whether CSI-RS can be configured without SSB (up to R1 to decide). - Do not create a new resource configuration just for the event triggered measurements. - Confirm on the previous agreement "For measurement reporting configuration, R18 LTM-CSI-ReportConfig is reused". - Ask RAN1 what should be the maximum number of beam measurement results that can be reported in event-triggered measurement report. RAN1 #118bis Agreements - The serving cell RS for event evaluation is at least derived from QCL RS or SSB QCLed with the QCL RS of the indicated joint/DL TCI state for the serving cell - QCL RS above is the RS w.r.t. QCL-TypeD when the indicated joint/DL TCI state is configured with two QCL RSs - FFS: Details on determination of QCL RS or SSB QCLed with QCL RS - Note: This does not imply the support of mTRP scenarios RAN1 #119 Agreements For the identification of the serving cell RS for event evaluation, If the RS(s) for candidate cell(s) are CSI-RS configured in a CSI-RS resource set configured with repetition, QCL RS of the indicated TCI-state is used for the serving cell; otherwise, SSB QCLed with QCL RS of the indicated TCI-state is used for the serving cell. - UE does not expect the following configuration: - CSI-RS resource in the indicated TCI state of serving cell is NOT configured in a CSI-RS resource set configured with repetition, and - CSI-RS is configured as measurement resource for the candidate cell(s).

RAN1/2 have already agreed to use same RS type for event evaluation. For each event, which type of RS is reported shall be explicitly configured in the report quantity, i.e. either SSBRI or CRI. SSBRI is the only supported report quantity in R18. For R18 gNB triggered reporting, the number of layers can be identified by number of SSB frequency configured for measurement. Hence, even multiple candidate cells with different SSB frequencies is configured in a same resourceSet, UE may perform SSB-based L1 measurement at different time occasions.

The issue would be similar for the event-triggered case. UE may identify the SSB frequency layer based on SSB frequency, and perform L1 measurement according to R18 rules. For LTM3 and LTM5, there is no differentiation between frequency layers, i.e., any L1 measurement results that meet the event-triggered condition may trigger L1 measurement reporting.

Firstly, event triggering and measurement reporting for L1 measurement are introduced. Similar to that for L3 measurement, it is better to define corresponding capabilities so that NW and UE can understand each other how many events triggering and reporting can be configured and supported.

For L3 measurement, capabilities for event triggering and reporting are defined as below.

#### 9.1.4.1 Introduction This clause contains requirements on UE capabilities for support of event triggering and reporting criteria. As long as the measurement configuration does not exceed the requirements stated in clause 9.1.4.2, the UE shall meet all other performance requirements defined in clause 9 and clause 10. The requirements in this clause also apply for a UE in EN-DC with PSCell on a carrier frequency with CCA or SA NR with PCell on a carrier frequency with CCA. The UE can be requested to make measurements under different measurement identities defined in TS 38.331 [2]. Each measurement identity corresponds to either event-based reporting, periodic reporting, or no reporting. In case of event-based reporting, each measurement identity is associated with an event triggering criterion. In case of periodic reporting, a measurement identity is associated with one periodic reporting criterion. In case of no reporting, a measurement identity is associated with one no reporting criterion. The purpose of this clause is to set some limits on the number of different event triggering, periodic, and no reporting criteria the UE may be requested to track in parallel. #### 9.1.4.2 Requirements In this clause a reporting criterion corresponds to either one event (in the case of event-based reporting), or one periodic reporting criterion (in case of periodic reporting), or one no reporting criterion (in case of no reporting). For event-based reporting, each instance of event, with the same or different event identities, is counted as separate reporting criterion in Table 9.1.4.2-1..

The requirements count criteria for event-based reporting, periodic reporting and no reporting. All the reporting is related to measurement identities. For L1 measurement, there is no measurement identities. Thus, it is okay to define a new UE capability for event triggering and reporting for LTM. However, it needs further discussion whether UE needs to share the capabilities between L3 measurements and L1 measurements.

Regarding how the UE capabilities are defined, RAN4 starts to work on SSB-based L1 measurements for LTM firstly. In general, intra-frequency measurement and inter-frequency measurement capabilities can be defined similar to L3 measurements. For CSI-RS based L1 measurements for LTM, it was agreed not to define requirements for gap-based measurement. Only intra-frequency measurement without gap is considered. It would be enough to consider 1 layer of measurement for CSI-RS based L1 measurements for LTM.

Proposal 2: RAN4 to introduce requirements for reporting criteria for LTM and define a separate capability for supported number of L1 measurement reporting.

Proposal 3: RAN4 to specify the reporting capabilities for SSB-based L1 measurements for LTM by considering intra-frequency measurement and inter-frequency measurement. For CSI-RS based L1 measurements for LTM, consider 1 layer of measurement for the reporting capability.

For LTM, following events were introduced.

- Event LTM2: Beam of serving cell becomes worse than absolute threshold;

- Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell;

- Event LTM4: Beam of candidate cell becomes better than absolute threshold;

- Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2.

For the number of frequency layers for L1-RSRP measurement, it is indicated by UE capabilities supportedMaxIntraInterFreqLayersWithoutGaps-r18 and supportedMaxInterFreqLayersWithGaps-r18. It is differentiated by measurement with gap and without gap, rather than intra frequency layer and inter frequency layer.

Considering above, it is suggested to define reporting criteria for L1-RSRP measurement as 4 per intra-frequency layer and 3 per inter-frequency layer. Then, total number would be 4*n1+3*n2, where n1 is the total number of intra-frequency layers for SSB-based L1-RSRP measurement based on UE capability plus one layer of CSI-RS based L1-RSRP measurement if configured, n2 is the total number inter-frequency layers for SSB-based L1-RSRP measurement based on UE capability. Due to it is not able to differentiate intra-frequency layer without gap, a simple way is to define reporting criteria as 4 for all the frequency layers.

Proposal 4: Reporting criteria for LTM event triggered reporting is 4 per frequency layer.

Proposal 5: The total number of reporting criteria for LTM event triggered reporting is 4*n, where n is the total number frequency layers for L1-RSRP measurement, including intra-frequency layer(s), inter-frequency layer(s) for SSB-based L1-RSRP measurement and 1 frequency layer for CSI-RS based L1-RSRP measurement if configured.

Leaving condition

#### Issue 2-4: Impact on other LTM requirements Agreement in RAN4#114bis: - For FR1: - UE has sent an event-triggered L1 report and the reporting condition continues to be met at the time of receiving the cell switch command or TCI state activation command, - Case 1: leaving condition of the event is configured and corresponding condition is not met, - the related cell switch delay and TCI state activation requirements are applicable - Case 2: leaving condition of the event is NOT configured, FFS - For FR2: FFS Candidate options: - Proposal 1: considering it is invalid that only event entering condition is configured and leaving condition is not configured, update previous agreement to: (Huawei) - For FR1: - UE has sent an event-triggered L1 report due to the entering condition is met and the reporting condition continues to be met at the time of receiving the cell switch command or TCI state activation command, and leaving condition of the event is not met - The related cell switch delay and TCI state activation requirements are applicable - The above optimization is not applicable to FR2 - Proposal 2: Apply to both FR1 and FR2: When UE has sent event-triggered L1 report and leave condition of the event is configured and UE has not sent report on leave before receiving the cell switch command, delay requirements are applicable. (Nokia) - Proposal 3: Either a) introduce no further conditions than UE having sent L1 event-triggered report and condition continuing to be met for the case without leave condition configured, or b) extend applicability from the existing e.g. to 5 seconds based on event-triggered L1 report. - Proposal 4: Add the following additional condition for Tfirst-RS and TRS-proc = 0: The target TCI state is on the active TCI state list, and the UE has sent an event-triggered L1 report associated to the target TCI state no longer than 160 ms before reception of the cell switch command. (Nokia) - Proposal 5: add the following condition on agreements under issue 2-7 in RAN4#114bis: (Apple) - Number of active TCI does not exceed that UE can support. - Proposal 6: For FR1, if UE has sent an event-triggered L1 report and the reporting condition continues to be met at the time of receiving the cell switch command or TCI state activation command, leaving condition of the event is configured and corresponding condition is not met, the related cell switch delay and TCI state activation requirements are applicable. Not to extend the optimization to other cases. (MTK) - Proposal 7: For both FR1 and FR2, the TCI state is considered as known during 1280ms after event-triggered L1 report. And the related cell switch delay and TCI state activation requirements are applicable. (ZTE) - Proposal 8: RAN4 to revisit the definition of TCI state known condition to accommodate event triggered LTM reporting. Potential spec change is highlighted in the following: (E///) - The UE has sent at least 1 valid L1-RSRP report for the target downlink TCI state before the MAC CE command is received. Or UE has sent at least one L3-RSRP report with beam index corresponding to the candidate cell on which the L1 event is configured

Firstly, event leaving condition and event entering condition share the same parameters so they are configured together. There would be no case as Case 2. However, report on leave is configurable. It means even if leaving condition is met, it could never report to network. On the other hand, without report on leave, NW may have other ways figuring out whether a beam can be configured as a new TCI state. There would be no need to add the condition report on leave should be enabled. With this, we agree the FR1 part of Proposal 1.

For FR2, based on discussion in the past meetings, there could be issue if the reported beam is not in the active TCI state list. However, the difference between FR1 and FR2 on this regard is unclear. The optimization can be applicable to both FR1 and FR2.

Proposal 6: The related cell switch delay and TCI state activation requirements in both FR1 and FR2 are applicable if UE has sent an event-triggered L1 report due to the entering condition is met and the reporting condition continues to be met at the time of receiving the cell switch command or TCI state activation command, and leaving condition of the event is not met.

# Summary

In this contribution, we further provided views on RRM requirements impact due to low band carrier aggregation with switching. Following proposals are present.

Proposal 1: RAN4 discuss at least following condition for the test in the performance part:

- No indication of TCI change during the evaluation period.

Proposal 2: RAN4 to introduce requirements for reporting criteria for LTM and define a separate capability for supported number of L1 measurement reporting.

Proposal 3: RAN4 to specify the reporting capabilities for SSB-based L1 measurements for LTM by considering intra-frequency measurement and inter-frequency measurement. For CSI-RS based L1 measurements for LTM, consider 1 layer of measurement for the reporting capability.

Proposal 4: Reporting criteria for LTM event triggered reporting is 4 per frequency layer.

Proposal 5: The total number of reporting criteria for LTM event triggered reporting is 4*n, where n is the total number frequency layers for L1-RSRP measurement, including intra-frequency layer(s), inter-frequency layer(s) for SSB-based L1-RSRP measurement and 1 frequency layer for CSI-RS based L1-RSRP measurement if configured.

Proposal 6: The related cell switch delay and TCI state activation requirements in both FR1 and FR2 are applicable if UE has sent an event-triggered L1 report due to the entering condition is met and the reporting condition continues to be met at the time of receiving the cell switch command or TCI state activation command, and leaving condition of the event is not met.

- References

- R4-2508282	WF on RRM requirements for NR_Mob_Ph4_Part1, Apple