3GPP TSG RAN WG4 Meeting #116	R4-2510330

Bengaluru, India, 25th – 29th August, 2025

Agenda item:			7.25.2

Source:	vivo

Title:	On UE capability for mobility enhancement phase 4 WI

Document for:	Discussion

# Introduction

In the previous RAN4 meetings, requirements for event triggered reporting for LTM and CSI-RS based LTM were discussed. In the last RAN4 meeting, UE capabilities for NR mobility enhancement phase 4 were agreed and capture in the RN4 R19 feature list [1].

In this contribution, we further provide our views on UE capabilities for event triggered reporting and CSI-RS based L1-RSRP measurement for LTM.

# Discussion

CSI-RS based L1-RSRP measurement for LTM

In the last RAN4 meeting, UE capabilities for NR mobility enhancement phase 4 was discussed. The agreed UE capabilities for CSI-RS based L1 measurement for LTM were capture in [1]. It is copied to Table 1 for convenience.

For FG 52-1, it is not concluded yet whether the maximum number of CSI-RS resources for L1-RSRP measurement for LTM is also counted in FG 2-24. In general, it will make no difference since the two FGs can be reported by UE with joint consideration. However, the maximum number of maxNumberSSB-CSI-RS-ResourceOneTx is 64. If it is also counted in FG2-24, the total number of CSI-RS based L1-RSRP measurement for serving cell may be compromised. Therefore, it is recommended that CSI-RS based L1-RSRP measurement for LTM is not counted in FG 2-24.

Proposal 1: For FG 52-1, the number of CSI-RS resources for L1-RSRP measurement within a slot is not counted in FG 2-24.

For the CSI-RS resources for LTM, the CSI-RS across all the candidate cells, not just neighbor cells, should be counted. This should be the same for both FG 52-1 and FG 52-2.

Proposal 2: For FG 52-1 and FG 52-2, the number of CSI-RS resources is counted across all candidate cells..

For FG 52-3, it needs further discussion whether indication of support skipping SSB-based L1-RSRP for CSI-RS-based L1-RSRP measurement is for neighbor cell or candidate cell. In general, it is related to the open issue whether step 2 is skipped for serving cell. In our contribution [2], we provided analysis and proposed it is not skipped for serving cell measurement.

Proposal 3: FG 52-3 is revised to Skip SSB based L1-RSRP measurement for neighboring cell CSI-RS-based L1-RSRP measurement.

Event triggered reporting for LTM

For event triggered reporting for LTM, the reporting criteria will be defined in the spec. Besides that, we don’t see any additional UE capabilities being needed.

Observation 1: No UE capabilities need to be introduced for event triggered reporting for LTM.

Table 1: UE feature for mobility enhancement phase 4 WI

- NR_Mob_Ph4

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 52. NR_Mob_Ph4 | 52-1 | Number of CSI-RS resources for L1-RSRP measurement within a slot | - The max number of CSI-RS resources for L1-RSRP measurement that UE can measure within a slot [across candidate cells] for L1-RSRP measurement | [RAN1 feature of CSI-RS based L1 RSRP measurement on LTM candidate cell] | Yes | No | There is no limitation on the number of CSI-RS resources for L1 measurement within a slot. | Per BC | No | No | N/A | Candidate value: {1,2,3,4,5,6,7,8,16,32, 48,64} - [Note: FFS It is also counted in FG 2-24] | Optional with capability signaling |
|  | 52-2 | Number of total CSI-RS resources to be measured | - The max number of total CSI-RS resources of serving cells and [candidate cells] [across all CCs] for L1 measurement. Both LTM candidate cell(s) and serving cells for CSI-RS based L1-RSRP measurement are counted, regardless of whether CSI-RS based L1-RSRP measurement on serving cell is configured in [LTM-CSI-ResourceConfig-r18] or not. | [RAN1 feature of CSI-RS based L1 RSRP measurement on LTM candidate cell] | Yes | No | There is no limitation on the total number of CSI-RS resources of serving cells and neighbouring cells across all CCs for L1 measurement. | Per BC | No | No | N/A | Candidate values: [{2,4,8,12,16,32,64}] - Note: the value should be not smaller than UE capability of beamManagementSSB-CSI-RS (Component 2 of 2-24) | Optional with capability signaling |
|  | 52-3 | Skip SSB based L1-RSRP measurement for [neighboring or candidate] cell CSI-RS-based L1-RSRP measurement | - Indicates support for skipping SSB-based L1-RSRP during [neighboring or candidate] cell CSI-RS-based L1-RSRP measurement. | [RAN1 feature of CSI-RS based L1 RSRP measurement on LTM candidate cell] | Yes | N/A | SSB based L1-RSRP cannot skipped while performing CSI-RS based L1-RSRP | Per UE | no | FR2-1 only | N/A |  | Optional with capability signalling |

# Summary

In this contribution, we further provided views on UE capabilities for event triggered reporting and CSI-RS based L1-RSRP measurement for LTM. Following proposals and observations are present.

Proposal 1: For FG 52-1, the number of CSI-RS resources for L1-RSRP measurement within a slot is not counted in FG 2-24.

Proposal 2: For FG 52-1 and FG 52-2, the number of CSI-RS resources is counted across all candidate cells..

Proposal 3: FG 52-3 is revised to Skip SSB based L1-RSRP measurement for neighboring cell CSI-RS-based L1-RSRP measurement.

Observation 1: No UE capabilities need to be introduced for event triggered reporting for LTM.

- References

- R4-2508077	R19 RAN4 UE feature list for NR, CMCC

- R4-2510328	On RRM requirements for LTM CSI-RS based L1 measurement, vivo