3GPP TSG RAN WG4 Meeting #116	R4-2510337

Bengaluru, India, 25th – 29th August, 2025

Agenda item:			7.17.4

Source:	vivo

Title:	Discussion on RRM core requirement and testing framework for beam management

Document for:	Discussion

- Introduction

In RAN4#115 meeting, there were some further discussions on key issues including KPIs related, the update on simulation assumption of measurement error, and the potential RRM requirements for beam prediction. The latest outcome was recorded in the latest WF [1].

In this contribution, we will further analyze the impacts on requirements for beam prediction and update some simulation results on proposed KPIs/ Test Metrics with consideration of the measurement error modelling derived from the updated simulation assumption [2].

- Requirements for beam predictions

In the previous meetings, the RRM impacts on beam management was discussed. The package of requirements for beam predictions was identlified as follows:

- Prediction delay requirements

- The KPIs on prediction accuracy

- TCI state known/unknown related

Based on the latest progress of each bullet, we will further provide our analysis on the requirements.

  - Prediction delay requirements

Agreement in RAN4#113 Issue 3-5: Prediction delay requirements Agreement: - Prediction delay includes: - Measurement for prediction - FFS on whether the legacy measurement delay can be reused - Inference delay - Other components are FFS Agreement in RAN4#115 Issue 2-2: Measurement period for inference Agreement: Case 1: The starting point is to use M, N, P (reference the values from an existing clause in the same version of the spec where the measurement period is defined) Case 2: postpone discussion until RAN1 concludes

For the latency on prediction, in the RAN4#113 meeting, it was agreed that RAN4 would define the requirement for an overall delay.

Furthermore, in the previous meeting, RAN4 conducted additional discussions on measurement delay. The focus of the discussion was whether the existing M, N, P in the measurement period could be reused. Additionally, if they can be reused, whether their corresponding values should follow those specified in the Rel-15 spec or the Rel-19 spec.

Next, we will separately analyze the measurement period and inference delay. Regarding the measurement period, we will provide our detailed perspective on these two questions.

- Measurement for prediction

This process refers to UE performing the L1-RSRP measurements for set B and obtaining the corresponding results as model inputs. Basically, the legacy measurement period on L1-RSRP can be as the reference for defining requirements on measurements of set B.

Table 9.5.4.1-2: Measurement period TL1-RSRP_Measurement_Period_SSB for FR2

| Configuration | TL1-RSRP_Measurement_Period_SSB (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*N)*TSSB) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P*N)*max(TDRX,TSSB)) |
| DRX cycle > 320ms | ceil(1.5*M*P*N)*TDRX |
| Note: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. |  |

To be specific, the related parameters to be considered at least include:

- The number of samples for set B measurements

The number of samples will affect the prediction accuracy.

For spatial prediction, previous discussions and simulations have been based on the assumption that the measurement of set B relies on one-shot measurement.

Proposal 1: For spatial prediction, the measurement of set B relies on one-shot measurement

- Beam sweeping factor N

For DL Tx beam prediction evaluations, RAN1 had some preliminary discussions and simulations of the hypothesis on the Rx beam as AI/ML model input for training and/or inference during the SI phase, including:

- Option 1: Measurements of the "best" Rx beam with exhaustive beam sweeping for each model input sample.

- Option 2: Measurements of specific Rx beam(s).

- Option 3: Measurements of random Rx beam(s) per model input sample.

- Option 4: Measurements of quasi-optimal Rx beam (i.e., not all the measurements as inputs of AI/ML are from the "best" Rx beam) with less measurement/RS overhead compared to exhaustive Rx beam sweeping.

Among them, option 1 is implemented in the same way as the existing requirement definition, which requires beam sweeping to find the best Rx beam. The value of the existing beam sweeping factor can be reused. However, since RAN1 did not further discuss the Rx beam assumption in the SI and WI phases, it is possible to implement it as any of the above four options. However, from the perspective of defining requirements, Option 1 can be considered as the worst-case assumption, where beam sweeping is required to find the best Rx beam. The definition of the beam sweeping factor follows existing requirements.

Proposal 2: For L1-RSRP measurement of set B, the definition and value of the beam sweeping factor N follows existing requirements.

- sharing factor P

The value of this parameter is related to factors such as measurement gap and SSB/SMTC period. However, due to the introduction of multiple feature enhancements in previous releases, such as measurement gap enhancement, significant improvements have been made to P. For example, the value of P may now be related to configurations like MUSIM gap and NCSG. This raises a question: when defining AI/ML-based L1-RSRP measurement, should these enhanced features introduced by other projects in previous releases be supported? Or, for the sake of spec simplification, should the requirement definition here be based on the R15 spec version.

A similar issue exists regarding the UE capabilities introduced after R15 in Clause 9.5.4 (L1-RSRP measurement requirements) of the current R19 spec. Following the agreed approach from the previous meeting "reference the values from an existing clause in the same version of the spec where the measurement period is defined", RAN4 would need to analyze each of these UE capabilities for coexistence with BM prediction. Based on our summary, the UE capabilities mentioned in the specification primarily include the following:

The UE capability specified in clause 9.5.4 L1-RSRP measurement requirements in Rel-19 spec - 1. bwpOperationMeasWithoutInterrupt-r18 - 2. ncd-SSB-BWP-Wor-r18 - 3. groupBasedBeamReporting-r17 - 4. multiDCI-IntraCellMultiTRP-TwoTA-r18 - 5. multiDCI-InterCellMultiTRP-TwoTA-r18 and rxTimingDiff-r18, - 6. rxTimingDiff-r18 - 7. highSpeedMeasFlagFR2-r17 - 8. concurrentMeasGap-r17 - 9. musim-GapPreference-r17 - 10. multiCellL1-measRTD-greaterThan-CP-r18 - 11. fastBeamSweepingMultiRx-r18

- For the capabilities highlighted in blue:

BWP-related capabilities (bwpOperationMeasWithoutInterrupt-r18 and ncd-SSB-BWP-Wor-r18) and gap-related capabilities (concurrentMeasGap-r17 and musim-GapPreference-r17) can coexist with and be supported by BM prediction.

- For the capabilities highlighted in red:

Multi-TRP-related capabilities (4, 5, 6): Since BM prediction is based on single TRP, these capabilities need not be mentioned in the BM prediction section. The same applies to capability 3.

HST capability: When a UE supports highSpeedMeasFlagFR2, it must apply enhanced inter-frequency RRM requirements to support speeds up to 350 km/h in FR2. However, BM prediction (e.g., time-domain prediction) in TR 38.843[3] assumes UE speeds of 30 km/h (baseline), 60 km/h (optional), 90 km/h (optional), and 120 km/h (optional). Enabling highSpeedMeasFlagFR2 could lead to a sharp decline in BM prediction performance due to the higher speed scenarios. Thus, this capability should not be configured alongside BM prediction.

multiCellL1-measRTD-greaterThan-CP-r18: This enhancement was introduced for LTM. Since the combination between LTM with AI is likely out of scope, it is also not recommended for coexistence with BM prediction.

fastBeamSweepingMultiRx-r18 was introduced in Multi-Rx, it is also not recommended for coexistence with BM prediction

Based on the above analysis, we propose the following:

Proposal 3: For requirements on AI/ML-based L1-RSRP measurement, when referencing values from an existing clause in the same spec version where the measurement period is defined, RAN4 will discuss the coexistence supportability of the prediction with other previously enhanced features. These enhanced features primarily include:

The UE capability specified in clause 9.5.4 L1-RSRP measurement requirements in Rel-19 spec：

­	1. bwpOperationMeasWithoutInterrupt-r18

­	2. ncd-SSB-BWP-Wor-r18

­	3. groupBasedBeamReporting-r17

­	4. multiDCI-IntraCellMultiTRP-TwoTA-r18

­	5. multiDCI-InterCellMultiTRP-TwoTA-r18 and rxTimingDiff-r18,

­	6. rxTimingDiff-r18

­	7. highSpeedMeasFlagFR2-r17

­	8. concurrentMeasGap-r17

­	9. musim-GapPreference-r17

­	10. multiCellL1-measRTD-greaterThan-CP-r18

­	11. fastBeamSweepingMultiRx-r18

Proposal 4: For the above enhanced features, RAN4 not to consider combining the AI/ML-based BM with the features related to MultiTRP, HST, group-based beam reporting, LTM, Multi-Rx when defining requirements for AI/ML-based L1-RSRP measurement. The specific UE capabilities include:

­	groupBasedBeamReporting-r17

­	multiDCI-IntraCellMultiTRP-TwoTA-r18

­	multiDCI-InterCellMultiTRP-TwoTA-r18 and rxTimingDiff-r18,

­	rxTimingDiff-r18

­	highSpeedMeasFlagFR2-r17

­	multiCellL1-measRTD-greaterThan-CP-r18

­	fastBeamSweepingMultiRx-r18

- inference delay

For model inference delay, the inference process is mainly from the data input to the model to the corresponding inference result is derived. Additionally, there may be some data processing time on top of that, which part of the delay may be related to the specific inference scenario (the number of Set B and Set A, time domain prediction or spatial prediction, etc.) and the processing capability of the model.

This delay is also reflected in RAN1 discussions and agreements, with corresponding conclusions and spec descriptions as follows. Here, (Z₃, Z₃') represents that Z₃ denotes the minimum time required for the UE to complete CSI calculation and reporting after receiving a CSI trigger. Meanwhile, d and d' represent the time required for AI inference. From this perspective, the inference time discussed in RAN4 is already included within the UE CSI computation time specified by RAN1.

RAN1#121 meeting Agreement For UE-sided model, regarding a CSI report with CSI-ReportConfig for inference for BM-Case1 and BM-Case 2, when applicable, extend legacy Z3/Z3’ to Z3+d / Z3’+d’, where d and d’ are reported by UE per SCS for BM-Case 1 and BM-Case 2 respectively - Detailed values of d and d’ can be further discussed in UE feature. CR to TS38.214（R1-2504992） ## 5.4 UE CSI computation time **The irrelevant parts are omitted*** Z, Z' and µ are defined as: Z=maxm=0,…,M-1(Z(m))⁡ and Z'=maxm=0,…,M-1(Z'(m)), where M is the number of updated CSI report(s) according to Clause 5.2.1.6, (Z(m),Z'(m)) corresponds to the m-th updated CSI report and is defined as - (Z1,Z1') of the table 5.4-1 if max{ µPDCCH, µCSI-RS, µUL} ≤ 3 and if the CSI is triggered without a PUSCH with either transport block or HARQ-ACK or both when L = 0 CPUs are occupied (according to Clause 5.2.1.6) and the CSI to be transmitted is a single CSI and corresponds to wideband frequency-granularity where the CSI corresponds to at most 4 CSI-RS ports in a single resource without CRI report and where CodebookType is set to 'typeI-SinglePanel' or where reportQuantity is set to 'cri-RI-CQI', or - (Z1,Z1') of the table 5.4-2 if the CSI to be transmitted corresponds to wideband frequency-granularity where the CSI corresponds to at most 4 CSI-RS ports in a single resource without CRI report and where CodebookType is set to 'typeI-SinglePanel' or where reportQuantity is set to 'cri-RI-CQI', or - (Z1,Z1') of the table 5.4-2 if the CSI to be transmitted corresponds to wideband frequency-granularity where the reportQuantity is set to 'ssb-Index-SINR', 'cri-SINR', 'ssb-Index-SINR- Index ', or 'cri-SINR- Index ', or - (Z3,Z3') of the table 5.4-2 if reportQuantity is set to 'cri-RSRP', 'ssb-Index-RSRP', 'cri-RSRP- Index' or 'ssb-Index-RSRP- Index ', where Xµ is according to UE reported capability beamReportTiming and KBl is according to UE reported capability beamSwitchTiming as defined in [13, TS 38.306], or if the CSI report is configured with ltm-CSI-ReportConfig for L1-RSRP measurement, or - (Z3+d,Z3'+d'), with (Z3,Z3') of the table 5.4-2 if reportQuantity is set to 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19', or 'p-ssb-index-RSRP-r19', where d and d' are according to UE reported capability as defined in [13, TS 38.306] and different values for d and d' may be reported by UE capability based on whether nroftimeinstance-r19 is configured or not within the CSI report, or

Secondly, taking the existing CSI-RS-based measurement period as an example, where TReport  is the configured periodicity for reporting, we understand that the UE has already completed inference before each report. The execution time of this inference can also be included within TReport.

Table 9.5.4.1-2: Measurement period TL1-RSRP_Measurement_Period_SSB for FR2

| Configuration | TL1-RSRP_Measurement_Period_SSB (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*N)*TSSB) |
| DRX cycle ≤ 320 ms | max(TReport, ceil(1.5*M*P*N)*max(TDRX,TSSB)) |
| DRX cycle > 320 ms | ceil(1.5*M*P*N)*TDRX |
| Note: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. |  |

Observation 1: The inference latency is reflected in the RAN1 specifications. As for the prediction delay, this inference latency is already included within TReport.

Therefore, based on the above two considerations, there is no need to define the inference latency separately from RAN4 requirement perspective.

Furthermore, based on this analysis, the prediction delay TL1-RSRP_Prediction_Period_CSI-RS can be defined as follows:

Table : Prediction period TL1-RSRP_Prediction_Period_CSI-RS for FR2

| Configuration | TL1-RSRP_Prediction_Period_CSI-RS (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*N)*TCSI-RS) |
| DRX cycle ≤ 320 ms | max(TReport, ceil(1.5*M*P*N)*max(TDRX,TCSI-RS)) |
| DRX cycle > 320 ms | ceil(M*P*N)*TDRX |
| NOTE 1: TCSI-RS is the periodicity of CSI-RS configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. NOTE 2: the requirements are applicable provided that the CSI-RS resource configured for L1-RSRP measurement is transmitted with Density = 3. |  |

Proposal 5: The prediction processing delay component does not need to be explicitly defined within the prediction delay requirements TL1-RSRP_Prediction_Period_CSI-RS.

Proposal 6:

- For CSI-RS based prediction , the prediction period requirements TL1-RSRP_Prediction_Period_CSI-RS for BM can be defined as

Table : Prediction period TL1-RSRP_Prediction_Period_CSI-RS for FR2

| Configuration | TL1-RSRP_Prediction_Period_CSI-RS (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*N)*TCSI-RS) |
| DRX cycle ≤ 320 ms | max(TReport, ceil(1.5*M*P*N)*max(TDRX,TCSI-RS)) |
| DRX cycle > 320 ms | ceil(M*P*N)*TDRX |
| NOTE 1: TCSI-RS is the periodicity of CSI-RS configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. NOTE 2: the requirements are applicable provided that the CSI-RS resource configured for L1-RSRP measurement is transmitted with Density = 3. |  |

- For SSB-based prediction , the prediction period requirements TL1-RSRP_Prediction_Period_SSB for BM can be defined as

Table : Prediction period TL1-RSRP_Prediction_Period_SSB for FR2

| Configuration | TL1-RSRP_Prediction_Period_SSB (ms) |
| --- | --- |
| non-DRX | Max (TReport, ceil (M*P*N) *TSSB) |
| DRX cycle ≤ 320 ms | Max (TReport, ceil (1.5*M*P*N) *max (TDRX, TSSB)) |
| DRX cycle > 320 ms | Ceil (1.5*M*P*N) *TDRX |
| Note: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. |  |

Additionally, for BM-Case 2, according to the following RAN1 conclusion, AP CSI-RS is not supported, and the corresponding requirements should exclude this scenario.

Conclusion For UE-sided model, for BM-Case 2, for inference, AP CSI-RS for Set B is not supported.

Proposal 7：For prediction delay requirement for BM-Case 2 (if defined), RAN4 not to consider aperiodic CSI-RS reporting scenario.

2.2	TCI state known/unknown

Agreement in RAN4#115 meeting Issue 2-3: Rx beam knowledge and TCI handling Agreement: - if the predicted Tx beam in Set A is QCL Type-D to a known Tx beam, the corresponding Rx beam is known. - If the predicted Tx beam in Set A is not QCL Type-D to a known Tx beam, depending on UE capability, the corresponding Rx beam may be known or unknown. - Known Rx beam means no additional Rx beam sweeping is needed - Unknown Rx beam means additional Rx beam sweeping is needed. - Rx beam is known mean TCI state is known if conditions below are also met The following conditions related to whether the TCI state is known or unknown is defined as the starting point. Companies are invited to bring analysis. - The target TCI state is known when the following is met: - The TCI state switch command is received within 1280 ms upon the last transmission of the RS resource for beam reporting or measurement and the UE has sent at least one L-1 RSRP report of the target TCI state (clause 8.10.2 of 38.133) - Applies for case 1 - FFS for case 2 - Whether RS resource is only from set B should be clarified - FFS on whether prediction also impacts if a TCI state is known or unknown - Companies are invited to bring analysis on how prediction would impact whether a TCI state is known or unknown - FFS on the time delay if prediction is used as a condition for a known TCI state

Significant progress was made on the TCI state known condition in the last meeting, with conclusions as shown above. The remaining aspects to be finalized mainly include:

- Q1: Whether/how prediction also impacts if a TCI state is known or unknown. Further, if there is the time delay when prediction is used as a condition for a known TCI state

- Q2: Does the agreed TCI state known condition apply/how does it apply to BM case 2?

For Q1, we will analyze the following two cases separately:

Case i): the predicted Tx beam in Set A is QCL Type-D to a known Tx beam.

Case ii): the predicted Tx beam in Set A is not QCL Type-D to a known Tx beam.

Case i: Here, the known Tx beam could be either a measured Tx beam in Set B or another previously measured beam. In both scenarios, the Rx beam is known. Taking the case where the known Tx beam is the measured Tx beam in Set B as an example, per the above conclusion, the following timing constraint must be met: The time gap between Point 1 (Time point of the transmission of the RS resource in Set B) and Point 3 (UE receiving the TCI state switch command) must not exceed 1280ms. Under this condition, it is sufficient to ensure that an inference result has been reported within this 1280ms window.

Fig. 1 Timeline analysis for case i

For Case ii, since there is no known Tx beam in Set A with a QCL Type-D relationship corresponding to the predicted Tx beam in this scenario, it relies on UE capability to determine whether the corresponding Rx beam is known. According to discussions, one idea is to bind the time constraint between the time point when the inference result is reported (Point 2) and when the UE receives the TCI state switch command (Point 3).

Fig. 2 Timeline analysis for case ii

However, the issue is that if the prediction time point (Point 2) is used as the starting point, it means the measurement time point of Set B is not considered. However, the measurement time point of Set B actually affects the prediction result. As shown in the figure below, when no restriction is imposed on the measurement time point of Set B, one possible scenario is that the measurement time point of Set B is already far removed from the time when the inference result is reported. This could mean that, by the time of reporting, the actual best Rx beam may have changed to another beam, and the corresponding prediction result may have also changed accordingly.

For example, if the prediction is based on Point 1-1, and the prediction result is reported at Point 2, then at Point 1-1, the best Tx beam determined through Rx beam sweeping is beam 1. However, if a long time has passed and the UE has moved or rotated, the signal received during the previous Rx beam sweeping may be entirely different from before. In other words, the model input has significantly changed, and thus the output would differ from the prediction based on Set B measurement results collected at Point 1-1.

From this perspective, it would be more appropriate to set the 1280ms restriction starting from the measurement time point of Set B rather than the time when the prediction result is reported. This would also align with the starting point defined in Case i.

Fig. 3 Timeline analysis for case ii

As for BM-case 2 (time domain prediction), the difference between this use case and BM-case 1 is that the prediction result of the former is for a future time point. The judgment of the TCI state known condition occurs when the TCI switch command is received and is based on measurements from historical time points before the TCI switch command. Therefore, the above analysis and conditions can also apply to BM-case 2.

Proposal 8: For BM-Case 1 and BM-Case 2, the target TCI state is known when the following is met:

- The TCI state switch command is received within 1280 ms upon the last transmission of the RS resource for beam reporting or measurement and the UE has sent at least one L-1 RSRP measurement or predicted RS quantities report of the target TCI state (clause 8.10.2 of 38.133)

- Prediction would not be used as a condition for known TCI state (i.e., not to be reflected as the time delay restriction between prediction results reporting and TCI switch command)

Based on the above analysis, the corresponding "Known condition of TCI state" can be defined as follows(Case i- in green text), Case ii- in purple text).

8.10.2 Known conditions for TCI state The TCI state is known if the following conditions are met: - During the period from T0 the last transmission of the RS resource used for the L1-RSRP measurement reporting for the target TCI state to the completion of active TCI state switch, where the RS resource for L1-RSRP measurement is the RS in target TCI state or QCLed to the target TCI state - TCI state switch command is received within 1280 ms upon the last transmission of the RS resource for beam reporting or measurement T0 - The UE has sent at least 1 L1-RSRP measurement or predicted RS quantities report for the target TCI state before the TCI state switch command - The TCI state remains detectable during the TCI state switching period - The SSB associated with the TCI state remain detectable during the TCI switching period - SNR of the TCI state ≥ -3 dB Where T0 is - the last transmission of the RS resource used for the L1-RSRP measurement reporting for the target TCI state, where the RS resource for L1-RSRP measurement is the RS in target TCI state or QCLed to the target TCI state [legacy case] - or the last transmission of the RS resource used for L1-RSRP measurement for predicted RS quantities reporting for the target TCI state, where the RS resource for L1-RSRP measurement for predicted RS quantities reporting is the RS in target TCI state or QCLed to the target TCI state - or the last transmission of the RS resources used for L1-RSRP measurement for predicted RS quantities reporting for the target TCI state, where the one of reported predicted RS is the RS in the target TCI state or QCLed to the target TCI state but not in the RS resources for L1-RSRP measurement for predicted RS quantities reporting, and UE supports [known TCI based on prediction capability], Otherwise, the TCI state is unknown.

Proposal 9: RAN4 to update the known TCI state conditions as

8.10.2 Known conditions for TCI state The TCI state is known if the following conditions are met: - During the period from T0 the last transmission of the RS resource used for the L1-RSRP measurement reporting for the target TCI state to the completion of active TCI state switch, where the RS resource for L1-RSRP measurement is the RS in target TCI state or QCLed to the target TCI state - TCI state switch command is received within 1280 ms upon the last transmission of the RS resource for beam reporting or measurement T0 - The UE has sent at least 1 L1-RSRP measurement or predicted RS quantities report for the target TCI state before the TCI state switch command - The TCI state remains detectable during the TCI state switching period - The SSB associated with the TCI state remain detectable during the TCI switching period - SNR of the TCI state ≥ -3 dB Where T0 is - the last transmission of the RS resource used for the L1-RSRP measurement reporting for the target TCI state, where the RS resource for L1-RSRP measurement is the RS in target TCI state or QCLed to the target TCI state [legacy case] - or the last transmission of the RS resource used for L1-RSRP measurement for predicted RS quantities reporting for the target TCI state, where the RS resource for L1-RSRP measurement for predicted RS quantities reporting is the RS in target TCI state or QCLed to the target TCI state - or the last transmission of the RS resources used for L1-RSRP measurement for predicted RS quantities reporting for the target TCI state, where the one of reported predicted RS is the RS in the target TCI state or QCLed to the target TCI state but not in the RS resources for L1-RSRP measurement for predicted RS quantities reporting, and UE supports [known TCI based on prediction capability], Otherwise, the TCI state is unknown.

After clarifying the discussion on the TCI state known condition, another requirements potentially affected is the MAC-CE-based TCI state switch delay. We will now analyze BM-Case 1 and BM-Case 2 separately.

- BM-Case 1

For Case i, since the predicted Tx beam in Set A is QCL Type-D to a known Tx beam and the corresponding Rx beam is known, the existing requirements can be applied. However, for Case ii, it depends on UE capability to determine whether the corresponding Rx beam is known or unknown. Therefore, we need to clarify the specific details of the UE capability. The key consideration is: if the UE supports this capability, when the target TCI state is known, whether fine time tracking still needs to be performed. This is reflected in the spec as follows: whether the value of TOk can be 0 (no fine time tracking needed) in certain cases, or whether fine time tracking is always required in all scenarios.

If we reuse the current definition of TOk, then when the target known TCI state is in the active TCI state list and the UE supports this capability, TOk can be 0 (no fine time tracking needed). However, for this scenario, we must avoid an error case: where the RS resource for Set A is not transmitted but is still configured in the active TCI state list. TOk = 0 should only apply when the RS resource for Set A is actually being transmitted.

An alternative assumption is that if the UE supports this capability, fine time tracking is always required (regardless of whether the target known TCI state is in the active TCI state list). In this case, TOk is always 1.

8.10.3 MAC-CE based TCI state switch delay If the target TCI state is known, upon receiving PDSCH carrying MAC-CE for indication of UE-specific PDCCH TCI state as defined in clause 6.1.3.15 of TS 38.321 [7] in slot n, UE shall be able to receive PDCCH with target TCI state of the serving cell on which TCI state switch occurs at the first slot that is after slot n+ THARQ + 3Nslotsubframe,µ+ TOk*(Tfirst-SSB + TSSB-proc) / NR slot length. The UE shall be able to receive PDCCH with the old TCI state until slot n+ THARQ + 3Nslotsubframe,µ .Where THARQ is the timing between DL data transmission and acknowledgement as specified in TS 38.213 [3]; - Tfirst-SSB is time to first SSB transmission after MAC CE command is decoded by the UE; The SSB shall be the QCL-TypeA or QCL-TypeC to target TCI state - TSSB-proc = 2 ms; - TOk = 1 if target TCI state is not in the active TCI state list for PDSCH, 0 otherwise. If the target TCI state is unknown, upon receiving PDSCH carrying MAC-CE for indication of UE-specific PDCCH TCI state as defined in clause 6.1.3.15 of TS 38.321 [7] in slot n, UE shall be able to receive PDCCH with target TCI state of the serving cell on which TCI state switch occurs at the first slot that is after slot n+ THARQ +3Nslotsubframe,µ + TL1-RSRP +TOuk*(Tfirst-SSB+ TSSB-proc) / NR slot length. The UE shall be able to receive PDCCH with the old TCI state until slot n+ THARQ + 3Nslotsubframe,µ . Where - T L1-RSRP = 0 in FR1 or when the TCI state switching not involving QCL-TypeD in FR2. Otherwise, - T L1-RSRP is the time for Rx beam refinement in FR2, defined as - TL1-RSPR_Measurement_Period_SSB for SSB as specified in clause 9.5.4.1, - with the assumption of M=1 - with TReport = 0 - TL1-RSRP_Measurement_Period_CSI-RS for CSI-RS as specified in clause 9.5.4.2 - configured with higher layer parameter repetition set to ON - with the assumption of M=1 for periodic CSI-RS - for aperiodic CSI-RS if number of resources in resource set at least equal to MaxNumberRxBeam - with TReport = 0 - TOuk = 1 for CSI-RS based L1-RSRP measurement, and 0 for SSB based L1-RSRP measurement when TCI state switching involves QCL-TypeD - TOuk = 1 when TCI state switching involves other QCL types only - Tfirst-SSB is time to first SSB transmission after L1-RSRP measurement when TCI state switching involves QCL-TypeD; - Tfirst-SSB is time to first SSB transmission after MAC CE command is decoded by the UE for other QCL types; - The SSB shall be the QCL-TypeA or QCL-TypeC to target TCI state

Proposal 10: For BM-Case 1, when the UE has the capability to know the corresponding Rx beam, RAN4 needs to further discuss:

- If the UE supports this capability, for MAC-CE-based TCI state switch delay for known TCI state, whether the UE still needs to perform fine time tracking—i.e., whether TOk can be 0 and under what conditions.

- BM-Case 2

For BM-Case 2, the analysis for Case i) is similar to that of BM-Case 1 above. Additionally, as for Case ii), for example, if the predicted Tx beam in Set A is not in Set B, then the best predicted beam may not have been measured during the TCI state switch. Additionally, since this is a prediction for a future time instance, when considering the MAC-CE-based TCI state switch delay, both known and unknown TCI states may require L1-RSRP measurements and fine time tracking. Specifically, the L1-RSRP measurement must also account for the following constraints:

Taking the above figure as an example, for MAC-CE-based TCI state switch, if the time point after THARQ +3Nslotsubframe,µ falls between TI2 and TI3, and if the predicted beam corresponding to TI3 is considered the target TCI state, then the corresponding L1-RSRP measurement can only be performed at TI3. The reason is that before TI3, the predicted beam (Beam 2) may not yet be available, making it impossible for the UE to measure it. Therefore, the uncertainty period from THARQ +3Nslotsubframe,µ to the earliest predicted time instance of the target TCI (denoted as Tuncertain in the figure) must also be factored into the switch time considerations.

Fig. 4 Timeline analysis for MAC-CE-based TCI state switch delay for BM-Case 2

Proposal 11: For BM-Case 2, regarding the MAC-CE-based TCI state switch delay, when the UE has the capability to know the corresponding Rx beam, RAN4 needs to further discuss:

- If the UE supports this capability, should the L1-RSRP measurement and fine time tracking delay always be defined according to the unknown TCI state case?

- Potential side condition: The L1-RSRP measurement must wait until the corresponding TCI state takes effect before it can be performed (i.e., the L1-RSRP measurement cannot be executed earlier than the earliest predicted time instance of the target TCI state).

  - KPIs related

2.4.1	RSRP prediction accuracy

2.4.1.1	Definition alignment

Issue 2-2: Absolute RSRP accuracy Agreement: - Absolute RSRP accuracy requirement applies to Top-1 of predicted beams at least - FFS whether to apply to other beams Issue 2-3: Relative RSRP accuracy Agreement: - RAN4 to use the follow as baseline for further discussions, if the relative RSRP accuracy requirement is defined - Relative RSRP accuracy for reported beams during inference reporting = (predicted L1-RSRP of beam index i - reported L1-RSRP of beam index n) - (ground truth of L1-RSRP of beam index i - ground truth of L1-RSRP of beam index n), [where the beam index n owns the largest reported value]. - Reported L1-RSRP can be predicted, - FFS whether reported L1-RSRP can be measured RSRP - Relative RSRP accuracy requirements apply for Case 1, and FFS on whether it can apply for Case 2

- Absolute RSRP accuracy

According to the following conclusions passed in the previous RAN1 meetings, it can be seen that only the RSRP reported for the predicted Top 1 beam corresponds to the absolute predicted RSRP. Therefore, for the absolute predicted RSRP, it can be applied to the Top-1 of predicted beams.

Agreement For UE-sided model inference, support the following report format (i.e., CSI field mapping order) for BM-Case1, for beam information on predicted Top K beam(s) among a set of beams and RSRP of predicted Top K beam(s) among a set of beams CRI or SSBRI #1. CRI or SSBRI #2. …. CRI or SSBRI #K. RSRP #1. differential RSRP #2. …. differential RSRP #K. - CRI or SSBRI #k is mapped to RSRP #k, where k = 1,2,…,K - RSRP #1 is absolute predicted RSRP - Differential RSRP #2~#K are differential predicted RSRP with reference to the largest predicted RSRP corresponding to CRI or SSBRI #1

Proposal 12: RAN4 to define absolute RSRP accuracy as one of metrics for beam management requirements/tests, the corresponding definition is:

- For BM-Case 1, the absolute RSRP accuracy = predicted L1-RSRP of beam index i – ground-truth of L1-RSRP of beam index i. The index i corresponds to the index of Top-1 beam of predicted beams

- Relative RSRP accuracy

According to the RAN1 reporting format for RSRP mentioned above, except for the Top 1 predicted beam, the other beams report the differential values of the predicted RSRP relative to the Top 1 predicted beam. Therefore, for beams other than the Top 1 predicted beam, the requirement to be met is the Relative RSRP accuracy, where the relative value refers to the comparison with the Top 1 predicted beam.

Regarding whether the Reported L1-RSRP can be predicted, based on the following RAN1 conclusion, the Reported L1-RSRP can only be predicted (assuming beam index n is also one of the predicted best Top K beams).

RAN1#116bis Agreement For report content of inference results for UE-sided model for BM-Case 1, for the RSRP of predicted Top K beam(s) in the report of inference results, when applicable, further study the following options: - Option A: Predicted RSRP - Option B: Predicted RSRP, if the beam is not configured for corresponding measurement, and measured L1-RSRP if the beam is configured for corresponding measurement - Where the predicted RSRP is based on AI/ML output - Note: Support both Option A and Option B is not precluded. RAN1#120 Agreement For report content of inference results for UE-sided model for BM-Case 1, the RSRP of predicted beam(s)in the report of inference results, is the predicted RSRP, where the predicted RSRP is based on AI/ML output. Note: how to capture it in the spec is a separate discussion.

"Regarding FFS whether the reported L1-RSRP can be measured RSRP, that is, considering beam index n is not a predicted beam but a purely measured beam, the currently agreed ground truth definition is based on the UE's reported measurement time. In this case, the reported L1-RSRP of beam index n will cancel out with the ground truth of L1-RSRP of beam index n."

Proposal 13: RAN4 to use the follow definition to define the relative RSRP accuracy requirement:

  - For BM-Case 1, Relative RSRP accuracy for reported beams during inference reporting = (predicted L1-RSRP of beam index i - Reported L1-RSRP of beam index n) - (ground truth of L1-RSRP of beam index i - ground truth of L1-RSRP of beam index n), where the beam index n owns the largest reported value

- Where, the reported L1-RSRP refers to predicted L1-RSRP

Furthermore, for BM-Case 2, before discussing the definition of KPIs, we believe RAN4 first needs to reach a consensus on "whether performance requirements should be defined for this case."

Current performance analysis has focused on BM-Case 1. If BM-Case 2 is to be considered, the simulation and evaluation workload could potentially double. Additionally, on top of BM-Case 1, time-domain considerations must be incorporated—such as different time gap lengths and the number of time instances—which directly impact prediction accuracy and require further simulation validation.

Given that, even for the performance part, we only have three meeting to discuss and define the metrics, RAN4 needs to clarify the work plan for BM-Case 2 as early as possible.

Proposal 14: Before discussing KPI definitions, we propose that RAN4 first reaches an agreement on whether performance requirements should be defined for this case.

2.4.1.2	Simulation results with consideration of measurement error

In this section, we present simulation results based on the latest assumptions [2] to evaluate the defined absolute RSRP accuracy requirements. We consider the performance impact of the following three cases on beam prediction accuracy in each KPI:

- Case 1: No error will be considered in training dataset, model input during inference and ground-truth.

- Case 2:

  - Case 2a No error will be considered in training dataset and ground-truth. Error will be considered in model input during inference

  - Case 2b No error will be considered in training dataset and ground truth for training. Error will be considered in model input during inference and ground-truth for inference

- Case 3: Error will be considered in training dataset, model input during inference and ground-truth.

Here, baseband (BB) error is modeled using LLS-based measurement error modeling with an assigned SNR of -3 dB. For RF error, Truncated Gaussian distribution under ±4.5dB RF error is used.

On the other hand, in addition to narrow-to-narrow spatial beam prediction (CSI-RS to CSI-RS), the previous meeting also introduced wide-to-narrow spatial beam prediction (SSB to CSI-RS) in the simulation assumptions. For this scenario, we have also provided corresponding simulation results.

Case 1: narrow-to-narrow spatial beam prediction (CSI-RS to CSI-RS)

Table 1-1 90%-tile RSRP difference between the predicted L1-RSRP of the Top-K (K=1,2,3,4,5) predicted beams and the ideal L1-RSRP of the same beams (narrow-to-narrow)

| Simulation results | L1-RSRP difference (dB) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Top K | 1 | 2 | 3 | 4 | 5 |
| Case 1 |  |  |  |  |  |
| No error | 2.51 | 2.81 | 3.07 | 3.17 | 3.23 |
| Case 2a |  |  |  |  |  |
| BB error | 3.57 | 3.74 | 4.06 | 4.08 | 4.30 |
| RF error | 4.98 | 4.82 | 5.54 | 5.45 | 5.37 |
| Both BB and RF | 5.72 | 5.77 | 6.13 | 6.00 | 6.19 |
| Case 2b |  |  |  |  |  |
| BB error | 4.16 | 4.38 | 4.53 | 4.52 | 4.56 |
| RF error | 5.44 | 5.37 | 5.81 | 5.91 | 5.68 |
| Both BB and RF | 5.80 | 6.08 | 6.31 | 6.29 | 6.09 |
| Case 3 |  |  |  |  |  |
| BB error | 4.50 | 4.77 | 4.88 | 4.71 | 5.05 |
| RF error | 5.21 | 5.32 | 5.57 | 5.88 | 5.84 |
| Both BB and RF | 5.06 | 5.61 | 5.95 | 5.98 | 5.94 |

Case 2: wide-to-narrow spatial beam prediction (SSB to CSI-RS)

Table 1-2 90%-tile RSRP difference between the predicted L1-RSRP of the Top-K (K=1,2,3,4,5) predicted beams and the ideal L1-RSRP of the same beams (wide-to-narrow)

| Simulation results | L1-RSRP difference (dB) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Top K | 1 | 2 | 3 | 4 | 5 |
| Case 1 |  |  |  |  |  |
| No error | 1.55 | 1.64 | 1.98 | 2.16 | 2.59 |
| Case 2a |  |  |  |  |  |
| BB error | 4.50 | 5.26 | 7.65 | 7.38 | 8.74 |
| RF error | 16.29 | 16.41 | 17.72 | 17.31 | 17.54 |
| Both BB and RF | 21.12 | 19.77 | 19.17 | 18.17 | 18.42 |
| Case 2b |  |  |  |  |  |
| BB error | 5.11 | 5.65 | 7.67 | 7.50 | 8.47 |
| RF error | 15.67 | 15.97 | 17.24 | 16.77 | 17.18 |
| Both BB and RF | 20.53 | 19.32 | 18.67 | 18.00 | 17.91 |
| Case 3 |  |  |  |  |  |
| BB error | 4.49 | 4.67 | 5.60 | 5.97 | 6.42 |
| RF error | 4.77 | 5.12 | 6.77 | 6.98 | 7.28 |
| Both BB and RF | 5.10 | 5.39 | 7.83 | 7.26 | 8.02 |

Table 1-1 and Table1-2 show the simulation results of RSRP difference between the predicted L1-RSRP of the Top-K predicted beams and the ideal L1-RSRP of the same beams, respectively for narrow to narrow and wide to narrow.

For narrow to narrow, for baseline scenario (Case 1: No Error), the RSRP difference for Top-1 to Top-5 ranges from 6.30 dB to 4.75 dB, decreasing as K increases. For Case 2a (Errors Only in Inference Input), with only BB Error, the difference increases by 0.7–1.12 dB (Top-1 is most sensitive). And for RF error, a significant increase in difference (+3.2–3.9 dB), showing that RF errors have a much larger impact than BB errors. When both B and RF error are considered: the difference reaches 10.05 dB (Top-1), demonstrating a clear additive effect of errors. And for Case 2b (Errors in Both Inference Input and Ground Truth), compared to Case 2a, the difference further increases (e.g., under RF error, Top-1 rises from 9.50 dB to 9.62 dB). For Case 3 (Errors in Training, Inference, and Ground Truth), with only BB Error is considered, the difference (6.69 dB–5.53 dB) is close to Case 1 and even outperforms Case 2a/2b for some K values. The possible reason is that when training data includes errors, the model may learn the error distribution, improving robustness. For RF Error, the difference (8.59 dB–7.30 dB) remains significant but is lower than in Case 2b.

For wide to narrow, for Case 1 (No Error), Lower RSRP differences compared to narrow-to-narrow (1.55 dB at K=1 vs. 2.51 dB), possibly due to wider beam coverage reducing sensitivity. And Case 2a & 2b, it has significant degradation (~3 dB increase over Case 1) with BB Error and extremely high degradation (~15 dB increase) with RF Error. It indicats RF errors are far more disruptive in wide-to-narrow prediction. When combining BB and RF Error, it has severe performance loss (~20 dB). For Case 3 (Full Error), unlike narrow-to-narrow, performance is worse than Case 2a/2b for RF errors but better for BB errors. Training with errors may partially mitigate BB errors but not RF errors in this scenario.

2.4.2	beam prediction accuracy

2.4.2.1	Definition alignment

Regarding the definition of beam prediction accuracy, significant progress was made in the last meeting, and the following conclusions were agreed upon through discussion.

Issue 2-1: Metrics/KPIs for beam ID prediction Agreement: - Metrics/KPIs for beam ID prediction, at least for the case if only beam ID is reported: The successful rate for the correct prediction, - The correct prediction is considered as maximum ground-truth RSRP among top-K predicted beams larger than or equal to the ground-truth RSRP of the strongest genie-aided beam(s) – x dB, - FFS on top-N (N</=K) predicted beams have to be considered in the success rate evaluation - FFS on N values - FFS on x values Note: if x=0 and N=1, it means Top-K/1 (%) : the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams"

Regarding this conclusion, there are still several points to be discussed. We will analyze these points one by one.

Issue 1: Regarding Top-N

The currently approved KPI definition compares the predicted largest beam's ideal RSRP with the actual largest beam's ideal RSRP. Based on our understanding of Top-N here (the conclusion above does not clearly elaborate on Top-N; this understanding is derived from the papers submitted by companies and the discussions, in addition to this comparison, it is also necessary to compare the predicted i-th largest beam's ideal RSRP (i = all predicted beams) with the actual i-th largest beam's ideal RSRP (i = all predicted beams). This method is intended to verify whether the difference between the reported beam's actual RSRP and the corresponding ranked beam's RSRP falls within a certain range, ensuring to some extent that all beams are usable.

However, we believe the necessity of verifying this KPI is not very strong, mainly for the following reasons:

This definition is very similar to what we previously discussed in the RSRP difference KPI: comparing the predicted RSRP of the Top-K beams with the actual RSRP of the corresponding ranked beams. The only difference here is that the comparison is between actual RSRP values. According to the conclusions of previous meetings, the RSRP difference has already been agreed upon as a KPI, and its definition is quite similar to the two definitions mentioned above.

To some extent, this involves redundant comparisons. For example, the second-largest predicted RSRP beam not only needs to be compared with the second-largest actual RSRP beam but also, based on the approved part of the definition, if the second-largest predicted RSRP beam happens to be the largest actual RSRP beam, it must also be compared with the largest actual RSRP beam. This leads to too many comparison combinations to consider.

Proposal 15: For Metrics/KPIs for beam ID prediction agreed in RAN4#114bis, RAN4 not to further consider the verification of top-N (N</=K) predicted beams in the success rate evaluation

Issue 2: Whether Beam ID Prediction Is Only Applied When Only Beam ID Is Reported

For cases where both RSRP and beam ID are reported, we prefer that beam ID accuracy should also be checked. The main reasons are:

- RSRP prediction (same ID) only ensures that the difference between the predicted and actual values falls within a certain range but does not guarantee that the Tx beam is the best. Legacy methods only consider RSRP accuracy because all values are measured. If the reported beam only satisfies RSRP accuracy (e.g., Beam 1 meets the requirement), Beam 1 may not necessarily be the strongest beam but could merely be among the actual Top-K beams. This prediction result does not reflect whether the model can correctly predict the best beam.

- If only RSRP is considered when both beam ID and RSRP are reported, and only the predicted Top-1 RSRP is checked, a possible scenario is: the predicted Top-1 is Beam 1, and its RSRP meets the accuracy requirement. However, Beam 1 might not even be among the reported Top-K beams (extreme case), making this beam practically unusable.

- From a monitoring perspective, the existing RAN1 metric for monitoring requires that at least one of the actual top-N beams (N=1 or 2) is within the predicted Top-K beams. It is unlikely that RSRP-related KPIs will be introduced. If the prediction metric is defined solely based on RSRP, a situation may arise where RSRP meets the accuracy requirement, but monitoring still fails because the two KPIs are evaluating different aspects.

Proposal 16: For Metrics/KPIs for beam ID prediction agreed in RAN4#114bis, it will apply to the case only beam ID is reported and the case both beam ID and L1-RSRP are reported.

Issue 3: FFS on X Values

Regarding the value of X, in the simulation assumptions [2], we included three values for X: 0, 1, 2, and 3. We believe this value can be determined based on the simulation results and analysis submitted by each company.

KPI 1: Beam prediction accuracy - The successful rate for the correct prediction which is considered as maximum ground truth RSRP among top-K predicted beams is larger than the ground truth RSRP of the strongest genie-aided beam – x dB, where K=1,2,3,4,5, X= 0, 1, 2, 3dB

Proposal 17: For the X value of Metrics/KPIs for beam ID prediction agreed in RAN4#114bis, it can be determined based on the simulation results

Specifically, according to the results discussed in RAN1#120 meeting, the step size of RSRP is 1dB. Therefore, the possible values of X can only be positive integers, which are relatively straightforward to determine.

Agreement For report content of inference results for UE-sided model, where the largest RSRP value is quantized to a 7-bit value in the range [-140, -44] dBm with 1dB step size, and the differential RSRP is quantized to a 4-bit value with 2 dB step size. Note: the model output is UE implementation and it doesn’t have to be RSRP subject to dBm value.

2.4.2.2	Simulation result with consideration of measurement error

Here, we also conducted simulation verification on the successful rate for the correct prediction which is considered as maximum ground truth RSRP among top-K predicted beams is larger than the ground truth RSRP of the strongest genie-aided beam – x dB, where K=1,2,3,4,5, X= 0, 1, 2, 3dB. The simulation results are shown below.

Case 1: narrow-to-narrow spatial beam prediction (CSI-RS to CSI-RS)

Table 2-1 Evaluation results of prediction accuracy for BM-Case1 DL Tx beam prediction (narrow-to-narrow)

| Beam prediction accuracy (%) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Top-1 | Top-2 | Top-3 | Top-4 | Top-5 |
| Case 1 |  |  |  |  |  |  |
| No error | X=0 | 77.70 | 91.18 | 94.53 | 96.27 | 97.27 |
|  | X=1 | 88.43 | 94.60 | 96.65 | 97.67 | 98.33 |
|  | X=2 | 92.97 | 96.38 | 97.75 | 98.40 | 98.80 |
|  | X=3 | 95.27 | 97.48 | 98.37 | 98.85 | 99.10 |
| Case 2a |  |  |  |  |  |  |
| BB error | X=0 | 74.53 | 89.80 | 93.93 | 95.98 | 96.95 |
|  | X=1 | 85.93 | 93.63 | 96.25 | 97.45 | 98.05 |
|  | X=2 | 91.55 | 96.02 | 97.63 | 98.28 | 98.62 |
|  | X=3 | 94.53 | 97.25 | 98.32 | 98.80 | 99.05 |
| RF error | X=0 | 66.03 | 84.95 | 91.33 | 94.38 | 95.75 |
|  | X=1 | 78.45 | 89.98 | 94.43 | 96.32 | 97.27 |
|  | X=2 | 85.77 | 93.30 | 96.25 | 97.45 | 98.08 |
|  | X=3 | 90.37 | 95.33 | 97.40 | 98.13 | 98.55 |
| Both BB and RF | X=0 | 64.92 | 83.38 | 90.07 | 93.43 | 95.12 |
|  | X=1 | 76.97 | 88.75 | 93.55 | 95.62 | 96.83 |
|  | X=2 | 84.42 | 92.27 | 95.72 | 96.95 | 97.67 |
|  | X=3 | 89.32 | 94.57 | 97.08 | 97.83 | 98.35 |
| Case 2b |  |  |  |  |  |  |
| BB error | X=0 | 59.80 | 82.50 | 89.85 | 93.98 | 96.03 |
|  | X=1 | 70.48 | 88.63 | 93.72 | 96.10 | 97.37 |
|  | X=2 | 79.57 | 92.65 | 96.17 | 97.40 | 98.07 |
|  | X=3 | 86.28 | 95.38 | 97.40 | 98.20 | 98.70 |
| RF error | X=0 | 55.05 | 78.18 | 87.00 | 92.03 | 94.32 |
|  | X=1 | 64.98 | 84.43 | 91.03 | 94.62 | 96.22 |
|  | X=2 | 73.75 | 88.82 | 93.93 | 96.37 | 97.35 |
|  | X=3 | 81.18 | 92.12 | 95.93 | 97.43 | 98.03 |
| Both BB and RF | X=0 | 56.45 | 79.40 | 87.63 | 91.85 | 94.22 |
|  | X=1 | 66.88 | 85.27 | 91.45 | 94.13 | 95.77 |
|  | X=2 | 76.05 | 89.95 | 94.33 | 95.73 | 96.95 |
|  | X=3 | 82.98 | 93.02 | 96.10 | 96.98 | 97.77 |
| Case 3 |  |  |  |  |  |  |
| BB error | X=0 | 58.40 | 82.15 | 89.43 | 93.65 | 95.48 |
|  | X=1 | 68.72 | 88.00 | 93.12 | 95.82 | 96.98 |
|  | X=2 | 77.72 | 91.90 | 95.48 | 97.12 | 97.88 |
|  | X=3 | 85.08 | 94.82 | 97.00 | 98.07 | 98.53 |
| RF error | X=0 | 54.28 | 78.50 | 87.85 | 92.02 | 94.65 |
|  | X=1 | 64.45 | 84.93 | 92.02 | 94.83 | 96.47 |
|  | X=2 | 73.30 | 89.42 | 94.78 | 96.57 | 97.62 |
|  | X=3 | 81.03 | 92.57 | 96.47 | 97.62 | 98.25 |
| Both BB and RF | X=0 | 58.05 | 80.68 | 88.78 | 92.72 | 94.75 |
|  | X=1 | 68.58 | 86.52 | 92.33 | 95.02 | 96.40 |
|  | X=2 | 77.22 | 90.55 | 94.73 | 96.50 | 97.37 |
|  | X=3 | 84.22 | 93.42 | 96.32 | 97.52 | 98.15 |

Case 2: wide-to-narrow spatial beam prediction (SSB to CSI-RS)

Table 2-2 Evaluation results of prediction accuracy for BM-Case1 DL Tx beam prediction (wide-to-narrow)

| Beam prediction accuracy (%) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Top-1 | Top-2 | Top-3 | Top-4 | Top-5 |
| Case 1 |  |  |  |  |  |  |
| No error | X=0 | 87.18 | 96.18 | 98.62 | 99.27 | 99.53 |
|  | X=1 | 95.40 | 98.53 | 99.37 | 99.62 | 99.75 |
|  | X=2 | 97.70 | 99.47 | 99.78 | 99.83 | 99.88 |
|  | X=3 | 98.60 | 99.73 | 99.93 | 99.95 | 99.98 |
| Case 2a |  |  |  |  |  |  |
| BB error | X=0 | 69.23 | 88.28 | 94.10 | 96.43 | 97.57 |
|  | X=1 | 81.20 | 93.48 | 96.88 | 98.25 | 98.77 |
|  | X=2 | 88.08 | 96.05 | 98.20 | 99.07 | 99.32 |
|  | X=3 | 92.42 | 97.47 | 98.90 | 99.47 | 99.67 |
| RF error | X=0 | 44.68 | 65.57 | 76.88 | 83.83 | 88.23 |
|  | X=1 | 54.38 | 73.67 | 83.28 | 88.93 | 92.38 |
|  | X=2 | 63.35 | 80.07 | 88.15 | 92.83 | 95.25 |
|  | X=3 | 70.45 | 84.52 | 91.52 | 95.07 | 96.80 |
| Both BB and RF | X=0 | 41.35 | 61.27 | 72.72 | 80.32 | 85.55 |
|  | X=1 | 50.70 | 69.10 | 79.33 | 85.72 | 89.97 |
|  | X=2 | 58.70 | 75.85 | 84.97 | 90.33 | 93.43 |
|  | X=3 | 66.05 | 81.27 | 89.05 | 93.37 | 95.78 |
| Case 2b |  |  |  |  |  |  |
| BB error | X=0 | 56.42 | 80.40 | 89.33 | 93.28 | 95.67 |
|  | X=1 | 66.73 | 86.50 | 93.28 | 96.05 | 97.45 |
|  | X=2 | 75.32 | 91.27 | 96.05 | 97.78 | 98.60 |
|  | X=3 | 82.85 | 94.18 | 97.58 | 98.68 | 99.17 |
| RF error | X=0 | 39.82 | 61.07 | 73.18 | 80.77 | 85.85 |
|  | X=1 | 48.25 | 68.58 | 79.55 | 85.93 | 89.87 |
|  | X=2 | 55.80 | 74.98 | 84.75 | 89.98 | 92.80 |
|  | X=3 | 63.28 | 80.42 | 88.85 | 92.88 | 95.02 |
| Both BB and RF | X=0 | 36.87 | 57.10 | 69.40 | 77.25 | 83.33 |
|  | X=1 | 45.22 | 64.05 | 75.80 | 82.52 | 87.40 |
|  | X=2 | 52.83 | 70.48 | 81.15 | 86.92 | 90.73 |
|  | X=3 | 60.05 | 76.38 | 85.53 | 90.45 | 93.52 |
| Case 3 |  |  |  |  |  |  |
| BB error | X=0 | 57.17 | 81.98 | 90.93 | 94.77 | 96.88 |
|  | X=1 | 67.68 | 88.22 | 94.48 | 96.95 | 98.12 |
|  | X=2 | 76.85 | 93.10 | 97.17 | 98.52 | 99.07 |
|  | X=3 | 84.30 | 96.02 | 98.43 | 99.27 | 99.58 |
| RF error | X=0 | 54.10 | 78.77 | 88.67 | 93.50 | 96.12 |
|  | X=1 | 64.52 | 85.62 | 92.70 | 96.10 | 97.70 |
|  | X=2 | 73.53 | 90.65 | 95.62 | 97.73 | 98.58 |
|  | X=3 | 81.50 | 94.15 | 97.50 | 98.68 | 99.20 |
| Both BB and RF | X=0 | 53.65 | 78.45 | 88.40 | 93.50 | 96.13 |
|  | X=1 | 64.07 | 85.15 | 92.57 | 96.07 | 97.70 |
|  | X=2 | 73.48 | 90.42 | 95.52 | 97.77 | 98.77 |
|  | X=3 | 81.15 | 94.37 | 97.53 | 98.73 | 99.42 |

For Narrow-to-Narrow Spatial Beam Prediction, for no error case(case 1), accuracy improves significantly with higher Top-K (from 77.7% at Top-1 to 97.27% at Top-5 for X=0). Relaxing X threshold (X=3) boosts Top-1 accuracy to 95.27%, indicating tolerance to minor RSRP differences. When BB error is consideted, it causes ~3-5% accuracy degradation. And for RF errors, it have more severe effects (max 11.67% drop at X=0/Top-1). For combined errors, it show additive effects, with RF errors remaining dominant. Specifically for Case 2b, it performs worse than Case 2a, especially with RF errors. And for Case 3, it shows limited improvement over Case 2b, suggesting marginal benefits from error-inclusive training.

For Wide-to-Narrow Spatial Beam Prediction, for no error case(case 1), baseline accuracy significantly higher than narrow-beam (87.18% at X=0/Top-1). For BB errors, it cause ~18% accuracy drop (X=0/Top-1). And for RF errors, it lead to catastrophic degradation (max 50.33% drop at X=0/Top-1). When these two errors are combined, it only has 36.87% accuracy at X=0/Top-1. And for Case 3, it outperforms Case 2b with BB errors, suggesting adaptive benefits from error training. For RF error scenarios, it show limited improvement even with relaxed X thresholds (<65% at X=3/Top-1)

- Testability issues for beam management

3.1 Channel model

In the RAN4#114 meeting, the following agreements about the channel models are captured

Issue 4-2: Channel model Agreement: - Use CDL-based channel model as starting point

As we discussed in the previous meeting, a flat channel (e.g., TDL channel model) could not reflect the real performance of AI model and channel models with spatial characteristic need to be emulated in the test system. Thus, RAN4 should at least focus on the CDL channel as the test environment. However, CDL channel with multi-clusters are too complicated in the whole sphere space to emulate. Fig. 5. shows the power angular spectrum (PAS) of AoA and ZoA of CDL-A, CDL-B, CDL-C channel model with NLOS clusters in a sphere space.

| (a) CDL-A | (b) CDL-B | (c) CDL-C |
| --- | --- | --- |

Fig. 5 Illustration of PAS of CDL-A, CDL-B and CDL-C channels

Based on the theory of prefaded signal (PFS) method to emulate the channel in OTA chamber, a great number of probes and high-number ports channel emulator will be implemented, which is not feasible for vendors. Thus, RAN4 needs to find a way to simplified the test system. For option 5, it is a feasible way to simplified the CDL channel model with a small number of clusters, but the test dependability will degrade. Based on the AI BM characteristic, we could have other feasible way and here is the analysis.

Observation 2: The simplified CDL channel with small number clusters will degrade the test dependability.

3.1.1 Feasible way to simplify the test system

UE always recognize the AoA direction of the strongest power path as the reference path for beamforming. When some certain different Tx beams are sweeping and transmitted in the channel environment, the AoA of the strongest power path will not change while the power is still changing. Fig.6 shows an example of the barlett power spectrum for AoA estimation with different Tx beams in fading channel environment, where the UE is assumed as a 4×4 antenna array. It can be observed that at least for beam sets 2-3, 5-10, 13-14-… the AoAs of the strongest path in UE side are the same.

Fig. 6 Bartlett power spectrum for AoA estimation with different Tx beams

Based on the above understanding, some Tx beams can be grouped together (e.g., beam 5-10) if the strongest receive power AoA are the same. Besides, there are different ways to group the Tx beams and needs to be studied. For example, setting a range for AoA of the strongest path and using this range to group Tx beams. Strongest path AoA based grouping is just an example for introducing this theory. An illustration of the Tx beam grouping is shown in Fig.7.

Fig.7 Tx beams grouping

Since only AoA of the signal needs to be emulated in the anechoic chamber while different Tx beams in one group are corresponding to only one AoA, which means that only one probe with one AoA to emulate multiple Tx beams in one group is feasible, and the number of groups equals to the number of probes. TE only needs to change the power of the specific probe in the specific time to emulate the power of the different Tx beams transmitted through fading channel, which makes the test system much easier. Fig. 4. shows an illustration of multi-probe anechoic chamber test system with the Tx beam grouping method. In this MPAC system, only power and direction need to be emulated which is applicable for AI/ML based BM case.

In Fig.8, the probe 1, 2, …, L is for emulating the Tx beams in group 1, 2, …, L, respectively. Only one probe is allowed to be active at the same time for transmitting the beams in the corresponding group. The Tx beams are realized in BS emulator and the fading channel is realized in channel emulator (CE). When the probe 1 is active and probe 2 to L are deactive, probe 1 could emulate the same AoA and different power of different beams in group 1, which are the basic elements for AI/ML based BM testing (L1-RSRPs and spatial characteristics).

Fig.8 Illustration of MPAC test system with Tx beam grouping for emulating AoA and power

Besides, if other channel characteristics needs to be reconstructed in quiet zoom, the test system in Fig. 5 can be referred to reconstruct the channel environment, e.g., spatial correlation, PAS, etc. However, more probes will be needed in this MPAC system.

In Fig.9, the probe set [1, 2, …,K] is for emulating the Tx beams in group 1, and another set […, i, j,…] is for emulating the Tx beams in group 2, and so on. Only one set of probes is allowed to be active at the same time for transmitting the beams in the corresponding group. When the probes in probe set [1, 2, …,K] are active and other probe sets are deactive, probe set [1, 2, …,K] could emulate the spatial correlation, PAS. This test system could emulate a more accurate environment that the test system in Fig. 5., but more probes are needed.

Fig.9 Illustration of MPAC test system with Tx beam grouping for reconstructing channel environment

Therefore, following procedures can be used for simplification:

Step 1. Generate the BS beamforming power pattern of every Tx beam

Step 2. Add the BS beamforming power pattern into the power of different clusters in specific ZoD and AoD, and derive the new power of every cluster

Step 3. Use the new power of clusters to generate the power angular spectrum (PAS) of ZoA and AoA

Step 4. Some Tx beams with the corresponding ZoA/AoA are to be grouped together based on a certain rule (e.g., the ZoA/AoA of the strongest power derived from the PAS in step 3 are the same angle or in a pre-defined range)

Step 5. Using one or small number of probes to emulate one beam group.

Step 6. Delete the unnecessary probes and merge some adjacent probes.

Observation 3: The number of probes can be greatly reduced by grouping different Tx beams together. Only one probe or one set of probes can emulate different Tx beams in one group.

Observation 4: Different test system with reduced cost and complexity can be introduced by using the Tx beam grouping method.

Proposal 18: RAN4 to consider methods to group Tx beams together to reduce the number of probes in the test system.

Proposal 19: RAN4 to consider the following procedures to simplify the test system:

- Step 1. Generate the BS beamforming power pattern of every Tx beam

- Step 2. Add the BS beamforming power pattern into the power of different clusters in specific ZoD and AoD, and derive the new power of every cluster

- Step 3. Use the new power of clusters to generate the power angular spectrum (PAS) of ZoA and AoA

- Step 4. Some Tx beams with the corresponding ZoA/AoA are to be grouped together based on a certain rule (e.g., the ZoA/AoA of the strongest power derived from the PAS in step 3 are the same angle or in a pre-defined range)

- Step 5. Using one or small number of probes to emulate one beam group.

- Step 6. Delete the unnecessary probes and merge some adjacent probes.

Proposal 20: RAN4 to consider the following two approaches for design the test system

approach a

approach b

3.2 Test setup

In RAN4#114 meeting, the following agreements have been achieved.

Issue 4-1: Baseline test setup Agreement: - Discuss both single AoA based and multiple AoA based set up - Continue to study the two multiple AoA candidate setups - One option to continue the discussion proposed as following: - Multi AoA setup – evaluation of already feasible systems for AI/ML BM testing suitability - AI/ML BM performance centric, not CDL channel emulation centric. - Define test system candidates which are already feasible - Perform system level simulations including the probe layout of candidate systems to address the impact on the UE AI ML BM performance. - Start with the set up with Set A = 32 beams, Set B = 8 beams, and develop the test procedures with single AoA and multiple AoAs

For this issue, we provide some simulation results for the OTA probe setup under the NLOS channel CDL-A, B, C based on the method, where the 128 Tx beams set A beamforming characteristics are given as:

Azimuth angle in degree: [-84.375 -73.125 -61.875 -50.625 -39.375 -28.125 -16.875 -5.625 5.625 16.875 28.125 39.375 50.625 61.875 73.125 84.375]

Zenith angle in degree: [81 84 87 90 93 96 99 102]

The power patterns of the 128 Tx beams in azimuth cut and elevation cut (elevation angle in degree = zenith angle in degree – 90degree) are shown below.

(a)

(b)

Fig.10 BS beamforming power pattern of 128 Tx beams

CDL-A

| (a) | (b) |
| --- | --- |
| (c) | (d) |

Fig.11 Probe allocation for CDL-A

CDL-B

| (a) | (b) |
| --- | --- |
| (c) | (d) |

Fig.12 Probe allocation for CDL-B

CDL-C

| (a) | (b) |
| --- | --- |
| (c) | (d) |

Fig.13 Probe allocation for CDL-C

Based on the probe allocation, it can be seen that some allocated probes are really close in angle-domain and can be merged and replaced by only one probe. Figure 10 shows an example for how to merging the probes in angle domain.

(a) original probe allocation for CDL-A, B, C (red: CDL-A, blue: CDL-B, green: CDL-C)

(b) new probe allocation after UE rotation in zenith angle domain

(c) 5 probe allocation after merge considering UE Rx resolution

Fig.14 probe allocation after merge and rotation

In Fig.14(a), the allocated probes for CDL-A, B, C are illustrated and the total number is 31, which is not feasible. However, test should be performed under different channel models and no need to consider channel model switch (e.g., CDL-A to CDL-B) in one test. Thus, we can rotate the channel (rotate DUT in 3D-turnable) to narrow the channel model range in angular domain, which is shown in Fig. 10 (b). In Fig.14 (b), all the channels are rotated in zenith angular domain and probes are symmetry about zenith angle = 90°. Besides, UE Rx beamforming has its resolution in angular domain, e.g., 30 deg. Fig.14 (c) shows an example of probe allocation when considering UE Rx 30 degrees resolution, which eventually provided 5 candidates probe locations and greatly decreases the cost of the test system.

Proposal 21: Companies to provide candidate probe locations for CDL-A/B/C for merging and rotating. E.g.,

| (a) CDL-A | (b) CDL-B | (c) CDL-C |
| --- | --- | --- |

Proposal 22: RAN4 to consider the UE Rx beamforming resolution in angular domain to perform OTA probe allocations.

- Summary

Proposal 1: For spatial prediction, the measurement of set B relies on one-shot measurement

Proposal 2: For L1-RSRP measurement of set B, the definition and value of the beam sweeping factor N follows existing requirements.

Proposal 3: For requirements on AI/ML-based L1-RSRP measurement, when referencing values from an existing clause in the same spec version where the measurement period is defined, RAN4 will discuss the coexistence supportability of the prediction with other previously enhanced features. These enhanced features primarily include:

The UE capability specified in clause 9.5.4 L1-RSRP measurement requirements in Rel-19 spec：

­	1. bwpOperationMeasWithoutInterrupt-r18

­	2. ncd-SSB-BWP-Wor-r18

­	3. groupBasedBeamReporting-r17

­	4. multiDCI-IntraCellMultiTRP-TwoTA-r18

­	5. multiDCI-InterCellMultiTRP-TwoTA-r18 and rxTimingDiff-r18,

­	6. rxTimingDiff-r18

­	7. highSpeedMeasFlagFR2-r17

­	8. concurrentMeasGap-r17

­	9. musim-GapPreference-r17

­	10. multiCellL1-measRTD-greaterThan-CP-r18

­	11. fastBeamSweepingMultiRx-r18

Proposal 4: For the above enhanced features, RAN4 not to consider combining the AI/ML-based BM with the features related to MultiTRP, HST, group-based beam reporting, LTM, Multi-Rx when defining requirements for AI/ML-based L1-RSRP measurement. The specific UE capabilities include:

­	groupBasedBeamReporting-r17

­	multiDCI-IntraCellMultiTRP-TwoTA-r18

­	multiDCI-InterCellMultiTRP-TwoTA-r18 and rxTimingDiff-r18,

­	rxTimingDiff-r18

­	highSpeedMeasFlagFR2-r17

­	multiCellL1-measRTD-greaterThan-CP-r18

­	fastBeamSweepingMultiRx-r18

Observation 1: The inference latency is reflected in the RAN1 specifications. As for the prediction delay, this inference latency is already included within TReport.

Proposal 5: The prediction processing delay component does not need to be explicitly defined within the prediction delay requirements TL1-RSRP_Prediction_Period_CSI-RS.

Proposal 6:

- For CSI-RS based prediction , the prediction period requirements TL1-RSRP_Prediction_Period_CSI-RS for BM  can be defined as

Table : Prediction period TL1-RSRP_Prediction_Period_CSI-RS for FR2

| Configuration | TL1-RSRP_Prediction_Period_CSI-RS (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*N)*TCSI-RS) |
| DRX cycle ≤ 320 ms | max(TReport, ceil(1.5*M*P*N)*max(TDRX,TCSI-RS)) |
| DRX cycle > 320 ms | ceil(M*P*N)*TDRX |
| NOTE 1: TCSI-RS is the periodicity of CSI-RS configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. NOTE 2: the requirements are applicable provided that the CSI-RS resource configured for L1-RSRP measurement is transmitted with Density = 3. |  |

- For SSB-based prediction , the prediction period requirements TL1-RSRP_Prediction_Period_SSB for BM  can be defined as

Table : Prediction period TL1-RSRP_Prediction_Period_SSB for FR2

| Configuration | TL1-RSRP_Prediction_Period_SSB (ms) |
| --- | --- |
| non-DRX | Max (TReport, ceil (M*P*N) *TSSB) |
| DRX cycle ≤ 320 ms | Max (TReport, ceil (1.5*M*P*N) *max (TDRX, TSSB)) |
| DRX cycle > 320 ms | Ceil (1.5*M*P*N) *TDRX |
| Note: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. |  |

Proposal 7：For prediction delay requirement for BM-Case 2 (if defined), RAN4 not to consider aperiodic CSI-RS reporting scenario.

Proposal 8: For BM-Case 1 and BM-Case 2, the target TCI state is known when the following is met:

- The TCI state switch command is received within 1280 ms upon the last transmission of the RS resource for beam reporting or measurement and the UE has sent at least one L-1 RSRP measurement or predicted RS quantities report of the target TCI state (clause 8.10.2 of 38.133)

- Prediction would not be used as a condition for known TCI state (i.e., not to be reflected as the time delay restriction between prediction results reporting and TCI switch command)

Proposal 9: RAN4 to update the known TCI state conditions as

Proposal 10: For BM-Case 1, when the UE has the capability to know the corresponding Rx beam, RAN4 needs to further discuss:

- If the UE supports this capability, for MAC-CE-based TCI state switch delay for known TCI state, whether the UE still needs to perform fine time tracking—i.e., whether TOk can be 0 and under what conditions.

Proposal 11: For BM-Case 2, regarding the MAC-CE-based TCI state switch delay, when the UE has the capability to know the corresponding Rx beam, RAN4 needs to further discuss:

- If the UE supports this capability, should the L1-RSRP measurement and fine time tracking delay always be defined according to the unknown TCI state case?

- Potential side condition: The L1-RSRP measurement must wait until the corresponding TCI state takes effect before it can be performed (i.e., the L1-RSRP measurement cannot be executed earlier than the earliest predicted time instance of the target TCI state).

Proposal 12: RAN4 to define absolute RSRP accuracy as one of metrics for beam management requirements/tests, the corresponding definition is:

- For BM-Case 1, the absolute RSRP accuracy = predicted L1-RSRP of beam index i – ground-truth of L1-RSRP of beam index i. The index i corresponds to the index of Top-1 beam of predicted beams

Proposal 13: RAN4 to use the follow definition to define the relative RSRP accuracy requirement:

  - For BM-Case 1, Relative RSRP accuracy for reported beams during inference reporting = (predicted L1-RSRP of beam index i - Reported L1-RSRP of beam index n) - (ground truth of L1-RSRP of beam index i - ground truth of L1-RSRP of beam index n), where the beam index n owns the largest reported value

- Where, the reported L1-RSRP refers to predicted L1-RSRP

Proposal 14: Before discussing KPI definitions, we propose that RAN4 first reaches an agreement on whether performance requirements should be defined for this case.

Proposal 15: For Metrics/KPIs for beam ID prediction agreed in RAN4#114bis, RAN4 not to further consider the verification of top-N (N</=K) predicted beams in the success rate evaluation

Proposal 16: For Metrics/KPIs for beam ID prediction agreed in RAN4#114bis, it will apply to the case only beam ID is reported and the case both beam ID and L1-RSRP are reported.

Proposal 17: For the X value of Metrics/KPIs for beam ID prediction agreed in RAN4#114bis, it can be determined based on the simulation results

Observation 2: The simplified CDL channel with small number clusters will degrade the test dependability.

Observation 3: The number of probes can be greatly reduced by grouping different Tx beams together. Only one probe or one set of probes can emulate different Tx beams in one group.

Observation 4: Different test system with reduced cost and complexity can be introduced by using the Tx beam grouping method.

Proposal 18: RAN4 to consider methods to group Tx beams together to reduce the number of probes in the test system.

Proposal 19: RAN4 to consider the following procedures to simplify the test system:

- Step 1. Generate the BS beamforming power pattern of every Tx beam

- Step 2. Add the BS beamforming power pattern into the power of different clusters in specific ZoD and AoD, and derive the new power of every cluster

- Step 3. Use the new power of clusters to generate the power angular spectrum (PAS) of ZoA and AoA

- Step 4. Some Tx beams with the corresponding ZoA/AoA are to be grouped together based on a certain rule (e.g., the ZoA/AoA of the strongest power derived from the PAS in step 3 are the same angle or in a pre-defined range)

- Step 5. Using one or small number of probes to emulate one beam group.

- Step 6. Delete the unnecessary probes and merge some adjacent probes.

Proposal 20: RAN4 to consider the following two approaches for design the test system

approach a

approach b

Proposal 21: Companies to provide candidate probe locations for CDL-A/B/C for merging and rotating. E.g.,

Proposal 22: RAN4 to consider the UE Rx beamforming resolution in angular domain to perform OTA probe allocations.

- References

- R4-2508080, WF on NR_AIML_air_part1, Qualcomm Incorporated

- R4-2508081, Updated simulation assumptions for BM, vivo, NTU, Nokia, Ericsson, Qualcomm, Xiaomi, Huawei, Hisilicon, Mediatek, OPPO, APPLE, Rohde & Schwarz, CATT, Samsung, Intel, ZTE Corporation, Sanechips, CAICT

- TR 38.843