3GPP TSG-RAN WG4 Meeting #116	R4-2510363

# Bengaluru, India, August 25th – 29th, 2025

# Source: 	ZTE Corporation, Sanechips

# Title: 	Discussion on UE RF requirements for NR NTN phase3

# Agenda item:	7.27.3.1

# Document for:	Approval

# 1 Introduction

In RAN#102 meeting, a new Work Item [1] was approved on Non-Terrestrial Networks (NTN) for NR Phase 3 with the following RAN4 objective. In this contribution, we want to share some further views from RedCap UE RF perspective.

- Support of Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN operating in FR1-NTN bands [RAN4, RAN1] - For full-duplex FDD RedCap and eRedCap UEs, define the RF and RRM requirements [RAN4] - For HD-FDD RedCap UEs and eRedCap UEs, check whether any essential changes are needed for their support (i.e. focusing on HD collision rules) by end of Q2/2024 [RAN1] - Depending on feasibility assessment above, define the RF and RRM requirements [RAN4] - Notes for this objective: - GNSS (Global Navigation Satellite Systems) capabilities and simultaneous GNSS and NR-NTN operation is supported in RedCap/eRedCap UE.

# 2 Discussion

  - 2.1 Simultaneous operation with NR NTN and GNSS

The above is reply LS from RAN2 to ask feedback on both duration and periodicity of the IDC free time to be applied to IDC TDM or autonomous denial solution. For IDC free time, we can use the value in IE GNSS-PositionFixDuration as a basis. IE GNSS-PositionFixDuration is introduced in 36.331, and indicates the time duration required for the UE to acquire a GNSS position. The values indicated in IE GNSS-PositionFixDuration are from 1 seconds to 31 seconds to accommodate various scenarios. In reality, GNSS receivers need to go through different start modes—cold, warm, and hot—each impacting the time to first fix (TTFF), or the time it takes for the receiver to compute its initial position. Cold start mode usually need above 20 seconds, while warm start mode needs about 10-20 seconds. For simultaneous operation with NR NTN and GNSS, we only need to consider hot start mode. Hence, the duration of IDC free time could be less than 10 seconds.

Proposal 1: The duration of IDC free time could be less than 10 seconds.

- – GNSS-PositionFixDuration The IE GNSS-PositionFixDuration indicates the time duration required for the UE to acquire a GNSS position. Value s1 corresponds to 1 second, s2 corresponds to 2 seconds and so on. GNSS-PositionFixDuration information element -- ASN1START GNSS-PositionFixDuration-r18 ::= ENUMERATED{ s1, s2, s3, s4, s5, s6, s7, s13, s19, s25, s31} -- ASN1STOP

  - 2.2 Output power enhancement for NR NTN PC3

- Topic 1: Output power enhancement for NR NTN PC3 Agreements: - Define new UE capabilities for UE to indicate support for the additional power boosting per band and network to enable the boosting - FFS for details, capabilities will be needed for feature list - When UE indicates support for boosting and network enables the boosting, UE reference power is increased, i.e. both Pcmax_L and Pcmax_H are increased - MPR is not changed for DFT-s-OFDM and QPSK and pi/2 BPSK, [all RB allocations] - For CP-OFDM and DFT-s-OFDM 16QAM and 64QAM, boosting is allowed by UE implementation and MPR is correspondingly increased by the amount of the boost - Boost is 0.5 dB - FFS on duplex modes - For information only: HD-FDD/FD-FDD support is indicated per band - This means that UE has option to e.g. be HD-FDD in band A and support boosting and be FD-FDD in band B and not support boosting - Rel-18 and Rel-19 power boosting schemes can be simultaneously supported - FFS for details - PC3 ACLR requirements apply for power boosted transmissions - Configured maximum output power and MPR updates related to power boosting need to be captured to TS 38.101-5

The above is the agreement and way forward we reached in last meeting [2]. Due to lower RF front-end insertion loss, a new UE capabilities to indicate support for the additional power boosting will be introduced per band and network needs to enable the boosting. For applicable FDD modes, Rel-19 power boosting could be applied to both FD-FDD and HD-FDD as Rel-19 power boosting depends on insertion loss reduction. For the details of the UE capability, we propose to define the following UE feature list table.

Proposal 2: Introduce a new UE capability to indicate support of Rel-19 power boosting with the following feature list:

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - NR_NTN_Ph3 | - 58-1 | Power boosting for NR-NTN | Support of UE power boosting for all modulation orders due to lower RF front-end insertion loss. The power boosting is only enabled via RRC powerboostingNTNRel-19. | N/A | Yes | N/A | UE cannot support power boosting. | Per band | FDD only | FR1 only | N/A | N/A |  | Optional with capability signalling |  |

# 3 Conclusion

In this contribution, we share some views on UE RF requirements for NR NTN phase3 and the proposals are made as follows:

Proposal 1: The duration of IDC free time could be less than 10 seconds.

Proposal 2: Introduce a new UE capability to indicate support of Rel-19 power boosting with the following feature list.

# 4 Reference

- RP-234078, New WID: Non-Terrestrial Networks (NTN) for NR Phase 3, Huawei

- R4-2508764, Way Forward for [115][309] NR_NTN_Ph3_General_UE_SAN_RF, Qualcomm Inc.