3GPP TSG-RAN WG4 Meeting #116	R4-2510368

Bengaluru, 25th – 29rd August 2025

Title: 	Power domain enhancements for single carrier

Agenda Item:	7.1.2.2.1

Source: 	Nokia

Document for:	Approval

# 1				Introduction

In this contribution we express our views to enable MPR reduction.

# 2	Discussion

In previous RAN4 meeting WF [1], several mechanisms to enhance power domain performance in NR systems, particularly focusing on MPR (Maximum Power Reduction) optimization and channel bandwidth (CBW) extension for single carrier scenarios has been discussed.

Sub-topic 1-1: Approaches to enable MPR reduction Issue 1-1-1: Extension values/ratios - Agreement in main session: - At least ½ and ¼ are introduced as the extension ratio to apply to both single-sided and dual-sided cases - It is FFS on the details of symmetric and asymmetric cases for dual-sided case where non-zero extension is applied for both sides. - It is FFS on details of the single side case. Issue 1-1-2: Guard band for extended CBW - WF - GB for extended CBW is a virtual concept to consider the boundary applying the shifted OOBE, which is not reflected in the specification and it can be calculated by extended CBW, UE CBW, NRB and extended NRB - Extended CBW is not limited to the existing UE CBW - RAN4 studies whether the extended BW emissions can be guaranteed by only measuring the existing CBW supported in the band (close to the extended BW) given the relaxed extended GB and preliminary measurements performed suggests Issue 1-1-3: Derivation of new inner RB region in extended CBW - WF - The following approach is considered as starting point to be further discussed in next meeting, notation should be further considered as well - For NRB based RB_start coordinate system origin, the formula of new inner region can be changed as below: - RBStart,Low_new = max(0, RBStart,Low – RBshift_low); - RBStart,High_new = min(NRB – LCRB, NRB + RBshift_high – RBStart,Low_new – LCRB); - RBStart,Low_new ≤ RBStart ≤ RBStart,High_new, and LCRB ≤ ceil((NRB+ RBshift_low + RBshift_high)/2); - RBshift_low, RBshift_high ≤ NRB/2 - Other approach is not precluded - The approach shall be generic to cover all possible extension cases, including symmetric, asymmetric and different extension ratios Sub-topic 1-2: Applicable requirements Issue 1-2-1: Derivation of the extended IBE - Agreement in main session: - Fixed IBE used for extended CBW at each side of UE CBW as the baseline, the value is derived from the outmost allocated RB of UE CBW (illustrated in a figure to be developed) - The details and the related test cases can be discussed and decided later. - If significant issue is identified, other options can be further considered. - WF Further discuss whether to apply the extended IBE in the GB of extended CBW. - Option1: Yes - Option2: No Issue 1-2-2: Relaxation of the extended IBE - Options - Option 1: using BPSK EVM in the IBE formula for all extended RBs - Option 2: no relaxation is needed. - WF - If there is no obvious system performance impact observed, option 1 is adopted Sub-topic 1-3: Signaling Issue 1-3-1: NW signaling - Agreement in main session: - The power enhancement feature is enabled by the gNB RRC signalling according to UE capabilities, - The details of the signaling including the contents are FFS - WF - It is up to the NW to judge the gap width on both sides of the UE - NW sends info of extension ratio (at least 1/2) per side Issue 1-3-2: UE capability - Agreement in main session: - No separate UE capabilities for scenario 1 and scenario 2 - Capability 1: Indicates whether UE supports MPR reduction for single UL carrier with 1/2*UE CBW extension only - Both dual-sided and single sided cases are supported. - Capability 2: Indicates whether UE supports MPR reduction for single UL carrier with both ½ and 1/4*UE CBW extension - Both dual-sided symmetric and single sided cases are supported. - For dual-sided case, it is FFS if asymmetric cases and a separated capability is introduced. - Note: the other ratio number depends on discussion of issue 1-1-1

Extension configuration flexibility:

Concerning the Issue 1-1-1: Extension values/ratios, the channel extension should be flexible enough to enhance MPR for wide range of RB allocation and different scenarios, so it is preferable to support symmetric and asymmetric dual-sided extension in addition to the single sided case. Hence, NW should be able to configure the extension factor for each side separately.

Proposal 1: RAN4 should support in rel-19 symmetric and asymmetric dual-sided extension in addition to the single-sided case based on NW configuration.

UE guard band definition for rel-19 power domain MPR enhancements:

Concerning the guard band definition in order to determine the boundary for the shifted OOBE, it should be defined per each side separately to allow gNB configuration flexibility of symmetric and asymmetric dual/single-sided extension (i.e., symmetric/asymmetric guard band size). Hence, UE RF specification should define a single unified guard band per side based on the extension factor, UE channel BW, etc. This new unified guard band covers all [left or right] frequency segments outside UE maximum transmission bandwidth, and it shall be larger than UE minimum guard band size defined in 38.101 Table 5.3.3-1 with extension factor/ratio greater than or equal to zero (legacy rel-18 UE).

Proposal 2: RAN4 shall specify in rel-19 a unified guard band per side to determine the boundary for the shifted OOBE for MPR enhancements with single/dual sided symmetric or asymmetric extension. The unified guard band size shall be larger than or equal to UE minimum guard band size defined in 38.101-1 Table 5.3.3-1 by considering at least ½, ¼  and zero extension factor/ratio per side as shown below.

IBE requirements with UE extended CBW:

The extended IBE (In-band Emission) requirements shall be extended up to UE OOBE boundaries (i.e. extended UE CBW boundaries) in order to limit the interferences to other UE possible allocation in the same and/or adjacent cell overlapping with the extended RB.

Proposal 3: To limit the high interference in the network, IBE requirements shall be specified within the extended UE CBW (i.e., the shifted OOBE boundary) for all non-allocated RBs and possible RB allocation for other UE(s) in the same and/or adjacent cells.

In addition, RAN4 shouldn’t define unnecessary IBE requirement overlapping with gNB’s guard band where there is no possible UE/gNB transmission.

Proposal 4: IBE requirement shouldn’t be defined in the frequency range overlapping with the guard band(s) of the BS CBW to avoid defining unnecessary IBE requirement where there is no possible transmission to/from any UE and thus against IBE main purpose to limit interference to other UEs.

Proposal 5: Before further IBE relaxation based on BPSK EVM if it is proven still necessary, RAN4 shall ensure a well-defined IBE limiting interference to other UE(s) and avoiding unnecessary UE RF requirement overlapping with BS guard bands as shown in Fig. 1a-b.

Figure 1a: IBE requirement range within UE extended CBW where BS CBW guard bands and UE guard bands are non-overlapping

Figure 1b: IBE requirement range within UE extended CBW where BS CBW guard bands and UE guard bands are overlapping in at least one side.

# 3	Conclusion

In this contribution we have made following proposal.

Proposal 1: RAN4 rel-19 should support symmetric and asymmetric dual-sided extension in addition to the single-sided case based on NW configuration.

Proposal 2: RAN4 rel-19 shall specify a unified guard band per side to determine the boundary for the shifted OOBE for MPR enhancements with single/dual sided symmetric or asymmetric extension. The unifed guard band size shall be larger than or equal to UE minimum guard band size defined in 38.101-1 Table 5.3.3-1 by considering at least ½, ¼ and zero extension factor/ratio per side as shown below.

Proposal 3: To limit the high interference in the network, IBE requirements shall be specified within the extended UE CBW (i.e., the shifted OOBE boundary) for all non-allocated RBs or frequency segments that are valid allocation for other UE.

Proposal 4: IBE requirement shouldn’t be defined in the frequency range overlapping with the guard band(s) of the BS CBW to avoid defining unnecessary IBE requirement where there is no possible transmission to/from any UE and thus against IBE main purpose to limit interference to other UEs.

Proposal 5: Before further IBE relaxation based on BPSK EVM if it is proven still necessary, RAN4 shall ensure a well-defined IBE limiting interference to other UE(s) and avoiding unnecessary UE requirement overlapping with BS guard bands as shown in Fig. 1a-b.

Figure 1a: IBE requirement range within UE extended CBW where BS CBW guard bands and UE guard bands are non-overlapping

Figure 1b: IBE requirement range within UE extended CBW where BS CBW guard bands and UE guard bands are overlapping in at least one side.

# 4	References

[1] 3GPP TSG-RAN WG4 Meeting #115, WF on NR_ENDC_RF_Ph4_part1 R4-2507930, Malta, MT, 19th – 23rd May, 2025