3GPP TSG-RAN WG4 Meeting #116	R4-2510371

Bengaluru, India, August 25th – 29th, 2025

Agenda item:	7.13.4

Source: 		NTT DOCOMO, INC.

Title: 	Simulation results for NR MMSE-IRC BS demodulation.

Document for:	Other

- Introduction

We show the simulation results based on WF in RAN4#115 [1].

- Simulations

Table 1: Simulation results for PUSCH with 30% BLER, Type A, 5MHz Channel bandwidth, 15kHz SCS

| Number of TX antennas | Number of RX antennas | Propagation conditions and correlation matrix |  | DIP set | MCS index | Waveform | Ideal SNR | Impairment SNR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Tested signal | Interferer |  |  |  |  |  |
| 1 | 2 (1 interferer) | TDLA30-10 | TDLC300-100 | 2 | 16 | CP-OFDM | 11.95 | 13.95 |
|  |  | TDLC300-100 |  | 1 |  |  | 11.96 | 13.96 |
|  | 8 (2 interferers) | TDLA30-10 |  | 2 |  |  | 6.13 | 8.13 |
|  |  | TDLC300-100 |  | 1 |  |  | 5.62 | 7.62 |

Table 2: Simulation results for PUSCH with 30% BLER, Type A, 20MHz Channel bandwidth, 15kHz SCS

| Number of TX antennas | Number of RX antennas | Propagation conditions and correlation matrix |  | DIP set | MCS index | Waveform | Ideal SNR | Impairment SNR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Tested signal | Interferer |  |  |  |  |  |
| 1 | 2 (1 interferer) | TDLA30-10 | TDLC300-100 | 2 | 16 | CP-OFDM | 12.39 | 14.39 |
|  |  | TDLC300-100 |  | 1 |  |  | 11.62 | 13.62 |
|  | 8 (2 interferers) | TDLA30-10 |  | 2 |  |  | 6.25 | 8.25 |
|  |  | TDLC300-100 |  | 1 |  |  | 6.04 | 8.04 |

Table 3: Simulation results for PUSCH with 30% BLER, Type A, 10MHz Channel bandwidth, 30kHz SCS

| Number of TX antennas | Number of RX antennas | Propagation conditions and correlation matrix |  | DIP set | MCS index | Waveform | Ideal SNR | Impairment SNR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Tested signal | Interferer |  |  |  |  |  |
| 1 | 2 (1 interferer) | TDLA30-10 | TDLC300-100 | 2 | 16 | CP-OFDM | 12.00 | 14.00 |
|  |  | TDLC300-100 |  | 1 |  |  | 11.69 | 13.69 |
|  | 8 (2 interferers) | TDLA30-10 |  | 2 |  |  | 6.33 | 8.33 |
|  |  | TDLC300-100 |  | 1 |  |  | 5.91 | 7.91 |

Table 4: Simulation results for PUSCH with 30% BLER, Type A, 100MHz Channel bandwidth, 30kHz SCS

| Number of TX antennas | Number of RX antennas | Propagation conditions and correlation matrix |  | DIP set | MCS index | Waveform | Ideal SNR | Impairment SNR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Tested signal | Interferer |  |  |  |  |  |
| 1 | 2 (1 interferer) | TDLA30-10 | TDLC300-100 | 2 | 16 | CP-OFDM | 11.96 | 13.96 |
|  |  | TDLC300-100 |  | 1 |  |  | 11.40 | 13.40 |
|  | 8 (2 interferers) | TDLA30-10 |  | 2 |  |  | 6.23 | 8.23 |
|  |  | TDLC300-100 |  | 1 |  |  | 5.90 | 6.90 |

Table 5: Simulation results for PUSCH with 30% BLER, Type B, 5MHz Channel bandwidth, 15kHz SCS

| Number of TX antennas | Number of RX antennas | Propagation conditions and correlation matrix |  | DIP set | MCS index | Waveform | Ideal SNR | Impairment SNR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Tested signal | Interferer |  |  |  |  |  |
| 1 | 2 (1 interferer) | TDLA30-10 | TDLC300-100 | 2 | 16 | CP-OFDM | 13.43 | 15.43 |
|  |  | TDLC300-100 |  | 1 |  |  | 16.16 | 18.16 |
|  | 8 (2 interferers) | TDLA30-10 |  | 2 |  |  | 8.70 | 10.70 |
|  |  | TDLC300-100 |  | 1 |  |  | 8.07 | 10.07 |

Table 6: Simulation results for PUSCH with 30% BLER, Type B, 20MHz Channel bandwidth, 15kHz SCS

| Number of TX antennas | Number of RX antennas | Propagation conditions and correlation matrix |  | DIP set | MCS index | Waveform | Ideal SNR | Impairment SNR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Tested signal | Interferer |  |  |  |  |  |
| 1 | 2 (1 interferer) | TDLA30-10 | TDLC300-100 | 2 | 16 | CP-OFDM | 13.63 | 15.63 |
|  |  | TDLC300-100 |  | 1 |  |  | 18.64 | 20.64 |
|  | 8 (2 interferers) | TDLA30-10 |  | 2 |  |  | 9.32 | 11.32 |
|  |  | TDLC300-100 |  | 1 |  |  | 8.97 | 10.97 |

Table 7: Simulation results for PUSCH with 30% BLER, Type B, 10MHz Channel bandwidth, 30kHz SCS

| Number of TX antennas | Number of RX antennas | Propagation conditions and correlation matrix |  | DIP set | MCS index | Waveform | Ideal SNR | Impairment SNR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Tested signal | Interferer |  |  |  |  |  |
| 1 | 2 (1 interferer) | TDLA30-10 | TDLC300-100 | 2 | 16 | CP-OFDM | 12.67 | 14.67 |
|  |  | TDLC300-100 |  | 1 |  |  | 12.27 | 14.27 |
|  | 8 (2 interferers) | TDLA30-10 |  | 2 |  |  | 7.06 | 9.06 |
|  |  | TDLC300-100 |  | 1 |  |  | 6.39 | 8.39 |

Table 8: Simulation results for PUSCH with 30% BLER, Type B, 100MHz Channel bandwidth, 30kHz SCS

| Number of TX antennas | Number of RX antennas | Propagation conditions and correlation matrix |  | DIP set | MCS index | Waveform | Ideal SNR | Impairment SNR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Tested signal | Interferer |  |  |  |  |  |
| 1 | 2 (1 interferer) | TDLA30-10 | TDLC300-100 | 2 | 16 | CP-OFDM | 11.99 | 13.99 |
|  |  | TDLC300-100 |  | 1 |  |  | 12.50 | 14.50 |
|  | 8 (2 interferers) | TDLA30-10 |  | 2 |  |  | 7.34 | 9.34 |
|  |  | TDLC300-100 |  | 1 |  |  | 6.42 | 8.42 |

- Conclusion

This contribution shows the simulation results of NR MMSE-IRC PUSCH demodulation.

References

- R4-2508618, Way Forward for [115][322] NR_demod_Ph5_Part1_General_BS, China Telecom, RAN4#115