3GPP TSG-RAN WG4 Meeting #116	R4-2510372

Bengaluru, India, August 25th – 29th, 2025

Agenda item:	7.13.4

Source: 		NTT DOCOMO, INC.

Title: 	Discussion on BS demodulation for NR MMSE-IRC.

Document for:	Discussion

- Introduction

In the last meeting RAN4 #115, it was discussed that RAN4 demodulation spec on NR MMSE-IRC. Some performance requirements are still open [1]. Thus, we contribute views based on our simulation results [2].

- Discussion

According to RAN4#115 WF [1], the status is noted as below,

- Sub-topic 1 Test parameters and simulation assumptions DMRS configuration - Ratio of PUSCH EPRE to DM-RS EPRE - Online agreement: - Ratio of PUSCH EPRE to DM-RS EPRE - -3dB with Number of DM-RS CDM group(s) without data = 2 for all cells Propagation condition - Online agreement: - Based on the previous agreement, the TDLC300-100 is for all interference cells and ‘TDLA30-10 for HetNet and TDLC300-100 for HomNet’ is for the serving cell. MMSE-IRC receiver assumption - estimation of interference covariance matrix - RAN4 agree that MMSE-IRC receiver assumption is not going to be captured in the RAN4 spec - There is no consensus whether to modify the previously agreed MMSE-IRC receiver assumption. […] - Sub-topic 3 SNR requirement value SNR requirement value deriving rule - Online agreement: - Reuse the SNR requirement value deriving rule for Rel-15 BS demodulation SNR requirement derivation procedure as agreed in R4-1904713 Procedure to derive the performance requirements: – Only inputs that consist of a pair of ideal and impaired results can be taken into account. – If the ideal span <= [2]dB: • The AVERAGE impairment results can be used for the performance requirement with [] in the draftCRs/CRs; – Else if the ideal span is larger than [2]dB: • The results farthest from the AVERAGE value is taken out for the AVERAGE and SPAN re-calculation until the ideal span is <=2dB but still with at least 3 companies’ results available: – The ultimate AVERAGE impairment results with corresponding ideal span <=2dB can be used for performance requirement with [] in the draftCRs/CRs. • Otherwise put TBD for the related performance requirements. – If the span of the impairment results after removal the outliers (if any) are larger than 4dB, then the procedure cannot be applied, related performance requirement remain TBD.. Collection of simulation assumptions Companies are encouraged to bring both ideal and impairment simulation results for all the agreed test cases in the next meeting.

We performed the simulations as described in R4-2510371 [2], based on the simulation assumptions agreed in the previous meeting. We assumed a 2dB gap between the ideal SNR and the SNR with impairments. RAN4 needs to define the requirements for NR MMSE-IRC based on the simulation results submitted by companies.

Proposal 1: RAN4 needs to define the requirements for NR MMSE-IRC based on the simulation results submitted by companies.

- Conclusion

This contribution states demodulation requirements of NR BS MMSE-IRC. The summary follows.

Proposal 1: RAN4 needs to define the requirements for NR MMSE-IRC based on the simulation results submitted by companies.

References

- R4-2508618, Way Forward for [115][322] NR_demod_Ph5_Part1_General_BS, China Telecom, RAN4#115

- R4-2510371, Simulation results for NR MMSE-IRC BS demodulation, NTT DOCOMO, INC., RAN4#116