3GPP TSG-RAN WG4 Meeting #116	R4-2510374

Bengaluru, India, 25th – 29th August 2025

Source: 	Ericsson

Title:  	on RRM core requirements and test framework for beam management

Agenda Item:	7.17.4

Document for:	Discussion

# Introduction

In last meeting 3GPP TSG RAN WG4 Meeting #115, RAN4 discussed RRM requirements for AI/ML based beam management and the discussion summary is listed in [1]. Following the discussions, we herein in this contribution present our views and proposals on the remaining open issues relevant to core requirements.

# Discussion

## Core requirements

### Issue 2-1: Metrics/KPIs for beam prediction

The summary of this issue in RAN4 meeting #115 is presented as follows:

Issue 2-1: Metrics/KPIs for beam ID prediction Proposals - Option 1: adopt N - N=2 - Further study the actual value - N>2 - N= K2+1 - Option 2: Do not use N, remove the FFS on top-N - Option 3: x<3dB - Option 4: further study x based on simulations - Option 5: Others (e.g. different distributions, etc) - Recommended WF - Option 1/2 and 3/4 to be discussed separately

Meanwhile, recall the WF in RAN4 meeting #114bis as the below:

Agreement: - Metrics/KPIs for beam ID prediction, at least for the case if only beam ID is reported: The successful rate for the correct prediction, - The correct prediction is considered as maximum ground-truth RSRP among top-K predicted beams larger than or equal to the ground-truth RSRP of the strongest genie-aided beam(s) – x dB, - FFS on top-N (N</=K) predicted beams have to be considered in the success rate evaluation - FFS on N values - FFS on x values Note: if x=0 and N=1, it means Top-K/1 (%) : the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams"

Furthermore, recall the WF in RAN4 meeting #114 as the below:

Beam prediction KPIs Different metrics/KPIs have been discussed and were captured in the TR: For metrics for beam management requirements/tests, the following test metrics are identified and could be considered - Option 1: RSRP accuracy - Option 2: Beam prediction accuracy -Top-1 (%) : the percentage of "the Top-1 strongest beam is Top-1 predicted beam" -Top-K/1 (%) : the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams" -Top-1/K (%) : the percentage of "the Top-1 predicted beam is one of the Top-K strongest beams" - Option 3: The successful rate for the correct prediction which is considered as maximum RSRP among top-K predicted beams is larger than the RSRP of the strongest beam – x dB, -Related measurement accuracy can be considered to determine x - Option 4: combinations of above options Proposals - Option 1: - In case of the AI/ML model with label (beam ID) output type - Beam prediction accuracy, i.e., Option 2 + Option 3 is adopted. - In case of the AI/ML model with label (beam ID) + RSRP output type - Beam prediction accuracy and RSRP accuracy, i.e., Option1, 2 an 3, are adopted simultaneously. - Option 2: Option 2 with a combination of Top-K/1 and Top 1/K - Option 2a: update option 2 to Top-N/K, the percentage of "the Top-N genie-aided beam is one of the Top-K predicted beams". - Option 3: Option 3 as it does not have the issue of UE failing if there are multiple beams within a small RSRP margin - Option 4: combinations of options, concrete proposals should be disucssed - Recommended WF - To be discussed The number of options should be downselected to make some progress or a clear path to choose an option should be discussed/agreed

First of all, regarding the baseline metric for beam prediction:

"A correct prediction is considered valid if the maximum ground-truth RSRP among the top K predicted beams is greater than or equal to the ground truth RSRP of the strongest genie aided beam(s) minus x dB."

In our view, this metric evaluates beam prediction accuracy by comparing the ground truth RSRP of the predicted beams against that of the genie aided beams. While this approach provides an agreed reference metric, it raises two concerns:

- Concern 1: Determination of the parameter K

- Concern 2: Determination of the parameter x

Regarding the first concern, although the choice of K may depend on practical implementation, it is critical for assessing prediction correctness.

It is observed that the top-K beams are ranked in a specific order, even when L1-RSRP is not reported by the model-according to the agreement reached in the RAN1 #121 meeting as follows:

Agreement For UE-sided model, for BM-Case 1 and BM-Case 2, for content in the report of inference results, for Opt 1 (only beam information of predicted Top K beam(s)), the ranking information of the predicted Top K beams for K > 1 is conveyed by the order of the beam information.

However, the interpretation of the ranking information for the top-K beams remains unresolved in RAN1. One proposal suggests that the predicted top-K beams should be defined as the best K beams based on either the predicted RSRP values, or the probability of each beam in Set A being the top-1 beam.

Observation 1: For the case that only beam ID is reported, the order of top-K beams indicates the order of RSRP or the probability of each beam in Set A to be the Top-1 beam of implicitly.

Returning to the test metrics, the ranking information is not considered in the beam prediction metrics defined in RAN4. For the baseline beam prediction metric, the ground-truth RSRP of each beam among the top-K predicted beams must be evaluated to identify the single beam with the maximum ground-truth RSRP.

The following example illustrates the issue related to the choice of parameter K.

Assume K=2, with two reported beams ordered as the 1st (rank 1) beam and the 2nd (rank 2) beam. Among these, the ground-truth RSRP of the 2nd beam is higher and meet the test metrics.  Under the current metric definition, the 1st beam is not required to pass any validation in this case, e.g., the AI/ML model has a defect causing the 1st beam to have a significantly lower ground-truth RSRP than the 2nd beam, it still pass the evaluation.

This creates a risk that the network might select the 1st beam based on the UE’s top-K beam report, potentially leading to degraded performance due to suboptimal or poor beam selection. Furthermore, as K increases, the evaluation criterion becomes more ‘relaxed’, leading to greater variability and inconsistency in UE performance.

Some opposing views argue that since the 2nd beam meets the test metrics, there is no need to impose constraints on the 1st beam—considering it an acceptable error tolerance within the top-K beam report.

In our opinion, this approach can lead to degraded performance in the field. The NW may select one or more beams from the top-K list whose quality is not guaranteed, potentially resulting in incorrect configurations or commands. Even if the network performs subsequent measurement procedures to verify the predicted beam quality, such steps introduce inevitable delays—contradicting the design target of applying AI/ML model for beam management..

Furthermore, from the RAN4 perspective, requirements should remain deterministic. RAN4 must ensure that all UEs are evaluated against consistent performance criteria; otherwise, it becomes difficult for the network to manage configurations across plenty of UEs.

Yet, ensuring the accuracy of all beams reported by the UE is essential, whether determined through prediction or legacy measurement. If error tolerance were acceptable for all reported beams, there would be no need to define strict accuracy requirements for each beam. However, in the legacy measurement solution, measurement accuracy is still defined for every beam. Based on this principle, prediction accuracy should not be more relaxed than measurement accuracy.

Observation 2: Only considering the baseline:

- Top-K/1 (%), and

- The correct prediction is considered as maximum ground-truth RSRP among top-K predicted beams larger than or equal to the ground-truth RSRP of the strongest genie-aided beam(s) – x dB

may result in a wrong, e.g., bad quality, beam being adopted by NW, and results in degraded performance than the legacy measurement solution.

Below, we briefly express our views with respect to the enhancement proposals based on the baseline metric for beam prediction.

Approach 1:  Verifying both K=1 case and K1 case, for top K/1 (%) based on the baseline of the metrics for beam prediction. Given the above analysis in a straightforward and intuitive way, the K=1 case serves as a proper complement to the K > 1 case. Specifically, the K=1 case requires the UE being able to report the best (rank 1) beam with sufficient ground-truth RSRP, regardless of whether there exists another beam with higher ground-truth RSRP in the K > 1 case.

Approach 2:  Verify more than one beam, for example, N beams within the top-K predicted beams. The purpose of considering N beams here is to leverage the prediction accuracies of multiple predicted beams to enhance the baseline: ‘the beam with maximum ground-truth RSRP among top-K predicted beams’.  From high level perspective, the approach shall consider the below:

- The verification shall guarantee the ground-truth RSRP of the N beams not lower than a threshold (e.g., -x dB) with respect to the ground-truth RSRP of the top-K beams.

  - The exact formula can be studied if the general approach is agreed.

- Along with the first bullet, we’re open to apply Top-K/1 (%) or Top-K/N (%) together with ground-truth RSRP verification.

- Wherein, the N beams may have highest ground-truth RSRP or be highest ranked among top-K beams.

However, Approach 2 deviates somewhat from the baseline, as it introduces N, which could be regarded as a variant of the Top-K/N (%) method.

Observation 3: From the network perspective, obtaining a plurality of predicted beams, each subject to specific prediction accuracy requirements, is advantageous for enhancing reliability and robustness in beam management.

Proposal 1:  Enhance the baseline metric for beam prediction by testing both cases: K = 1 and K = [2], with the beam prediction metrics baseline as follow:

- ‘Top-K/1 (%)’, and

- ‘The correct prediction is considered as maximum ground-truth RSRP among top-K predicted beams larger than or equal to the ground-truth RSRP of the strongest genie-aided beam(s) – x dB’

Regarding the second concern, the parameter ‘x’ dB, which essentially represents a margin factor, it may be influenced by measurement errors as well as other prediction errors occurring during both training and inference phases. Therefore, it is expected that ‘x’ should be chosen to match or closely approximate the current measurement error. While TR 38.843 references 1 dB as a margin for simulations, this value remains subject to doubt.

Proposal 2: The parameter x in beam prediction accuracy metrics should be set to a value that reflects the expected magnitude of measurement error and other errors, [1] dB may be taken as a baseline.

Another issue concerns the SNR side condition in the test metrics.

Conventionally, the prerequisite for measurement accuracy requirements is the fulfillment of an SNR side condition. It is generally understood that all reference signals (RS) with SNR above this threshold are expected to meet consistent measurement accuracy criteria, regardless of the RS resource set configuration.

With the introduction of prediction, a key question arises: Should beams in Set A and/or Set B be required to meet certain SNR side conditions as a prerequisite for applying measurement accuracy requirements to the top‑K beams?

We observe the below variants:

- Option 1: If Set B beams meet the SNR side condition, then the top-K beams shall meet the measurement accuracy requirements.

- Option 2: If Set B beams meet the SNR side condition, then the top-K beams—which also meet the SNR side condition—shall meet the measurement accuracy requirements.

- Option 3: Both Set A and Set B beams shall meet the SNR side condition, after which the top-K beams shall meet the measurement accuracy requirements.

To our understanding, Both Set A and Set B beams shall meet the SNR side condition, no matter the beams are measured or predicted, in order to ensure the measurement accuracy requirements of the top-K beams.

Proposal 3: RAN4 to investigate the SNR side condition for prediction. Potential options are under consideration in the context of beam ID reporting:

- Option 1: If Set B beams meet the SNR side condition, then the top-K beams shall meet the measurement accuracy requirements.

- Option 2: If Set B beams meet the SNR side condition, then the top-K beams—which also meet the SNR side condition—shall meet the measurement accuracy requirements.

- Option 3: Both Set A and Set B beams shall meet the SNR side condition, after which the top-K beams shall meet the measurement accuracy requirements.

Among the options, Option 3 is preferred.

Furthermore, regarding AI/ML models supporting RSRP reporting, a key issue was raised in the last meeting: Should both RSRP and beam ID be subject to testing?

To our understanding, for AI/ML models that output both a label (i.e., beam ID) and RSRP, it is essential that test metrics account for both prediction accuracy and RSRP accuracy.

An objection to this approach may be that test metrics could follow the legacy method, where only RSRP is verified. However, this may be applicable only when testing a single beam or a very limited number of beams with deterministic beam ID/index information. In contrast, beam prediction test typically involves the UE reporting a set of beams, each predicted, indexed, and sorted in descending order of RSRP.

Using the top-1 predicted beam as an example, two accuracy requirements can be distinguished:

- RSRP accuracy: The difference between the predicted L1-RSRP and the ground-truth L1-RSRP of the top-1 predicted beam should be less than or equal to y dB.

- Beam accuracy: The difference between the ground-truth RSRP of the top-1 predicted beam and that of the top-1 genie-aided beam should be less than or equal to y dB.

If only RSRP accuracy is assessed —without any comparison to the genie-aided beam (i.e., the ideal beam determined with full knowledge)—there is no guarantee that the top-1 predicted beam is meaningfully related to the optimal beam since no comparison to genie-aided beam. In such case, a poorly performing prediction model could still satisfy RSRP accuracy requirements while selecting a beam that is far from optimal, undermining the purpose of accurate beam prediction.

Observation 4: For AI/ML model supporting RSRP and beam ID reporting, if only RSRP accuracy is required, an underlining problem is that the Top-1/top-K predicted beam is far away from the Top-1/top-K genie-aided beam since no comparison to genie-aided beam is performed.

Proposal 4: Metrics/KPIs for beam management requirements/tests are outlined by the below options:

- In case of the AI/ML model with label (beam ID) output type

  - Beam prediction accuracy is adopted.

- In case of the AI/ML model with label (beam ID) + RSRP output type

  - Beam prediction accuracy and RSRP accuracy, are adopted simultaneously.

Even so, It is predictable that adding RSRP accuracy to beam ID prediction demands higher model precision and robustness.

Observation 5: For the case of the AI/ML model with label (beam ID) + RSRP output type, fulfilling beam prediction accuracy and RSRP accuracy may result in more stringent requirements compared with the case of the AI/ML model only with label (beam ID) output type.

### Issue 2-2: Absolute RSRP accuracy

Agreement: - Absolute RSRP accuracy requirement applies to Top-1 of predicted beams at least - FFS whether to apply to other beams

Agreement in RAN4#112: - If the absolute RSRP is agreed to be metric for the beam management prediction, the absolute RSRP accuracy for AI/ML based beam prediction = predicted L1-RSRP of beam index i – groundtruth of L1-RSRP of beam index i. The index i may be any beam index [in top-K beams based on groundtruth L1-RSRP].

In the conventional measured L1-RSRP definition, only top-1 (largest measured value) of measured beams is reported in absolute RSRP format. The precise wording from TS 38.214, Section 5.2.1.4.3 is reproduced below.

For L1-RSRP reporting, if the higher layer parameter nrofReportedRS in CSI-ReportConfig is configured to be one, the reported L1-RSRP value is defined by a 7-bit value in the range [-140, -44] dBm with 1dB step size. If the higher layer parameter nrofReportedRS is configured to be larger than one, or if the higher layer parameter groupBasedBeamReporting is configured as 'enabled', the UE shall use differential L1-RSRP based reporting, where the largest measured value of L1-RSRP is quantized to a 7-bit value in the range [-140, -44] dBm with 1dB step size, and the differential L1-RSRP is quantized to a 4-bit value. The differential L1-RSRP value is computed with 2 dB step size with reference to the largest measured L1-RSRP value which is part of the same L1-RSRP reporting instance.

In a similar manner, one could agree that the absolute RSRP accuracy requirement applies only to the 1st beam in top-K predicted beam.

However, it’s worth noting that this approach defined by RAN1 optimizes the reporting by reducing the amount of data transmitted, especially when multiple beams are involved, which is served from RAN1 perspective. On the contrary, from the perspective of test robustness, we propose that the absolute RSRP accuracy requirement should apply to the other predicted beams besides of top-1 predicted beam.

Yet, based on the definitions in TS 38.133, as quoted below, RAN4 does not differentiate between absolute L1-RSRP accuracy and relative L1-RSRP accuracy for different SSB resource sets. All measurements are required to meet the absolute L1-RSRP accuracy requirement at a minimum.

##### 10.1.19.1 SSB based L1-RSRP accuracy requirements ##### Unless otherwise specified, the requirements for absolute and relative SSB based L1-RSRP accuracy in this clause apply to all SSBs of the serving cell configured for L1-RSRP measurement and all SSBs of cell(s) with different PCI from serving cell configured for L1-RSRP measurement in FR1.

##### 10.1.19.1.2 Relative Accuracy The relative SSB based L1-RSRP accuracy is defined as the L1-RSRP measured from one SSB compared to the largest measured value of L1-RSRP among all SSBs of the cell (serving cell or cell with different PCI from serving cell) on which UE performs L1-RSRP measurements.

Given all the above, we suggest to define the absolute RSRP accuracy requirement for each of Top-K beams.

Proposal 5: The absolute RSRP accuracy requirement applies to each of Top-K beams, not only the 1st beam in top-K predicted beam.

Yet there exists a leftover regarding the absolute RSRP accuracy with respect to the texts in the bracket in the agreement: ‘[in top-K beams based on ground truth L1-RSRP]’.

In our opinion, the focus of RAN4 should be on the predicted beams—that is, the prediction accuracy should concentrate on the beams reported by the UE, rather than those provided by the genie-aided method. It's questionable to adopt ‘the top-K beams w.r.t. the ground truth RSRP’. Two concerns are provided below:

- If the requirement is only relevant to the top-K beams with respect to the ground truth RSRP, does the beam's RSRP have no requirements if it is present in the top-K predicted beams but not in the genie beams?

- Conversely, why should we need the RSRP accuracy of a beam that the UE doesn't even report if it is present in the top-K genie beams but not in the predicted beams?

Proposal 6: For the absolute RSRP, the index i shall be any beam index in all beams based on predicted L1-RSRP as per UE reporting, i.e.,

- The absolute RSRP accuracy = predicted L1-RSRP of beam index i – groundtruth of L1-RSRP of beam index i. The index i may be any beam index in top-K beams based on prediction report.

### Issue 2-3: Relative RSRP accuracy

Agreement: - RAN4 to use the follow as baseline for further discussions, if the relative RSRP accuracy requirement is defined - Relative RSRP accuracy for reported beams during inference reporting = (predicted L1-RSRP of beam index i - reported L1-RSRP of beam index n) - (ground truth of L1-RSRP of beam index i - ground truth of L1-RSRP of beam index n), [where the beam index n owns the largest reported value]. - Reported L1-RSRP can be predicted, - FFS whether reported L1-RSRP can be measured RSRP - Relative RSRP accuracy requirements apply for Case 1, and FFS on whether it can apply for Case 2

Regarding the first FFS on ‘whether reported L1-RSRP can be measured RSRP’, the origin of the question comes from RAN1 study, which considered whether the UE could use measured RSRP instead of predicted RSRP in the prediction report, as outlined below:

Agreement For report content of inference results for UE-sided model for BM-Case 1, for the RSRP of predicted Top K beam(s) in the report of inference results, when applicable, further study the following options: - Option A: Predicted RSRP - Option B: Predicted RSRP, if the beam is not configured for corresponding measurement, and measured L1-RSRP if the beam is configured for corresponding measurement - Where the predicted RSRP is based on AI/ML output - Note: Support both Option A and Option B is not precluded. Working Assumption For report content of inference results for UE-sided model for BM-Case 2, the RSRP of predicted beam(s) in the report of inference results, is the predicted RSRP, where the predicted RSRP is based on AI/ML output

Later, in RAN1 #120, the following agreement was reached:

Agreement For report content of inference results for UE-sided model for BM-Case 1, the RSRP of predicted beam(s)in the report of inference results, is the predicted RSRP, where the predicted RSRP is based on AI/ML output.

Based on this, we understand that the reported L1-RSRP shall be the predicted L1-RSRP.

Observation 6: RAN1 has agreed that BM-Case 1, RSRP of predicted beam(s)in the report of inference results, is the predicted RSRP. It shall be also be valid for BM-Case 2.

Furthermore, from the RAN4 perspective, we understand that the evaluation of prediction should clearly separate predicted and measured data by design. This separation prevents any bias or overlap that could compromise the integrity of prediction performance assessment.

Proposal 7: Relative RSRP accuracy for reported beams during inference reporting = (predicted L1-RSRP of beam index i - predicted L1-RSRP of beam index n) -  (ground truth of L1-RSRP of beam index i - ground truth of L1-RSRP of beam index n), [where the beam index n owns the largest reported value].

Regarding the second FFS on ‘Relative RSRP accuracy requirements apply for Case 1, and FFS on whether it can apply for Case 2’, we do not identify any technical impediments to extending the relative RSRP accuracy requirements to Case 2. In our view, applying these requirements to Case 2 is both technically feasible and consistent with the principles already established for Case 1.

Besides, as a reference, RAN1 agreed to define relative RSRP for Case 1 and Case 2 both, which is cited as follows.

Agreement For UE-sided model inference, support the following report format (i.e., CSI field mapping order) for BM-Case1, for beam information on predicted Top K beam(s) among a set of beams and RSRP of predicted Top K beam(s) among a set of beams CRI or SSBRI #1. CRI or SSBRI #2. …. CRI or SSBRI #K. RSRP #1. differential RSRP #2. …. differential RSRP #K. - CRI or SSBRI #k is mapped to RSRP #k, where k = 1,2,…,K - RSRP #1 is absolute predicted RSRP - Differential RSRP #2~#K are differential predicted RSRP with reference to the largest predicted RSRP corresponding to CRI or SSBRI #1 Agreement For UE-sided model inference, support the following report format (i.e., CSI field mapping order) for BM-Case2, for beam information on predicted Top K beam(s) among a set of beams and RSRP of predicted Top K beam(s) among a set of beams Time instance indicator. CRI or SSBRI #1 of time instance #1. CRI or SSBRI #2 of time instance #1. …. CRI or SSBRI #K of time instance #1. CRI or SSBRI #1 of time instance #2. CRI or SSBRI #2 of time instance #2. …. CRI or SSBRI #K of time instance #2. …. CRI or SSBRI #1 of time instance #N. CRI or SSBRI #2 of time instance #N. …. CRI or SSBRI #K of time instance #N. RSRP #1 of time instance #1. Differential RSRP #2 of time instance #1. …. Differential RSRP #K of time instance #1. Differential RSRP #1 of time instance #2. Differential RSRP #2 of time instance #2. …. Differential RSRP #K of time instance #2. …. Differential RSRP #1 of time instance #N. Differential RSRP #2 of time instance #N. …. Differential RSRP #K of time instance #N. - Time instance indicator exist if N > 1 - The size of CSI field for time instance indicator is - The value of time instance indicator n (n≥0) corresponds to the (n+1)-th earliest time instance in the N time instances - Time instance #1 corresponds to the time instance indicated by the time instance indicator - Time instance #2~#N are mapped to the remaining N-1 time instance(s) other than time instance #1 based on the time domain order of the time instances - where time instance #2 is mapped to the earliest time instance from the N-1 time instance(s) - CRI or SSBRI #k is mapped to RSRP #k with the same time instance, where k = 1,2,…,K - RSRP #1 of time instance #1 is absolute RSRP; and the remaining RSRP are differential RSRP with reference to the largest predicted RSRP corresponding to CRI or SSBRI #1 of time instance #1

Proposal 8: Relative RSRP accuracy requirement is applicable for Case 1 and Case 2.

### Issue 2-4: Measurement period for inference

- Agreement: - Case 1: The starting point is to use M, N, P (reference the values from an existing clause in the same version of the spec where the measurement period is defined) - Case 2: postpone discussion until RAN1 concludes

In conventional way, measurement delay normally refers to Clause 9.5.4 of TS38.133, ‘When groupBasedBeamReporting-r17 is not configured, the UE shall be capable of performing L1-RSRP measurements based on the configured SSB resource for L1-RSRP computation, and the UE physical layer shall be capable of reporting L1-RSRP measured over the measurement period of TL1-RSRP_Measurement_Period_SSB.’. As a result, the requirement derives the minimal delay in which the UE shall be able to complete an entire L1-RSRP measurement while satisfying the accuracy.

The following outlines the delays in the prediction procedure.

The timeline concerning measurement and inference phases in prediction in Figure 1. Note: The variants illustrated in the figure are intended solely for interpretative demonstration.

Figure 1 Timeline of measurement and inference phase in prediction

It’s noted that BM case 1 and BM case 2 may have different set A and set B configurations. Also, the length of measurement/inference for BM case 1 and BM case 2 are typically different, e.g.,

- m, n for BM case 1 typically equal to 1, i.e., one-shot L1-RSRP measurement and one-shot inference.

- On the contrary,  m, n for BM case 2 typically are larger than 1, for tracking beam changes.

- Here, m and n denote the number of reference signal measurement occasions or predication occasions respectively. Normally, the exact number of m, n shall be configured by the NW or up to UE implementation/capability or predefined in specification with respect to RAN1 progress.

Regarding measurement delay, which may be defined by one of the below options:

- Option 1: Measurement delay to get a full L1-RSRP measurement result with qualified accuracy. Note: it is identical to measurement delay in legacy.

- Option 2: Measurement delay to get more than one L1-RSRP measurement result to track the L1-RSRP change.

In our opinion, Option 2, especially for BM case 2, typically are not intended to obtain an accumulated higher measurement accuracy. Instead, they mainly target facilitating beam characteristic analysis, which is typically a proprietary process managed by AI/ML model vendors, such as analyzing UE movements. It’s not in the scope of RAN4 conformance test.

Proposal 9: Regarding measurement period for prediction, for both BM-case1 and BM-case2, the legacy L1-RSRP measurement delay shall be applied (i.e., reusing M, N, P), with the assumption that the measurement accuracy remains consistent.

Proposal 10: For BM-case 2, RAN4 not to design a new measurement period for tracking changes of L1-RSRP by obtaining more than one L1-RSRP measurement result.

There may exist the other two delays, reporting delay and inference delay.

- The reporting delay may be the delay from the first/last measurement to sending prediction report.

  - As legacy, we don’t take reporting time into the delay definition, but we can define measurement reporting requirements, similar to the legacy ones, if needed.

- The inference delay, which may be defined as the delay from the first/last measurement to the first predicted reference signal occasion.

  - It may reflect the UE capability handling prediction procedure. RAN1 have concludes the time gap between the reference time and the first future time instance for prediction.

As reference, agreements in RAN1 #120 are duplicated as follow.

Agreement For inference, for BM-Case 2 of UE-side model, - The time gap between two consecutive future time instances is configured by RRC, and the number of future time instance(s) N is configured by RRC. - time gap is [10ms, 20ms, 40ms, 80ms, 160ms] - N = [1, 2, 4, 8] - Reference time of the earliest time instance for the predicted results is based on the most recent occasion of the CSI-RS/SSB resource in Set B for measurement - Where the most recent occasion of the CSI-RS/SSB resource of set B is the latest CSI-RS/SSB occasion no later than the corresponding CSI reference resource of the corresponding inference report.

And agreements in RAN1 #120bis are duplicated as follow.

For BM-Case 2 of UE-side model, one RRC parameter represents the time gap configured for between two consecutive future time instances, and also represents the time gap between the reference time and the first future time instance for prediction.

In our view, the time gap defined by RAN1 represents an interference delay between the completion of Set B measurements and the time at which Set A prediction is valid.

Observation 7: RAN1 has defined inference delay, i.e., time gap from completion of Set B measurements to the time at which Set A prediction is valid.

Proposal 11: For BM-case 2, RAN4 not to design inference delay, since time gap/delay from completion of Set B measurements until the ready of Set A prediction is configurable according to RAN1 agreements.

### Issue 2-5: Rx beam knowledge and TCI handling

Agreement: - if the predicted Tx beam in Set A is QCL Type-D to a known Tx beam, the corresponding Rx beam is known. - If the predicted Tx beam in Set A is not QCL Type-D to a known Tx beam, depending on UE capability, the corresponding Rx beam may be known or unknown. - Known Rx beam means no additional Rx beam sweeping is needed - Unknown Rx beam means additional Rx beam sweeping is needed. - Rx beam is known mean TCI state is known if conditions below are also met The following conditions related to whether the TCI state is known or unknown is defined as the starting point. Companies are invited to bring analysis. - The target TCI state is known when the following is met: - The TCI state switch command is received within 1280 ms upon the last transmission of the RS resource for beam reporting or measurement and the UE has sent at least one L-1 RSRP report of the target TCI state (clause 8.10.2 of 38.133) - Applies for case 1 - FFS for case 2 - Whether RS resource is only from set B should be clarified - FFS on whether prediction also impacts if a TCI state is known or unknown - Companies are invited to bring analysis on how prediction would impact whether a TCI state is known or unknown - FFS on the time delay if prediction is used as a condition for a known TCI state

To our understanding of the agreement, the QCL relationship to a known Tx beam isn’t the prerequisite of known Rx beam for the predicted beam. In other words, even no QCL to a known Tx beam, the UE is able to know the Rx beam of TCI state if the prediction beam is the RS in target TCI state or QCLed to the target TCI state.

Observation 8: Even no QCL to a known Tx beam, the UE is able to know the Rx beam of TCI state if the prediction beam is the RS in target TCI state or QCLed to the target TCI state.

In the last meeting, the following condition was regarded as the starting point, at least for BM-case1: ‘ The target TCI state is known when the following is met:

‘ The TCI state switch command is received within 1280 ms upon the last transmission of the RS resource for beam reporting or measurement and the UE has sent at least one L-1 RSRP report of the target TCI state (clause 8.10.2 of 38.133)’

It refers to the existing known conditions for TCI state as duplicated in the following:

The TCI state is known if the following conditions are met: - During the period from the last transmission of the RS resource used for the L1-RSRP measurement reporting for the target TCI state to the completion of active TCI state switch, where the RS resource for L1-RSRP measurement is the RS in target TCI state or QCLed to the target TCI state - TCI state switch command is received within 1280 ms of the last transmission of the RS resource for beam reporting or measurement - The UE has sent at least 1 L1-RSRP report for the target TCI state before the TCI state switch command - The TCI state remain detectable during the TCI state switching period in the SSB occasions available at the UE - The SSB associated with the TCI state remain detectable during the TCI switching period in the SSB occasions available at the UE - SNR of the TCI state is ≥ -3 dB Otherwise, the TCI state is unknown.

The agreement regarding the prediction procedure remains unclear. Considering the application of AI/ML models, if the known conditions for TCI state transition continue to rely solely on the measured RS, then the implementation of AI/ML models doesn’t bring no additional benefit.

Therefore, the criteria for the known conditions should be redefined to focus on the predicted RS rather than the measured RS. In this context, knowing the Rx beams is a crucial factor for enhancing TCI state switching through prediction.

Proposal 12: RAN4 to define the known conditions based on prediction, i.e., the known condition with respect to Set A.  the known conditions for Set B, since it is measured, follows the legacy way.

It’s observed that the known ‘predicted’ known conditions differ between BM-case 1 and BM-case 2:

For BM-case 1:

- The predicted Set A beams are output and reported by the UE contiguously upon receiving Set B beams. Consequently, the 1280 ms time window remains valid for predicted beams as well.

- The proposed “predicted” known conditions for BM-case 1 are as follows:

  - During the period from the last time instance of the RS resource for beam prediction for the target TCI state to the completion of active TCI state switch, where the RS resource for beam prediction is the RS in target TCI state or QCLed to the target TCI state

  - The TCI state switch command is received within 1280 ms upon the last time instance of the RS resource for beam prediction

  - The UE has sent at least 1 inference report for the target TCI state before the TCI state switch command

  - SNR of the TCI state is ≥ -3 dB

For BM-case 2:

- The prediction to the future time instances is defined with the prediction window. In the prediction window, the prediction remains valid, which has no relationship with 1280ms.

Observation 9: The known conditions based on prediction are different for BM-case 1 and BM-case 2.

Proposal 13: The known conditions for BM-case 1 based on prediction are proposed as follow.

- During the period from the last time instance of the RS resource for beam prediction for the target TCI state to the completion of active TCI state switch, where the RS resource for beam prediction is the RS in target TCI state or QCLed to the target TCI state

  - The TCI state switch command is received within 1280 ms upon the last time instance of the RS resource for beam prediction

  - The UE has sent at least 1 inference report for the target TCI state before the TCI state switch command

- TCI state and SSB of TCI state remain detectable during TCI state switching period

  - SNR of the TCI state is ≥ -3 dB

Proposal 14: The known conditions for BM-case 2 based on prediction are proposed as follow.

- During the period from the last time instance of the RS resource for beam prediction for the target TCI state to the completion of active TCI state switch, where the RS resource for beam prediction is the RS in target TCI state or QCLed to the target TCI state

  - The TCI state switch command is received within the valid prediction window of the RS resource for beam prediction

  - The UE has sent at least 1 inference report for the target TCI state before the TCI state switch command

- TCI state and SSB of TCI state remain detectable during TCI state switching period

  - SNR of the TCI state is ≥ -3 dB

Another important question that must be addressed is how to handle the following two legacy known conditions, which remain unclear:

- The TCI state remain detectable during the TCI state switching period in the SSB occasions available at the UE

- The SSB associated with the TCI state remain detectable during the TCI switching period in the SSB occasions available at the UE

Here, ‘detectable’ means the RS associated with the TCI state continues to transmit. From the UE’s perspective, if the UE relies on prediction instead of measurement (as in BM-case 1 and BM-case 2), it may not be necessary to request the network to continue RS transmission.

Observation 10: In prediction, the TCI state or The SSB associated with the TCI state may not keep transmitting during the TCI state switching period.

Proposal 15: The term “detectable” in the condition “The TCI state and the SSB of the TCI state remain detectable during the TCI state switching period” should be updated to reflect prediction procedures, in which the predicted RS may not be transmitted.

- Example: Replace “detectable” with “predictable.”

### Issue 2-6: Performance monitoring

Issue 2-7: LCM Requirements - Proposals - Option 1: the delay components of LCM operations should be considered for different CSI-report configurations (i) periodic CSI report (ii) semi-persistent CSI report and (iii) aperiodic CSI report for both BM-Case1 and BM-Case2. - Option 2: For performance monitoring for BM, the corresponding delay is defined as - Tmonitoring_period_BM = Nmonitor* TL1-RSRP_Prediction_Period_CSI-RS, - Where Nmonitor is the configured number of the latest transmission occasion(s) of monitoring resources with linked inference report no later than CSI reference resource corresponding to the CSI report for monitoring - Option 3:postpone discussion until RAN1/2 monitoring framework is clear

In RAN2#126, the following agreements are achieved on LCM for UE-sided model:

Agreements 1 RAN2 will support functionality activation/deactivation after inference configuration. FFS initial state of configuration and how activation/deactivation is achieved. FFS what Deactivation refers to: examples discussed: 1) fallback to legacy 2) switching, etc. 2 We will work offline on the definitions for functionality types and define what is availability. 3 The UE will indicate the gNB/LMF whether the AI/ML functionality is available/applicable. For a functionality to be applicable at least there should at least one model available within it. FFS other details on what is applicability/non-applicability. 4 For NW-side additional conditions, RAN2 assumes that RRC signaling from gNB to UE can be designed for consistency between inference and training. RAN2 will wait for RAN1 input for further details. FFS if the same applies to positioning 5 For BM use case, As a baseline the UE determines whether a functionality is applicable. Existing UAI framework is used at least for proactive reporting of applicable functionality. FFS reactive

In RAN4#115, below agreements were achieved as duplicated in the below:

Agreements: Activation delay: - Activation delay requirements created for the periodic, semi-persistent and aperiodic CSI reporting for beam prediction - It is FFS for CSI prediction - For Option A, RAN4 will send LS to RAN2 to clarify the related mechanism. - For semi-persistent CSI reporting - Activation delay starts at the reception of the MAC-CE/DCI - For aperiodic CSI reporting - Activation delay starts at the reception of the DCI Switching delay and Fallback delay: - Deprioritize the discussion on switching and fallback delay requirements and focus on activation/deactivation delay requirement, including both AI/nonAI, discussion. Monitoring reporting delay and accuracy: Discuss how to define delay and accuracy requirements for UE sided monitoring where the UE reports the metric: - BM-Case1 and possibly BM-Case 2 o Performance monitoring type 1 option 2 - If it proves to be not possible to define, then reverse the agreement - Note: For BM-Case2, it may not be completed in Rel-19. If it is not covered in Rel-19, there would not be a monitoring delay requirement. If it would be in Rel-19, a monitoring delay requirement is needed.

From the above agreements, the LCM operations for AI/ML BM, e.g., activation, deactivation could be supported by CSI report configurationg.By reusing the existing capability framework, NW can configure CSI report configurations to manage LCM operations.

For both BM-case 1 and BM-case 2, CSI reports can be based on periodic, semi-persistent, or aperiodic CSI reporting schemes.

Upon receiving such configurations, the UE should be able to activate or deactivate the AI/ML model in a timely manner. But only ativation delay is prioritized.

The LCM-related activation may comprise the below paramters:

- Processing time, including the time for UE readying AI/ML mode and signal processing time from the reception of signaling/command, e.g., TRRC_processing , THARQ  or time required by the UE to perform PDCCH reception.

- prediction delay, including measurement and inference

- CSI reporting delay, including the time waiting for CSI reporting containing interference report

In legacy, CSI reporting delay isn’t included in L1-RSRP measurement delay, as duplicated in the below:

When groupBasedBeamReporting-r17 is not configured, the UE shall be capable of performing L1-RSRP measurements based on the configured SSB resource for L1-RSRP computation, and the UE physical layer shall be capable of reporting L1-RSRP measured over the measurement period of TL1-RSRP_Measurement_Period_SSB.

The same approach can be applied to the prediction process.

Proposal 16: RAN4 to study activation delay requirements for LCM operations, with respect to to different CSI-report configurations (i) periodic CSI report (ii) semi-persistent CSI report and (iii) aperiodic CSI report for both BM-Case1 and BM-Case2.

Proposal 17: Activation delay requirements for periodic CSI report shall comprise the time delays:

- Processing time, from sending RRCReconfigurationComplete ( if the inference configuration is provided in Step 3), including the time for UE readying AI/ML mode and signal processing time e.g., TRRC_processing.

- prediction delay, covering the time needed for measurement acquisition and preparation of the AI/ML inference.

Proposal 18: Activation delay requirements for semi-persistent and aperiodic CSI report shall comprise the time delays:

- Processing time, from MAC-CE/DCI, including the time for UE readying AI/ML mode and signal processing time, e.g., THARQ  or time required by the UE to perform PDCCH reception.

- prediction delay, covering the time needed for measurement acquisition and preparation of the AI/ML inference.

The second issue appears to be more closely related to performance monitoring rather. Based on RAN1’s agreements, as cited in the below, we may focus on Type 1, Option 2 of UE-side model monitoring.

Agreement For BM-Case1 and BM-Case2 with a UE-side AI/ML model: - Support Type 1 performance monitoring, including the following two options: - Option 1 (NW-side performance monitoring): - UE sends a report to NW (for the calculation of performance metric at NW) - Measurement results from resource set for monitoring, e.g., L1-RSRP and/or RS index is supported as the content of the report - FFS on other contents - The report is at least configured/triggered by NW - Note: this may or may not have additional spec impact - Option 2 (UE-assisted performance monitoring): - UE calculates performance metric(s) - FFS how to report and what to report - FFS whether to trigger the report based on event(s) for Option 1 and/or Option 2 - FFS Type 2 performance monitoring

In RAN WG1 #118, the below agreements were achieved:

Agreement For BM-Case1 and BM-Case2 with a UE-sided AI/ML model, for Option 2 (UE-assisted performance monitoring), further study at least the following alternatives, including: - Alt 1: Top 1 or Top K beam prediction accuracy (with or without margin) by comparing the prediction results and the Top 1 or Top K beam based on the measurements from a resource set/resources for monitoring - Alt 2: The L1-RSRP difference information based on actual measurement of the L1-RSRP of one or more of Top K predicted beam, and L1-RSRP measurements from a resource set/resources for monitoring - Alt 3: The RSRP difference information between the predicted RSRP and measured L1-RSRP of corresponding beam(s) of a resource set/resources for monitoring - Note: resources for Set B for monitoring are not precluded and can be study. - Note: this is only applicable when the model can predict RSRP - Alt 4: The probability information of the predicted beam(s) to be the Top 1 or Top K beam - Note: this is only applicable when the model can generate probability information - FFS: for Alt 1/2/3, on other details including how to configure the resource set/resources for monitoring, including - E.g. whether/how to use full set of Set A for measurement. If not, whether/how to obtain the measurement of the predicted Top 1 or Top K beam for calculating the prediction accuracy or the RSRP difference. - For all alternatives, study whether the performance information is calculated per sample (one-shot), or per set of samples (window)

In RAN WG1 #118bis, the below agreements were achieved.

Agreement For BM-Case1 and BM-Case2 with a UE-sided AI/ML model, for Option 2 (UE-assisted performance monitoring), - At least support Alt 1: Top 1 or Top K beam prediction accuracy (with or without margin) by comparing the prediction results and the Top 1 or Top K beam based on the measurements from a resource set/ resources for monitoring - FFS on detail definition of the metric, including whether/how to configure or define a window for calculation - FFS: on other details including how to configure the resource set/resources for monitoring, including - E.g. whether/how to use full set of Set A for measurement. If the full set A is not configured, whether/how to define the metric - FFS other alternatives

At least for the monitoring Type 1 Option 2 of UE-side model monitoring (when applicable), consider the following options with potential down selection for the configuration for monitoring: - Option 1: The resource set(s) for monitoring and report configuration for monitoring are configured (when applicable) within CSI report configuration used for inference - FFS: the resource set(s) for monitoring - UE measures the resource set(s) for monitoring. - FFS how/when to report the monitoring results. - Option 2: Dedicated resource set(s) for monitoring and report configuration for monitoring are configured in a dedicated CSI report configuration used for monitoring - The dedicated report configuration used for monitoring links to an inference report configuration - FFS how to identify the connection between RSs in the resource set(s) for monitoring and Set A beams - UE measures the resource set(s) for monitoring. - FFS when to report the monitoring results.

In RAN WG1 #119, the below agreements were achieved:

Agreement At least for the monitoring Type 1 Option 2 of UE-side model monitoring (when applicable), support to reuse CSI framework for the configuration for monitoring result report in L1 signaling: - Dedicated resource set(s) for monitoring and report configuration for monitoring are configured in a dedicated CSI report configuration used for monitoring - The ID of an inference report configuration is configured in the configuration for monitoring to link the inference report configuration and monitoring report configuration - FFS how to identify the connection between RSs in the resource set(s) for monitoring and Set A beams - FFS on whether to support all the combination on time domain behavior of the reportConfigType for infernece report and the reportConfigType for monitoring report - FFS on the timing related issues - UE measures the dedicated resource set(s) for monitoring.

Our understanding is that, for both BM‑case 1 and BM‑case 2, when a UE‑side AI/ML model is used under Option 2 (UE‑assisted performance monitoring), the UE should be capable of comparing the differences between the prediction results and the measurement results over a designated resource set or specific resources used for monitoring.

And further relevant agreements in RAN #121 defines the timeline of monitoring for BM-Case 1:

Agreement - At least for the monitoring Type 1 Option 2 of UE-side model monitoring, for calculation of metric for monitoring, - for BM-Case 1, the measurement result of nth (n = 1,..,N) latest transmission occasion of the CSI-RS/SSB resources for monitoring is linked with an inference report, where the CSI reference resource of the corresponding inference report has the minimal slot offset to the nth transmission occasion of the CSI-RS/SSB resources for monitoring. - Wherein, the corresponding inference report, and the transmission occasion of the CSI-RS/SSB resources for monitoring are no later than the CSI reference resource corresponding to the CSI report for monitoring - Predefines a threshold X = 64 for the minimal slot offset, which is configured by RRC, where the minimal slot offset k is no larger than X; otherwise, the transmission occasion for monitoring has no linked inference report. - Note: CSI reference resource corresponding to the CSI report for monitoring and inference is determined based on legacy for all types (P/SP/AP) of CSI report carrying L1-RSRP, considering discussing associated timeline separately - The associated working assumption made in RAN1#120b will not be confirmed.

Agreement For beam prediction accuracy report for monitoring, the report quantity RS-PAI is Np (0 ≤ Np≤ N) - Where Np is the total count of accurate reference signal prediction instance(s) that meets the condition, among N latest transmission occasion(s) of monitoring resources that no later than CSI reference resource corresponding to the CSI report for monitoring - condition: - for the transmission occasion of monitoring resources, it has a linked inference report - at least one of the nrofBestBeamforMonitoring-r19 identified CSI-RS resources, or SS/PBCH Block resources mapped to one of the nrofreportedpredictedrs-r19 reported P-CRI(s) or P-SSBRI(s), of the linked report of the CSI Reporting Setting for inference - if this condition is met, the transmission occasion is counted as an accurate reference signal prediction instance; otherwise, it is not counted as an accurate reference signal prediction instance. - Where N = 1, 3, 7, 15 is configured in CSI-ReportConfig - the size of CSI field associated with the RS-PAI is log2(N+1)

Based on RAN1 agreements to date, we have the below observations.

- The performance monitoring will be considered for different CSI-report configurations: (i) periodic CSI report (ii) semi-persistent CSI report and (iii) aperiodic CSI report for both BM-Case1 and BM-Case2. RAN1 will continue the discussions on the associated timeline separately.

- Measurement result on each RS occasions will be reported, there are total N measurement results.

- Report quantity RS-PAI is Np is defined as the total count of accurate reference signal prediction instance(s) that meets the condition, among N latest transmission occasion(s).

Proposal 19: RAN4 to define prediction monitoring requirements for performance monitoring Type 1 Option 2 (UE-assisted performance monitoring)

Proposal 20: RAN4 shall take the below RAN1 agreements into performance monitoring requirements:

- Evaluation period for performance monitoring:

  - N transmission occasion of the CSI-RS/SSB resources for monitoring, at least for BM-case 1, Where N = 1, 3, 7, 15 is configured in CSI-ReportConfig.

- Report quantity RS-PAI,

  - Np is defined as the total count of accurate reference signal prediction instance(s) that meets the condition, among N latest transmission occasion(s).

### Issue 2-7:	Simulation results

Impact to other requirements Use of beam prediction could impact other RRM requirements which rely on measurement reports. Possible impact to CBD was brought up in one of the contributions. RAN4 could discuss how to handle such cases Issue 2-6: Simulation results - Proposals - Option 1: RAN4 to further analyze which requirements could be impacted. Companies to bring analysis until next meeting on impacted requirements and potential changes. - Option 2: RAN4 to only discuss the impact to CBD evaluation period: - RAN4 should consider the impact in RAN4 requirements for the Evaluation period due to the presence of predicted beams - Option 3: RAN4 does need to discuss such impact, the changes would implicitly apply by assume a predicted beam as measured/reported - Option 4: There is no impact, this is not in scope - Option 5: others - Recommended WF - To be discussed

Until now, the beam management procedures, e.g., BFD, RLM and CBD, have not been studied by RAN1/2. Therefore, it would not be reasonable to initiate a study on the CBD evaluation period at this stage.

Proposal 21: RAN4 not to study BFD, RLM, or CBD until clear solutions have been defined and agreed upon in RAN1/2.

## Testability and interoperability

### Issue 4-1: Baseline test setup

- Proposals - Option 1: Single AoA setup (different Tx beams emulated only with power difference?) - Is it possible to test if UE can also make a prediction when set B beams come from different directions? - How many Tx beams can be emulated when the spatial domain properties are lost? - Will UE need just a simple interpolation model or is a more complex model necessary to pass such a test? - Can additional (L1-)RSRP measurement accuracy requirement (e.g. spherical coverage) be used to complement Single AoA testing with directional/spatial UE performance? - Option 2: Single AoA setup with “two stage approach” - Will a UE which uses Rx beam information in the prediction model have to also implement a model which does not use the Rx beam information? - Are there any implementation constraints on a UE to execute/pass this test? - How will the TE know the UE Rx beam pattern? - Option 3: multi AoA setup– RRM test setup as baseline (including enhancement for up to 4AoA active probes) - How many Tx beams can be emulated with such a setup, what is the relationship between number of probes and number of set B Tx beams? - Can AoAs between the probes also be emulated (is it possible to “make it look like” signals are coming from other angles than where the probes are placed?) - Can a simplified CDL be emulated or only TDL from each probe? - Option 4: multi AoA setup – 3D MPAC system as baseline - How many Tx beams can be emulated with the baseline setup? - 4Tx beams with dual pol(set B?) can be emulated with 2 additional probes Pros&cons of different test setups - Option 5: multi AoA setup – evaluation of already feasible systems for AI/ML BM testing suitability - AI/ML BM performance centric, not CDL channel emulation centric. - Define test system candidates which are already feasible - Perform system level simulations including the probe layout of candidate systems to address the impact on the UE AI ML BM performance. Single AoA; Single AoA with two stage; Multi AoA – RRM test setup as baseline; Multi AoA 3D MPAC. Tests that UE uses spatial information(UE actually makes measurement in multiple directions to make a prediction); No; Maybe?; Yes; Yes. Relevance for field performance; Low; Maybe?; Good/High; Good/High. Simplified CDL emulation; No; Yes; Yes. Test setup complexity; Low; Low; Medium; Medium/High?. Test method complexity; Low; High; Medium; Medium/High?. Reuse of test setup for other tests; Medium (multi AoA testing already exists in RRM); High?; Low/medium? Can only reuse the test setup for FR2 MIMO OTA?. - Recommended WF - Do not further discuss single AoA test setups, continue to study the two multi AoA candidate setups

Agreement: - Discuss both single AoA based and multiple AoA based set up - Continue to study the two multiple AoA candidate setups - One option to continue the discussion proposed as following: - Multi AoA setup – evaluation of already feasible systems for AI/ML BM testing suitability - AI/ML BM performance centric, not CDL channel emulation centric. - Define test system candidates which are already feasible - Perform system level simulations including the probe layout of candidate systems to address the impact on the UE AI ML BM performance. - Start with the set up with Set A = 32 beams, Set B = 8 beams, and develop the test procedures with single AoA and multiple AoAs

### Issue 4-2: Channel model

- Proposals - Option 1: simplified CDL - Option 2: TDL - Option 3: others - Recommended WF - To be discussed If simplified CDL is to be further considered, more detailed proposals on what to study/analyze further should be discussed (e.g. RX PAS, number of relevant clusters to emulate, dependency on number of clusters and number of probes in the chamber, etc)

Agreement: - Use CDL-based channel model as starting point

Regarding Issue 4-1 and Issue 4-2, from AI/ML model testing perspective, it’s clear that there exist two options in last meetings.

- Option 1:  only inputting L1-RSRP of beams, single AoA with CDL/TDL channel can be applied.

- Option 2:  inputting beam information comprising L1-RSRP, Tx beam spatial distribution and Rx beam spatial distribution, multiple AOA with CDL channel shall be applied.

Option1 supposes that by emulating artificial Tx/Rx beam into the incoming beams, the single AoA test configuration can generate the needed L1-RSRP of beams.

Figure 2 Single AOA flow

However the practical Rx beam plays a crucial role in FR2 UE performance, a question to Option 1 is whether the difference between really tested Tx/Rx beam and the artificial Tx/Rx beam can impact AI/ML model.

Proposal 22: RAN4 to study the two questions regarding multiple/single AoA:

- What is the (maximal) difference between really tested Tx/Rx beam and the artificial Tx/Rx beam?

- If the difference can impact AI/ML model and the prediction accuracy?

The simplification of the CDL channel is achieved by constraining the directions Tx beams and and AOA/AOD. Observations regarding possible approaches to simplifying AOA/AOD can be found in R4-2500825, R4-2411254, and other relevant contributions. The simplified CDL channel consolidates similar AOA/AOD angles to enable spatial evaluation with a reduced number of probes.

Given that the maximal number of beams to be measured is 32, a important consideration is whether the 32 beams after simplified CDL channel can be produced by 8 or 12 probes thereby achieving the balance between performance and implementation complexity.

Proposal 23: RAN4 to check if the 32 beams after simplified CDL channel can be produced by [8] (if it is feasible to be realized in test bed) probes which may achieve the balance between performance and implementation complexity.

A test solution involving the measurement of different Tx/Rx beams using a time-division multiplexing (TDM) approach was proposed in previous meetings. While we remain open to this approach, the method for controlling Rx switching requires further clarification.

Proposal 24: Measuring Tx/Rx beams using a TDM approach may be a valid method; however, the control mechanism for Rx switching must be explicitly addressed.

The simplified CDL channel can, in principle, be applied to both training and inference. However, within the scope of common test configurations, only inference needs to be specified, as training is not part of the conformance test. Consequently, a comparison between the simplified CDL channel and the reference CDL channel during training is not required. In other words, training data may be generated using the reference CDL channel.

Proposal 25: The simplified CDL channel only is considered to be applied in inference, no need to be applied in training.

Proposal 26: The metric to estimate the “goodness” of a simplified CDL channel model may be represented by the below steps:

- Step 1: Test (inference) with the reference CDL channel.

- Step 2: Test (inference) with the simplified CDL channel.

- Step 3: The prediction difference between the results of the two tests in the above steps is less than a threshold with [95] percentile.

### Issue 4-3: Beam prediction testing process

- Proposals - Option 1: Test needs to have at least 2 parts: - 1. inference part of the test (UE has to make the actual prediction) - 2. predicted beam measurement (UE has to measure and report “the ground truth”) - Option 2: others - Recommended WF - Take Option 1 as baseline, discuss further details of tests such as under what condition should the ground truth be measured, how to ensure the UE does not cheat, etc.

If the ideal measurement of RSRP is acquirable, then the test procedure shall at least comprise the below three steps:

- Acquire the predicted RSRP under the required corresponding SNR condition

- Acquire the ideal measurement of RSRP under high SNR condition

- Compare the two RSRPs acquired in the above steps

Still, arranging the details on the sequence and test configuration may need further investigation. In essence, to our understanding, the UE is deemed to execute the RSRP measurement and prediction with same test configurations except for different SNR conditions.

Proposal 27: Test procedure shall at least comprise the below three steps:

- Acquire the predicted RSRP under the required corresponding SNR condition

- Acquire the ideal measurement of RSRP under high SNR condition

- Compare the two RSRPs acquired in the above steps

Note 1: Test configurations are kept same in the steps except for different SNR conditions.

Note 2: Arranging the details on the sequence and test configuration may need further investigation in performance part.

### Issue 4-4: OTA Test setup for beam management

In this issue, we discuss the setup of channels to be applied in measurement and inference.

For BM-case1, which predicts the best beams in Set A (different from Set B) with respect to the measurements on Set B, at the same time domain with respect to spatial correlation between Set B and Set A. Since the time delay of measurement delay and inference delay is in dozens or hundreds of milli-seconds order, there’s no chance to rearrange random distributions and variations between measurement and prediction, the channel for measurement and prediction have to apply a same channel.

For BM-case2, which harnesses the historical predictable non-random variation of Set B to anticipate Set A (same as Set B) for one or more future time instances with respect to temporal correlation between Set B and Set A, from test perspective, it needs some trackable changes, e.g., UE monotonous movements which reflect the temporal correlation as prerequisite of the temporal prediction.

A question is relevant to measurement window and the inference window, which are configured by NW, may have respective minimal length with respect to a certain prediction performance. Such that a set of consequent changes of channel parameters shall be designed during measurement window and the inference window. At the least, establishing a set of channel instants from channel #a+1 to channel #a+m for measurement window and channel #b+1 to channel #b+n for inference window.

These channels contain a set of parameters which update sequentially and gladfully starting from measurement until end of inference. The parameters wherein to be updated may comprise one or more than one of the below:

- Departure angles (AOD, ZOD)

- Arrival angles (AOA, ZOA)

- UE position and/or rotation

- Channel coefficient

In an illuminative example, for BM-case 2, the updates of parameters, in order to mimic the UE’s movement, may include the below:

- AOD in channel #1 is 10 degrees,

- AOD in channel #2 is 20 degrees,

- …,

- AOD in channel #k is (10*k) degree

Proposal 28: For BM-case1, same channel is applied for measurement and inference.

Proposal 29: For BM-case2, plural channel instants with a series of changes of the channel parameters are necessary to reflect temporal correlation between channels. At least, it includes:

- Generating a set of channel instants, from channel #a+1 to channel #a+m, for measurement, and

- Generating a set of channel instants, from channel #b+1 to channel #b+n for inference

Where,

- The channels instants comprise sequent and grade updates of a set of parameters.

- The numbers of m and n depend on the measurement window and prediction window of BM-case2.

- The channel parameters may comprise one or more than one of

  - Departure angles (AOD, ZOD)

  - Arrival angles (AOA, ZOA)

  - UE position and/or rotation

  - Channel coefficient

# Conclusion

Observation 1: For the case that only beam ID is reported, the order of top-K beams indicates the order of RSRP or the probability of each beam in Set A to be the Top-1 beam of implicitly.

Observation 2: Only considering the baseline:

- Top-K/1 (%), and

- The correct prediction is considered as maximum ground-truth RSRP among top-K predicted beams larger than or equal to the ground-truth RSRP of the strongest genie-aided beam(s) – x dB

may result in a wrong, e.g., bad quality, beam being adopted by NW, and results in degraded performance than the legacy measurement solution.

Observation 3: From the network perspective, obtaining a plurality of predicted beams, each subject to specific prediction accuracy requirements, is advantageous for enhancing reliability and robustness in beam management.

Observation 4: For AI/ML model supporting RSRP and beam ID reporting, if only RSRP accuracy is required, an underlining problem is that the Top-1/top-K predicted beam is far away from the Top-1/top-K genie-aided beam since no comparison to genie-aided beam is performed.

Observation 5: For the case of the AI/ML model with label (beam ID) + RSRP output type, fulfilling beam prediction accuracy and RSRP accuracy may result in more stringent requirements compared with the case of the AI/ML model only with label (beam ID) output type.

Observation 6: RAN1 has agreed that BM-Case 1, RSRP of predicted beam(s)in the report of inference results, is the predicted RSRP. It shall be also be valid for BM-Case 2.

Observation 7: RAN1 has defined inference delay, i.e., time gap from completion of Set B measurements to the time at which Set A prediction is valid.

Observation 8: Even no QCL to a known Tx beam, the UE is able to know the Rx beam of TCI state if the prediction beam is the RS in target TCI state or QCLed to the target TCI state.

Observation 9: The known conditions based on prediction are different for BM-case 1 and BM-case 2.

Observation 10: In prediction, the TCI state or The SSB associated with the TCI state may not keep transmitting during the TCI state switching period.

Proposal 1: Enhance the baseline metric for beam prediction by testing both cases: K = 1 and K = [2], with the beam prediction metrics baseline as follow:

- ‘Top-K/1 (%)’, and

- ‘The correct prediction is considered as maximum ground-truth RSRP among top-K predicted beams larger than or equal to the ground-truth RSRP of the strongest genie-aided beam(s) – x dB’

Proposal 2: The parameter x in beam prediction accuracy metrics should be set to a value that reflects the expected magnitude of measurement error and other errors, [1] dB may be taken as a baseline.

Proposal 3: RAN4 to investigate the SNR side condition for prediction. Potential options are under consideration in the context of beam ID reporting:

- Option 1: If Set B beams meet the SNR side condition, then the top-K beams shall meet the measurement accuracy requirements.

- Option 2: If Set B beams meet the SNR side condition, then the top-K beams—which also meet the SNR side condition—shall meet the measurement accuracy requirements.

- Option 3: Both Set A and Set B beams shall meet the SNR side condition, after which the top-K beams shall meet the measurement accuracy requirements.

Among the options, Option 3 is preferred.

Proposal 4: Metrics/KPIs for beam management requirements/tests are outlined by the below options:

- In case of the AI/ML model with label (beam ID) output type

  - Beam prediction accuracy is adopted.

- In case of the AI/ML model with label (beam ID) + RSRP output type

  - Beam prediction accuracy and RSRP accuracy, are adopted simultaneously.

Proposal5: The absolute RSRP accuracy requirement applies to each of Top-K beams, not only the 1st beam in top-K predicted beam.

Proposal 6: For the absolute RSRP, the index i shall be any beam index in all beams based on predicted L1-RSRP as per UE reporting, i.e.,

- The absolute RSRP accuracy = predicted L1-RSRP of beam index i – groundtruth of L1-RSRP of beam index i. The index i may be any beam index in top-K beams based on prediction report.

Proposal 7: Relative RSRP accuracy for reported beams during inference reporting = (predicted L1-RSRP of beam index i - predicted L1-RSRP of beam index n) -  (ground truth of L1-RSRP of beam index i - ground truth of L1-RSRP of beam index n), [where the beam index n owns the largest reported value].

Proposal 8: Relative RSRP accuracy requirement is applicable for Case 1 and Case 2.

Proposal 9: Regarding measurement period for prediction, for both BM-case1 and BM-case2, the legacy L1-RSRP measurement delay shall be applied (i.e., reusing M, N, P), with the assumption that the measurement accuracy remains consistent.

Proposal 10: For BM-case 2, RAN4 not to design a new measurement period for tracking changes of L1-RSRP by obtaining more than one L1-RSRP measurement result.

Proposal 11: For BM-case 2, RAN4 not to design inference delay, since time gap/delay from completion of Set B measurements until the ready of Set A prediction is configurable according to RAN1 agreements.

Proposal 12: RAN4 to define the known conditions based on prediction, i.e., the known condition with respect to Set A.  the known conditions for Set B, since it is measured, follows the legacy way.

Proposal 13: The known conditions for BM-case 1 based on prediction are proposed as follow.

- During the period from the last time instance of the RS resource for beam prediction for the target TCI state to the completion of active TCI state switch, where the RS resource for beam prediction is the RS in target TCI state or QCLed to the target TCI state

Proposal 14: The known conditions for BM-case 2 based on prediction are proposed as follow.

- During the period from the last time instance of the RS resource for beam prediction for the target TCI state to the completion of active TCI state switch, where the RS resource for beam prediction is the RS in target TCI state or QCLed to the target TCI state

  - The TCI state switch command is received within the valid prediction window of the RS resource for beam prediction

  - The UE has sent at least 1 inference report for the target TCI state before the TCI state switch command

- TCI state and SSB of TCI state remain detectable during TCI state switching period

  - SNR of the TCI state is ≥ -3 dB

Proposal 15: The term “detectable” in the condition “The TCI state and the SSB of the TCI state remain detectable during the TCI state switching period” should be updated to reflect prediction procedures, in which the predicted RS may not be transmitted.

- Example: Replace “detectable” with “predictable.”

Proposal 16: RAN4 to study activation delay requirements for LCM operations, with respect to to different CSI-report configurations (i) periodic CSI report (ii) semi-persistent CSI report and (iii) aperiodic CSI report for both BM-Case1 and BM-Case2.

Proposal 17: Activation delay requirements for periodic CSI report shall comprise the time delays:

- Processing time, from sending RRCReconfigurationComplete ( if the inference configuration is provided in Step 3), including the time for UE readying AI/ML mode and signal processing time e.g., TRRC_processing.

- prediction delay, covering the time needed for measurement acquisition and preparation of the AI/ML inference.

Proposal 18: Activation delay requirements for semi-persistent and aperiodic CSI report shall comprise the time delays:

- Processing time, from MAC-CE/DCI, including the time for UE readying AI/ML mode and signal processing time, e.g., THARQ  or time required by the UE to perform PDCCH reception.

- prediction delay, covering the time needed for measurement acquisition and preparation of the AI/ML inference.

Proposal 19: RAN4 to define prediction monitoring requirements for performance monitoring Type 1 Option 2 (UE-assisted performance monitoring)

Proposal 20: RAN4 shall take the below RAN1 agreements into performance monitoring requirements:

- Evaluation period for performance monitoring:

  - N transmission occasion of the CSI-RS/SSB resources for monitoring, at least for BM-case 1, Where N = 1, 3, 7, 15 is configured in CSI-ReportConfig.

- Report quantity RS-PAI,

  - Np is defined as the total count of accurate reference signal prediction instance(s) that meets the condition, among N latest transmission occasion(s).

Proposal 21: RAN4 not to study BFD, RLM, or CBD until clear solutions have been defined and agreed upon in RAN1/2.

Proposal 22: RAN4 to study the two questions regarding multiple/single AoA:

- What is the (maximal) difference between really tested Tx/Rx beam and the artificial Tx/Rx beam?

- If the difference can impact AI/ML model and the prediction accuracy?

Proposal 23: RAN4 to check if the 32 beams after simplified CDL channel can be produced by [8] (if it is feasible to be realized in test bed) probes which may achieve the balance between performance and implementation complexity.

Proposal 24: Measuring Tx/Rx beams using a TDM approach may be a valid method; however, the control mechanism for Rx switching must be explicitly addressed.

Proposal 25: The simplified CDL channel only is considered to be applied in inference, no need to be applied in training.

Proposal 26: The metric to estimate the “goodness” of a simplified CDL channel model may be represented by the below steps:

- Step 1: Test (inference) with the reference CDL channel.

- Step 2: Test (inference) with the simplified CDL channel.

- Step 3: The prediction difference between the results of the two tests in the above steps is less than a threshold with [95] percentile.

Proposal 27: Test procedure shall at least comprise the below three steps:

- Acquire the predicted RSRP under the required corresponding SNR condition

- Acquire the ideal measurement of RSRP under high SNR condition

- Compare the two RSRPs acquired in the above steps

Note 1: Test configurations are kept same in the steps except for different SNR conditions.

Note 2: Arranging the details on the sequence and test configuration may need further investigation in performance part.

Proposal 28: For BM-case1, same channel is applied for measurement and inference.

Proposal 29: For BM-case2, plural channel instants with a series of changes of the channel parameters are necessary to reflect temporal correlation between channels. At least, it includes:

- Generating a set of channel instants, from channel #a+1 to channel #a+m, for measurement, and

- Generating a set of channel instants, from channel #b+1 to channel #b+n for inference

Where,

- The channels instants comprise sequent and grade updates of a set of parameters.

- The numbers of m and n depend on the measurement window and prediction window of BM-case2.

- The channel parameters may comprise one or more than one of

  - Departure angles (AOD, ZOD)

  - Arrival angles (AOA, ZOA)

  - UE position and/or rotation

  - Channel coefficient

# References

- R4-2508080 WF on AI/ML, Qualcomm