3GPP TSG-RAN WG4 Meeting #116	R4-2510375

Bengaluru, India, 25th – 29th August 2025

Source: 	Ericsson

Title:  	Discussion on performance requirements for Ambient-IoT

Agenda Item:	7.22.7

Document for:	Discussion

# Introduction

This contribution outlines our point of view on the performance requirements for Ambient-IoT based on the agreements in last meetings.

# Discussion

Recall the agreements in the last meeting RAN4 #115as shown in the below:

Agreement: - RAN4 to include following aspects for RRM requirements for A-IoT random access. The details of UE behaviour will be updated based on RAN2 spec. - CBRA procedure - the correct behaviour when transmitting A-IoT MSG1(Random ID transmission) - the correct behaviour when receiving A-IoT MSG2 - the correct behaviour when not receiving A-IoT MSG2 - CFRA procedure - the correct behaviour when transmitting First D2R message

Agreement: - RAN4 define D2R transmit timing requirements. - Define D2R transmit timing error requirements as follows: - For MSG1 X=1, - The reference point for the device transmission timing is Toffset1 after the end of corresponding R2D transmission as define in TS 38.219. The device transmission timing error shall be less than or equal to Te. The value of Te is equal to 10%*TSFO, where TSFO is the length in microseconds from the last edge of the corresponding R2D transmission to the starting time of MSG1. - For MSG1 X=2, follow the same principle as starting point. - Note 1: The timing error requirements for other D2R could be updated accordingly based on further RAN1 agreements. - Note 2: The end of the R2D transmission and last edge could be updated based on further RAN1 agreements if any.

With respect to core requirements, accordingly it is expected to define 2 test cases:

- Random Access, including contention based random access test and non-contention based random access test.

- D2R transmit timing, to verify accuracy of the TSFO  from the last edge of the corresponding R2D transmission to the starting time of MSG1.

Proposal 1: RAN4 to define 2 test cases:

- Random Access, including

  - Contention based random access test

  - Non-contention based random access test.

- D2R transmit timing

# Conclusion

Proposal 1: RAN4 to define 2 test cases:

- Random Access, including

  - Contention based random access test

  - Non-contention based random access test.

- D2R transmit timing

# References

- R4-2508275, WF on RRM requirements for Ambient_IoT_Solutions, CMCC