3GPP TSG-RAN WG4 Meeting #116	R4-2510381

Bengaluru, India, 25th – 29th August 2025

Source: 	Ericsson

Title:  	Discussion on RedCap RRM requirements for NTN for NR Phase 3

Agenda Item:	7.27.5.1

Document for:	Discussion

# Introduction

In the light of the WI NR Non-Terrestrial Networks (NTN) for NR Phase 3, the outcomes of the last meeting's debate, encompassing consensuses and unresolved issues, were documented in WF [1].

This contribution outlines our point of view on on the underlying impacts to RedCap RRM requirements mandated by the issues listed in the WF.

# Discussion

## Topic #1: General consideration on RRM requirements

#### Issue 1-1-2: MDT requirements

- Background The agreement in the previous meeting: RAN4#114: #### Issue 1-1-3: MDT requirements Agreement: - For MDT requirements in RRC_IDLE and RRC_INACTIVE state, - Deprioritize to define Minimization of Drive Tests (MDT) requirements in RRC_IDLE state and RRC_INACTIVE state for (e)RedCap UE with FR1-NTN.. <Way Forward>: - Proposal 1: Not to define the MDT requirements in RRC_IDLE and RRC_INACTIVE state for RedCap with NTN. - Proposal 2: Define the MDT requirements in RRC_IDLE and RRC_INACTIVE state for Redcap over NTN. The legacy requirement from NTN can be reused. - Proposal 3: RAN4 to discuss whether to send LS to RAN2 / RAN3 informing them about the RAN4 agreement on specifying MDT requirements in RRC_IDLE and RRC_INACTIVE for (e)RedCap UE supporting MDT. - Proposal 4: - RAN4 shall consider the necessity of defining MDT requirements and then decide whether to define relevant MDT requirements. - For measurement requirements, the legacy measurement requirements shall be reused for 1Rx and 2Rx RedCap UEs in NTN. - For accuracy relative time stamp requirements, RAN4 shall reuse the same drift of relative time stamp for 1Rx and 2Rx RedCap UEs in NTN.

Proposal 1, as outlined in the way forward, aligns well with the original design intent of RedCap UEs. When RedCap was introduced in 3GPP Release 17, the objective was explicitly to reduce device complexity, lower cost, and extend battery life by minimizing measurement and signaling requirements—particularly during RRC_IDLE and RRC_INACTIVE states.

In NTN scenarios, the rationale for Proposal 1 is even stronger. Running MDT in RRC_IDLE or RRC_INACTIVE states for NTN, especially considering satellite switching, would require additional procedures and lead to increased power consumption. Moreover, satellites serve both standard NTN NR UEs and NTN RedCap UEs simultaneously over the same propagation links, making it unnecessary for RedCap UEs to support MDT functions for NTN scenarios beyond what is already provided for NTN NR UEs.

Proposal 1: Not to define MDT requirements in RRC_IDLE and RRC_INACTIVE for RedCap with NTN.

#### Issue 1-2-2: SMTC adjustment in idle/inactive

<Way Forward>: - Proposal 1: SMTC adjustment in idle/inactive is kept mandatory for (e)RedCap UEs with NTN. - Proposal 2: RAN4 will not make any modifications on the feature of SMTC adjustment in idle/inactive for RedCap UE with NTN. - Proposal 2a: Maintain the same principle used in NR NTN for RedCap devices: the requirements are transparent to whether the UE adjusts or not the SMTC window in IDLE/INACTIVE mode. - Proposal 3: For SMTC adjustment in idle/inactive, we recommend keeping it optional, yet we would support defining it as mandatory if companies have strong concern on this.

We propose to maintain the same principle for NTN RedCap UE as for NTN NR UE, since there is no strong reason to modify the existing function that ensures correct measurements in NTN scenarios.

Proposal 2: Support Proposal 2 and Proposal 2a: maintain the same principle used in NR NTN for RedCap devices, whereby the requirements are agnostic to whether and how the UE adjusts the SMTC window in Idle/Inactive mode.

## Topic #3: The impact of HD-FDD

#### Issue 1-3-5: The impact on NR Handover due to HD-FDD

<Way Forward>: - Proposal 1: For the impact on NR handover for HD-FDD, the principle in option 1 is accepted. - Proposal 1a: Regarding Issue 3-2-1 (The impact on NR Handover due to HD-FDD), adopt Option 1 with 'availability' defined as 'no collision with UL transmission at the UE'. - Proposal 2: Adopt option 2 (no spec impact) for HO, CHO and satellite switch for HD-FDD UE.

Duplicate Option 1 in the below:

- Option 1: agree on the principle below and further discuss the wording in the CR:

    - For RACH-based HO, RACH-lhaess HO, Time/location-based CHO without L3 measurement and Satellite switching with re-synchronization, following applicability rules to adapt HD-FDD case should be added:

      - For RedCap NTN UE with HD-FDD, the requirements are met provided that

- SSB is available at the UE once every SMTC period during Tsearch

- One SSB is available during T∆

- One SSB is available during TIU

    - For Time/location-based CHO with L3 measurement, following applicability rules to adapt HD-FDD case should be added:

      - For RedCap NTN UE with HD-FDD, the requirements are met provided that

- One SSB is available during T∆

- One SSB is available during TIU

In legacy specifications, the handover requirement with interruption time is defined as: Tinterrupt = Tsearch + TIU + Tprocessing  + T∆ + Tmargin ms.

For RedCap UEs with HD-FDD, the following restrictions apply:

For RedCap UE with HD-FDD, the handover requirements are met provided that - SSB is available at the UE once every SMTC period during Tsearch - One SSB is available during T∆ - One SSB is available during TIU.

For Conditional Handover (CHO) with L3 measurements (Tmeasure), these HD‑FDD restrictions are not required, since measurement handling for HD‑FDD can be managed through network configuration.

In cases without measurements (Tmeasure), e.g., handover or CHO without L3 measurement, the legacy HD‑FDD restrictions should continue to apply.

Proposal 3: Agree to Proposal 1, the content is presented in the below:

    - For RACH-based HO, RACH-lhaess HO, Time/location-based CHO without L3 measurement and Satellite switching with re-synchronization, following applicability rules to adapt HD-FDD case should be added:

      - For RedCap NTN UE with HD-FDD, the requirements are met provided that

- SSB is available at the UE once every SMTC period during Tsearch

- One SSB is available during T∆

- One SSB is available during TIU

    - For Time/location-based CHO with L3 measurement, following applicability rules to adapt HD-FDD case should be added:

      - For RedCap NTN UE with HD-FDD, the requirements are met provided that

- One SSB is available during T∆

#### Issue 1-4-1: General principle for designing the UE capabilities for (e)RedCap UE with NTN

<Way Forward>: - Option 1: Declare the capabilities for (e)RedCap UE with FR1-NTN bands through implicitly by declaring support of RedCap (supportOfRedCap-r17 or supportOfERedCap-r18) and support of NTN (nonTerrestrialNetwork-r17). (CATT, CMCC, ZTE, Xiaomi, E///, Samsung - compromise, Apple, QC) - FFS: The Rel-19 requirement can be release independent from Rel-17. - Option 2: RAN4 to introduce new capability to declare support over RedCap over NTN. (vivo, Samsung) - Option 3: to wait for RAN1 to confirm the working assumption (UE that supports RedCap and NTN shall support RedCap over NTN). (Nokia, QC, MTK)

We’d like to preliminarily analyse the revision compliance.

For a Rel‑17/18 UE, there are no requirements defined for RedCap in NTN, even if the UE supports both NTN and RedCap in its capability. Therefore, in an NTN scenario, such a UE operates as an NR NTN UE and cannot operate as a RedCap device to meet RedCap in NTN requirements.

While, for a Rel-19 UE support both NTN and RedCap in capability,

- In TN, the UE shall follow RedCap in TN requirements.

- In NTN, the UE shall follow RedCap in NTN requirements.

Observation 1: For a Rel-17/18 UE supporting both NTN and RedCap in capability, in NTN scenarios the UE functions as an NR NTN UE and cannot operate as a RedCap device to meet RedCap in NTN requirements.

Observation 2: For a Rel-19 UE support both NTN and RedCap in capability:

- In TN, the UE shall follow RedCap in TN requirements.

- In NTN, the UE shall follow RedCap in NTN requirements.

Proposal 4: Agree to Option 1, declare the capabilities for (e)RedCap UE with FR1-NTN bands through implicitly by declaring support of RedCap (supportOfRedCap-r17 or supportOfERedCap-r18) and support of NTN (nonTerrestrialNetwork-r17).

# Conclusion

Observation 1: For a Rel-17/18 UE supporting both NTN and RedCap in capability, in NTN scenarios the UE functions as an NR NTN UE and cannot operate as a RedCap device to meet RedCap in NTN requirements.

Observation 2: For a Rel-19 UE support both NTN and RedCap in capability:

- In TN, the UE shall follow RedCap in TN requirements.

- In NTN, the UE shall follow RedCap in NTN requirements.

Proposal 1: Not to define MDT requirements in RRC_IDLE and RRC_INACTIVE for RedCap with NTN.

Proposal 2: Support Proposal 2 and Proposal 2a: maintain the same principle used in NR NTN for RedCap devices, whereby the requirements are agnostic to whether and how the UE adjusts the SMTC window in Idle/Inactive mode.

Proposal 3: Agree to Proposal 1, the content is presented in the below:

    - For RACH-based HO, RACH-lhaess HO, Time/location-based CHO without L3 measurement and Satellite switching with re-synchronization, following applicability rules to adapt HD-FDD case should be added:

      - For RedCap NTN UE with HD-FDD, the requirements are met provided that

- SSB is available at the UE once every SMTC period during Tsearch

- One SSB is available during T∆

- One SSB is available during TIU

    - For Time/location-based CHO with L3 measurement, following applicability rules to adapt HD-FDD case should be added:

      - For RedCap NTN UE with HD-FDD, the requirements are met provided that

- One SSB is available during T∆

Proposal 4: Agree to Option 1, declare the capabilities for (e)RedCap UE with FR1-NTN bands through implicitly by declaring support of RedCap (supportOfRedCap-r17 or supportOfERedCap-r18) and support of NTN (nonTerrestrialNetwork-r17).

# References

- R4-2504912, The WF on RRM requirements for Rel-19 NR NTN phase3, CATT