3GPP TSG-RAN WG4 Meeting #116	R4-2510382

Bengaluru, India, 25th – 29th August 2025

Source: 	Ericsson

Title:  	Discussion on other RRM requirements for NTN for NR Phase 3

Agenda Item:	7.27.5.2

Document for:	Discussion

# Introduction

In the light of the WI NR Non-Terrestrial Networks (NTN) for NR Phase 3, the outcomes of the last meeting's debate, encompassing consensuses and unresolved issues, were documented in WF [1].

This contribution outlines our point of view on on the underlying impacts to RRM requirements mandated by the issues listed in the WF.

# Discussion

## Topic #1: SSB periodicity larger than 20 ms

## Issue 0: LS from RAN2 on multiple SMTCs with different periodicities

Agreement: (Ad-hoc) From RAN4 perspective, it is feasible to support configuring more than 4 SMTCs per frequency (i.e. 6) for idle/inactive UEs. - In the reply LS (to be prepared by Xiaomi), the following information highlighted in yellow for both intra- and inter-frequency will be added for reference: - FFS on the exact RRM requirement change. - FFS whether to limit the number of SMTCs to no more than 4 is desirable in an earth-moving cell deployment Agreement: online From RAN4 perspective, it is feasible to support SMTCs with up to 2 periodicities and different offsets but requires some clarifications/modifications to the existing RRM requirements. - RAN4 assumes that the configured SMTC periodicity is equal to or greater than the largest SSB periodicity among the cells associated with the relevant SMTC on the frequency layer. Agreement: online The progress of RRM requirement definition for multiple SMTCs with up to two periodicities does not block the completion of RAN4 RRM core part of this WI.

Regarding 4SMTCs for idle/inactive UE:

Recap the existing related requirements in TS 38.133 in the below:

The requirements in this clause apply provided that the number of SMTCs for intra-frequency carrier does not exceed the parallelSMTC-r17, otherwise UE may select one or subset of all the configured SMTCs sequentially for performing the measurements until all of the SMTCs can be measured. The selection of SMTCs to be used is up to UE implementation, and in this case, measurement period longer than the corresponding measurement period specified in table 4.2C.2.3-1 and table 4.2C.2.3-2 is expected.

The configured number of SMTCs does not impact the RRM requirements.

Observation 1: More than 4 configured SMTCs doesn’t impact the actual RRM requirements.

Another thing is ‘SSB periodicity among the cells associated with the relevant SMTC on the frequency layer.’ We understand the wording ‘associated’ doesn’t mean that UE in idle state is able to obtain association information between cells and SMTC before detecting SSB of the cell from SI.

Observation 2: The wording “associated” in the above LS does not imply that a UE in idle state must obtain association information between cells and SMTC before detecting the SSB of a cell. In this context, “associated” refers solely to the SSB(s) covered by the configured SMTC.

Regarding the concern “FFS whether to limit the number of SMTCs to no more than four in an earth‑moving cell deployment,” we note that while some UEs may have limitations in managing more than 4 SMTCs, existing RRM requirements do not request UEs to measure multiple SMTCs simultaneously out of its capability. Therefore, supporting more than four SMTCs in such deployments does not introduce additional computational burden at the UE side.

Proposal 1: Support more than 4 SMTCs in the earth-moving cell deployment.

Regarding 2SMTC periodicities for idle/inactive/connected UE:

Recap the relevant content in RAN2 LS as follows:

- RAN2 considers to support configuring two different SMTC periodicities (with different offsets) in one frequency layer for idle, inactive and connected mode neighbor cell measurements.

From the RAN2 perspective, two SMTC periodicities may be applied to different cells within the same frequency layer, primarily to accommodate distinct measurement delay requirements for each cell.

We understand the utilization of 2 SMTCs corresponds to the case 1 in Figure 1, where for sake of simplification, SSB1 is by cell 1 and SSB2 is by cell2.

In last meeting, RAN4 achieved an agreement:

‘RAN4 assumes that the configured SMTC periodicity is equal to or greater than the largest SSB periodicity among the cells associated with the relevant SMTC on the frequency layer.’

This agreement effectively rules out Case 2 in Figure 1, while allowing Cases 3 and 4. The distinction between these two cases is:

- In case 3, one SMTC covers two cells, the measurement delays for the two cells are same.

- In case 4, two SMTCs covers two cells, even SSB1 is measured by 2 SMTCs, the measurement delays for the two cells are differentiated.

Based on the above observations, Case 1 and Case 4 appear to align with the motivation for introducing two SMTCs with different periodicities.

Figure 1 SMTC(s) in a same frequency layer

Regarding measurements of intra/inter-frequency NR cells for UE in idle mode, measurement delay is is determined by DRX. Configuring SMTCs with two periodicities, even if signaled via SIB2 or SIB4, does not impact the existing idle-mode measurement procedures.

Proposal 2: Confirm that configuring two SMTC periodicities does not impact the existing idle mode measurement requirements for intra/inter-frequency NR cells.

Regarding measurements of intra/inter-frequency NR cells for UE in connected mode, for each frequency layer, during each layer 1 measurement period, the UE shall be capable of performing measurements for multiple cells.

Under current specification, the UE is able to measure multiple SMTCs in a same measurement delay if SMTC periodicity is same.  And it is noted in delay requirements, e.g., Table 9.2C.5.1-1, Table 9.2C.5.1-2 and Table 9.2C.5.1-3, ‘if different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified.’  It implies that the UE may receive different periodicity SMTCs (SMTC2) in MO for different cells,  a simplified example is presented in the below:

- If a cell in cell not in PCI-list is detected, its requirement follows smtc1 (longer periodicity).

- If a cell in cell in PCI-list is detected, its requirement follows smtc2 (shorter periodicity).

Going back to  Figure 1, only case1 and case 4 have two SMTCs with different periodicities:

- For  case 1 in Figure 1, It’s straightforward  that only SSB1 is detected in SMTC1 which is used to evaluate Cell1, and only SSB2 is detected in SMTC2 which is used to evaluate Cell2.

- For case 4 in Figure 1, Even SMTC2 covers two SSBs/cells, the UE can distinguish cells/satellites linked to each SMTC based on information from SIB and SMTC configuration. Thus, SSB1/Cell1 can be measured with SMTC1, and SSB2/Cell 2 can be measured with SMTC2, without ambiguity.

Thus, the UE is capable of understanding the relationship between the SSB/cell and the corresponding SMTC, in line with the agreement that ‘the configured SMTC periodicity is equal to or greater than the largest SSB periodicity among the cells associated with the relevant SMTC on the frequency layer’.

So, we can reuse the note ‘if different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified.’ to reflect the interpretation that the UE understands the relationship between the SSB/cell and the corresponding SMTC.

Proposal 3: Confirm that the existing note: ‘If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified’ in the existing measurements of intra/inter-frequency NR cells for UE in connected mode remains valid when SMTCs with different periodicities are configured.

Regarding handover, it is noted that the SMTC periodicity is included as part of the interruption time. The interpretation of this has been duplicated as follows:

Trs is the SMTC periodicity of the target NR SAN cell if the UE has been provided with an SMTC configuration for the target cell in the handover command, otherwise Trs is the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing. If the UE is not provided SMTC configuration or measurement object on this frequency, the requirement in this clause is applied with Trs=5 ms assuming the SSB transmission periodicity is 5 ms. There is no requirement if the SSB transmission periodicity is not 5 ms. If the UE has been provided with higher layer in TS 38.331 [2] signalling of smtc2 prior to the handover command, Trs follows smtc1 or smtc2 according to the physical cell ID of the target cell.

There are some observations of impacts due to SMTCs with different periodicities:

- In handover, the target NR SAN cell is expected to provide a single SMTC in the handover command.

- If no explicit SMTC is included in the handover command, Trs is determined based on the SMTC configured in the measObjectNR with the same SSB frequency and subcarrier spacing.

  - It is possible that two SMTCs with different periodicities may be configured for the same frequency and subcarrier spacing (e.g., one linked to the serving cell, one linked to a non‑serving cell). In such cases, ambiguity could arise regarding which SMTC to apply for Trs .

  - In this case, the UE shall apply the longer SMTC periodicity.

Proposal 4: In handover, Trs is the SMTC, or the longer SMTC if SMTCs with different periodicities are configured, configured in the measObjectNR having the same SSB frequency and subcarrier spacing.

At the end, for requirements with SMTC periodicity, it is recommended to add a note to reflect the agreed content in the reply LS: ‘no configured SMTC periodicity is equal to or greater than the largest SSB periodicity among the cells associated with the relevant SMTC on the frequency layer’.

Proposal 5: For requirements with SMTC periodicity, add a note: ‘no configured SMTC periodicity is equal to or greater than the largest SSB periodicity among the cells associated with the relevant SMTC on the frequency layer’.

## Another relevant concern is the enhancements of 4SMTCs for idle/inactive UE and 2SMTC periodicities for idle/inactive /connected UE are applicable for FR2-NTN, since the scenario isn’t precluded.

## Proposal 6: Confirm that enhancements of 4SMTCs for idle/inactive UEs and 2SMTC periodicities for idle/inactive/connected UEs are applicable to FR2-NTN.

## Issue 1-2-B: RRC Re-establishment (SSB Ês/Iot < -8dB and SSB periodicity >20 ms)

Agreement: online - In RRC re-establishment requirements, for the case SSB Ês/Iot < -8dB and unknown NR cell, - Remove the [] around the value 22 in the following two bullets. - Tidentify_intra_NR: MAX (800 ms, [22] x TSMTC) for intra-frequency carrier - Tidentify_inter_NR,i: MAX (800 ms, [22] x TSMTC,i) for inter-frequency carrier - Remove Note 1 in Table 6.2C.1.2.1-1 and Table 6.2C.1.2.1-2 for the applicable scenarios, and the value of TSMTC and TSMTC,i is FFS

During initial access, a UE may encounter SSB periodicities of both 20ms and 160ms, and the UE may detect either one. This occurs because neighboring cells can have different periodicities, especially when they belong to different frequency layers.

Since the RRC re-establishment procedure occurs after the UE has already completed initial cell selection—successfully detecting and decoding the SS/PBCH block—the expectation is that the SSB periodicity should match the one used during initial cell selection.

However, neighbor cells, which mostly is inter-frequency layer of the serving cell, may use an SSB periodicity different from that of the serving cell. In such cases, the UE can leverage 160ms measurement opportunities to detect cells transmitting with a 20ms initial access periodicity.

Observation 3: Depending on network implementation, 20ms or 160ms both may coexist in neighbor cells in practice.

Proposal 7: RAN4 shall define RRC re-establishment requirements to ensure that a Rel-19 capable UE can complete RRC re-establishment with respect to a 20 ms SSB periodicity when neighboring cells provide 20 ms periodicity SSBs.

Proposal 8: For RRC re-establishment to NR intra-frequency cell, the SSB periodicity, e.g., TSMTC, without SMTC configuration, is assumed same as the SSB periodicity used during initial cell selection, provided that same SSB periodicity is applied for neighbor cells.

Proposal 9: For RRC re-establishment to NR inter-frequency cell, if different SSB periodicity, is applied for neighbor cells, i.e., some cells apply DL-CE and others don’t apply DL-CE, then, TSMTC,i without SMTC configuration, assumed to be 160ms.

# Conclusion

Observation 1: More than 4 configured SMTCs doesn’t impact the actual RRM requirements.

Observation 2: The wording “associated” in the above LS does not imply that a UE in idle state must obtain association information between cells and SMTC before detecting the SSB of a cell. In this context, “associated” refers solely to the SSB(s) covered by the configured SMTC.

Observation 3: Depending on network implementation, 20ms or 160ms both may coexist in neighbor cells in practice.

Proposal 1: Support more than 4 SMTCs in the earth-moving cell deployment.

Proposal 2: Confirm that configuring two SMTC periodicities does not impact the existing idle mode measurement requirements for intra/inter-frequency NR cells.

Proposal 3: Confirm that the existing note: ‘If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified’ in the existing measurements of intra/inter-frequency NR cells for UE in connected mode remains valid when two SMTCs with different periodicities are configured.

Proposal 4: In handover, Trs is the SMTC, or the longer SMTC if SMTCs with different periodicities are configured, configured in the measObjectNR having the same SSB frequency and subcarrier spacing.

Proposal 5: For requirements with SMTC periodicity, add a note: ‘no configured SMTC periodicity is equal to or greater than the largest SSB periodicity among the cells associated with the relevant SMTC on the frequency layer’.

## Proposal 6: Confirm that enhancements of 4SMTCs for idle/inactive UEs and 2SMTC periodicities for idle/inactive/connected UEs are applicable to FR2-NTN.

Proposal 7: RAN4 shall define RRC re-establishment requirements to ensure that a Rel-19 capable UE can complete RRC re-establishment with respect to a 20 ms SSB periodicity when neighboring cells provide 20 ms periodicity SSBs.

Proposal 8: For RRC re-establishment to NR intra-frequency cell, the SSB periodicity, e.g., TSMTC, without SMTC configuration, is assumed same as the SSB periodicity used during initial cell selection, provided that same SSB periodicity is applied for neighbor cells.

Proposal 9: For RRC re-establishment to NR inter-frequency cell, if different SSB periodicity, is applied for neighbor cells, i.e., some cells apply DL-CE and others don’t apply DL-CE, then, TSMTC,i without SMTC configuration, assumed to be 160ms.

# References

- R4-2508288, The WF on RRM requirements for Rel-19 NR NTN phase3, QC