3GPP TSG-RAN WG4 Meeting #116	R4-2510383

Bengaluru, India, 25th – 29th August 2025

Source: 	Ericsson

Title:  	Discussion on RRM Performance requirements for NTN for NR Phase 3

Agenda Item:	7.27.6

Document for:	Discussion

# Introduction

This contribution outlines our point of view on on the RRM Performance requirements for NTN for NR Phase 3.

# Discussion

### Sub-topic 2-1: (e)RedCap UE with FR1-NTN

#### Issue 2-1-1: Measurement accuracy requirement

- Background The agreement in the previous meeting: RAN4#114-bis: #### Issue 2-2-1: L1/L3 measurement accuracy for 1Rx/2Rx (e)RedCap UEs with FR1-NTN Agreement: - For 2Rx (e)RedCap UEs with FR1-NTN bands, RAN4 to reuse the measurement accuracy requirements for normal UEs defined in NTN. - For 1Rx (e)RedCap UEs with FR1-NTN bands, RAN4 to relax the L1/L3 measurement accuracy for SS-RSRP, SS-RSRQ, SS-SINR and L1-RSRP compared to those defined for 2Rx (e)RedCap UEs. - The same relaxation defined in TN for 1Rx (e)RedCap UEs can be reused. - SS-RSRP, SS-RSRQ, SS-SINR: relax by 1dB. - L1-RSRP: relax by 3dB. RAN4#114: #### Issue 2-3-2: Network verified UE location for 1Rx/2Rx (e)RedCap UEs with FR1-NTN Agreement: - For 2Rx (e)RedCap UEs with FR1-NTN bands, the measurement period and measurement accuracy requirements for Network verified UE location defined in NTN can be reused. - For 1Rx (e)RedCap UEs with FR1-NTN bands, the measurement accuracy requirements defined for 1Rx RedCap positioning can be referred, and the same relaxation (as the measurement accuracy requirements defined for 1Rx TN-RedCap ) for accuracy should be reused.. <Way Forward>: - Proposal 1 (CMCC): - For 2Rx RedCap UE, reuse the measurement accuracy requirement of NTN. - For 1Rx RedCap UE, define new measurement accuracy requirement on NTN operating bands with reusing the RSRP accuracy and side conditions from legacy 1Rx RedCap UE. - Proposal 2 (vivo): For the performance requirement in NTN for NR Phase 3, measurement accuracy requirements need to be defined for (e)Redcap UEs with FR1-NTN. - Proposal 3 (HW): RAN4 to define accuracy requirements for following measurements for RedCap in NTN. - Intra- and inter-frequency RSRP - Intra- and inter-frequency RSRQ - Intra- and inter-frequency SINR - L1-RSRP - Rx-Tx for NW verified location

Combining the proposals, we understand that the common agreement on Proposal 1 and Proposal 2 is:

- To reuse the NTN NR measurement accuracy requirements for 2Rx RedCap in NTN.

- To reuse the legacy 1Rx TN RedCap measurement accuracy requirements for 2Rx RedCap in NTN.

Regarding Proposal 3, since UE Rx-Tx time difference measurement has not been verified in NR NTN, the same approach can be applied to RedCap in NTN.

Proposal 1: RAN4 to define accuracy requirements for following measurements for RedCap in NTN.

- For 2Rx RedCap UE, reuse the measurement accuracy requirements of NTN.

- For 1Rx RedCap UE, define measurement accuracy requirements on NTN operating bands with reusing the RSRP accuracy and side conditions from legacy 1Rx RedCap UE.

- Measurement accuracy requirements including:

  - Intra- and inter-frequency RSRP

  - Intra- and inter-frequency RSRQ

  - Intra- and inter-frequency SINR

  - L1-RSRP

  - Rx-Tx time difference

#### Issue 2-1-2: Test configuration and parameters for (e)RedCap UE with FR1-NTN

<Way Forward>: - Proposal 1 (CATT): - Both FDD and HD-FDD duplex mode should be considered in test configuration for (e)RedCap UE with FR1-NTN. - 15 kHz SCS and 10MHz bandwidth can be configured in test cases. - Both GSO and NGSO are configured in test cases for RedCap UE with FR1-NTN. - Proposal 1a (ZTE): RAN4 shall consider the following general test configurations as follows: Configuration; Description. 1; GSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz. 2; NGSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz. 3; GSO, NR HD-FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz. 4; NGSO, NR HD-FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz. NOTE: The UE is only required to be tested in one of the supported test configurations. - Proposal 2 (HW): Adapt the test case for reduced BW, e.g. test configuration, some RMC and Io values - Proposal 3 (CMCC): Take the NTN test setups as the reference, and use the ‘refer to’ method when drafting the spec to avoid duplicated contents. - Proposal 4 (ZTE): RAN4 only clarify the procedure and parameters which can be reused from NTN, and then define new parameters applicable to RedCap UEs.

We agree on the general consideration in Proposal 1, 2, 3, 4.

Proposal 2: Approve Proposal 1a as detailed in the table below, and incorporate the general considerations from Proposals 2, 3, and 4 into the drafting of the corresponding CRs.

| Configuration | Description |
| --- | --- |
| 1 | GSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 2 | NGSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 3 | GSO, NR HD-FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 4 | NGSO, NR HD-FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| NOTE: The UE is only required to be tested in one of the supported test configurations |  |

#### Issue 2-1-3: How and where to capture the test cases for 1Rx and 2Rx (e)RedCap UE with NTN,

<Way Forward>: - Proposal 1(ZTE): The existing NTN test cases can be used as a baseline for RedCap UEs in NTN scenario. - Proposal 1a (HW): Re-use existing test cases for NTN in A.14 for RedCap in NTN - Adapt the test case for reduced BW, e.g. test configuration, some RMC and Io values - Proposal 2 (ZTE): For RedCap UEs in NTN scenario, some new test cases will be added in the existing NTN test cases, which will also be defined in Clause A.14.

Since RedCap in NTN largely reuses the requirements from NR NTN, it is straightforward to add clauses for RedCap in NTN in Clause A.14.

Proposal 3: Agree to Proposals 1 and 2, and incorporate clauses for RedCap in NTN within Clause A.14.

- Proposal 1: The existing NTN test cases can be used as a baseline for RedCap UEs in NTN scenario.

  - Proposal 1a: Re-use existing test cases for NTN in A.14 for RedCap in NTN

    - Adapt the test case for reduced BW, e.g. test configuration, some RMC and Io values

- Proposal 2: For RedCap UEs in NTN scenario, some new test cases will be added in the existing NTN test cases, which will also be defined in Clause A.14.

#### Issue 2-1-4-1: Whether to define separate test cases for 1Rx and 2Rx (e)RedCap UE with NTN

<Way Forward>: - Proposal 1 (CATT, CMCC): RAN4 to define the separate test cases for 1Rx and 2Rx (e)RedCap UE with NTN. - Proposal 2 (HW): Further discuss whether to define separate test cases for 1Rx and 2Rx RedCap UEs - Proposal 3 (ZTE): Discuss case by case.

Proposal 4: Following the same approach as for 1Rx and 2Rx RedCap in TN, apply separate test cases for 1Rx and 2Rx RedCap in NTN.

#### Issue 2-2-1: Test scope for downlink coverage enhancement

<Way Forward>: - Proposal 1 (ZTE): For downlink coverage enhancement, some new test cases will be added in the existing NTN test cases, which will also be defined in Clause A.14. - Proposal 2 (Ericsson, CATT, HW, ZTE): RAN4 to consider adding a new test case for RRC Re-establishment for the UE capable of coverage enhancement. - Proposal 2a (HW): Define test case for RRC Re-establishment with 160ms SSB periodicity and without serving cell based on existing test case in A.6.3.2.1.3. - Proposal 3 (CATT, HW): Other test cases need to wait for the progress in core part. - Proposal 3a (HW): Wait for conclusion on potential SMTC enhancement before discussing whether to define new test cases for cell reselection and L3 measurement. - Proposal 4 (ZTE): For RRC connection release with redirection to NR, handover, satellite switch with re-sync and cell re-selection, no new test cases shall be defined.

As noted by several companies, to support the 160 ms SSB periodicity, RAN4 has defined updated RRC re-establishment requirements:

- When SMTC is not provided for an inter-frequency carrier, the UE assumes an SMTC period of 160 ms instead of the legacy 20 ms.

- The RRC re-establishment requirements include provisions based on the 160 ms periodicity for cases where SSB Ês/Iot < -8 dB and the NR cell is unknown.

Since the 160 ms SSB periodicity effectively relaxes the overall delay constraints, we do not consider it necessary to design for all three cases.

Proposal 5: Downselect the intra-frequency RRC re-establishment test case to the scenario with SSB Ês/Iot < -8 dB and unknown NR cell.

# Conclusion

Proposal 1: RAN4 to define accuracy requirements for following measurements for RedCap in NTN.

- For 2Rx RedCap UE, reuse the measurement accuracy requirements of NTN.

- For 1Rx RedCap UE, define measurement accuracy requirements on NTN operating bands with reusing the RSRP accuracy and side conditions from legacy 1Rx RedCap UE.

- Measurement accuracy requirements including:

  - Intra- and inter-frequency RSRP

  - Intra- and inter-frequency RSRQ

  - Intra- and inter-frequency SINR

  - L1-RSRP

  - Rx-Tx time difference

Proposal 2: Approve Proposal 1a as detailed in the table below, and incorporate the general considerations from Proposals 2, 3, and 4 into the drafting of the corresponding CRs.

| Configuration | Description |
| --- | --- |
| 1 | GSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 2 | NGSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 3 | GSO, NR HD-FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 4 | NGSO, NR HD-FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| NOTE: The UE is only required to be tested in one of the supported test configurations |  |

Proposal 3: Agree to Proposals 1 and 2, and incorporate clauses for RedCap in NTN within Clause A.14.

- Proposal 1: The existing NTN test cases can be used as a baseline for RedCap UEs in NTN scenario.

  - Proposal 1a: Re-use existing test cases for NTN in A.14 for RedCap in NTN

    - Adapt the test case for reduced BW, e.g. test configuration, some RMC and Io values

- Proposal 2: For RedCap UEs in NTN scenario, some new test cases will be added in the existing NTN test cases, which will also be defined in Clause A.14.

Proposal 4: Following the same approach as for 1Rx and 2Rx RedCap in TN, apply separate test cases for 1Rx and 2Rx RedCap in NTN.

Proposal 5: Downselect the intra-frequency RRC re-establishment test case to the scenario with SSB Ês/Iot < -8 dB and unknown NR cell.

# References

- R4-2508287, WF on RRM requirements for NR_NTN_Ph3_Part1, CATT